3GPP TSG RAN WG1 #103-e		R1-2007701

e-Meeting, October 26th – November 13th, 2020

Source:         GuangDong Communications and Networks Institute

Title:	Extension to Rel-16 DCI-based power saving adaptation during DRX Active Time

Agenda Item:    8.7.2

Document for:   Discussion/Decision

- Introduction

In Rel-17, there is a UE power saving WID agreed in RAN 86 [1], the scope of which is given as follows:

- …

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

- NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

In this contribution, several aspects of extension to Rel-16 DCI-based power saving adaptation during DRX Active Time are discussed.

- Potential Power Saving technique

  - PDCCH skipping

PDCCH monitoring is periodicity action, so when it is repeated frequently, it will constantly monitor even if there is no data transmission which cost power consumption as wasted. Base station can send PDCCH skipping indication to UE to skip the next or several monitoring period or directly go to sleeping state until the next DRX period. However, existing mechanism WUS indication and dormancy adaptation cannot skip PDCCH monitoring in certain short durations. Therefore, we propose a DCI based PDCCH skipping technique.

In Rel-16, DCI format 2_6 is working as WUS indication with the 1 bit as wake-up indication and L-1 bit as SCell go-to-sleep indication as shown in Figure 1. Total length as L and the second part can be 0 bit with different length to different UEs.

Figure 1

We propose that add 1 bit before WUS indication as skipping indication as shown in Figure 2. 0 as not to skip PDCCH monitoring and 1 as to skip PDCCH monitoring until next DRX period. This part can be further study to enable more skipping selections for example skipping two PDCCH monitoring occasions instead of to the next DRX period.

Figure 2

Proposal 1: DCI-based PDCCH monitoring should be considered. DCI format 2_6 can be extended to support PDCCH skipping

  - Cross-slot scheduling and multi PDSCH scheduling

In Rel-16, power saving based on cross-slot scheduling has been supported. Unnecessary PDSCH sampling and buffering can be avoided and also PDCCH processing speed is decreased with certain parts go to sleeping-mode. Only when data transmission happens will UE wake up to process PDSCH sampling and buffering, thus reduces the power consumption unnecessary. Meanwhile, if multi PDSCH scheduling is supported, less PDCCH occasion will be needed also reducing the power consumption.

UE will be informed if it is cross-slot scheduling by configuring Kmin in advance. The configuration of minimum applicable value for K0/K2, i.e. Kmin is related to PDCCH monitoring periodicity with restriction as shown in Figure 3. To avoid the accumulated PDCCH monitoring occasions, periodicity should larger than Kmin. In Figure 3, PDCCH 0 schedules PDSCH0, PDSCH1 and PDSCH 2 in the first periodicity and same as PDCCH1 which scheduled PDSCH 3 and PDSCH4 in the second periodicity.

Figure 3

The following proposal should be considered:

Proposal 2: UE adapts PDCCH periodicity to be no smaller than Kmin.

Proposal 3: Study necessary enhancement to support multi-PDSCH/multi-PUSCH scheduling.

  - Search Space Set Extension

NR introduced a dynamic search space set switching mechanism, and provide explicit and implicit switching conditions. The UE can switch between sparse and dense PDCCH monitoring which is used for power saving. Currently the maximum configuration is two search space sets with explicit/ implicit switching condition provided to switch between sparse and dense PDCCH monitoring occasions. We propose to set an extreme condition: PDCCH sleeping as another basic search space set to meet the requirement of PDCCH sleeping/ activating switching. The following figure shows the switching structure: UE can switch from SS set 1 which represent for example sparse monitoring to SS set 2 dense monitoring, and when there is no PDCCH monitoring requirement during active time, it returns to SS set 0 to sleeping condition until next data burst arrive. The switching condition can be further discussed.

Figure 4

Proposal 4: Extend search space sets configuration to reduce PDCCH monitoring during active time.

- Conclusion

Proposal 1: DCI-based PDCCH monitoring should be considered.

Proposal 2: UE adapts PDCCH periodicity to be no smaller than Kmin.

Proposal 3: Study necessary enhancement to support multi-PDSCH/multi-PUSCH scheduling.

Proposal 4: Extend search space sets configuration to reduce PDCCH monitoring during active time.

- Reference

[1] RP-193239, “New SID on support of UE power saving”, MTK, 3GPP TSG RAN #86 meeting.