3GPP TSG-RAN WG1 Meeting #103-e	Tdoc R1-2007704

Online, October 26th – November 13th 2020

Agenda Item:	7.2.5

Source:	Ericsson

Title:	Maintenance of UCI for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss the remaining issues on codebook generation.

# 2	Discussion

# 2.1	HARQ-ACK PUCCH determination with different SCSs in UL and DL

- In Section 9.1.2, TS 38.213, it is mentioned that the UE reports HARQ-ACK information for a PDSCH reception in case of repetition as the following:

If the UE is provided pdsch-AggregationFactor in SPS-Config or PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 includes repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor in SPS-Config or PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from slot n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat is provided by pdsch-AggregationFactor [6, TS 38.214], or - from slot n-repetitionNumber-r16+1 to slot n, if the Time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber-r16, or - in slot n, otherwise

- In this slot n corresponds to last PDSCH reception (with or without repetition) which mapped to UL slot size. However, the quantity (aggregation factor) RepNum16 or NPDSCHrepeat is measured in DL slot size and therefore the the combination slot n-NPDSCHrepeat+1 or slot n-repetitionNumber-r16+1 is not correctly presented in the specification. To correct this, the quantity (aggregation factor) RepNum16 or NPDSCHrepeat should be mapped to UL slot size.

- Proposal 1		To determine HARQ-ACK information for the PDSCH reception in the PUCCH in slot n+k, the PDSCH reception should be considered from slot  n-NPDSCHrepeat2μ,PUCCH2μ,PDSCH+1 or n-repetitionNumber-r162μ,PUCCH2μ,PDSCH+1.

- Below is Text proposal reflecting above proposal in TS38.213:

### 9.1.2 Type-1 HARQ-ACK codebook determination *** Unchanged text is omitted *** If the UE is provided pdsch-AggregationFactor in SPS-Config or PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 includes repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor in SPS-Config or PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from slot n-NPDSCHrepeat2μ,PUCCH2μ,PDSCH+1 n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat is provided by pdsch-AggregationFactor [6, TS 38.214], or - from slot n-repetitionNumber-r162μ,PUCCH2μ,PDSCH+1 n-repetitionNumber-r16+1 to slot n, if the Time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber-r16, or - in slot n, otherwise only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK. *** Unchanged text is omitted ***

# 3	Conclusion

Based on the discussion in the previous sections we propose the following, together with the Text Proposal to TS 38.213 section 9.1.2:

Proposal 1	To determine HARQ-ACK information for the PDSCH reception in the PUCCH in slot n+k, the PDSCH reception should be considered from slot  n-NPDSCHrepeat2μ,PUCCH2μ,PDSCH+1 or n-repetitionNumber-r162μ,PUCCH2μ,PDSCH+1.

### 9.1.2	Type-1 HARQ-ACK codebook determination

*** Unchanged text is omitted ***

If the UE is provided pdsch-AggregationFactor in SPS-Config or PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 includes repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor in SPS-Config or PDSCH-Config; otherwise  NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception

-	from slot n-NPDSCHrepeat2μ,PUCCH2μ,PDSCH+1 n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat is provided by pdsch-AggregationFactor [6, TS 38.214], or

-	from slot n-repetitionNumber-r162μ,PUCCH2μ,PDSCH+1 n-repetitionNumber-r16+1  to slot n, if the Time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber-r16, or

-	in slot n, otherwise

only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

*** Unchanged text is omitted ***