3GPP TSG-RAN WG1 Meeting #103-e	Tdoc R1-2007705

Online, October 26th – November 13th 2020

Agenda Item:	7.2.5

Source:	Ericsson

Title:	Maintenance of Scheduling/HARQ for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we would like to focus on two maintenance issues:

1. Processing order of intra-UE prioritization and multiplexing (last meeting the working assumption was agreed):

Working assumption

- Multiplexing/overriding/etc. is performed similar to Rel.15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc., are performed before cancellation.

- A UE cancels the transmission of a LP channel including any intermediate scheduled LP transmission that does not overlap with any LP channel, if any DCI schedules an overlapping HP transmission with the LP channel, before performing multiplexing/overriding HP channels if any.

- Multiplexing/overriding of HP channels is performed as if LP channels do not exist.

- A final HP channel is prioritized if it overlaps with a final LP channel, after performing multiplexing of HP channels

2. PUSCH repetition type B in presence of A-CSI.

# 2	Discussion

## 2.1	Processing Order of Intra-UE Prioritization and Multiplexing

In RAN1#102e, the issue of multiplexing order for intra-UE prioritization was discussed in email discussion thread [102-e-NR-L1enh-URLLC-Scheduling and HARQ-02], which was concluded by summary [1] and working assumption for processing order (copied in Section 1).

We observe that the WA may result in overly aggressive cancellation. One example is illustrated Figure 1 below. On the other hand, considering the very late stage of Rel-16 maintenance, we are fine to confirm the working assumption and keep the current specification as Section 9 in 38.213 V16.3.0, which captures the WA.

Figure 1. An example of unnecessary cancellation when applying the working assumption to resolve the overlap between low-priority channels and high-priority channels.

Confirm working assumption and no specification change is needed.

## 2.2	Number of Repetitions for PUSCH Repetition Type B

In RAN1#102e meeting, the following TP was agreed [2] for Section 6 of TS38.214:

#### TS 38.214 6.1 UE procedure for transmitting the physical uplink shared channel A UE shall upon detection of a DCI format scheduling a PUSCH transmit the corresponding PUSCH unless the UE does not generate a transport block as described in [10, TS38.321].

With the understanding that MAC may not generated a transport block for an uplink grant, the phrase “no transport block” in section 6.1.2.1 is problematic. It does not reflect the original intention of the specification text, if a transport block may or may not be generated by MAC according to UE decision. Additionally, it implies that the gNB may need to receive PUSCH repetition Type B differently, depending on UE internal decision of generating a TB or not. Thus the gNB has to test two hypothesis: (a) MAC generated a TB; (b) MAC didn’t generate a TB. To avoid gNB hypothesis testing and reflect the original intention of the specification text, the following TP is recommended. Consequently, for PUSCH with triggered A-CSI and SP-CSI, the gNB knows exactly how to receive the PUSCH repetition Type B, completely based on the DCI fields.

Additionally, the current specification is confusing, since a TB is never generated for semi-persistent CSI report(s) on PUSCH.

Based on the discussion above, the text proposal is provided below for TS 38.214.

Adopt the TP to TS 38.214 section 6.1.2.1 when determining the number of repetitions for PUSCH repetition Type B.

#### TS 38.214

#### 6.1.2.1	Resource allocation in time domain

<unchanged part omitted>

…

For PUSCH repetition Type B with 1-bit UL-SCH indicator field of value “1”, except for PUSCH transmitting CSI report(s) with no transport block, the number of nominal repetitions is given by numberOfRepetitions-r16. For the n-th nominal repetition, n = 0, …, numberOfRepetitions-r16 - 1,

…

For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) with 1-bit UL-SCH indicator field of value “0” or activates semi-persistent CSI report(s) on PUSCH with no transport block by a CSI request field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions-r16. When the UE is scheduled to transmit a PUSCH repetition Type B with 1-bit UL-SCH indicator field of value “0” with no transport block and with aperiodic or semi-persistent CSI report(s) by a CSI request field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a CSI request field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].

<unchanged part omitted>

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm working assumption with conclusion that specification change is not needed.

Proposal 2	Adopt the TP to TS 38.214 section 6.1.2.1 when determining the number of repetitions for PUSCH repetition Type B.

# References

R1-2007432, Summary #3 of [102-e-NR-L1enh-URLLC-Scheduling and HARQ-02], Moderator (Qualcomm), RAN1#102e, August 17th – 28th, 2020.

R1-2007337, Correction on 38.214 for PUSCH with UL skipping, vivo, RAN1#102e, August 17th – 28th, 2020.