3GPP TSG RAN WG1 Meeting #103-e	R1-2007725

e-Meeting, October 26th–November 13th, 2020

Source: 	ZTE

Title: 	Remaining Maintenance Issues of UL Tx Switching

Agenda item:	5.1

Document for:	Discussion/Decision

Introduction

After extensive discussion during the last quarters, the framework and most of the details of UL Tx switching for UL CA, EN-DC and SUL are stable now from RAN1 perspective. In this contribution, some TPs for UL Tx switching are presented.

Discussion

SCS ambiguity issue and back-to-back switching

During RAN1#101e, the following agreements were reached. To avoid frequent UL switching, a UE does not expect to perform more than one UL Tx switching in a slot with larger SCS between two uplink carriers.

Agreements: - For inter-band UL CA, SUL and EN-DC, a UE does not expect to perform more than one UL Tx switching in a slot with larger SCS between two uplink carriers.

SCS ambiguity issue

In the current specification, the following description is adopted to reflect the above agreements. However, the current description is not clear due to the following two reasons. (1) The “subcarrier spacing of the uplink transmitted before the switching gap” and “subcarrier spacing of the uplink transmitted after the switching gap” are ambiguous in case when there are UL transmissions in both carriers, especially when the subcarrier spacings of the uplink transmitted in these two carriers are different. (2) The SCS of the uplink carrier should be determined by the SCS of the active UL BWP, instead of the uplink transmission. If the uplink transmission is PRACH, the SCS of the uplink transmission (i.e., PRACH) may be different from the SCS of the active UL BWP.

The current spec description The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL,carrier1, µUL,carrier2), where the µUL,carrier1 corresponds to the subcarrier spacing of the uplink transmitted before the switching gap and the µUL,carrier2 corresponds to the subcarrier spacing of the uplink transmitted after the switching gap.

To address the above issue, the following TP is proposed.

Proposal 1: Adopt the following TP1 for 38.214 UL Tx switching.

TP1: {38.214, 6.1.6 Uplink switching}

<---------------------------- Other parts omitted ----------------------------> If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in subclause 5.3, subclause 5.4, subclause 6.2.1, subclause 6.4 and in subclause 9 of [6, TS 38.213]. The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL,carrier1, µUL,carrier2), where the µUL,carrier1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier and the µUL,carrier2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier. <---------------------------- Other parts omitted ---------------------------->

Back-to-back switching

During RAN1#102e email discussion [102-e-LS-TxSwitching-01], companies raised the concern about back-to-back UL Tx switching, which may complicate the UE implementation design. For example, as shown in the following figure, with the current specification, network is allowed to trigger an UL Tx switching at the end of one slot and trigger another UL Tx switching at the start of the next slot. However, such kind of scheduling is problematic as the network triggers the UE to Case2 without doing anything during Case2 and then triggers the UE back to Case1.

Figure 1. Back-to-back UL Tx switching

Note that, it is possible that network may trigger the UE to Case2 to transmit 1-symbol, 2-symbol or 4-symbol SRS transmission in carrier 2 and then switch back to Case1. Thus, specification should not preclude this kind of scheduling.

To preclude such kind of back-to-back switching, one way is to preclude back-to-back switching in the specification and another way is to leave it to network scheduling because network of course won’t schedule such a dummy UL Tx switching.

If we want to explicitly preclude the back-to-back switching in the spec, then two alternatives can be considered.

  - Alternative#1: The UE does not expect to perform an uplink switching if the gap between the start of this uplink switching and the end of the previous uplink switching is smaller than 1 symbol based on numerology µUL

  - Alternative#2: The switching gap can only be placed at the slot boundary or the switching point for S slot.

Based on the above analysis, we have the following proposal.

Proposal 2: Consider the following two alternatives to address the back-to-back switching issue.

Alternative#1: The UE does not expect to perform an uplink switching if the gap between the start of this uplink switching and the end of the previous uplink switching is smaller than 1 symbol based on numerology µUL

Alternative#2: The switching gap can only be placed at the slot boundary or the switching point for S slot.

Clarification on Tmuxproc,CSI

According the FL summary in RAN1#102e [1], the following TP was discussed but didn’t reach any consensus on it.

9.2.5 UE procedure for reporting multiple UCI types < unchanged text omitted> - N1, N2, d1,1, d2,1, d2,2, Z and Tswitch are defined in [6, TS 38.214], Tswitch is applied only if Z1 of table 5.4-1 in [6, TS 38.214] is applied to the determination of Z, and κ and TC are defined in [4, TS 38.211]. < unchanged text omitted>

It seems the above TP is only to cover the case of SR+Aperioidc CSI on PUSCH. If the TP is adopted, it means non-zero switching gap will be adopted for SR+Aperioidc CSI on PUSCH when Z1 is used. Otherwise, zero switching gap is always assumed. From our perspective, we prefer not to include T switch for this particular case if UE vendors feel it is feasible.

Proposal 3: Tswitch is not needed for the case of SR+Aperioidc CSI multiplexing on PUSCH.

Conclusion

In this contribution, the following TP is provided for UL Tx switching.

Proposal 1: Adopt the following TP1 for 38.214 UL Tx switching.

TP1: {38.214, 6.1.6 Uplink switching}

<---------------------------- Other parts omitted ----------------------------> If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in subclause 5.3, subclause 5.4, subclause 6.2.1, subclause 6.4 and in subclause 9 of [6, TS 38.213]. The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL,carrier1, µUL,carrier2), where the µUL,carrier1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier and the µUL,carrier2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier. <---------------------------- Other parts omitted ---------------------------->

Proposal 2: Consider the following two alternatives to address the back-to-back switching issue.

Alternative#1: The UE does not expect to perform an uplink switching if the gap between the start of this uplink switching and the end of the previous uplink switching is smaller than 1 symbol based on numerology µUL

Alternative#2: The switching gap can only be placed at the slot boundary or the switching point for S slot.

Proposal 3: Tswitch is not needed for the case of SR+Aperioidc CSI multiplexing on PUSCH.

Reference

R1-2007402, [102-e-LS-TxSwitching-01] Email discussion/approval on maintenance of uplink Tx switching thread #1, Moderator (China Telecom)