3GPP TSG RAN WG1 Meeting #103-e	R1-2007758

e-Meeting, October 26th – November 13th, 2020

Source:	ZTE

Title:	Discussion on determining PHR for PUSCH in CA

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

The draft CR and discussion paper about an issue in determining PHR for CA for Rel-15 were submitted in RAN1#102e with the following conclusion.

R1-2005493 Discussion on determining PHR for PUSCH in CA ZTE R1-2005494 Draft CR on determining PHR for PUSCH in CA ZTE Rejected for Rel-15 but further consider for Rel-16 (in RAN1#103-e)

In this contribution, we provide our further views on this issue only for Rel-16.

- Discussion

Both configured grant (CG) PUSCH and dynamic scheduled (also called dynamic grant, DG) PUSCH can carry PHR. The timing for determining PHR (also called PHR timing) on DG PUSCH depends on the DCI scheduling the DG PUSCH. On the other hand, the PHR timing on CG PUSCH depends on the moment that the first uplink symbol of the CG PUSCH transmission minus T’proc,2.

In 38.213[1], it describes a situation that the first PUSCH transmission overlaps with the second PUSCH transmission, and the PHR is carried on the first PUSCH transmission. In some cases the second PUSCH transmission is not considered when computing PHR on the first PUSCH as below.

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP of carrier of serving cell , a second PUSCH transmission on active UL BWP of carrier of serving cell that overlaps with the first PUSCH transmission if - the second PUSCH transmission is scheduled by a DCI format 0_0 or a DCI format 0_1 in a PDCCH received in a second PDCCH monitoring occasion, and - the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format 0_0 or DCI format 0_1 scheduling an initial transmission of a transport block after a power headroom report was triggered or - the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 =1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the first PUSCH transmission is on a configured grant after a power headroom report was triggered. ...

As seen from above text, only the following cases have been specified:

- Case 1: DG first PUSCH + DG second PUSCH

- Case 2: CG first PUSCH + DG second PUSCH

- Case 3: CG first PUSCH + CG second PUSCH

In practice, a Case 4 with DG first PUSCH + CG second PUSCH should also be allowed. Unfortunately, there is NO specified UE behavior herein. Consequently, we propose to specify the UE behavior for a Case 4 with DG first PUSCH + CG second PUSCH.

For CG second PUSCH transmission, the moment that the first uplink symbol of the CG second PUSCH transmission minus T’proc,2 should be used to decide whether the second PUSCH transmission can be “seen” or not.

Specifically, the following examples show the details of how it works for case4.

- Case 4: DG first PUSCH + CG second PUSCH, for example, as shown in figure 1,

- for case 4-1, the moment that the first uplink symbol of the CG second PUSCH transmission minus T’proc,2 is earlier than PHR timing, so the CG second PUSCH transmission can be “seen” when computing PHR in first PUSCH;

- for case 4-2, the moment that the first uplink symbol of the CG second PUSCH transmission minus T’proc,2 is later than PHR timing, so the CG second PUSCH transmission can NOT be “seen” when computing PHR in first PUSCH.

Figure 1 case 4-1 and case 4-2

Consequently, we have the following proposed text change.

Proposed text change: {38.213: 7.7.1}

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP of carrier of serving cell , a second PUSCH transmission on active UL BWP of carrier of serving cell that overlaps with the first PUSCH transmission if - the second PUSCH transmission is scheduled by a DCI format 0_0 or a DCI format 0_1 in a PDCCH received in a second PDCCH monitoring occasion, and - the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format 0_0 or DCI format 0_1 scheduling an initial transmission of a transport block after a power headroom report was triggered or - the second PUSCH transmission is configured by ConfiguredGrantConfig, and - the first uplink symbol of the second PUSCH transmission minus T’proc,2 =Tproc,2 is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format 0_0 or DCI format 0_1 scheduling an initial transmission of a transport block after a power headroom report was triggered, or - the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 if the first PUSCH transmission is on a configured grant after a power headroom report was triggered, where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 =1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant. <Unchanged part omitted>

- Conclusion

In this contribution, we discuss the issue on determining PHR for PUSCH in CA and provide proposed text change as shown above. The corresponding draft CR can be found in [2].

- References

- 3GPP TS 38.213-g30 NR; Physical layer procedures for control

- R1-2007759 Draft CR on determining PHR for PUSCH in CA, ZTE