3GPP TSG RAN WG1 Meeting #103-e                                                  	              R1-2007766

e-Meeting, October 26th – November 13th, 2020

Source:              ZTE

Title:                   Enhancements on beam management for multi-TRP

Agenda Item:     8.1.2.3

Document for:   Discussion and Decision

- Introduction

Regarding beam management for multi-TRP which is one of the objectives in the WID [1], some potential enhancements on beam management for multi-TRP including DL beam measurement and reporting with multi-panel reception, TRP-specific beam recovery and simultaneous transmission of different DL channel(s) and RS(s) are discussed for NR Rel-16 in this contribution.

- Enhancement(s) on DL beam measurement and reporting

In this subsection, some potential enhancements including CSI framework for group based beam reporting, multi-panel/TRP based beam measurement, and non-group based reporting are discussed for NR Rel-17.

  - Enhancements on group based beam reporting

Group-based reporting that informs gNB which pairs of multiple beams can be received simultaneously (which implicitly provides the mapping between UE panel and gNB beam) should be regarded as an essential solution for supporting the multi-panel operation of both DL and UL transmission (including DL multi-panel reception and UL fast panel selection). In Rel-15, the following two alternative criteria for group based beam reporting were discussed and agreed for down-selection or merging.

Alt-1 (beam group based reporting): UE reports information about TRP Tx Beam(s) that can be received using selected UE Rx beam set(s) where a Rx beam set refers to a set of UE Rx beams that are used for receiving a DL signal. - NOTE: Different TRP Tx beams reported for the same Rx beam set can be received simultaneously at the UE. - NOTE: Different TRP TX beams reported for different UE Rx beam set may not be possible to be received simultaneously at the UE Alt-2 (antenna group based reporting): UE reports information about TRP Tx Beam(s) per UE antenna group basis where UE antenna group refers to receive UE antenna panel or subarray. - NOTE: Different TX beams reported for different antenna groups can be received simultaneously at the UE. - NOTE: Different TX beams reported for the same UE antenna group may not be possible to be received simultaneously at the UE

  - In our views, antenna group based reporting (Alt-2) is beneficial for DL panel-specific beam management and is also relevant to fast UL panel switching. For instance, for mDCI case in mTRP, the different groups can be associated with two different TPRs, and consequently the subsequent beam refinement and CSI measurement for two TRPs can be performed independently.

  - But, in a specific case, e.g., L1-SINR reporting and sDCI in mTRP, the UE can well consider the inter-beam/panel interference through using beam group based reporting (Alt-1)  and can provide a better spatial multiplexing (high-RANK DL transmission).

Consequently, from our perspective, the separate usages of beam group based reporting (Alt-1) and antenna group based reporting (Alt-2) can be identified in multi-panel and multi-TRP case.

  - Beam group based reporting (Alt-1) is used for sDCI-mTRP and/or optimizing inter-beam inference in SINR reporting, and antenna group based reporting (Alt-2) is used for mDCI-mTRP and/or facilitating panel-specific DL and UL beam management via RSRP reporting.

Observation 1: Group-based reporting should be regarded as an essential solution for the case of multi-panel operation including both DL multi-panel reception and UL fast panel selection.

  - Regarding criteria for group based reporting, the antenna group based reporting (Alt-2) is very beneficial for DL and UL panel-specific beam management, but beam group based reporting (Alt-1) is more suitable for optimizing inter-beam/panel interference.

Proposal 1: Support both beam group based reporting and antenna group based reporting for multi-TRP operation.

  - Information on grouping one or more RS(s) (e.g., beam group ID, or antenna group ID) can be reported along with RS ID(s) and RSRP/SINR in a report instance.

The Rel-15/16 group based reporting with only N=2 Tx beams to be reported significantly restricts system flexibility and weakens diversity gain through dynamically indicating one of multiple candidate beams in multi-panel/TRP cases. According to the grouping criteria of antenna group based reporting (Alt-2), we evaluate SLS performance for the case that a UE with 2 Rx panels is served by 2 TRP in indoor hotspot and dense urban under UE blockage model of TS 38.901. In dense urban, we consider the high-mobility case, i.e., speed of UE = 60 km/h. The evaluation assumptions can be found in the appendix.

The simulation results for indoor hotspot and dense urban are provided as a function of number of beams per group in a report instance and can be found in Table 1 and Table 2 respectively. Rel-15 group based reporting (up to 1 beam for each of 2 UE panels is reporting) is considered as baseline. From the evaluation results, it can be observed that increase number of beams per group can bring performance benefit due to combat blockage and UE mobility. Specifically, we have the further observation as follows.

- With the increase of the number of groups to be reports M from only 1 to 4, the performance gain of 7.7% and 5.6% in terms of average UPT can be observed in indoor hotspot and dense urban.

- For 5% cell edge UE in dense urban, we can find the performance gain of 36.8% compared with Rel-15 group based reporting, which proves that the increase of candidate beams for each UE panel can distinctly enhance the performance of cell-edge UE with high mobility and be against the blockage and UE mobility.

Table 1	SLS for group based reporting with M beams per group to be reported and N groups in indoor hotspot

| UPT (Mbps) | mean | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| (N = 2 group, M = 1 beam) i.e., Rel-15 group reporting | 89.48 (100%) | 52.02 (100%) | 79.20 (100%) | 176.47 (100%) |
| (N = 2 group, M = 2 beams) | 93.34 (+4.3%↑) | 55.50 (+6.7%↑) | 84.19 (+6.3%↑) | 182.06 (+3.2%↑) |
| (N = 2 group, M = 4 beams) | 96.36 (+7.7%↑) | 55.82 (+7.3%↑) | 84.82 (+7.1%↑) | 183.05 (+3.7%↑) |

Table 2	SLS for group based reporting with M beams per group to be reported and N groups in dense urban

| UPT (Mbps) | mean | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| (N = 2 group, M = 1 beam) i.e., Rel-15 group reporting | 83.42 (100%) | 13.47 (100%) | 77.65 (100%) | 165.66 (100%) |
| (N = 2 group, M = 2 beams) | 86.19 (+3.3%↑) | 15.15 (+12.5%↑) | 80.86 (+4.1%↑) | 170.71 (+3.0%↑) |
| (N = 2 group, M = 4 beams) | 88.09 (+5.6%↑) | 18.42 (+36.8%↑) | 81.40 (+4.8%↑) | 170.07 (+2.7%↑) |

For beam group based reporting (Alt-1), LLS evaluation results can be found in our companion contribution [2], and it can also be observed that, with the increase of number of groups and/or number of beams within a group, significant performance gains to combat blockage and UE mobility can be observed.

Besides, up to 2 groups (e.g., UE panels) in Rel-15 group reporting are also unsuitable for the enhanced UE in Rel-16. Three or four UE panels may be applied to a handheld device, e.g., mobile phone. Regarding EVM for multi-beam operation in Item-1 of this FeMIMO, evaluation assumption of 3 UE panels (left, right, and back) has also been agreed for subsequent evaluation. That means that the number of groups in the group based reporting should also be enhanced, e.g., up to 4 groups.

Observation 2: From the evaluation results for group based reporting, it can be observed that:

  - Increase number of beams per group can bring significantly higher performance gain to combat blockage and UE mobility, especially for cell-edge UE in dense urban, through improving system flexibility with more candidate beams.

  - Increase number of groups can be more suitable for advanced UE with 3, 4 or more panels.

Proposal 2:  Extension of Rel-15 group based beam reporting should be considered to support more Tx beams and/or more groups to be reported in Rel-17 NR-FeMIMO.

  - Regarding beam group based reporting, a UE can be configured to report N (N>=1) groups and M (M>1) beams per group.

  - Regarding antenna group based reporting, a UE can be configured to report N (N>1) groups and M (M>=1) beams per group.

  - CSI framework for multi-TRP based beam measurement

Consideration on constraints of TRP antenna panels should be taken for selecting Tx beam(s) for downlink, like the maximum number of SRS resource sets that can be configured for UL beam management in NR Rel-15. For instance, Tx beams generated from one Tx panel cannot be transmitted simultaneously. Therefore, enhanced signaling or configuration would allow gNB to inform this kind of information of UE for beam reporting.

- For one TRP, there may have more than one TRP panels, but these multiple panels often share the same boresight and similar large-scale channel properties. If beam management is done for each individual panel, the overheads of beam measurement and reporting per Tx panel per TRP is very high and NOT necessary.

- Therefore, from beam management perspective, these TRP panels from one same TRP can be represented by one out of these panel, and the beam(s) generated by the selected panel can be virtualized as one group, in which no more than N beams can be simultaneously transmitted, where N depends on gNB capability. Furthermore, in multi-TRP case, gNB can configure multiple groups, respectively, each of which is associated with one TRP.

Consequently, when a UE selects beams for simultaneous receiving in group based beam reporting, the selection rules should follow the constraint that the number of these beams (to be reported for simultaneous receiving) from one same TRP should satisfy its capability of simultaneous transmission, e.g., no more than N.

- One example for multi-beam measurement in multi-panel/TRP case can be found in Figure 1, where there are two TRPs that are deployed with 4 panels and 1 panel, respectively. For TRP-1, only one panel is used for generating candidate Tx beam(s) for sweeping, rather than the whole 4 panels.

Observation 3: For one TRP, there may have multiple TRP panels sharing the same boresight and similar large scale properties, and consequently beam measurement for one out of the multiple panels (instead of each of the panels) is sufficient for supporting the subsequent CSI acquisition and data transmission.

  - In such case, the overhead of measurement RS and UL reporting can be significantly saved compared with per-panel per-TRP measurement.

Figure 1 DL multi-beam measurement scenario for multi-TRP transmission with multi-panel reception

Consequently, information on simultaneous transmission capability from each of TRP can be indicated for the assist of DL beam determination in UE reporting and facilitating inter-TRP beam pairing. In general, according to offline discussion in RAN1#102-e, there are two following options for providing the above information by gNB.

- Opt-1: an ID, e.g., TRP ID, is introduced for CSI framework, and a CSI-RS resource can be configured with the ID directly.

- Opt-2: A CSI-RS resource set corresponds to a TRP transparently, and for multi-TRP case, more than one CSI-RS resource sets can be configured for CSI resource setting.

The two related examples are shown as follows.

- Example 1: introduce higher-layer configured indices (labelled SSI, for discussion purpose only) configured to CMR resources, where UE is requested to report beams with the same or different SSIs within a reported group - Example 2: One CSI resource setting can be configured with multiple CSI-RS resource sets (each of which corresponds to a TRP transparently), with a restriction/requirement for group based report. - E.g.-1 the maximum number of CSI-RS in a set to be reported in a group, e.g.,1, 2 or 4. - E.g.-2, each set should be with at least one CSI-RS resource to be reported, if multiple CSI-RS resources reported in a group.

From our perspective, when down-selecting above options for informing the TRP-related information to UE side from gNB side, consideration on flexible CSI framework in NR should be well taken. Specifically, there is NOT unique/fixed mapping between a CSI-RS resource and CSI-RS resource set/setting, and instead one CSI-RS resource can be mapped into more than one different CSI-RS resource set by NR CSI framework. But, if going with Opt-1, there is a strong restriction that a CSI-RS resource, e.g., aperiodic CSI-RS, can NOT be shared with different TRP(s).  Therefore we prefer Opt-2.

- Regarding beam group based report (i.e., different RSs to be reported within a group can be received simultaneously), we have the the following restriction/requirement, i.e., above Example 2.

  - The maximum number of CSI-RS in a set, e.g., 1, 2 or 4, can be reported in a group. That corresponds to the TRP related simultaneous transmission capability.

  - Each set should be with at least one CSI-RS resource to be reported for facilitating inter-TRP beam pairing.

- Regarding antenna group based reporting, some further studies on the reporting restriction/requirement, e.g., mapping between a TRP and a UE panel, are needed.

Proposal 3: A CSI-RS resource set corresponds to a TRP transparently, and for multi-TRP operation, more than one CSI-RS resource sets can be configured for CSI resource setting.

  - The following restriction/requirement can be configured for beam group based report (i.e., different RSs within a reported group can be received simultaneously).

    - Reporting restriction-1: the maximum number of CSI-RS in a set, e.g., 1, 2 or 4, to be reported in a group, in order to inform TRP related simultaneous transmission capability implicitly.

    - Reporting restriction-2: at least one CSI-RS resource from one CSI-RS resource set to be reported for facilitating inter-TRP beam pairing.

  - FFS: restriction/requirement for antenna group based reporting (i.e., different RSs from different reported group can be received simultaneously)

  - Enhancements on non-group based beam reporting

In RAN1#102-e, besides group based reporting, non-group based reporting was additionally identified for beam management for multi-TRP. The two related examples are shown as follows.

- Example 1: Two non-group based reporting settings are configured for a UE to report L1-RSRP measurement results, with reporting setting 1 containing resources from TRP1, and another reporting setting 2 containing resources from the second TRP2. The two reporting settings are configured that UE knows the reported beams in the two reports could be received simultaneously - Example 2: Two non-group based reporting settings are configured for a UE to report L1-RSRP measurement results, with reporting setting 1 containing resources from TRP1, and another reporting setting 2 containing resources from the second TRP2. An explicit ID is associated with each reported resources. For the reported resources associated with different IDs, they can be received simultaneously by the UE. For the reported resources associated with the same ID, they cannot be received simultaneously by the UE.

The motivation of enhancing non-group based reporting is to handle multi-TRP transmission with non-ideal backhaul. UE can determine the one by one mapping between UE panel(s) and TRP(s), and then panel-specific reporting can be carried by multiple reporting instances, each of which is associated with a respective group ID/panel ID and is sent to a respective TRP. Compared with group based reporting, the each of groups can be reported by different reporting instance, and the latency of sharing group based reporting from one TRP to another TRP may be saved.

From our perspective, the group based reporting can still leverage non-ideal backhaul to coordinate certain delay tolerant information for beam reporting. Consequently, the benefit of non-group based beam reporting may be limited, and how to achieve the association between two different report instances should be further studied. Also, the non-group based reporting may only be relevant to antenna group based reporting (Alt-2).

Proposal 4: Further study benefits and identify usages of non-group based reporting approach in multi-TRP operation, compared with group based reporting.

  - Notes that, for the non-group based reporting approach, multiple Tx beam group(s) corresponding to different UE panel(s) can be carried by respective reporting instances, each of which is associated with a different antenna group ID/panel ID and is sent to a respective TRP.

- Enhancement(s) on TRP-specific beam recovery

In general, there are independent beam pair links for each of multiple TRP serving a single UE, and consequently only beam pair link for partial TRPs fails instead of all beam pair links. For instance, in Figure 2, Link-1 is blocked by a human or a vehicle, and the channel quality (e.g., BLER of PDCCH/CORESET(s) with the same CORESET pool) may be worse than a threshold. But, there is no blockage occurrence for Link-2, and channel quality is still better than the threshold. Consequently, UE can NOT initialize any beam failure recovery procedure according to the rule of existing beam failure recovery procedure.

Considering the non-ideal backhaul between TRPs, communication link for the failed TRP may NOT be recovered quickly with the assist of a work-well TRP. Straightforwardly, the PCell and SCell beam recovery procedure can be enhanced as a TRP-specific one. In Rel-16 eMIMO, there was some initial discussion for TRP-specific beam recovery, but, unfortunately, there was no further progress.

In our views, in order to guarantee TRP-specific transmission, e.g., for mDCI in mTRP, TRP-specific beam recovery procedure should be further enhanced in Rel-17 FeMIMO.

- Considering the existing beam recovery procedures for PCell and SCell, beam recovery for SCell is more suitable for this TRP-specific beam recovery procedure. As an enhancement, we need to consider per-TRP procedure for beam recovery.

  - Regarding beam failure detection, BFD RS can be determined per TRP.

  - Regarding new candidate beam identification, RS for new candidate beam identification is configured per TRP. The explicit and implicit approaches can be reused accordingly.

  - Regarding beam failure recovery request, PUCCH + MAC-CE based recovery procedure is reused as a baseline.

  - Regarding gNB response for beam failure recovery request, toggling NDI with the same HARQ procedure ID as the PUSCH carrying MAC-CE is reused as a baseline.

  - Regarding UE behavior after receiving gNB response, CORESET and PUCCH’s beam is updated per TRP according to the new candidate beam reported in MAC-CE. Then, UL power control parameter(s) corresponding to PUCCH are updated accordingly, and we can further study the UE behavior for PUSCH and SRS transmission after receiving gNB response.

Figure 2 Multi-beam transmission for multi-TRP transmission with multi UE-panel reception

Besides, it may occur that all TRP(s) are failed simultaneously, and consequently we need to consider a fall-back mechanism from TRP-specific beam recovery to normal PRACH based PCell beam recovery.

Proposal 5: For TRP-specific BFR procedure, Rel-16 SCell-BFR procedure can be considered as a baseline

  - For beam failure detection (BFD), BFD related RS(s) can be determined per TRP

  - For new beam identification (NBI), NBI related RS(s) can be configured per TRP.

  - For beam failure recovery request, PUCCH + MAC-CE based approach as in SCell-BFR is reused as a baseline.

  - For gNB response, toggling NDI with the same HARQ procedure ID as the PUSCH carrying MAC-CE as in SCell-BFR is reused as a baseline.

  - For UE behavior after receiving gNB response, CORESET and PUCCH’s beam, and UL power control parameter for PUCCH is updated per TRP according to the new candidate beam reported in MAC-CE.

    - FFS: PUSCH and SRS

- Enhancement(s) on simultaneous transmission with multi-panel reception

In this section, the enhancement(s) on simultaneous transmission with multi-panel reception, including antenna group/panel indication for DL transmission and simultaneous transmission of different DL channel(s) and RS(s), are discussed.

  - Antenna group/panel indication for DL transmission

In order to achieve an effective multi-panel operation, the enhancements on associating the DL transmission with antenna group(s) should be considered. One Tx beam may be received by different UE antenna group(s) with corresponding UE Rx beams, and consequently there may be some ambiguities between gNB and UE sides about which UE antenna group(s) are used when only the TCI state (i.e., DL Tx beam) is indicated for a DL transmission. Subsequently, considering that different DL channel(s) or RS(s) with different antenna group(s) may be received simultaneously, associating the DL transmission with the antenna group(s) is beneficial for specifying whether/when the different DL channel(s) and RS(s) can be transmitted simultaneously.

- Note that the DL channel(s) and RS(s) with different spatial QCL parameters and same antenna group may NOT be received simultaneously.

- Note that the DL channel(s) and RS(s) with different spatial QCL parameters and different antenna groups may be received simultaneously.

Regarding the antenna group/panel indication for DL transmission, the spatial QCL parameter in the TCI state can be further configured with an antenna group/panel ID. When the TCI state is applied to a DL transmission, the Rx spatial domain filter of receiving the DL transmission should be generated by the UE Rx panel indicated by the antenna group ID.

Alternatively, for non-ideal backhaul or mDCI-mTRP, dynamic beam indication for transmission/reception sharing the same antenna group among multiple TRPs is very difficult considering the constraint of simultaneous transmission. Consequently, when multiple CORESET group IDs are configured in a UE, the different CORESET group(s) and the associated transmission resource(s), e.g., the scheduled PDSCH and AP-CSI-RS, can be associated with respective antenna group(s), and the subsequent beam management for a CORESET group is only performed based on its associated antenna groups until re-associating.

Proposal 6:  In multi-panel reception, DL channel(s) and RS(s) can be associated with the information about antenna group(s).

- Study mechanism(s), e.g., associating CORESET group(s) or TCI state(s) with antenna group(s).

  - Simultaneous transmission of different DL channel(s) and RS(s)

In NR Rel-16, 2-Tx-beam transmission is supported, but the collision rules for different DL Tx beam(s) at a given time instant are still based on the single DL Tx beam transmission, i.e., according to Rel-15. With the support of multi-panel operation for DL transmission, it is assumed that DL channels or RSs from the same or different CCs (or TRPs) can be received with different spatial QCL parameters simultaneously, according to the UE capability. Furthermore, considering constraints of antenna group(s), different spatial QCL parameters should be associated with respective antenna groups, e.g., panel, of the UE side as one condition.

- For instance, for a multi-panel UE, there can be only one spatial QCL parameter applied for a single antenna group at a given time instant.

- Additionally, different spatial QCL parameters can be applied to the different antenna groups at a given time instant. As shown in Figure 3, DL channel/RS-0 and DL channel/RS-1 should be transmitted simultaneously based on two different gNB Tx beams, i.e., the green beam and the blue beam, respectively. Herein, the two different gNB Tx beams corresponds the panel-0 and panel-1 (or panel-1 and panel-0), respectively.

Figure 3 Simultaneous transmission of different channel(s) and RS(s) based on different beams and different panels

However, different spatial QCL parameters of different DL channel(s) and RS(s) may be applied to the same antenna group at a given time instant in some cases. For example, as shown in Figure 4, in a non-ideal backhaul scenario (e.g., multi-DCI in multi-TRP), the UE rotates at a given time instant, and only panel-0 faces to 2 TRPs. Thereby, for the reception of DL channel/RS-0 and DL channel/RS-1, the two best (Rx) beam are likely to be associated with panel-0. With this collision, the UE fails to receive the DL channel(s) and RS(s) simultaneously due to the restriction of its capability. In order to avoid such dynamical collisions, one straightforward way is that the gNB shall always transmit the different DL channel(s) and RS(s) with different spatial QCL parameter by TDMed scheme, e.g., go back to Rel-15 design. No doubt that this approach does not achieve any performance gain through using DL simultaneous reception with multiple UE antenna groups, and meanwhile it also greatly limits the flexibility of the gNB scheduling.

Figure 4 Transmission of different channel(s) and RS(s) based on different beams and the same panel for a multi-panel UE

Therefore, in order to ensure the flexibility of the gNB scheduling, it is necessary to study mechanism or rules that can resolve such collisions. Straightforwardly, the UE can prioritize the reception of a DL channel or RS with higher priority. Alternatively, when the reliability requirement for receiving these two DL channel or RS is low, the UE can use a predefined beam (or default beam) to receive these two DL channel or RS, e.g., a wider beam can be used as a predefined beam. Additionally, considering that 2 panels may be in active state at the same time, so, the other active panel can be used to be receive the other DL channel or RS (with lower priority). For instance, as shown in Figure 4, when the two best (Rx) beam are associated with panel-0, the UE can use panel-0 to receive the DL channel or RS with higher priority based on this best beam, at the same time, the UE can use panel-1 to receive the other DL channel or RS with lower priority based on a predefined beam. Furthermore, the following cases for simultaneous transmission of different DL channel(s) and RS(s) can be studied: PDCCH+PDSCH, PDSCH+PDSCH, PDSCH+CSI-RS, CSI-RS+CSI-RS, etc.

Proposal 7:  In the case of multi-TRP transmission with UE multi-panel reception, at least one of the following methods should be considered when beam collision between different DL channel(s) and RS(s) occurs.

- Alt 1: UE can prioritize the reception of a DL channel or RS with higher priority.

- Alt 2: UE can use a predefined beam to receive the conflicting DL channels or RSs.

- Alt 3: UE can use the indicated beam and corresponding panel to receive the DL channel or RS with higher priority, and meanwhile UE can use the other active panel and a predefined beam to receive the other DL channel or RS with lower priority.

- FFS: definition of beam collision between different DL channel(s) and RS(s), e.g., different QCL Type D RS(s) under the same panel.

- Conclusion

In this contribution, we provide the observations and proposals for enhancements on beam management for multi-TRP.

Observation 1: Group-based reporting should be regarded as an essential solution for the case of multi-panel operation including both DL multi-panel reception and UL fast panel selection.

  - Regarding criteria for group based reporting, the antenna group based reporting (Alt-2) is very beneficial for DL and UL panel-specific beam management, but beam group based reporting (Alt-1) is more suitable for optimizing inter-beam/panel interference.

Observation 2: From the evaluation results for group based reporting, it can be observed that:

  - Increase number of beams per group can bring significantly higher performance gain to combat blockage and UE mobility, especially for cell-edge UE in dense urban, through improving system flexibility with more candidate beams.

  - Increase number of groups can be more suitable for advanced UE with 3, 4 or more panels.

Observation 3: For one TRP, there may have multiple TRP panels sharing the same boresight and similar large scale properties, and consequently beam measurement for one out of the multiple panels (instead of each of the panels) is sufficient for supporting the subsequent CSI acquisition and data transmission.

  - In such case, the overhead of measurement RS and UL reporting can be significantly saved compared with per-panel per-TRP measurement.

Proposal 1: Support both beam group based reporting and antenna group based reporting for multi-TRP operation.

  - Information on grouping one or more RS(s) (e.g., beam group ID, or antenna group ID) can be reported along with RS ID(s) and RSRP/SINR in a report instance.

Proposal 2:  Extension of Rel-15 group based beam reporting should be considered to support more Tx beams and/or more groups to be reported in Rel-17 NR-FeMIMO.

  - Regarding beam group based reporting, a UE can be configured to report N (N>=1) groups and M (M>1) beams per group.

  - Regarding antenna group based reporting, a UE can be configured to report N (N>1) groups and M (M>=1) beams per group.

Proposal 3: A CSI-RS resource set corresponds to a TRP transparently, and for multi-TRP operation, more than one CSI-RS resource sets can be configured for CSI resource setting.

  - The following restriction/requirement can be configured for beam group based report (i.e., different RSs within a reported group can be received simultaneously).

    - Reporting restriction-1: the maximum number of CSI-RS in a set, e.g., 1, 2 or 4, to be reported in a group, in order to inform TRP related simultaneous transmission capability implicitly.

    - Reporting restriction-2: at least one CSI-RS resource from one CSI-RS resource set to be reported for facilitating inter-TRP beam pairing.

  - FFS: restriction/requirement for antenna group based reporting (i.e., different RSs from different reported group can be received simultaneously)

Proposal 4: Further study benefits and identify usages of non-group based reporting approach in multi-TRP operation, compared with group based reporting.

  - Notes that, for the non-group based reporting approach, multiple Tx beam group(s) corresponding to different UE panel(s) can be carried by respective reporting instances, each of which is associated with a different antenna group ID/panel ID and is sent to a respective TRP.

Proposal 5: For TRP-specific BFR procedure, Rel-16 SCell-BFR procedure can be considered as a baseline

  - For beam failure detection (BFD), BFD related RS(s) can be determined per TRP

  - For new beam identification (NBI), NBI related RS(s) can be configured per TRP.

  - For beam failure recovery request, PUCCH + MAC-CE based approach as in SCell-BFR is reused as a baseline.

  - For gNB response, toggling NDI with the same HARQ procedure ID as the PUSCH carrying MAC-CE as in SCell-BFR is reused as a baseline.

  - For UE behavior after receiving gNB response, CORESET and PUCCH’s beam, and UL power control parameter for PUCCH is updated per TRP according to the new candidate beam reported in MAC-CE.

    - FFS: PUSCH and SRS

Proposal 6:  In multi-panel reception, DL channel(s) and RS(s) can be associated with the information about antenna group(s).

- Study mechanism(s), e.g., associating CORESET group(s) or TCI state(s) with antenna group(s).

Proposal 7:  In the case of multi-TRP transmission with UE multi-panel reception, at least one of the following methods should be considered when beam collision between different DL channel(s) and RS(s) occurs.

- Alt 1: UE can prioritize the reception of a DL channel or RS with higher priority.

- Alt 2: UE can use a predefined beam to receive the conflicting DL channels or RSs.

- Alt 3: UE can use the indicated beam and corresponding panel to receive the DL channel or RS with higher priority, and meanwhile UE can use the other active panel and a predefined beam to receive the other DL channel or RS with lower priority.

- FFS: definition of beam collision between different DL channel(s) and RS(s), e.g., different QCL Type D RS(s) under the same panel.

- References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung

[2] R1-2007770, Further details on Multi-beam and Multi-TRP operation, ZTE

- Appendix:

Table 3 Simulation assumptions for indoor-hotspot system-level evaluation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 80MHz |
| Subcarrier Spacing | 120kHz |
| Channel Model | Indoor in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | Two UE panels are attached with two respective TRP. |
| ISD | 20m |
| BS Tx power | 23dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1), (dV,dH) = (0.5, 0.5)λ Boresight direction is perpendicular to the ceiling |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See wall-mount in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 3m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 3km/h, 10 users per BS |
| UE receiver type | MMSE-IRC |
| Transmission scheme | SU, Rank adaptation |

Notes: Any other parameters not specified here remain the same as those in NR evaluation assumption for beam management in Rel-16 eNR-MIMO.

Table 4 Simulation assumptions for dense urban system-level evaluation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 80MHz |
| Subcarrier Spacing | 120kHz |
| Channel Model | Dense Urban Micro layer only in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | Two UE panels are attached with two respective TRP. |
| ISD | 200m |
| BS Tx power | 43dBm |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | According to TR38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 25m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 60km/h, 10 users per BS |
| UE receiver type | MMSE-IRC |
| Transmission scheme | SU, Rank adaptation |

Notes: Any other parameters not specified here remain the same as those in NR evaluation assumption for beam management in Rel-16 eNR-MIMO.