3GPP TSG RAN WG1 #103-e                                              R1-2007769

e-Meeting, October 26th – November 13th, 2020

Source:         ZTE

Title:            CSI enhancements for Multi-TRP and FR1 FDD reciprocity

Agenda item:    8.1.4

Document for:	Discussion and Decision

Introduction

In RAN#86, the Rel-17 WID for further enhancements on NR MIMO was approved [1], in which one potential enhancement is for CSI measurement and reporting as shown as follows

Enhancement on CSI measurement and reporting: - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2 - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

To fairly compare the performance among companies, the consensus was achieved for simulation assumptions in RAN1#102-e meeting. Meanwhile, some agreements were also made to categorize the potential enhancement solutions for both MTRP-CSI and FDD reciprocity which correspond to the above sub-bullet a and b respectively.

In this contribution, we provide our views and simulation results for these potential enhancements.

Multi-TRP CSI enhancement

As shown in the following agreement made in RAN1#102 meeting, the potential solutions for MTRP CSI enhancement are categorized into two general options.

Agreement For CSI enhancement for multi-TRP, study following aspects taking into account trade-off among UE complexity, performance and reporting/RS overhead - Category 1 - For a reporting setting CSI-ReportConfig, more than one CSI-RS port groups in a resource or resources or resource sets are associated to different TRPs/TCI states, - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/UE-selected channel and interference hypotheses across TRPs /TCI states - and then report one or more CSIs within a single CSI report. - Category 2 – Within an implicit/explicit set of reporting settings CSI-ReportConfigs, which are associated to different TRPs/TCI states, - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/ UE-selected channel and interference hypotheses - and then report multiple CSIs with multiple CSI reports (including one or more CSIs per report or selected CSI with single CSI report) - Other enhancement are not excluded, e.g. CQI enhancements for multi-TRP transmission including CQI format, CQI reporting mechanism Note that companies are encouraged to clarify applicable transmission schemes/scenarios and strive to unify Rel-17 MTRP CSI framework enhancements

In Category 1, two parts of CSI-RS corresponding to two TRPs respectively are configured in one CSI report setting. To achieve accurate CSI measurement, UE can report two sets of RI/PMI/CQI where each set corresponds to one part of CSI-RS. When UE computes CSI, it takes into account the interference between two parts of CSI-RS. It reflects inter-TRP interference of MTRP PDSCH transmission very well especially for SDM scheme based on single-DCI MTRP. Since two CSI-RS parts can correspond to two CSI-RS port groups, two CSI-RS resources or two CSI-RS resource sets within one CSI report setting, the current structure of CSI configuration can be reused. The spec impact is just how to set up association between two CSI-RS parts.

In Category 2, two parts of CSI-RS corresponding to two TRPs respectively are configured in different CSI report settings. The association between two CSI reporting should be introduced. More RRC structure enhancement is needed. In such case, two CSI report settings are linked and for one instance of MTRP CSI feedback, RRC signaling overhead is increased accordingly compared with Category 1.

The use case of Category 2 was proposed mainly for multi-DCI based MTRP with non-ideal backhaul. However, the scheduling of two TRPs are usually independent. In such case, even inter-TRP interference is considered for CSI feedback, two PDSCHs scheduled by two TRPs may be non-overlapped or partially overlapped, which means the scheduling decision from gNB can have different level of inter-TRP interference as the assumption when UE derives CSI. This makes Category 2 less useful or more restrictive in practical deployment.

From specification impact and signaling overhead perspectives, we support Category 1.

Proposal 1: Support Category 1, i.e.

- For a reporting setting CSI-ReportConfig, more than one CSI-RS port groups in a resource or resources or resource sets are associated to different TRPs/TCI states,

  - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/UE-selected channel and interference hypotheses across TRPs /TCI states

  - and then report one or more CSIs within a single CSI report.

In order to justify the performance of the CSI enhancement, we provide our simulation results in Table 2-1 and 2-2 for UMa and Indoor hotspot respectively. The simulation assumptions can be found in Table 6-1 in Appendix.

In the simulation, SDM based MTRP transmission is used. For Rel-16 CSI, CSI for two TRPs are separately reported, the inter-TRP interference is not considered. For the joint CSI, RI/PMI/CQI is jointly selected considering inter-TRP interference. Although some performance gain based on improved CSI feedback scheme can be observed from the results, it is not so large even we assume SDM based MTRP transmission. It can be expected that there will not be performance improvement in the case of multi-DCI based MTRP with non-ideal backhaul where two PDSCHs may not be overlapped.

Table 2-1 Performance of CSI enhancement for UMa

|  | RU | Mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- |
| Rel-16 CSI | 23.23% | 54.31 | 20.51 | 58.26 | 94.16 |
| Joint CSI | 22.22% | 56.86(4.7%) | 21.75(6.0%) | 60.64(4.1%) | 101.56(7.9%) |
| Rel-16 CSI | 36.96% | 46.24 | 15.06 | 45.52 | 83.94 |
| Joint CSI | 35.77% | 48.58(5.1%) | 16.25(7.9%) | 48.21(5.9%) | 88.55(5.5%) |
| Rel-16 CSI | 65.48% | 29.89 | 5.74 | 23.81 | 65.11 |
| Joint CSI | 63.17% | 32.93(10.2%) | 7.28(26.8%) | 28.54(19.9%) | 65.11(0.0%) |

Table 2-2 Performance of CSI enhancement for Indoor

|  | RU | Mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- |
| Rel-16 CSI | 27.60% | 48.65 | 14.97 | 48.82 | 91.38 |
| Joint CSI | 26.32% | 51.10(5.1%) | 16.23(8.4%) | 51.36(5.2%) | 99.74(9.1%) |
| Rel-16 CSI | 43.72% | 36.61 | 8.76 | 32.08 | 65.11 |
| Joint CSI | 41.55% | 39.44(7.7%) | 9.78(11.6%) | 34.89(8.8%) | 73.04(12.2%) |
| Rel-16 CSI | 68.30% | 22.92 | 3.08 | 17.26 | 62.83 |
| Joint CSI | 66.53% | 25.32(10.5%) | 3.67(19.2%) | 19.63(13.7%) | 64.67(2.9%) |

In Category 1, two parts of CSI-RS corresponding to two TRPs respectively should be associated at least for FR2. Since two CSI-RS parts are usually configured with different TCI states, UE has to use two corresponding receive beams for the reception of the two parts of CSI-RS in order to calculate the inter-beam interference. There are three options to implement Category 1 as follows

- Category 1-1: two port groups of one CSI-RS resource are configured with two TCI states

  - This scheme is flexible. In one CSI-RS resource set, some CSI-RS resources can be configured with two TCI states for MTRP CSI feedback, and some other CSI-RS resources can be configured with single TCI state for single-TRP CSI feedback.

    - When UE selects a CRI corresponding to a CSI-RS resource with two TCI states, e.g. resource a1 as shown in Figure 2-1, UE has to feed back two sets of CSI, e.g. two RI/PMI/LI and one combined CQI.

    - When UE selects a CRI corresponding to a CSI-RS resource with a single TCI state, e.g. resource a2 or a3 as shown in Figure 2-1, the RI/PMI/LI/CQI feedback overhead is the same as Rel-16.

  - However, it may cause some problem to support 32-ports per each TRP since the current spec does not support 64-ports for a CSI-RS resource.

Figure 2-1 Two CSI-RS port groups

- Category 1-2: two CSI-RS resources within one resource set are associated

  - Some CSI-RS resources can be configured with two TCI states. The association can be implicitly set up if two CSI-RS resources are configured with the same two TCI states.

    - Assuming CSI-RS resource a1 and a3 are configured with the same two TCI states as shown in Figure 2-2, when UE selects a CRI corresponding to a CSI-RS resource a1 with two TCI states, UE has to feed back two sets of CSI for CSI-RS resource a1 and its associated CSI-RS resource a3 respectively. In such case, the interference between CSI-RS resource a1 and a3 should be considered.

    - When UE selects a CRI corresponding to a CSI-RS resource with single TCI state, e.g. resource a2 or a4 as shown in Figure 2-2, the CSI feedback is the same as Rel-16 for single TRP transmission.

  - The current structure of CSI configuration can be reused with support of two TCI states for one CSI-RS resource. The specification impact is marginal.

Figure 2-2 Two CSI-RS resources are associated

- Category 1-3: two CSI-RS resource sets are associated

  - This scheme is similar with Category 1-2. It can be assumed that the CSI-RS resources with the same relative index within the two CSI-RS resource sets are associated. The number of CSI-RS resource sets within one CSI-RS resource setting should be increased.

    - When UE selects a CRI which corresponds to two CSI-RS resources in both resource sets, e.g. CRI=0 corresponds to resource a1 and b1, or CRI=1 corresponds to resource a2 and b2 as shown in Figure 2-3, UE has to feed back two sets of CSI. In such case, the interference between two TRPs should be considered.

    - When UE selects a CRI which corresponds to a CSI-RS resource only contained in one of two resource sets, e.g. CRI=2 corresponds to resource a3 as shown in Figure 2-3, the CSI feedback is the same as Rel-16 for single TRP transmission.

Figure 2-3 Two CSI-RS resource sets are associated

All of the tree options can work for MTRP CSI enhancement. However, Category 1-1 has some problems to support 32 CSI-RS ports for each TRP. Thus, one of Category 1-2 and 1-3 can be considered. Compared with Category 1-3, Category 1-2 does not need to configure multiple CSI-RS resource sets, we slightly prefer Category 1-2.

Proposal 2: In one CSI-RS resource set for channel estimation, two CSI-RS resources configured with the same two TCI states are associated to different TRPs,

- If UE selects a CRI corresponding to a CSI-RS resource with two TCI states, UE will determine CSI based on the interference between the CSI-RS resource and its associated CSI-RS resource.

  - The CSI includes two RI, PMI, LI and one combined CQI.

Figure 2-4 elaborates the details of MTRP CSI computation in FR2, two CSI-RS resources are from different TRPs, where UE uses beam 0 and beam 1 to receive CSI-RS resource 0 and resource 1 respectively. For CSI calculation based on channel measurement in CSI-RS resource 0, CSI-RS resource 1 is for interference measurement. Beam and precoder used for CSI-RS resource 1 should be considered in order to simulate interference from TRP 1 as the inter-layer interference for SDM PDSCH transmission. Likewise, for CSI calculation on CSI-RS resource 1, CSI-RS resource 0 is interference. Beam and precoder used for CSI-RS resource 0 should be considered.

Figure 2-4 Multi-TRP CSI measurement

Therefore, SINR for two CSI-RS resources can be derived as follows.

- For CSI-RS resource 0:

- For CSI-RS resource 1:

wherein bi means beam i, RSi means CSI-RS resource i,  is channel matrix between UE and CSI-RS resource i in the case when UE uses receive beam j, Wi is precoding matrix which will be used by TRP i , and Ii is other interference for CSI-RS resource i. SINRi refers to SINR for CSI-RS resource i.

In order to acquire the optimal W0 and W1, UE needs to calculate channel matrix  and . For instance, the optimal W0 and W1 should target to maximize the sum of throughput of TRP0 and TRP1, or maximize the sum or other function of SINRb0 and SINRb1. For SINR0 calculation, the interference part caused by CSI-RS resource 1 should consider the precoding matrix W1. Also, for SINR1 calculation, the interference part caused by CSI-RS resource 0 should consider the precoding matrix W0.

In summary, UE should calculate interference from the coordinated TRP considering optimal precoder and beam used by the coordinated TRP.

Proposal 3: UE shall calculate interference from the coordinated TRP considering the selected precoder and beam used by the coordinated TRP.

CSI enhancement based on FDD reciprocity

- 3.1 General procedure

This item assumes partial reciprocity for FDD scenario. gNB can derive angular and delay for multiple paths and use it to assist CSI reporting. In general, the procedure of this FDD reciprocity based CSI reporting is depicted in Fig. 3-1.

Fig. 3-1 FDD reciprocity based CSI acquisition

Consider DL is operated in frequency A, and UL is operated in frequency B, three step are considered in this procedure.

- Step 1: gNB estimates angles and delays in frequency A by SRS in frequency B;

- Step 2: gNB uses the angles and delays to precode CSI-RS in frequency A;

- Step 3: UE measures the CSI-RS and report CSI (e.g., selected “ports” and coefficients) for frequency A.

Specifically, in Step 2, each CSI-RS port is precoded by one pair of SD basis and FD basis. For one CSI-RS tone of one CSI-RS port, the transmitted signal is v⋅fs, where v is an SD basis, f is an element of the FD basis, and s is one element in the CSI-RS sequence. In Step 3, for each CSI-RS port, i.e., each pair of SD basis and FD basis, what UE can observe is the beamformed channel by setting the delay to FD basis 0. UE can do average in frequency domain to decompress the noise, interference and residual delay vectors. Then UE can select the best ports and perform a wideband SVD to derive the coefficients.

- 3.2 Codebook structure and reporting details

Based on the discussion above, with DL channel delays and angles estimated in gNB side based on FDD reciprocity, the benefit for the network is that gNB can flexibly design the FD and SD basis vectors in precoded CSI-RS ports without the limitation from a specified codebook. For example, gNB can use eigenvectors as FD or SD vectors, or DFT vectors with higher oversampling than Rel-15/Rel-16 codebooks. Further, as FD vectors are invisible to UEs, gNB can use a smaller frequency-domain granularity, i.e., a larger R value. Hence we propose that Rel-17 codebook structure should be designed based on CSI-RS precoded in both frequency domain and spatial domain.

Proposal 4: Rel-17 codebook structure based on FDD reciprocity is designed based on CSI-RS precoded in both frequency domain and spatial domain.

With FD and SD vectors precoded in CSI-RS, UE needs to report the selected {FD vector, SD vector} pairs to gNB. Two approaches have been identified for this reporting.

- Alt 1: UE selects FD vectors and SD vectors separately. To achieve this, UE needs to know which CSI-RS ports are mapped from the same FD vectors. One example can be shown in Fig. 3-2, where different RBs use different FD basis vectors in CSI-RS precoding, and a same set of P SD basis vectors are precoded in CSI-RS in different RBs. The total number of SD-FD pairs in the entire frequency is 4P. In this case, the codebook structure of precoding the entire frequency domain should contain three matrices as follows,

W=W1W2WfT

where W1 and Wf contain selection vectors to indicate port selection corresponding to SD selection and FD selection, respectively. In each layer of this structure, the precoding vectors of different frequency units form a matrix.

Fig. 3-2 An example of separate SD-FD selection in UE side

- Alt 2: UE selects SD-FD pairs jointly. gNB derives the SD-FD pairs and uses them in CSI-RS precoding. UE selects the best SD-FD pairs and report them in port selection codebook. One example is shown in Fig. 3-3, where each RB contains P SD-FD pairs, and in total 2P SD-FD pairs are mapped to P CSI-RS ports in 2 RBs. In this case, UE just selects and reports SD-FD pairs from the 2P pairs without the necessity of distinguishing SD and FD pairs. The codebook structure of precoding in the entire frequency domain only contains two matrices as following,

W=W1W2

where W1 is a selection matrix and indicates SD-FD pair selection. In each layer of this structure, the precoding vectors of different frequency units form a long vector.

Fig. 3-3 An example of joint SD-FD selection in UE side

Comparing Alt 1 and Alt 2, Alt 1 is essentially a special case of Alt 2. Considering precoding vectors of one layer in the whole frequency domain, the codebook structure of Alt 1 can be rewritten as the following, which has the same structure of Alt 2.

vecW=vecW1W2WfT=Wf⊗W1vecW2=W1'w2

Since Alt 2 is a more general framework which can enable more flexible SD-FD pair selection and CSI-RS precoding in gNB side, we have the following proposal.

Proposal 5: For codebook enhancement in Rel-17 based on FDD reciprocity, support UE selects SD-FD pairs jointly, and the codebook structure is W=W1W2.

The next issue is how to map the X SD-FD pairs selected by gNB into P CSI-RS ports configured for the CSI reporting. Two mapping approaches can be identified.

- Mapping 1: X=P with one to one mapping from SD-FD pairs to CSI-RS ports

- Mapping 2: X can be larger than P, and the mapping from SD-FD pairs to CSI-RS ports can be multiple to one

Examples of the two mapping approaches are depicted in Fig. 3-4. Fig. 3-4(a) is an example of mapping 1, where gNB selects P SD-FD pairs one-to-one mapped to P CSI-RS ports. Fig. 3-4(b) is an example of mapping 2, where gNB selects 2P SD-FD pairs mapped to P CSI-RS ports. P SD-FD pairs out of 2P are one-to-one mapped to P CSI-RS ports in one RB, and the other P SD-FD pairs are one to one mapped to P CSI-RS ports in the next RB. In CSI reporting, UE will report port selection information to select ports from the two RBs.

(a)                            (b)

Fig. 3-4 Examples of mapping SD-FD pairs to P CSI-RS ports

Generally, mapping 2 can allow gNB to select more SD-FD pairs, or reduce CSI-RS overhead. Hence potentially it has better performance than mapping 1. This is part of the aspects in the evaluation of section 3.3.

- 3.3 Evaluation results

We conduct system level simulation to compare the proposed Rel-17 enhancements with Rel-16 eType II port selection codebook. We compare the following codebook schemes.

- Baseline: Rel-16 eType II PS codebook with 8 or 32 CSI-RS ports, and with R=1 or 2.

- The proposed Rel-17 enhancements with the following cases

- Mapping 1, i.e., X pairs/RB, with R=1, 2 or 4

- Mapping 2, i.e., X pairs/2RB, with R=1, 2 or 4

Simulation results are given in Fig. 3-5.

- In each of the Rel-17 curve, X values include 16, 32, 48 and 64, which are similar as 2K0 in Rel-16.

- UE selects maximum X/2 non-zero coefficients per layer, and maximum X non-zero coefficients across all layers.

- The maximum rank is 4 per UE.

- FD vectors in CSI-RS precoding are 16x-oversampling DFT vectors, and SD vectors are CSI-RS precoding is 4x-4x-oversampling 2D-DFT vectors.

- UE-specific CSI-RS overhead is taken into account when calculating throughput. CSI-RS is assumed in a semi-persistent way. When data arrives for a UE, CSI-RS starts to be transmitted every 5 ms, and the CSI-RS transmission terminates when all the data transmission (including initial transmission and retransmission) completes.

- The other simulation assumptions are given in Table 6-2 of Appendix.

(a) FTP-3, 70% load, SU/MU

(b) FTP-3, 20% load, SU

Fig. 3-5 Simulation results to compare the proposed Rel-17 enhancements with Rel-16 eType II

The following observations can be drawn from the simulation results.

Observation:

- With mapping X SD-FD pairs to CSI-RS ports in more than one RBs, the proposed Rel-17 enhancement can achieve about up to 12% gain compared with the best performance of Rel-16 for R=2. If R is increased to 4, the maximum gain is about 14%.

- If the X SD-FD pairs are only mapped to CSI-RS ports in one RB, there is no gain for the proposed Rel-17 enhancement under the restriction of up to 32 ports per CSI-RS resource.

- To allow MU scheduling can achieve larger gain than SU-only scheduling.

Based on the above observation, we have the following proposal.

Proposal 6: Support Rel-17 port selection codebook enhancement with mapping X>PCSI-RS SD-FD pairs to CSI-RS ports in more than one RB, where PCSI-RS is the number of configured CSI-RS ports.

Conclusions

In this contribution, we provide our views to enhance CSI measurement and reporting for MTRP and FDD reciprocity.

For MTRP CSI enhancement:

Proposal 1: Support Category 1, i.e.

- For a reporting setting CSI-ReportConfig, more than one CSI-RS port groups in a resource or resources or resource sets are associated to different TRPs/TCI states,

  - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/UE-selected channel and interference hypotheses across TRPs /TCI states

  - and then report one or more CSIs within a single CSI report.

Proposal 2: In one CSI-RS resource set for channel estimation, two CSI-RS resources configured with the same two TCI states are associated to different TRPs,

- If UE selects a CRI corresponding to a CSI-RS resource with two TCI states, UE will determine CSI based on the interference between the CSI-RS resource and its associated CSI-RS resource.

  - The CSI includes two RI, PMI, LI and one combined CQI.

Proposal 3: UE shall calculate interference from the coordinated TRP considering the selected precoder and beam used by the coordinated TRP.

For CSI enhancement based on FDD reciprocity:

Proposal 4: Rel-17 codebook structure based on FDD reciprocity is designed based on CSI-RS precoded in both frequency domain and spatial domain.

Proposal 5: For codebook enhancement in Rel-17 based on FDD reciprocity, support UE selects SD-FD pairs jointly, and the codebook structure is W=W1W2.

Observation:

- With mapping X SD-FD pairs to CSI-RS ports in more than one RBs, the proposed Rel-17 enhancement can achieve about up to 12% gain compared with the best performance of Rel-16 for R=2. If R is increased to 4, the maximum gain is about 14%.

- If the X SD-FD pairs are only mapped to CSI-RS ports in one RB, there is no gain for the proposed Rel-17 enhancement under the restriction of up to 32 ports per CSI-RS resource.

- To allow MU scheduling can achieve larger gain than SU-only scheduling.

Proposal 6: Support Rel-17 port selection codebook enhancement with mapping X>PCSI-RS SD-FD pairs to CSI-RS ports in more than one RB, where PCSI-RS is the number of configured CSI-RS ports.

References

- RP-193133, New WID: Further enhancements on MIMO for NR, Samsung,

- Offline email discussion on FeMIMO evaluation methodology: Item 4

- R1-1704425, Enhancement to CSI feedback, ZTE, ZTE Microelectronics

Appendix

Table 6-1 SLS assumptions for multi-TRP performance evaluation

| Parameters | Dense urban (Macro Only) | Indoor hotspot |
| --- | --- | --- |
| Center frequency | 2GHz | 4GHz |
| TP antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) | 2 ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) |
| UE antenna configuration | 4Rx Port: (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ | 4Rx Port: (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| UE distribution | 80% of users are outdoors and 20% of users are indoors | 100% of users are indoor: 3 km/h UE-speed |
| BS Tx power | 46dBm | 21dBm |
| Simulation bandwidth and SCS | 20MHz BW and 30kHz SCS |  |
| Backhaul | Ideal backhaul, Non-ideal backhaul |  |
| Coordination assumptions | 2 coordinated TRPs |  |
| Traffic model | FTP traffic model 3 |  |
| Scheduler | PF |  |
| HARQ scheme | CC with up to 4 retransmissions |  |
| CSI feedback | Type I codebook, 5 slots period Multiple CSI feedback with or without interference between two coordinating TRPs |  |
| UE receiver noise figure | 9dB |  |
| Transmission scheme | SU-MIMO with rank adaptation |  |

Table 6-2 SLS assumptions for CSI based on FDD reciprocity

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban |
| Frequency Range |  | 2GHz with duplexing gap of 200MHz between DL and UL, |
| Inter-BS distance |  | 200m |
| Channel model |  | The reciprocity model of DL/UL channel is based on Section 5.3 of TR 36.897 |
| Antenna setup and port layouts at gNB |  | (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 44 dBm for 20 MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15 kHz |
| Simulation bandwidth |  | 20 MHz (104 RBs) for 15 kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | For 70% RU, SU/MU-MIMO with rank adaptation is assumed. For 20% RU, SU-MIMO with rank adaptation is assumed. |
| MIMO layers |  | The maximum number of MU layers is 12. The maximum number of SU layers is 4. |
| CSI feedback |  | CSI feedback periodicity: 5 ms, Scheduling delay: 4 ms |
| Overhead |  | 2 OFDM symbols for PDCCH，type 2 for DMRS(24 REs/PRB/slot) CSI-RS overhead is taken into account as specified in the main content |
| Traffic model |  | FTP 3 with packet size 0.5 Mbytes |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| SRS modeling for UL channel estimation |  | 5ms SRS periodicity SRS error model in Table A.1-2 in 36.897 with Δ=9 dB. |
| FDD DL/UL calibration error model at gNB |  | With amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. |