3GPP TSG RAN WG1 #103-e        	                                                                                 R1-2007796

e-Meeting, October 26th – November 13th, 2020

Title:	Draft reply LS to RAN3 on full slot formats support in TDD UL-DL configuration

Response to: 	R1-2007513(R3-205794)

Release:	Release 16

Work Item:	NR_CLI_RIM

Source:	CATT

To:	RAN3

CC:

Contact Person:

Name:	Lei Wang

E-mail Address:	wanglei@catt.cn

1. Overall Description:

RAN1 would like to thank RAN3 for the LS R3-205794 on full slot formats support in TDD UL-DL configuration.

In R3-205794, RAN3 asked RAN1 to provide answer to the following question:

Question to RAN1:

Whether all the slot formats defined in TS 38.213 shall be supported in the intended TDD UL-DL configuration exchange?

The slot formats defined in TS38.213 are defined to support full flexibility of TDD UL-DL configuration in NR system to carter to the various traffic and scheduling scenarios. For slot formats 46-53 (Table 11.1.1-1 of TS 38.213), there are two switching points within a slot in order to satisfy the requirement of low latency traffic, i.e. URLLC, wherein the self-contained scheduling can be achieved. For slot formats 54 and 55 (Table 11.1.1-1 of TS 38.213), they are defined for special cases, i.e. relay and latency reduction (considering SSB configuration in the nearby cell) [1]. In order to guarantee the flexibility and feasibility of TDD UL-DL configuration, all the slot formats defined in TS38.213 should be supported.

2   Actions:

To RAN3

ACTION: RAN1 respectfully asks RAN3 to take the above information into account.

3   Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #104-e                                       25 Jan. – 05 Feb. 2021                       e-Meeting

TSG-RAN WG1 Meeting #104bis-e                                   12 Apr. – 20 Apr. 2021                       e-Meeting

4 References:

R1-1800069, ‘Summary of remaining issues on group-common PDCCH’, Huawei, HiSilicon