3GPP TSG RAN WG1 #103-e                                              R1-2007844

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.15.1

Source:	CATT

Title:	         Application scenarios discussion on NB-IoT/eMTC

Document for:	Discussion and Decision

Introduction

Based on SID RP-193235[1], the first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

According to this objective, we anlayzed and prosented our views about potential application scenarios for eMTC and NB-IoT over NTN.

Discussion

  - Satellite related parameters configuration

In general, MTC deployment is targeted to provide wide coverage, especially for non-terrestrial network. Then the minimal beam footprint size should not be less than 100km. Regarding GEO and LEO deployment option, typically both scenarios should be supported, where for GEO case, the beam coverage is larger and beam mobility issue can be avoided, but for LEO, link budget is more optimistic. For sake of deployment flexibility, earth fixed beam and moving beam can be considered both. In sum, table 1 summarized our initial thinking to satellite parameters configuration.

Table 1 satellite parameters configuration for IoT NTN

|  | GEO | LEO |
| --- | --- | --- |
| Satellite altitude | 35786km | 1200/600 km |
| Beam diameter | >300km | >100km |
| Beam type | Earth fixed beam | Steerable beam and moving beam |
| Frequency reuse mode for adjacent beam/cells | FDM | FDM |
| Antenna port | 1 | 1 |

Proposal 1: Take into account the table 1 on satellite parameters configuration for NB-IoT/eMTC over NTN.

  - UE parameters configuration

For NTN IoT UE, power consumption and UE complexity should be controllable. In order to reduce power consumption and UE cost, only one anchor carrier can be assumed for satellite IoT device. Table 2 summarized our initial thinking on UE parameters configuration.

Table 2 UE parameters configuration for IoT NTN

|  | eMTC | NB-IoT |
| --- | --- | --- |
| Transission power | 23/20 dBm | 23/20 dBm |
| Duplex mode | FD-FDD HD-FDD | HD-FDD |
| TX/RX Antenna port | 1 | 1 |
| Anetnna type | Linear polarization Omnidirectional antenna | Linear polarization Omnidirectional antenna |
| Bandwidth | 1.4Mhz | 200Khz Only one anchor carrier is considered. |

Proposal 2: Take into account the table 2 on UE parameters configuration for NB-IoT/eMTC over NTN.

  - Targeted scenarios

For satellite IoT case, one obvious advantage over terrestrial IoT is wide coverage, which may lead to less network cost and simplified deployment. In general, Satellite IoT may provide more diverse traffic services due to more wide deployment, but the transmission latency will be increased. Of course, in most of case, it is not critical. If keeping same transmission power for IoT device, link budget will be determined by satellite parameters configuration. Since in LEO case, the path loss is better compared to GEO case and also satellite moving is challenaging, so quick compeletion of data packet is preferred, and finally, better link budget of LEO case is expected to speed up data packet delievery. Regarding frequency band, L, S or C band can be considered. From path loss perspective, C band is only applicable for LEO case. For UE speed, in general GEO IoT will provide larger coverage, e.g high speed train monitoring. So the UE speed can be targetted to up to 300 km/h for GEO. In sum, table 3 listed a few parameters related to application scenarios.

Table 3 Some scenario related parameters configuration for IoT NTN

|  | GEO | LEO |
| --- | --- | --- |
| UE speed | Up to 300km/h | Up to 120km/h |
| Link budget target | Can be less than -10dB | Not less than -5dB |
| Frequency band | S or L band | C or S band |

Proposal 3: Take into account the table 3 on scenario configuration for NB-IoT/eMTC over NTN.

Conclusion

In this contribution, we analzyed typical scenarions and related parameters for NB-IoT and eMTC over satellite. In sum, three tables are shown as follows:

Proposal 1: Take into account the table 1 on satellite parameters configuration for NB-IoT/eMTC over NTN.

Proposal 2: Take into account the table 2 on UE parameters configuration for NB-IoT/eMTC over NTN.

Proposal 3: Take into account the table 3 on scenario configuration for NB-IoT/eMTC over NTN.

Table 1 Satellite parameters configuration for IoT NTN

|  | GEO | LEO |
| --- | --- | --- |
| Satellite altitude | 35786km | 1200/600 km |
| Beam diameter | >300km | >100km |
| Beam type | Earth fixed beam | Steerable beam and moving beam |
| Frequency reuse mode for adjacent beam/cells | FDM | FDM |
| Antenna port | 1 | 1 |

Table 2 UE parameters configuration for IoT NTN

|  | eMTC | NB-IoT |
| --- | --- | --- |
| Transission power | 23/20 dBm | 23/20 dBm |
| Duplex mode | FD-FDD HD-FDD | HD-FDD |
| TX/RX Antenna port | 1 | 1 |
| Anetnna type | Linear polarization Omnidirectional antenna | Linear polarization Omnidirectional antenna |
| Bandwidth | 1.4Mhz | 200Khz Only one anchor carrier is considered. |

Table 3 Some scenario related parameters configuration for IoT NTN

|  | GEO | LEO |
| --- | --- | --- |
| UE speed | Up to 300km/h | Up to 120km/h |
| Link budget target | Can be less than -10dB | Not less than -5dB |
| Frequency band | S or L band | C or S band |

References

- RP-193235, New Study WID on NB-IoT/eTMC support for NTN, MediaTek