3GPP TSG RAN WG1 Meeting #103-e		                     	                  R1-2007858

e-Meeting, October 26th – November 13th, 2020

Source:	CATT

Title:	Remaining issues on additional scenarios for evaluation of NR Positioning Enhancements

Agenda item:	8.5.1

Document for:	Discussion and Decision

Introduction

In RAN1#102-e meeting, the following agreements were achieved on additional scenarios for evaluation of NR Positioning Enhancements [1]. In this contribution, we discuss some remaining issues on additional scenarios for evaluation of NR Positioning Enhancements, including Rel-17 target positioning requirements and definitions of PRS/SRS resource utilization.

Agreement: Physical Layer Latency Start and End times are defined as follows: Method; Start; End. UE assisted DL-only & DL-ECID & Multi-RTT; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message; Successful decoding of the PUSCH carrying the LPP Provide Location Information message. UL-only method & UL ECID & Multi-RTT; Reception by the gNB of the NRPPa measurement request message; The transmission by the gNB of the NRPPa measurement response message. UE-based; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives.; Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE. Conclusion: RAN1 will not define additional optional values for UE and gNB antenna heights for evaluations. Conclusion: RAN1 will not define additional details for the optional UE antenna configuration of 4 UE panels for evaluations. Conclusion: For power consumption evaluation, it is up to each company to detail their methodology (including power model) for evaluation. Agreement: Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. Conclusion: For UE mobility, the details of the optional mobility model are left to companies. Agreement: PRS/SRS resource utilization is the metric used to evaluate network efficiency - FFS: what is included in resource utilization, e.g. PRS/SRS/MG configurations, beam sweeping assumptions Agreement: For the absolute time of arrival modelling in IOO, UMa, Umi, companies may provide the details of their model, if any.

Discussion

Rel-17 target positioning requirements

In RAN1#101-e meeting, the following agreements were achieved on Rel-17 target positioning requirements for commercial use cases and IIoT use cases.

Agreement: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for [90%] of UEs - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs - End-to-end latency for position estimation of UE (< [100 ms]) - FFS: Physical layer latency for position estimation of UE (< [10 ms]) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X m) for [90%] of UEs - X = [0.2 or 0.5] m - Vertical position accuracy (< Y m) for [90%] of UEs - Y = [0.2 or 1] m - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms]) - FFS: Physical layer latency for position estimation of UE (< [10ms]) - Note: Target positioning requirements may not necessarily be reached for all scenarios

In RAN1#102-e meeting, there was very dense discussion for the issue of Rel-17 target positioning requirements; however, no agreements are achieved during the email discussion. The latest Feature lead proposal [2] as follows,

Feature lead Proposal 1d: In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for [X1] of UEs - X1 =85% - Vertical position accuracy (< 3 m) for [X2] of UEs - X2 =85% - End-to-end latency for position estimation of UE (< 100 ms) - Physical layer latency for position estimation of UE (< X3) - X3 =[10ms] OR (to be determined based on the definition of physical layer latency and higher layer latency consideration) - Note: X3 may have multiple value for corresponding to different service level. In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X4) for 90% of UEs - X4 = 0.2m, or 0.5m - FFS: accuracy correspond to first fix or it can be the one in the stable tracking period - Vertical position accuracy (< X5) for 90% of UEs - X5 = 0.2m or 1m - FFS: accuracy correspond to first fix or it can be the one in the stable tracking period - End-to-end latency for position estimation of UE (< X6) - X6 = [10ms] or [100ms] - Physical layer latency for position estimation of UE (< X7) - X7 =[10ms] (to be determined based on the definition of physical layer latency and higher layer latency consideration) Note1: companies to report values for X1-X7 (alternatively, the service level, if agreed) when presenting potential enhancement results Note2: X6, X7 may have multiple values for corresponding to different service level. Note3: Depending on the End-To-End latency, X7 target may need to be set to a smaller value than [10 ms]

The above FL proposal introduced the concept of service level into the definition of Rel-17 target positioning requirements and proposed the targets may have multiple values for corresponding to different service levels. In our point of view, such a proposal seems to make the issue more complicated and difficult to converge. We prefer to move forward based on the agreements in RAN1#101-e meeting, and only confirm the TBD values in the brackets. Moreover, in all scenarios, there is only one target value independent of service levels. In addition, as we agreed in RAN1#101-e meeting, target positioning requirements may not necessarily be reached for all scenarios.

Observation: The introduction of service level makes the definition of Rel-17 target positioning requirements more complicated and difficult to converge.

As shown in above, we had reached the agreements on framework of Rel-17 target requirements in RAN1#101-e meeting, but most of the numbers are still in the brackets. We prefer the following numbers marked on GREEN background as Rel-17 target positioning requirements:

In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for X1 of UEs - X1 = 85% - Vertical position accuracy (< 3 m) for X2 of UEs - X2 = 85% - End-to-end latency for position estimation of UE (< X3) - X3 = 100ms - Physical layer latency for position estimation of UE (< X4) - X4 = 10ms In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X5) for 90% of UEs - X5 = 0.2m - Vertical position accuracy (< X6) for 90% of UEs - X6 = 1m - End-to-end latency for position estimation of UE (< X7) - X7 = 100ms - Physical layer latency for position estimation of UE (< X8) - X8 = 10ms

As shown in above, for vertical position accuracy, we would like to select relatively loose target requirements, as the vertical positioning needs the deployment of multi-layer base stations in the vertical direction, and the number of base stations that the UE can used for positioning in the vertical direction is less than that in the horizontal direction. It should be pointed out that synchronization error among gNBs will degrade the performance of UE positioning. About end-to-end latency for position estimation of UE, we prefer 100ms as the target requirement, as it is very difficult for current network to achieve 10ms or 20ms E2E latency, even if it is optimized. About physical layer latency for position estimation of UE, we prefer 10ms as the target requirement in order to reduce the physical layer latency as much as possible.

Proposal 1: We prefer the following numbers for Rel-17 target positioning requirements:

In Rel-17 target positioning requirements for commercial use cases are defined as follows:

- Horizontal position accuracy (< 1 m) for X1 of UEs

  - X1 = 85%

- Vertical position accuracy (< 3 m) for X2 of UEs

  - X2 = 85%

- End-to-end latency for position estimation of UE (< X3)

  - X3 = 100ms

- Physical layer latency for position estimation of UE (< X4)

  - X4 = 10ms

In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

- Horizontal position accuracy (< X5) for 90% of UEs

  - X5 = 0.2m

- Vertical position accuracy (< X6) for 90% of UEs

  - X6 = 1m

- End-to-end latency for position estimation of UE (< X7)

  - X7 = 100ms

- Physical layer latency for position estimation of UE (< X8)

  - X8 = 10ms

Definition of PRS/SRS resource utilization

In RAN1#102-e meeting, the following agreements were achieved on the PRS/SRS resource utilization. [1]

Agreement: PRS/SRS resource utilization is the metric used to evaluate network efficiency - FFS: what is included in resource utilization, e.g. PRS/SRS/MG configurations, beam sweeping assumptions

In this section, we discuss the definition of PRS/SRS resource utilization according to the configuration parameters of PRS/SRS resource.

    - Definition of DL-PRS resource utilization

Assume gNB x has M TRPs (the index of TRP is j, and j=1~M-1) which are configured to transmit DL-PRS, and each TRP is configured with N DL-PRS resource sets (the index of DL-PRS resource set is i, i=1~N-1). In order to avoid the interference among these DL-PRS resource sets, either the time-domain resources or the frequency-domain resources are orthogonal among these DL-PRS resource sets belongs to the same gNB.

We use DL_PRS_RU to denote DL-PRS resource utilization, then the DL-PRS resource utilization of gNB x (DL_PRS_RUgNB x) can be derived as follows,

DL_PRS_RUPRS resource set i = DL_PRS_RUPRS resource set i, time-domain×DL_PRS_RUPRS resource set i, frequency-domain

DL_PRS_RUPRS resource set i, time-domain= NumOfSymbolsPRS resourcePeriodicityPRS resource set i×NumOfSymbolsPer millisecond×NumofPRSResourcesPRS resource set i×RepetitionFactorPRS resource set i

DL_PRS_RUPRS resource set i, frequency-domain=BandWidthPRS resource set iBandWidthSystem×1CombSizeNPRS resource set i

DL_PRS_RUTRP j = i=1NDL_PRS_RUPRS resource set i

DL_PRS_RUgNB x = j=1MDL_PRS_RUTRP j

    - Definition of SRS-Pos resource utilization

Similar to Section 2.2.1 above, Assume gNB x serves M UEs (the index of UE is j, and j=1~M-1) which are configured to transmit SRS-Pos, and each UE is configured with N SRS-Pos resource sets (the index of SRS-Pos resource set is i, i=1~N-1). In order to avoid the interference among these SRS-Pos resource sets, either the time-domain resources or the frequency-domain resources are orthogonal among these SRS-Pos resource sets belongs to the same gNB.

In addition to periodic SRS-Pos, semi-persistent or aperiodic SRS-Pos may also be configured. For the sake of simplification, only the resource utilization of periodic SRS-Pos is analyzed here.

We use SRS_Pos_RU to denote SRS-Pos resource utilization, then the SRS-Pos resource utilization of gNB x (SRS_Pos_RUgNB x) can be derived as follows,

SRS_Pos_RUSRS resource set i = SRS_Pos_RUSRS resource set i, time-domain×SRS_Pos_RUSRS resource set i, frequency-domain

SRS_Pos_RUSRS resource set i, time-domain= NumOfSymbolsSRS resourcePeriodicitySRS resource set i×NumOfSymbolsPer millisecond×NumofSRSResourcesSRS resource set i

SRS_Pos_RUSRS resource set i, frequency-domain=BandWidthSRS resource set iBandWidthSystem×1CombSizeNSRS resource set i

SRS_Pos_RUUE j = i=1NSRS_Pos_RUSRS resource set i

SRS_Pos_RUgNB x = j=1MSRS_Pos_RUUE j

Proposal 2: The PRS/SRS resource utilization can be defined according to the configuration parameters of PRS/SRS resource as shown in the section 2.2.1and 2.2.2.

Conclusions

In this contribution, we discuss several remaining issues on additional scenarios for evaluation of NR Positioning Enhancements. Based on the discussion, we propose:

Proposal 1: We prefer the following numbers for Rel-17 target positioning requirements:

In Rel-17 target positioning requirements for commercial use cases are defined as follows:

- Horizontal position accuracy (< 1 m) for X1 of UEs

  - X1 = 85%

- Vertical position accuracy (< 3 m) for X2 of UEs

  - X2 = 85%

- End-to-end latency for position estimation of UE (< X3)

  - X3 = 100ms

- Physical layer latency for position estimation of UE (< X4)

  - X4 = 10ms

In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

- Horizontal position accuracy (< X5) for 90% of UEs

  - X5 = 0.2m

- Vertical position accuracy (< X6) for 90% of UEs

  - X6 = 1m

- End-to-end latency for position estimation of UE (< X7)

  - X7 = 100ms

- Physical layer latency for position estimation of UE (< X8)

  - X8 = 10ms

Proposal 2: The PRS/SRS resource utilization can be defined according to the configuration parameters of PRS/SRS resource as shown in the section 2.2.1and 2.2.2.

References

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #102-e.

- R1-2007367, FL summary for additional scenarios for evaluation of NR positioning enhancements, Moderator (Ericsson)