3GPP TSG RAN WG1 Meeting #103   	  		                                              R1-2007859

e-meeting, October 26th – November 13th, 2020

Source:	CATT

Title:	Discussion of evaluation of NR positioning performance

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

At RAN1#102e meeting, following agreements on evaluation of NR positioning performance were approved [1].

Agreement:

Text proposal for LS to RAN WG2 and CC SA WG2 and RAN WG3 for analysis of latency of NR positioning protocols defined in Rel.16:

RAN1 evaluates physical layer latency and its potential reduction for NR Rel-17 positioning solutions. In order to evaluate End-To-End latency of NR positioning solutions the input from RAN2 is needed on latency components of NR/NG-RAN/5GC higher layer positioning protocols. RAN1 respectfully asks if RAN2 can provide a list of latency components with corresponding range of values for the existing and any potential enhanced NR positioning solutions, keeping in mind the End-To-End latency described as desired in the study item description (RP-200928)

Agreement:

Physical layer latency for DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted approaches are separately studied

Agreement:

Capture the following in TR as an observation:

- Performance analysis of baseline I-IoT InF scenarios shows that InF-SH scenario is characterized by high probability of LOS links. In InF-DH the probability of LOS links is reduced substantially while probability of NLOS links is increased accordingly.

Agreement:

- FFS whether Rel.16 granularity of timing measurement reports is enough to avoid degradation in I-IoT scenarios and meet positioning requirements

Agreement:

- At least the following information is provided for positioning physical layer latency analysis:

  - Source initiating request for positioning measurements/location for a given UE (UE, Network)

  - Destination awaiting for positioning measurements/location for a given UE (UE, Network)

  - Start and end triggers/events for physical layer latency evaluation

    - For Rel.16 solutions, it is based on specification for each solution

  - Initial and final RRC State of positioned UE (RRC IDLE, INACTIVE, CONNECTED) at the start and end time for the physical layer latency evaluation

  - Positioning

    - technique (enumeration): (1) DL-TDOA, (2) DL AoD, (3) UL-TDoA, (4) UL-AoA, (5) Multi-RTT, (6) E-CID

    - type: DL, UL, DL+UL

    - mode: UE-based, UE-assisted

  - Latency component w/ value range and description, including information on any parallel (simultaneous) components

  - Total latency value

- Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest one:

| Source [UE, NW]/Destination [UE, NW] Positioning technique [DL-TDOA, E-CID, …], type [DL, UL, DL+UL], mode [UE-A, UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  |
| Name of component 1 |  |  |
| Name of component 2 |  |  |
| Name of last component |  |  |
| End trigger |  |  |
| Total values |  |  |

Conclusion:

- Evaluations show that high probability of NLOS links and propagation delay offset imposed by NLOS links may cause performance degradation of positioning accuracy, that was especially observed in InF-DH scenario

- Initial evaluations have also shown that under certain ideal assumptions (e.g., synchronization error, Rx/Tx calibration error) the effective LOS/NLOS classification/detection, outlier determination/rejection techniques may be beneficial to improve NR positioning accuracy

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Conclusion:

- It is observed that calibration errors of UE/gNB Tx/Rx timing may negatively impact accuracy of timing-based methods of Rel.16 positioning solutions when precise UE positioning is targeted.

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Conclusion:

- Evaluations show that network synchronization errors may cause accuracy degradation of the DL-TDOA or UL-TDOA Rel-16 positioning solutions

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

This contribution discusses the evaluation of NR positioning performance with simulation results in InF and IOO scenarios.

Simulation evaluation of NR positioning performance

This section discusses the simulation evaluation results of NR positioning performance in InF and IOO scenarios. Simulation assumptions are based on the agreements on baseline InF and IOO scenarios in RAN1#101-e and RAN1#102-e. The evaluation results are provided according to the template discussed in [102-e-Post-NR-ePos-01] “Email discussion/approval on collection of simulation results” [2].

# 8	Performance evaluations for R17 performance targets

## 8.1	Performance analysis of Rel-16 positioning solutions

### 8.1.1	Positioning accuracy analysis

#### 8.1.1.1	Results from CATT[R1-2007859]

##### 8.1.1.1.1	Description of evaluation scenarios

In this section, evaluation scenarios and parameters for positioning accuracy analysis of Rel-16 positioning solution are given from Table 8.1.1.1.1-1a to Table 8.1.1.1.1-1f.

- Case IDs: 1 to 36.

- Scenarios: InF-SH-2D, InF-DH-2D, InF-SH-3D, InF-DH-3D, IOO.

- Frequency Bands: FR1, FR2.

- Positioning Techniques: R.16 DL-TDOA, R.16 UL-TDOA, R.16 Multi-RTT, R.16 DL-AOD, R.16 UL-TDOA +R.16 UL-AOA.

Note 1：

- InF-SH-2D: 300x150 m, Horizontal: gNB antenna height (8m), UE antenna height (1.5m), low clutter density {20%, 2m, 10m}.

- InF-SH-3D: 300x150 m, Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m], low clutter density {20%, 2m, 10m}

- InF-DH-2D: 120x60 m, Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter density #1{40%, 2m, 2m}

- InF-DH-3D: 120x60 m, Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter density #2{40%, 3m, 5m}.

Table 8.1.1.1.1-1a: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR1]

| Parameter | [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | [Case 2], [InF- SH-2D], [FR1], [UL-TDOA] | [Case 3], [InF- SH-2D], [FR1], [Multi-RTT] | [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | [Case 5], [InF- SH-3D], [FR1], [UL-TDOA] | [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-SH-2D | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | No | No | No | No | No | No | No |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No |
| Additional Notes, if any | No | No | No | No | No | No | No |

Table 8.1.1.1.1-1b: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR1]

| Parameter | [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | [Case 9], [InF- DH-2D], [FR1], [UL-TDOA] | [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-DH-2D | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | No | No | No | No | No | No | No |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions | No | No | No | No | No | No | No |
| Additional Notes, if any | No | No | No | No | No | No | No |

Table 8.1.1.1.1-1c: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR1]

| Parameter | [Case 15], [IOO], [FR1], [DL-TDOA] | [Case 16], [IOO], [FR1], [UL-TDOA] | [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 18], [IOO], [FR1], [Multi-RTT] | [Case 19], [IOO], [FR1], [DL-TDOA] | [Case 20], [IOO], [FR1], [UL-TDOA] | [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 22], [IOO], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | IOO | IOO | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | No | No | No | No | Yes | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No | No |
| Additional Notes, if any | No | No | No | No | No | No | No | No |

Table 8.1.1.1.1-1d: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR2]

| Parameter | [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | [Case 24], [InF- SH-2D], [FR2], [UL-TDOA] | [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | No | No | No | No |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any | No | No | No | No |

Table 8.1.1.1.1-1e: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR2]

| Parameter | [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | No | No | No | No |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any | No | No | No | No |

Table 8.1.1.1.1-1f: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR2]

| Parameter | [Case 31], [IOO], [FR2], [DL-TDOA] | [Case 32], [IOO], [FR2], [UL-TDOA] | [Case 33], [IOO], [FR2], [Multi-RTT] | [Case 34], [IOO], [FR2], [DL-TDOA] | [Case 35], [IOO], [FR2], [UL-TDOA] | [Case 36], [IOO], [FR2], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | No | No | No | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No |
| Additional Notes, if any | No | No | No | No | No | No |

##### 8.1.1.1.2	Positioning accuracy evaluation results

Evaluation results are presented from Table 8.1.1.1.2-1 to Table 8.1.1.1.2-2.

Table 8.1.1.1.2-1: Rel.16 NR positioning - horizontal location error results from CATT[R1-2007859]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | Convex UEs | 0.0418 | 0.0603 | 0.0836 | 0.1650 |
|  | (Optional) All UEs | 0.0996 | 0.104 | 0.1196 | 0.3159 |
| [Case 2], [InF-SH-2D], [FR1], [UL-TDOA] | Convex UEs | 0.0430 | 0.0621 | 0.0823 | 0.1551 |
| [Case 3], [InF-SH-2D], [FR1], [Multi-RTT] | Convex UEs | 0.03969 | 0.05828 | 0.08561 | 0.1650 |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.079 | 0.1083 | 0.1379 | 0.2045 |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.0754 | 0.1143 | 0.1728 | 0.2574 |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.033 | 0.054 | 0.1428 | 0.2677 |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.08 | 0.108 | 0.153 | 0.2540 |
| [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | Convex UEs | 0.0599 | 0.088 | 0.1185 | 0.1693 |
| [Case 9], [InF-DH-2D], [FR1], [UL-TDOA] | Convex UEs | 0.0488 | 0.0695 | 0.0856 | 0.1184 |
| [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | Convex UEs | 0.04744 | 0.06356 | 0.08704 | 0.1237 |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.1201 | 0.2499 | 0.4116 | 0.7089 |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.108 | 0.155 | 0.3692 | 0.6937 |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.044 | 0.062 | 0.118 | 0.151 |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.115 | 0.179 | 0.391 | 0.692 |
| [Case 15], [IOO], [FR1], [DL-TDOA] | Convex UEs | 0.079 | 0.1028 | 0.1338 | 0.2288 |
| [Case 16], [IOO], [FR1], [UL-TDOA] | Convex UEs | 0.0646 | 0.0984 | 0.127 | 0.1836 |
| [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.013 | 0.031 | 0.049 | 0.1219 |
| [Case 18], [IOO], [FR1], [Multi-RTT] | Convex UEs | 0.073 | 0.1024 | 0.1331 | 0.283 |
| [Case 19], [IOO], [FR1], [DL-TDOA] | Convex UEs | 7.4192 | 13.0891 | 18.592 | 32.4509 |
| [Case 20], [IOO], [FR1], [UL-TDOA] | Convex UEs | 6.6813 | 12.6521 | 18.281 | 32.0927 |
| [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 2.4591 | 3.7312 | 5.603 | 9.2356 |
| [Case 22], [IOO], [FR1], [Multi-RTT] | Convex UEs | 0.3828 | 0.4918 | 0.6728 | 1.3668 |
| [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | Convex UEs | 0.0136 | 0.0223 | 0.028 | 0.0372 |
| [Case 24], [InF-SH-2D], [FR2], [UL-TDOA] | Convex UEs | 0.022 | 0.0296 | 0.041 | 0.0538 |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.025 | 0.0324 | 0.046 | 0.0789 |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.0223 | 0.0276 | 0.0383 | 0.0817 |
| [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | Convex UEs | 0.0178 | 0.023 | 0.0277 | 0.0388 |
| [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | Convex UEs | 0.0204 | 0.0296 | 0.0416 | 0.0553 |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.0314 | 0.089 | 0.2996 | 0.7033 |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.0282 | 0.0561 | 0.2827 | 0.6848 |
| [Case 31], [IOO], [FR2], [DL-TDOA] | Convex UEs | 0.0166 | 0.0225 | 0.0301 | 0.0406 |
| [Case 32], [IOO], [FR2], [UL-TDOA] | Convex UEs | 0.011 | 0.0179 | 0.0243 | 0.0397 |
| [Case 33], [IOO], [FR2], [Multi-RTT] | Convex UEs | 0.01636 | 0.022 | 0.03 | 0.0402 |
| [Case 34], [IOO], [FR2], [DL-TDOA] | Convex UEs | 7.3189 | 13.0765 | 18.572 | 32.3809 |
| [Case 35], [IOO], [FR2], [UL-TDOA] | Convex UEs | 6.6798 | 12.5968 | 18.2581 | 32.0887 |
| [Case 36], [IOO], [FR2], [Multi-RTT] | Convex UEs | 0.3475 | 0.4718 | 0.6565 | 1.2681 |

Table 8.1.1.1.2-2: Rel.16 NR positioning - altitude location error results from CATT[R1-2007859]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.608 | 0.9636 | 1.3383 | 1.8954 |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.608 | 1.036 | 1.3383 | 2.0599 |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.8572 | 1.154 | 1.516 | 2.256 |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.639 | 0.917 | 1.60 | 2.18 |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.461 | 0.8138 | 1.3474 | 2.9917 |
| [Case 12], [InF- DH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.4167 | 0.6408 | 1.0091 | 2.049 |
| [Case 13], [InF- DH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.334 | 0.486 | 0.593 | 0.633 |
| [Case 14], [InF- DH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.507 | 0.723 | 1.59 | 2.82 |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.1581 | 0.277 | 0.3961 | 0.6283 |
| [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.2018 | 0.3084 | 0.4611 | 0.8304 |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.1337 | 0.2703 | 0.6779 | 3.0578 |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.116 | 0.2358 | 0.5051 | 3.1267 |

##### 8.1.1.1.3	Observations on Rel-16 NR positioning accuracy

The summaries of horizontal and vertical accuracy with Rel-16 NR positioning techniques (R.16 DL-TDOA, R.16 UL-TDOA, R.16 Multi-RTT, R.16 DL-AOD, R.16 UL-TDOA +R.16 UL-AOA) are given in Table 8.1.1.1.3-1 and Table 8.1.1.1.3-2, respectively. For horizontal accuracy performance of commercial use case (IOO), the requirements [1]m @[90]% are met by CASE {15~18, 31~33}, but not met by CASE {19~22, 34~36}. For horizontal accuracy performance of IIoT use cases (InF-SH and InF-DH), the requirements [0.2]m @[90]% are met by CASE {1~3,8~10, 13, 23~28} , but not met by CASE {4~7, 11, 12, 14, 29, 30}. For horizontal accuracy performance of IIoT use cases (InF-SH and InF-DH), requirements [0.5]m @[90]% are met by CASE {1~10, 13, 23~28} , but not met by CASE {11, 12, 14, 29, 30}. For vertical accuracy performance of IIoT use cases (InF-SH-3D and InF-DH-3D), the requirements of [0.2]m @[90]% are not met by CASE {4~7, 11~14, 25,26,29,30}. For vertical accuracy performance of IIoT use cases (InF-SH-3D and InF-DH-3D), the requirements of [1]m @[90]% are met by CASE {13, 25, 26} , but not met by CASE {4~7, 11, 12, 14, 29,30}.

Note that network synchronization error and UE/gNB Tx/Rx calibration error are included in CASE {19~22,34~36}, but not simulated in CASE {1~18, 23~33}. The following observations can be get:

Observation 1: Without considering the network synchronization error and UE/gNB Tx/Rx calibration error, Rel-16 NR positioning techniques can meet the commercial use case (IOO) horizontal accuracy performance requirements [1]m @[90]%, and IIoT use cases (InF-SH and InF-DH) horizontal accuracy performance requirements [0.5]m @[90]%, but cannot meet IIoT use cases (InF-SH and InF-DH) horizontal accuracy performance requirements [0.2]m @[90]%.

Observation 2: Without considering the network synchronization error and UE/gNB Tx/Rx calibration error, none of the simulated IIoT cases can meet the vertical accuracy performance requirement of [0.2m] @[90]%. Most of the simulated IIoT cases cannot meet the vertical accuracy performance requirements of [1m] @[90]%.

Observation 3: Network synchronization error and UE/gNB Tx/Rx calibration error have great impact on Rel-16 NR positioning accuracy performance.

Table 8.1.1.1.3-1: Rel.16 NR positioning – horizontal accuracy performance summary CATT[R1-2007859]

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If No, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | —— | Yes | Yes |
| [Case 2], [InF-SH-2D], [FR1], [UL-TDOA] | —— | Yes | Yes |
| [Case 3], [InF-SH-2D], [FR1], [Multi-RTT] | —— | Yes | Yes |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | —— | No （0.0045） | Yes |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | —— | No （0.0574） | Yes |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | —— | No （0.0677） | Yes |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | —— | No (0.054) | Yes |
| [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | —— | Yes | Yes |
| [Case 9], [InF-DH-2D], [FR1], [UL-TDOA] | —— | Yes | Yes |
| [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | —— | Yes | Yes |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | —— | No (0.5089) | No (0.2089) |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | —— | No （0.4937） | No (0.1937) |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | —— | Yes | Yes |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | —— | No (0.362) | No (0.062) |
| [Case 15], [IOO], [FR1], [DL-TDOA] | Yes | —— | —— |
| [Case 16], [IOO], [FR1], [UL-TDOA] | Yes | —— | —— |
| [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | Yes | —— | —— |
| [Case 18], [IOO], [FR1], [Multi-RTT] | Yes | —— | —— |
| [Case 19], [IOO], [FR1], [DL-TDOA] | No （31.4509） | —— | —— |
| [Case 20], [IOO], [FR1], [UL-TDOA] | No （31.0927） | —— | —— |
| [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | No （8.2356） | —— | —— |
| [Case 22], [IOO], [FR1], [Multi-RTT] | No (31.39) | —— | —— |
| [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | —— | Yes | Yes |
| [Case 24], [InF-SH-2D], [FR2], [UL-TDOA] | —— | Yes | Yes |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | —— | Yes | Yes |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | —— | Yes | Yes |
| [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | —— | Yes | Yes |
| [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | —— | Yes | Yes |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | —— | No （0.5033） | No （0.2033） |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | —— | No （0.4848） | No （0.1848） |
| [Case 31], [IOO], [FR2], [DL-TDOA] | Yes | —— | —— |
| [Case 32], [IOO], [FR2], [UL-TDOA] | Yes | —— | —— |
| [Case 33], [IOO], [FR2], [Multi-RTT] | Yes | —— | —— |
| [Case 34], [IOO], [FR2], [DL-TDOA] | No （31.3809） | —— | —— |
| [Case 35], [IOO], [FR2], [UL-TDOA] | No （31.0887） | —— | —— |
| [Case 36], [IOO], [FR2], [Multi-RTT] | No (0. 2681) | —— | —— |

Table 8.1.1.1.3-2: Rel.16 NR positioning – vertical accuracy performance summary CATT[R1-2007859]

| Simulation case (Vertical Error) | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If No, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If No, provide performance gaps@[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If No, provide performance gaps@[90]% |
| --- | --- | --- | --- |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | —— | No （1.6954） | No （0.8954） |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | —— | No （1.6599） | No （1.0599） |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | —— | No (2.056) | No (1.256) |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | —— | No (1.98) | No (1.18) |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | —— | No （2.7917） | No （1.9917） |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | —— | No （1.849） | No （1.049） |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | —— | No (0.433) | Yes |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | —— | No (2.62) | No (1.82) |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | —— | No （0.4283） | Yes |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | —— | No （0.6304） | Yes |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | —— | No （2.8578） | No （2.0578） |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | —— | No （2.9267） | No （2.1267） |

### 8.1.2	Physical layer latency analysis for Rel-16

#### 8.1.1.2	Results from CATT[R1-2007859]

In this section, we analyse the physical layer latency for both DL-TDOA and UL-TDOA. For DL-TDOA, the physical layer procedure is shown in Figure 1.

Figure 1: PHY procedure for DL-TDOA

According to Figure 1, the total PHY delay could be calculated as

Ttotal=Tgap,req+Tgap,cfg+Tariv+Tprocess+Treport

where,

- Tgap,req denotes the time to require measurement gap, which is 1ms according to the result of URLLC latency in TR 38.824 [5].

- Tgap,cfg denotes the time for measurement gap configuration, which could be assumed as 10ms according to RRC reconfiguration procedure delay in TS 38.331 [6].

- Tariv denotes the delay between the time when DL PRS is received and the time when measurement gap configuration is received, which is related to the maximum value of the configured periodicity of PRS and measurement gap.

- Tprocess denotes the time from UE begins to measure PRS until the measurement result is ready to report. Tprocessis related to UE capability and the number of PRS resources needed to be measured.

- Treport denotes the time for measurement reporting, which includes SR request, PDCCH-based UL grant and PUSCH-based measurement result reporting.

The value of Treport could be assumed as 0.5ms for URLLC according to section 5.2.1 of TR 36.881 [7]. The value of Tprocess may be inferred from the minimum periodicity of PRS and measurement gap, since the measurement result has to be reported before the next PRS occasion.

In Rel-16, the periodicity of PRS includes the following values: 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots. For the periodicity of measurement gap, the following values are supported: 20, 40, 80, 160, [320, 640]ms.

For example, assume SCS is 15kHz, the periodicity of PRS is 4 slots and the periodicity of measurement gap is 20ms, the PHY latency could be summarized in Table 8.1.1.2-1.

Table 8.1.1.2-1: Rel.16 NR positioning latency for DL-TDOA

| Case 1, 15kHz, FR1, DL-TDOA Source UE/Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 1 | Measurement gap request |
| Measurement gap configuration | 10 | Measurement gap configuration delay by RRC |
| PRS arrival delay | ≤X1, X1=max⁡{TcPRS,Tcmgap} (e.g. X1=4 when TcPRS=4 and Tcmgap=20) | DL PRS arrived for measurement, which is related to the maximum value of the configured periodicity of PRS TcPRS and the configured periodicity of measurement gap Tcmgap |
| Process time | ≤X2, X2=max⁡{min⁡(TPRS),min⁡(Tmgap)} (X2=20, as min⁡(TPRS)=4 and min⁡(Tmgap)=20) | UE begins to measure PRS until the measurement result is ready. This value depends on UE capability the number of PRS resources to be measured. The value is assumed to be determined by the supported minimum periodicity of PRS TPRS and measurement gap Tmgap. |
| End trigger | 0.5 | SR request, PDCCH UL grant and PUSCH-based measurement result reporting are included, wherein URLLC is assumed |
| Total values | ≤51.5 |  |

For UL-TDOA, the physical layer procedure is shown in Figure 2.

Figure 2: PHY procedure for UL-TDOA

The total delay could be calculated as

Ttotal=TSRS,act+Tprep+Tprocess

where,

- TSRS,act denotes the time to activate the SRS transmission.

- Tprep denotes the delay from effective time of SRS activation until UE begins to transmit SRS, which is related to the value of the configured periodicity of SRS.

- Tprocess denotes the time from gNB begins to measure SRS until the measurement result is ready, which is related to UE capability and the number of SRS resources needed to be measured.

If SRS is activated by MAC-CE, 3ms delay could be assumed. Similar to Case 1, the value of Tprocess may be inferred from the minimum periodicity of SRS. In Rel-16, the periodicity of SRS for 15 kHz SCS includes the following values:

{1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots.

Assume the periodicity of SRS is 1 slot, the PHY latency could be summarized in Table 8.1.1.2-2.

Table 8.1.1.2-2: Rel.16 NR positioning latency for UL-TDOA

| Case 2, 15kHz, FR1, UL-TDOA Source UE/Destination NW Positioning technique UL-TDOA, type UL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 3 | UE SRS transmission is activated |
| UL SRS arrival delay | ≤X1, X1=TcSRS (e.g. X1=1 when TcSRS=1) | UL SRS arrived for measurement, which is related to the value of the configured periodicity of SRS TcSRS |
| End trigger | ≤X2, X2=min⁡(TSRS) (X2=1, as min⁡(TSRS)=1) | gNB begins to measure SRS until the measurement result is ready. This value depends on gNB capability, and the number of SRS resources to be measured. The value is assumed to be determined by the minimum periodicity of SRS, TSRS. |
| Total values | ≤5 |  |

##### 8.1.1.2.1	Latency analysis for Rel.16 solutions

According to the analysis above, the DL and UL PHY latency could be summarized as below, where the minimum possible latency values for both UL and DL are provided:

- Table 8.1.1.2.1-1: NR Rel.16 positioning - latency performance summary CATT[R1-2007859]

| Description Evaluation Case | L1 Latency, ms | Commercial requirements [100]ms are met -Yes/No - If No, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If No, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case 1, 15kHz, FR1, DL-TDOA | 51.5 | Yes | No (41.5ms gaps) | Yes |
| Case 2, 15kHz, FR1, UL-TDOA | 5 | Yes | Yes | Yes |

We have the following observation:

Observation 4: Physical layer latency for DL positioning needs to be significantly reduced in R17 to meet the 10ms requirement.

## 8.2	Performance of studied NR positioning enhancements

### 8.2.1	Positioning accuracy analysis for NR positioning enhancements

#### 8.2.1.1	Results from CATT[R1-2007859]

##### 8.2.1.1.1	Description of evaluation scenarios

In this section, evaluation scenarios and parameters for positioning accuracy analysis of Rel-17 enhanced positioning solution are given in Table 8.2.1.1.1-1.

- Case ID: 1 to 2.

- Scenario: InF-HH-2D.

- Frequency Band: FR1.

- Positioning Technique: DL-TDOA +DL-CPP, UL-TDOA + UL-CPP [R1-2007860].

Table 8.2.1.1.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case 1], [InF-HH-2D], [FR1], [DL-TDOA+ DL-CPP] | [Case 2], [InF-HH-2D], [FR1], [UL-TDOA+ UL-CPP] |
| --- | --- | --- |
| Channel model (baseline) | InF-HH-2D | InF-HH-2D |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 |
| Number of symbols used per occasion | 12 | 12 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC + PLL | MUSIC + PLL |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | EKF + Chan | EKF + Chan |
| Network synchronization assumptions | Yes | Yes |
| UE/gNB Tx/Rx Calibration Error | No | No |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No |
| Evaluated Enhancement for Rel.17 | DL-TDOA+ DL-CPP | UL-TDOA+ UL-CPP |
| Additional Notes, if any | Double differential with reference UE to overcome network synchronization error [R1-2007860] | Double differential with reference UE to overcome network synchronization error [R1-2007860] |

##### 8.2.1.1.2	Positioning accuracy evaluation results for NR positioning enhancements

Table 8.2.1.1.2-1: NR positioning enhancements - horizontal location error results from CATT[R1-2007859]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-HH-2D], [FR1], [DL- TDOA+ DL-CPP] | Convex UEs | 0.0218 | 0.0321 | 0.0387 | 0.051 |
| [Case 2], [InF-HH-2D], [FR1], [UL- TDOA+ UL-CPP] | Convex UEs | 0.0196 | 0.0306 | 0.0368 | 0.049 |

##### 8.2.1.1.3	Observations on NR positioning enhancements

Horizontal accuracy performance summary of studied NR positioning (DL-TDOA + DL-CPP, and UL-TDOA + UL-CPP) are given in Table 8.2.1.1.3-1.

For IIoT (InF-HH-2D), the horizontal accuracy performance requirements [0.2]m and [0.5]m @[90]% are met by CASE {1, 2}.

Table 8.2.1.1.3-1: NR positioning enhancements – horizontal accuracy performance summary [R1-2007859]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If No, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| [Case 1], [InF-HH-2D], [FR1], [DL- TDOA+ DL-CPP] | 0.133m Vs CASE 1 in section 8.1.1.1 | —— | YES | YES |
| [Case 2], [InF-HH-2D], [FR1], [UL- TDOA+ UL-CPP] | 0.1261m Vs CASE 2 in section 8.1.1.1 | —— | YES | YES |

Observation 5: CPP (DL-TDOA + DL-CPP, and UL-TDOA + UL-CPP) with double differential techniques can meet the IIoT (InF-HH-2D) horizontal accuracy performance requirements [0.2]m@[90]% even when the network synchronization error is simulated.

### 8.2.2	Physical layer latency analysis for NR positioning enhancements

#### 8.2.2.1	Results from CATT[R1-2007859]

##### 8.2.2.1.1	Description of evaluation scenarios

For DL positioning enhancement, aperiodic PRS (A-PRS) could be introduced to reduce positioning latency. In this section, evaluation scenarios and parameters for positioning latency analysis of A-PRS based positioning solution are given as below:

- Case ID: 1.

- SCS: 15kHz.

- Frequency Band: FR1.

- Positioning Technique: DL-TDOA [R1-2007860].

- Periodicity of PRS: Aperiodic PRS

- Duration time of PRS: 4ms

##### 8.2.2.1.2	Latency analysis of NR positioning enhancements

For DL positioning enhancement, aperiodic PRS could be introduced. From our point of view, aperiodic measurement gap can be configured for aperiodic PRS [R1-2007860]. Therefore, whenever triggering, UE should measure those PRS resources. In addition, for measurement reporting, the SR request and UL grant procedures could be arranged before A-PRS triggering. Therefore, the reporting time could be reduced further. The enhanced procedure of DL-TDOA is shown in Figure 3.

Figure 3: Enhanced PHY procedure for DL-TDOA

The total delay could be calculated as

Ttotal=Ttrig+Tariv+Tprocess+Treport

where,

- Ttrig denotes the time to receive and decode the A-PRS triggering signaling.

- Tariv denotes the delay when DL PRS is received, which is related to the PRS offset value configured by the A-PRS triggering signaling

- Tprocess denotes the time from UE begins to measure PRS until the measurement result is ready, which is related to UE capability and the number of PRS resources needed to be measured.

- Treport denotes the time for measurement reporting, which includes SR request, PDCCH-based UL grant and PUSCH-based measurement result reporting..

Assume DCI signalling is used to trigger A-PRS, 1ms could be assumed for Ttrig. Current UE capability is defined as N ms of DL PRS symbols being processed in every T ms for a given maximum bandwidth (B) in MHz supported by UE. N = {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50}ms and T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}ms. Assume Y ms of PRS resources are configured, the processing time could be calculated as Tprocess=YN∙T.

Assume SCS is 15 kHz, and a moderate UE capability, i.e. N=4, T=8, the PHY latency of enhanced DL-TDOA could be summarized in Table 8.2.2.1.2-1.

Table 8.2.2.1.2-1: NR positioning enhancements – latency analysis CATT[R1-2007859]

| Case 1, 15kHz, FR1, DL-TDOA Source UE / Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | 1 | receive and decode the A-PRS triggering signaling |
| PRS arrival delay | 4 | DL PRS arrived for measurement, which is related to the PRS offset value configured by the A-PRS triggering signaling |
| Process time | ≤X, X=YN∙T (e.g. X=8 for Y=4, N=4, T=8 ) | UE begins to measure PRS until the measurement result is ready. This value depends on UE capability the number of PRS resources to be measured. |
| End trigger | 0.5 | PUSCH-based measurement result reporting are included |
| Total values | 13.5 |  |

##### 8.2.2.1.3	Observations on NR positioning latency enhancements

According to the analysis above, the enhanced DL-TDOA PHY latency could be summarized as below:

Table 8.2.2.1.3-1: NR positioning enhancements - physical layer latency performance summary CATT[R1-2007859]

| Description Evaluation Case | L1 Latency, ms | Gain over R16, ms | Commercial requirements [100]ms are met Yes/No. If No, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If No, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| Case 1, 15kHz, FR1, DL-TDOA | 13.5 | 38 | Yes | No (3.5ms gap) | Yes |

### 8.2.3	Network efficiency analysis for NR positioning enhancements

#### 8.2.3.1	Results from CATT[R1-2007859]

##### 8.2.3.1.1	Description of evaluation scenarios

In this section, evaluation scenarios and parameters of network efficiency analysis for NR positioning enhancements are given in Table 8.2.3.1.1-1.

Table 8.2.3.1.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case 1], [InF-HH-2D], [FR1], [DL-TDOA+ DL-CPP] | [Case 2], [InF-HH-2D], [FR1], [UL-TDOA+ UL-CPP] |
| --- | --- | --- |
| Channel model (baseline) | InF-HH-2D | InF-HH-2D |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz |
| System Bandwidth | 100MHz | 100MHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 |
| Number of symbols used per occasion | 12 | 12 |
| number of occasions used per positioning estimate | 1 | 1 |
| Evaluated Enhancement for Rel.17 | DL-TDOA+ DL-CPP | UL-TDOA+ UL-CPP |
| Periodicity of reference signal | 20ms | 20ms |
| Number of PRS resource sets in one periodicity | 1 | 1 |
| Number of PRS Resources per PRS resource set | 1 | 1 |
| Number of users per site | 1,30 | 1,30 |

##### 8.2.3.1.2	Network efficiency analysis of NR positioning enhancements

In RAN1#102-e meeting, the following agreements were achieved on the PRS/SRS resource utilization.

Agreement: PRS/SRS resource utilization is the metric used to evaluate network efficiency - FFS: what is included in resource utilization, e.g. PRS/SRS/MG configurations, beam sweeping assumptions

For the proposed NR positioning enhancements, we use DL-TDOA+DL-CPP or UL-TDOA+UL-CPP as the Rel.17 enhancement scheme. Carrier Phase Positioning can be supported by using Rel.16 DL-PRS or UL-SRS, i.e., the UE or gNB can get the carrier phase measurements together with other NR positioning measurements without additional RF resources and new reference signals. Therefore, the network efficiency of our proposed NR positioning enhancements is the same as that of Rel.16 positioning solutions.

The definition of DL-PRS/UL-SRS resource utilization and the calculation of DL-PRS/UL-SRS resource utilization according to the described definition are shown as in below.

- Definition of DL-PRS resource utilization

Assume gNB x has M TRPs (the index of TRP is j, and j=1~M-1) which are configured to transmit DL-PRS, and each TRP is configured with N DL-PRS resource sets (the index of DL-PRS resource set is i, i=1~N-1). In order to avoid the interference among these DL-PRS resource sets, either the time-domain resources or the frequency-domain resources are orthogonal among these DL-PRS resource sets belongs to the same gNB.

We use DL_PRS_RU to denote DL-PRS resource utilization, then the DL-PRS resource utilization of gNB x (DL_PRS_RUgNB x) can be derived as follows,

DL_PRS_RUPRS resource set i = DL_PRS_RUPRS resource set i, time-domain×DL_PRS_RUPRS resource set i, frequency-domain

DL_PRS_RUPRS resource set i, time-domain= NumOfSymbolsPRS resourcePeriodicityPRS resource set i×NumOfSymbolsPer millisecond×NumofPRSResourcesPRS resource set i×RepetitionFactorPRS resource set i

DL_PRS_RUPRS resource set i, frequency-domain=BandWidthPRS resource set iBandWidthSystem×1CombSizeNPRS resource set i

DL_PRS_RUTRP j = i=1NDL_PRS_RUPRS resource set i

DL_PRS_RUgNB x = j=1MDL_PRS_RUTRP j

- Calculation of DL-PRS resource utilization

The calculation of DL-PRS resource utilization according to the above described definition are shown as follows,

DL_PRS_RUPRS resource set i, time-domain=1220×28×1×1=2.1%

DL_PRS_RUPRS resource set i, frequency-domain=100MHz100MHz×16=16.7%

DLPRSRUPRS resource set i = DL_PRS_RUPRS resource set i, time-domain×DL_PRS_RUPRS resource set i, frequency-domain=2.1%×16.7%=0.36%

DL_PRS_RUTRP j = i=11DL_PRS_RUPRS resource set i=0.36%

DL_PRS_RUgNB x = j=16DL_PRS_RUTRP j=2.1%

- Definition of SRS-Pos resource utilization

Assume gNB x serves M UEs (the index of UE is j, and j=1~M-1) which are configured to transmit SRS-Pos, and each UE is configured with N SRS-Pos resource sets (the index of SRS-Pos resource set is i, i=1~N-1). In order to avoid the interference among these SRS-Pos resource sets, either the time-domain resources or the frequency-domain resources are orthogonal among these SRS-Pos resource sets belongs to the same gNB.

We use SRS_Pos_RU to denote SRS-Pos resource utilization, then the SRS-Pos resource utilization of gNB x (SRS_Pos_RUgNB x) can be derived as follows,

SRS_Pos_RUSRS resource set i = SRS_Pos_RUSRS resource set i, time-domain×SRS_Pos_RUSRS resource set i, frequency-domain

SRS_Pos_RUSRS resource set i, time-domain= NumOfSymbolsSRS resourcePeriodicitySRS resource set i×NumOfSymbolsPer millisecond×NumofSRSResourcesSRS resource set i

SRS_Pos_RUSRS resource set i, frequency-domain=BandWidthSRS resource set iBandWidthSystem×1CombSizeNSRS resource set i

SRS_Pos_RUUE j = i=1NSRS_Pos_RUSRS resource set i

SRS_Pos_RUgNB x = j=1MSRS_Pos_RUUE j

- Calculation of SRS-Pos resource utilization

The calculation of UL-SRS resource utilization according to the above described definition are shown as follows,

SRS_Pos_RUSRS resource set i, time-domain= 1220×28×1=2.1%

SRS_Pos_RUSRS resource set i, frequency-domain=100MHz100MHz×18=12.5%

SRS_Pos_RUSRS resource set i = SRS_Pos_RUSRS resource set i, time-domain×SRS_Pos_RUSRS resource set i, frequency-domain=4.3%×12.5%=0.5%

SRS_Pos_RUUE j = i=11SRS_Pos_RUSRS resource set i=0.27%

When there are 1 user per site,

SRS_Pos_RUgNB x = j=11×6SRS_Pos_RUUE j=1.6%

When there are 30 users per site,

SRS_Pos_RUgNB x = j=130×6SRS_Pos_RUUE j=48.2%

##### 8.2.3.1.3	Observations on network efficiency of NR positioning enhancements

Although the carrier phase positioning (CPP) scheme needs to measure the carrier phase to calculate the UE position, considering Rel.16 DL-PRS or UL-SRS can be reused to complete the carrier phase measurement, there is no increase in terms of reference signal resource overhead as the metric for network efficiency, compared with Rel.16 positioning solutions.

Observation 6: NR positioning enhancements with carrier phase measurements has no impact on UE and network RF resource usage efficiency.

--------------------------------------- End of template for collection of NR positioning results ---------------------------------

Conclusions

In this contribution, we discussed the evaluation of NR positioning performance. The following observations are given.

Observation 1: Without considering the network synchronization error and UE/gNB Tx/Rx calibration error, Rel-16 NR positioning techniques can meet the commercial use case (IOO) horizontal accuracy performance requirements [1]m @[90]%, and IIoT use cases (InF-SH and InF-DH) horizontal accuracy performance requirements [0.5]m @[90]%, but cannot meet IIoT use cases (InF-SH and InF-DH) horizontal accuracy performance requirements [0.2]m @[90]%.

Observation 2: Without considering the network synchronization error and UE/gNB Tx/Rx calibration error, none of the simulated IIoT cases can meet the vertical accuracy performance requirement of [0.2m] @[90]%. Most of the simulated IIoT cases cannot meet the vertical accuracy performance requirements of [1m] @[90]%.

Observation 3: Network synchronization error and UE/gNB Tx/Rx calibration error have great impact on Rel-16 NR positioning accuracy performance.

Observation 4: Physical layer latency for DL positioning needs to be significantly reduced in R17 to meet the 10ms requirement.

Observation 5: CPP (DL-TDOA + DL-CPP, and UL-TDOA + UL-CPP) with double differential techniques can meet the IIoT (InF-HH-2D) horizontal accuracy performance requirements [0.2]m@[90]% even when the network synchronization error is simulated.

Observation 6: NR positioning enhancements with carrier phase measurements has no impact on UE and network RF resource usage efficiency.

References

- RAN1 Chairman's Notes 3GPP TSG RAN WG1 Meeting #102-e, August 17th – 28th, 2020.

- [102-e-Post-NR-ePos-01] Email discussion/approval on collection of simulation results (10/12-10/21).

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V16.1.0 (2019-12)

- Chan Y T, Ho K C. A simple and efficient estimator for hyperbolic location[J]. IEEE Transactions on Signal Processing, 2002, 42(8):1905-1915.

- 3GPP TR 38.824, “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”, V16.0.0 (2019-03)

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification,V16.2.0 (2020-09)

- 3GPP TR 36.881, “study on latency reduction techniques for LTE”, V14.0.0 (2016-06)

- R1-2007860, Discussion of NR positioning enhancements, RAN1#103e, CATT