3GPP TSG RAN WG1 Meeting #103-e                                                                	R1- 2007873

e-Meeting, October 26th – November 13th, 2020

Source:	CATT

Title:	Baseline coverage performance for FR2

Agenda Item:	8.8.1.2

Document for:	Discussion and Decision

Introduction

At RAN1#102 e-meeting, the link level simulation assumptions and the link budget template were further discussed and the following agreements were achieved[1]:

Agreements:

- for SIP invite message

  - Payload of 1500 bytes can be a starting point.

  - The assumptions (TB size, time period etc.) are reported by companies.

  - Contributions R1-2003464 and R1-2005259 are taken into account for the evaluation.

  - In addition, 1 second time period can also be considered

Agreements:

- For link level simulations in FR2, only PUCCH format 1 and format 3 are considered for baseline performance evaluation.

- For link level simulations in FR2, only PUCCH duration of 14 OFDM symbols is considered for baseline performance evaluation.

- For link level simulations in FR2, consider 4 DMRS symbol for PUCCH Format 3.

- Consider only one panel at the UE in link budget in FR2.

- For link budget calculation in FR2, downlink transmit power is scaled by the occupied bandwidth. The following downlink transmit power vs occupied bandwidth values are considered as baseline for the calculations:

  - 40 dBm for 100 MHz Urban scenario,

  - 23 dBm for 100 MHz Indoor scenario.

- For link budget calculation in FR2, an uplink transmit power of 23dBm is considered for baseline performance evaluations. Other values can be reported by companies.

- Confirm the target throughput values of the REL-17 SID for the suburban scenario:

  - DL: 1 Mbps, UL: 50 kbps

- Study performance of PUSCH in FR2 only for DFT-s-OFDM.

- For link level simulations, only 1% BLER should be considered for baseline performance evaluation of PDDCH in FR2.

- For link level simulations in FR2, only PUSCH repetition type A is considered for baseline performance evaluation.

  - Note: companies are not precluded to report results for repetition type B.

- Suburban scenario is deprioritized for NR coverage enhancement SI.

- Baseline performance evaluation of msg1 transmission is studied for 1% missed detection probability in FR2.

- Only 1% BLER target should be considered for baseline performance evaluation of PUCCH in FR2, regardless of whether UCI includes CSI feedback or not.

In this contribution, we provide the baseline performance based on different evaluation metrics in FR2. Accordingly, the bottleneck channels are identified.

Evaluation results

As discussed during in RAN1#102 e-meeting, MIL and MCL are adopted to identify the bottleneck channels for LLS based methodology. Furthermore, MPL can be used as supplemental information for coverage bottleneck(s) identification. Accordingly, we provide the baseline performance based on all these three metrics, which are illustrated as below:

- For MPL, the same definition as in IMT-2020 self-evaluation with some modifications.[1]

- For MIL, the template adopted by email discussion [102-e-Post-NR-CovEnh-02] is used to calculate MIL

- For MCL, the template adopted by email discussion [102-e-Post-NR-CovEnh-02] is used to calculate MIL

Target MPL corresponding to target ISD of 200m is calculated. NLOS outdoor-to-indoor case with 3km/h UE speed is considered in the evaluation. Required SINR for LLS with TDL-C channel model are evaluated.

The channel would be identified as the bottleneck if it satisfies the following condition:

- For MPL, a channel is identified as the bottleneck if the baseline performance is worse than the target performance.

- For MIL, a UL channel is identified as the bottleneck if it is the worst, or second worst or third worst channel. It should be noted that even the worst downlink channel has a much better coverage than the best uplink channel. Hence only uplink channels are considered.

- For MCL, a UL channel is identified as the bottleneck if it is the worst, or second worst or third worst channel. It should be noted that even the worst downlink channel has a much better coverage than the best uplink channel. Hence only uplink channels are considered.

- Urban scenario

The baseline performance in terms of MIL and MCL are provided in Table 1-4. Additionally, target MPL corresponding to target ISD of 200m are also provided. NLOS outdoor-to-outdoor case with 30km/h UE speed and outdoor-to-indoor case with 3km/h UE speed are considered in the evaluation. Required SINR for LLS with both TDL-A and TDL-C channel model are evaluated. Detailed simulation assumptions for Urban scenario and evaluation results are listed in Appendix.

Table 1: evaluation results for Urban scenario TDLA-3km

| Scneario | Physical channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 124.29 | 160.38 | 128.76 | 121.75 | -1.3 |
|  | PDSCH with VoIP | 138.23 | 164.32 | 132.7 | 121.75 | -5.24 |
|  | SSB | 133.55 | 165.63 | 134.01 | 121.75 | -9.56 |
|  | PDCCH | 139.39 | 165.48 | 130.42 | 121.75 | -6.40 |
|  | PUCCH PF1 | 114.77 | 154.89 | 119.83 | 121.75 | -5.4 |
|  | PUCCH PF3/11bits/rep=1 | 112.52 | 152.64 | 117.58 | 121.75 | -3.15 |
|  | PUCCH PF3/11bits/rep=2 | 114.62 | 154.74 | 119.68 | 121.75 | -5.25 |
|  | PUCCH PF3/22bits/rep=1 | 110.32 | 150.44 | 115.38 | 121.75 | -0.95 |
|  | PUCCH PF3/22bits/rep=2 | 112.67 | 152.79 | 117.73 | 121.75 | -3.3 |
|  | Msg 3 PUSCH | 111.04 | 148.14 | 116.52 | 121.75 | -1.6 |
|  | eMBB PUSCH | 96.78 | 133.38 | 101.76 | 121.75 | 0.9 |
|  | VoIP PUSCH w/o repetition | 101.83 | 138.93 | 107.31 | 121.75 | 4.6 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.33 | 117.71 | 121.75 | -5.8 |
| 28GHz DDSU | PDSCH with eMBB | 123.29 | 159.38 | 127.76 | 121.75 | -0.3 |
|  | PDSCH with VoIP | 128.23 | 164.32 | 132.7 | 121.75 | -5.24 |
|  | SSB | 132.55 | 175.63 | 144.01 | 121.75 | -9.56 |
|  | PDCCH | 129.39 | 165.48 | 130.42 | 121.75 | -6.40 |
|  | PUCCH PF1 | 114.77 | 154.89 | 119.83 | 121.75 | -5.4 |
|  | PUCCH PF3/11bits/rep=1 | 112.52 | 152.64 | 117.58 | 121.75 | -3.15 |
|  | PUCCH PF3/11bits/rep=2 | 114.62 | 154.74 | 119.68 | 121.75 | -5.25 |
|  | PUCCH PF3/22bits/rep=1 | 110.32 | 150.44 | 115.38 | 121.75 | -0.95 |
|  | PUCCH PF3/22bits/rep=2 | 112.67 | 152.79 | 117.73 | 121.75 | -3.3 |
|  | Msg 3 PUSCH | 111.04 | 148.14 | 116.52 | 121.75 | -1.6 |
|  | eMBB PUSCH | 97.68 | 134.78 | 103.16 | 121.75 | 0 |
|  | VoIP PUSCH w/o repetition | 101.83 | 138.93 | 107.31 | 121.75 | 4.6 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.33 | 117.71 | 121.75 | -5.8 |

Table 2: evaluation results for Urban scenario TDLA-30km

| Scenario | Physical channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 123.49 | 159.58 | 147.22 | 121.75 | -0.5 |
|  | PDSCH with VoIP | 127.54 | 165.63 | 153.27 | 121.75 | -4.55 |
|  | SSB | 131.63 | 164.71 | 152.35 | 121.75 | -8.64 |
|  | PDCCH | 129.88 | 165.97 | 150.86 | 121.75 | -6.89 |
|  | PUCCH PF1 | 115.23 | 155.35 | 140.24 | 121.75 | -5.86 |
|  | PUCCH PF3/11bits/rep=1 | 112.33 | 152.45 | 137.34 | 121.75 | -2.96 |
|  | PUCCH PF3/11bits/rep=2 | 116.69 | 156.81 | 141.7 | 121.75 | -7.32 |
|  | PUCCH PF3/22bits/rep=1 | 110.33 | 150.45 | 135.34 | 121.75 | -0.96 |
|  | PUCCH PF3/22bits/rep=2 | 114.93 | 155.05 | 139.94 | 121.75 | -5.56 |
|  | Msg 3 PUSCH | 110.74 | 147.84 | 135.48 | 121.75 | -1.3 |
|  | eMBB PUSCH | 97.18 | 134.28 | 121.92 | 121.75 | 0.5 |
|  | VoIP PUSCH w/o repetition | 101.73 | 138.83 | 126.47 | 121.75 | 4.7 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 141.17 | 121.75 | -10 |
| 28GHz DDSU | PDSCH with eMBB | 123.49 | 159.58 | 147.22 | 121.75 | -0.5 |
|  | PDSCH with VoIP | 127.54 | 163.63 | 151.27 | 121.75 | -4.55 |
|  | SSB | 131.63 | 164.71 | 152.35 | 121.75 | -8.64 |
|  | PDCCH | 129.88 | 165.97 | 150.86 | 121.75 | -6.89 |
|  | PUCCH PF1 | 115.23 | 155.35 | 140.24 | 121.75 | -5.86 |
|  | PUCCH PF3/11bits/rep=1 | 112.33 | 152.45 | 137.34 | 121.75 | -2.96 |
|  | PUCCH PF3/11bits/rep=2 | 116.69 | 156.81 | 141.7 | 121.75 | -7.32 |
|  | PUCCH PF3/22bits/rep=1 | 110.33 | 150.45 | 135.34 | 121.75 | -0.96 |
|  | PUCCH PF3/22bits/rep=2 | 114.93 | 155.05 | 139.94 | 121.75 | -5.56 |
|  | Msg 3 PUSCH | 110.74 | 147.84 | 135.48 | 121.75 | -1.3 |
|  | eMBB PUSCH | 98.98 | 135.48 | 123.12 | 121.75 | -0.7 |
|  | VoIP PUSCH w/o repetition | 101.73 | 138.83 | 126.47 | 121.75 | 4.7 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 141.17 | 121.75 | -10 |

Table 3: evaluation results for Urban scenario TDLC-3km

| Scenario | Physical channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 124.51 | 160.6 | 128.98 | 121.75 | -1.52 |
|  | PDSCH with VoIP | 128.34 | 164.43 | 132.81 | 121.75 | -5.35 |
|  | SSB | 132.55 | 164.71 | 133.09 | 121.75 | -9.56 |
|  | PDCCH | 130.09 | 166.18 | 131.12 | 121.75 | -7.10 |
|  | PUCCH PF1 | 114.77 | 154.89 | 119.83 | 121.75 | -5.4 |
|  | PUCCH PF3/11bits/rep=1 | 111.67 | 151.79 | 116.73 | 121.75 | -2.3 |
|  | PUCCH PF3/11bits/rep=2 | 114.19 | 154.31 | 119.25 | 121.75 | -4.82 |
|  | PUCCH PF3/22bits/rep=1 | 109.37 | 149.49 | 114.43 | 121.75 | 0 |
|  | PUCCH PF3/22bits/rep=2 | 112.12 | 152.24 | 117.18 | 121.75 | -2.75 |
|  | Msg 3 PUSCH | 110.44 | 147.54 | 115.92 | 121.75 | -1 |
|  | eMBB PUSCH | 96.18 | 133.28 | 101.66 | 121.75 | 1.5 |
|  | VoIP PUSCH w/o repetition | 101.63 | 138.73 | 107.11 | 121.75 | 4.8 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.33 | 117.71 | 121.75 | -5.8 |
| 28GHz DDSU | PDSCH with eMBB | 123.51 | 159.6 | 127.98 | 121.75 | -0.52 |
|  | PDSCH with VoIP | 128.34 | 164.43 | 132.81 | 121.75 | -5.35 |
|  | SSB | 132.55 | 165.63 | 134.01 | 121.75 | -9.56 |
|  | PDCCH | 130.09 | 166.18 | 131.12 | 121.75 | -7.10 |
|  | PUCCH PF1 | 114.77 | 154.89 | 119.83 | 121.75 | -5.4 |
|  | PUCCH PF3/11bits/rep=1 | 111.67 | 151.79 | 116.73 | 121.75 | -2.3 |
|  | PUCCH PF3/11bits/rep=2 | 114.19 | 154.31 | 119.25 | 121.75 | -4.82 |
|  | PUCCH PF3/22bits/rep=1 | 109.37 | 149.49 | 114.43 | 121.75 | 0 |
|  | PUCCH PF3/22bits/rep=2 | 112.12 | 152.24 | 117.18 | 121.75 | -2.75 |
|  | Msg 3 PUSCH | 110.44 | 147.54 | 115.92 | 121.75 | -1 |
|  | eMBB PUSCH | 97.68 | 134.78 | 103.16 | 121.75 | 0 |
|  | VoIP PUSCH w/o repetition | 101.63 | 138.73 | 107.11 | 121.75 | 4.8 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.33 | 117.71 | 121.75 | -5.8 |

Table 4: evaluation results for Urban scenario TDLC-30km

| Scenario | Physical channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 123.85 | 159.94 | 146.15 | 121.75 | -0.86 |
|  | PDSCH with VoIP | 128.04 | 164.13 | 150.34 | 121.75 | -5.05 |
|  | SSB | 128.52 | 165.63 | 151.84 | 121.75 | -9.53 |
|  | PDCCH | 130.14 | 166.23 | 149.2 | 121.75 | -7.15 |
|  | PUCCH PF1 | 115.23 | 155.35 | 138.32 | 121.75 | -5.86 |
|  | PUCCH PF3/11bits/rep=1 | 111.55 | 151.67 | 134.64 | 121.75 | -2.18 |
|  | PUCCH PF3/11bits/rep=2 | 116.02 | 156.14 | 139.11 | 121.75 | -6.65 |
|  | PUCCH PF3/22bits/rep=1 | 109.51 | 149.63 | 132.6 | 121.75 | -0.14 |
|  | PUCCH PF3/22bits/rep=2 | 113.64 | 153.76 | 136.73 | 121.75 | -4.27 |
|  | Msg 3 PUSCH | 110.34 | 147.44 | 133.65 | 121.75 | -0.9 |
|  | eMBB PUSCH | 97.08 | 134.18 | 120.39 | 121.75 | 0.6 |
|  | VoIP PUSCH w/o repetition | 101.73 | 138.83 | 125.04 | 121.75 | 4.7 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 139.74 | 121.75 | -10 |
| 28GHz DDSU | PDSCH with eMBB | 123.85 | 159.94 | 146.15 | 121.75 | -0.86 |
|  | PDSCH with VoIP | 128.04 | 164.13 | 150.34 | 121.75 | -5.05 |
|  | SSB | 128.52 | 165.63 | 151.84 | 121.75 | -9.53 |
|  | PDCCH | 130.14 | 166.23 | 149.2 | 121.75 | -7.15 |
|  | PUCCH PF1 | 115.23 | 155.35 | 138.32 | 121.75 | -5.86 |
|  | PUCCH PF3/11bits/rep=1 | 111.55 | 151.67 | 134.64 | 121.75 | -2.18 |
|  | PUCCH PF3/11bits/rep=2 | 116.02 | 156.14 | 139.11 | 121.75 | -6.65 |
|  | PUCCH PF3/22bits/rep=1 | 109.51 | 149.63 | 132.6 | 121.75 | -0.14 |
|  | PUCCH PF3/22bits/rep=2 | 113.64 | 153.76 | 136.73 | 121.75 | -4.27 |
|  | Msg 3 PUSCH | 110.34 | 147.44 | 133.65 | 121.75 | -0.9 |
|  | eMBB PUSCH | 98.28 | 135.38 | 121.59 | 121.75 | -0.6 |
|  | VoIP PUSCH w/o repetition | 101.73 | 138.83 | 125.04 | 121.75 | 4.7 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 139.74 | 121.75 | -10 |

Observation 1: For Urban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- Based on MPL evaluation results, PUCCH, Msg3 PUSCH, eMBB PUSCH and VOIP PUSCH coverage are the bottleneck channel.

- Based on MIL/MCL evaluation results, eMBB PUSCH, Msg3 PUSCH and PUCCH format3 coverage are the bottleneck channel.

- Indoor scenario

The baseline performance in terms of MIL and MCL are provided in Table 5. Additionally, target MPL corresponding to target ISD of 20m is assumed. NLOS indoor case with 3km/h UE speed is considered in the evaluation. Required SINR for LLS with TDL-A channel model is evaluated. Detailed simulation assumptions for Indoor scenario are listed in Appendix.

Table 5: Evaluation results for Indoor scenario TDLA-3km

| Scenario | Physical channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=20 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 125.77 | 158.85 | 157.16 | 94 | -1.56 |
|  | PDSCH with VoIP | 129.51 | 162.59 | 160.9 | 94 | -5.3 |
|  | SSB | 133.85 | 163.93 | 162.24 | 94 | -9.64 |
|  | PDCCH | 130.92 | 164 | 161.31 | 94 | -6.71 |
|  | PUCCH PF1 | 122.97 | 156.07 | 153.38 | 94 | -6.55 |
|  | PUCCH PF3/11bits/rep=1 | 119.52 | 153.62 | 150.93 | 94 | -3.1 |
|  | PUCCH PF3/11bits/rep=2 | 121.76 | 155.86 | 153.17 | 94 | -5.34 |
|  | PUCCH PF3/22bits/rep=1 | 117.17 | 151.27 | 148.58 | 94 | -0.75 |
|  | PUCCH PF3/22bits/rep=2 | 119.34 | 153.44 | 150.75 | 94 | -2.92 |
|  | Msg 3 PUSCH | 115.01 | 149.11 | 147.42 | 94 | -1.6 |
|  | eMBB PUSCH | 100.85 | 134.94 | 133.25 | 94 | 0.8 |
|  | VoIP PUSCH w/o repetition | 105.8 | 139.89 | 138.2 | 94 | 4.6 |
|  | VoIP PUSCH w/ repetition | 116.2 | 150.29 | 148.6 | 94 | -5.8 |
| 28GHz DDSU | PDSCH with eMBB | 124.66 | 157.74 | 156.05 | 94 | -0.45 |
|  | PDSCH with VoIP | 129.51 | 162.59 | 160.9 | 94 | -5.3 |
|  | SSB | 133.85 | 163.93 | 162.24 | 94 | -9.64 |
|  | PDCCH | 130.92 | 164 | 161.31 | 94 | -6.71 |
|  | PUCCH PF1 | 122.97 | 156.07 | 153.38 | 94 | -6.55 |
|  | PUCCH PF3/11bits/rep=1 | 119.52 | 153.62 | 150.93 | 94 | -3.1 |
|  | PUCCH PF3/11bits/rep=2 | 121.76 | 155.86 | 153.17 | 94 | -5.34 |
|  | PUCCH PF3/22bits/rep=1 | 117.17 | 151.27 | 148.58 | 94 | -0.75 |
|  | PUCCH PF3/22bits/rep=2 | 119.34 | 153.44 | 150.75 | 94 | -2.92 |
|  | Msg 3 PUSCH | 115.01 | 149.11 | 147.42 | 94 | -1.6 |
|  | eMBB PUSCH | 101.65 | 135.74 | 134.05 | 94 | 0 |
|  | VoIP PUSCH w/o repetition | 105.8 | 139.89 | 138.2 | 94 | 4.6 |
|  | VoIP PUSCH w/ repetition | 116.2 | 150.29 | 148.6 | 94 | -5.8 |

According to table 5, we can have the following observation:

Observation 2: For Indoor scenario,

- Based on MPL evaluation results, no bottleneck channel is identified.

- Based on MIL/MCL evaluation results, eMBB PUSCH, Msg3 PUSCH and VoIP PUSCH coverage are the bottleneck channel.

- SubUrban scenario

For SubUrban scenario, the undefined parameters reuse that of Urban Scenario. The baseline performance in terms of MIL and MCL are provided in Table 6-9. Additionally, target MPL corresponding to target ISD of 200m are assumed. NLOS outdoor-to-outdoor case with 30km/h UE speed and outdoor-to-indoor case with 3km/h UE speed are considered in the evaluation. Required SINR for LLS with both TDL-A and TDL-C channel model are evaluated. Detailed simulation assumptions for SubUrban scenario are listed in Appendix.

Table 6: evaluation results for SubUrban scenario TDLA-3km

| Scneario | Physical channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 130.39 | 166.48 | 134.86 | 121.75 | -7.4 |
|  | PDSCH with VoIP | 128.23 | 164.32 | 132.7 | 121.75 | -5.24 |
|  | SSB | 132.55 | 165.63 | 134.01 | 121.75 | -9.56 |
|  | PDCCH | 129.39 | 165.48 | 130.42 | 121.75 | -6.40 |
|  | PUCCH PF1 | 114.87 | 154.99 | 119.93 | 121.75 | -5.5 |
|  | PUCCH PF3/11bits/rep=1 | 112.47 | 152.59 | 117.53 | 121.75 | -3.1 |
|  | PUCCH PF3/11bits/rep=2 | 114.69 | 154.81 | 119.75 | 121.75 | -5.32 |
|  | PUCCH PF3/22bits/rep=1 | 110.11 | 150.23 | 115.17 | 121.75 | -0.74 |
|  | PUCCH PF3/22bits/rep=2 | 112.3 | 152.42 | 117.36 | 121.75 | -2.93 |
|  | Msg 3 PUSCH | 111.04 | 148.14 | 116.52 | 121.75 | -1.6 |
|  | eMBB PUSCH | 103.68 | 140.78 | 109.16 | 121.75 | -6 |
|  | VoIP PUSCH w/o repetition | 101.83 | 138.93 | 107.31 | 121.75 | 4.6 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.33 | 117.71 | 121.75 | -5.8 |
| 28GHz DDSU | PDSCH with eMBB | 130.39 | 166.48 | 134.86 | 121.75 | -7.4 |
|  | PDSCH with VoIP | 128.23 | 164.32 | 132.7 | 121.75 | -5.24 |
|  | SSB | 132.55 | 165.63 | 134.01 | 121.75 | -9.56 |
|  | PDCCH | 129.39 | 165.48 | 130.42 | 121.75 | -6.40 |
|  | PUCCH PF1 | 114.87 | 154.99 | 119.93 | 121.75 | -5.5 |
|  | PUCCH PF3/11bits/rep=1 | 112.47 | 152.59 | 117.53 | 121.75 | -3.1 |
|  | PUCCH PF3/11bits/rep=2 | 114.69 | 154.81 | 119.75 | 121.75 | -5.32 |
|  | PUCCH PF3/22bits/rep=1 | 110.11 | 150.23 | 115.17 | 121.75 | -0.74 |
|  | PUCCH PF3/22bits/rep=2 | 112.3 | 152.42 | 117.36 | 121.75 | -2.93 |
|  | Msg 3 PUSCH | 111.04 | 148.14 | 116.52 | 121.75 | -1.6 |
|  | eMBB PUSCH | 103.68 | 140.78 | 109.16 | 121.75 | -6 |
|  | VoIP PUSCH w/o repetition | 101.83 | 138.93 | 107.31 | 121.75 | 4.6 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.33 | 117.71 | 121.75 | -5.8 |

Table 7: evaluation results for SubUrban scenario TDLA-30km

| Scneario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 130.19 | 166.28 | 153.92 | 121.75 | -7.2 |
|  | PDSCH with VoIP | 127.54 | 163.63 | 151.27 | 121.75 | -4.55 |
|  | SSB | 131.63 | 164.71 | 152.35 | 121.75 | -8.64 |
|  | PDCCH | 129.88 | 165.97 | 150.86 | 121.75 | -6.89 |
|  | PUCCH PF1 | 114.98 | 155.1 | 139.99 | 121.75 | -5.61 |
|  | PUCCH PF3/11bits/rep=1 | 112.37 | 152.49 | 137.38 | 121.75 | -3 |
|  | PUCCH PF3/11bits/rep=2 | 116.7 | 156.82 | 141.71 | 121.75 | -7.33 |
|  | PUCCH PF3/22bits/rep=1 | 110.15 | 150.27 | 135.16 | 121.75 | -0.78 |
|  | PUCCH PF3/22bits/rep=2 | 114.77 | 154.89 | 139.78 | 121.75 | -5.4 |
|  | Msg 3 PUSCH | 110.74 | 147.84 | 135.48 | 121.75 | -1.3 |
|  | eMBB PUSCH | 103.68 | 140.78 | 128.42 | 121.75 | -6 |
|  | VoIP PUSCH w/o repetition | 102.23 | 139.33 | 126.97 | 121.75 | 4.2 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 141.17 | 121.75 | -10 |
| 28GHz DDSU | PDSCH with eMBB | 130.19 | 166.28 | 153.92 | 121.75 | -7.2 |
|  | PDSCH with VoIP | 127.54 | 163.63 | 151.27 | 121.75 | -4.55 |
|  | SSB | 131.63 | 164.71 | 152.35 | 121.75 | -8.64 |
|  | PDCCH | 129.88 | 165.97 | 150.86 | 121.75 | -6.89 |
|  | PUCCH PF1 | 114.98 | 155.1 | 139.99 | 121.75 | -5.61 |
|  | PUCCH PF3/11bits/rep=1 | 112.37 | 152.49 | 137.38 | 121.75 | -3 |
|  | PUCCH PF3/11bits/rep=2 | 116.7 | 156.82 | 141.71 | 121.75 | -7.33 |
|  | PUCCH PF3/22bits/rep=1 | 110.15 | 150.27 | 135.16 | 121.75 | -0.78 |
|  | PUCCH PF3/22bits/rep=2 | 114.77 | 154.89 | 139.78 | 121.75 | -5.4 |
|  | Msg 3 PUSCH | 110.74 | 147.84 | 135.48 | 121.75 | -1.3 |
|  | eMBB PUSCH | 103.68 | 140.78 | 128.42 | 121.75 | -6 |
|  | VoIP PUSCH w/o repetition | 102.23 | 139.33 | 126.97 | 121.75 | 4.2 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 141.17 | 121.75 | -10 |

Table 8: evaluation results for SubUrban scenario TDLC-3km

| Scneario | Physical channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 130.79 | 166.88 | 135.26 | 121.75 | -7.8 |
|  | PDSCH with VoIP | 128.34 | 164.43 | 132.81 | 121.75 | -5.35 |
|  | SSB | 132.55 | 165.63 | 134.01 | 121.75 | -9.56 |
|  | PDCCH | 130.09 | 166.18 | 131.12 | 121.75 | -7.10 |
|  | PUCCH PF1 | 114.87 | 154.99 | 119.93 | 121.75 | -5.5 |
|  | PUCCH PF3/11bits/rep=1 | 112.47 | 152.59 | 117.53 | 121.75 | -3.1 |
|  | PUCCH PF3/11bits/rep=2 | 114.42 | 154.54 | 119.48 | 121.75 | -5.05 |
|  | PUCCH PF3/22bits/rep=1 | 110.09 | 150.21 | 115.15 | 121.75 | -0.72 |
|  | PUCCH PF3/22bits/rep=2 | 112.59 | 152.71 | 117.65 | 121.75 | -3.22 |
|  | Msg 3 PUSCH | 110.44 | 147.54 | 115.92 | 121.75 | -1 |
|  | eMBB PUSCH | 102.48 | 139.58 | 107.96 | 121.75 | -4.8 |
|  | VoIP PUSCH w/o repetition | 101.63 | 138.73 | 107.11 | 121.75 | 4.8 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.53 | 117.91 | 121.75 | -5.8 |
| 28GHz DDSU | PDSCH with eMBB | 113.79 | 166.88 | 135.26 | 121.75 | -7.8 |
|  | PDSCH with VoIP | 128.34 | 164.43 | 132.81 | 121.75 | -5.35 |
|  | SSB | 132.55 | 165.63 | 134.01 | 121.75 | -9.56 |
|  | PDCCH | 130.09 | 166.18 | 131.12 | 121.75 | -7.10 |
|  | PUCCH PF1 | 131.87 | 154.99 | 119.93 | 121.75 | -5.5 |
|  | PUCCH PF3/11bits/rep=1 | 112.47 | 152.59 | 117.53 | 121.75 | -3.1 |
|  | PUCCH PF3/11bits/rep=2 | 114.42 | 154.54 | 119.48 | 121.75 | -5.05 |
|  | PUCCH PF3/22bits/rep=1 | 110.09 | 150.21 | 115.15 | 121.75 | -0.72 |
|  | PUCCH PF3/22bits/rep=2 | 112.59 | 152.71 | 117.65 | 121.75 | -3.22 |
|  | Msg 3 PUSCH | 110.44 | 147.54 | 115.92 | 121.75 | -1 |
|  | eMBB PUSCH | 102.48 | 139.58 | 107.96 | 121.75 | -4.8 |
|  | VoIP PUSCH w/o repetition | 101.63 | 138.73 | 107.11 | 121.75 | 4.8 |
|  | VoIP PUSCH w/ repetition | 112.23 | 149.53 | 117.91 | 121.75 | -5.8 |

Table9: evaluation results for SubUrban scenario TDLC-30km

| Scneario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=200 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz DDDSU | PDSCH with eMBB | 130.69 | 166.78 | 152.99 | 121.75 | -7.7 |
|  | PDSCH with VoIP | 128.04 | 164.13 | 150.34 | 121.75 | -5.05 |
|  | SSB | 132.09 | 165.17 | 151.38 | 121.75 | -9.1 |
|  | PDCCH | 130.14 | 166.23 | 149.2 | 121.75 | -7.15 |
|  | PUCCH PF1 | 114.98 | 155.10 | 138.07 | 121.75 | -5.61 |
|  | PUCCH PF3/11bits/rep=1 | 112.14 | 152.26 | 135.23 | 121.75 | -2.77 |
|  | PUCCH PF3/11bits/rep=2 | 116.55 | 156.67 | 139.64 | 121.75 | -7.18 |
|  | PUCCH PF3/22bits/rep=1 | 109.92 | 150.04 | 133.01 | 121.75 | -0.55 |
|  | PUCCH PF3/22bits/rep=2 | 114.4 | 154.52 | 137.49 | 121.75 | -5.03 |
|  | Msg 3 PUSCH | 110.34 | 147.44 | 133.65 | 121.75 | -0.9 |
|  | eMBB PUSCH | 103.68 | 140.78 | 126.99 | 121.75 | -6 |
|  | VoIP PUSCH w/o repetition | 101.73 | 138.83 | 125.04 | 121.75 | 4.7 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 139.74 | 121.75 | -10 |
| 28GHz DDSU | PDSCH with eMBB | 130.69 | 166.78 | 152.99 | 121.75 | -7.7 |
|  | PDSCH with VoIP | 128.04 | 164.13 | 150.34 | 121.75 | -5.05 |
|  | SSB | 132.09 | 165.17 | 151.38 | 121.75 | -9.1 |
|  | PDCCH | 130.14 | 166.23 | 149.2 | 121.75 | -7.15 |
|  | PUCCH PF1 | 114.98 | 155.10 | 138.07 | 121.75 | -5.61 |
|  | PUCCH PF3/11bits/rep=1 | 112.14 | 152.26 | 135.23 | 121.75 | -2.77 |
|  | PUCCH PF3/11bits/rep=2 | 116.55 | 156.67 | 139.64 | 121.75 | -7.18 |
|  | PUCCH PF3/22bits/rep=1 | 109.92 | 150.04 | 133.01 | 121.75 | -0.55 |
|  | PUCCH PF3/22bits/rep=2 | 114.4 | 154.52 | 137.49 | 121.75 | -5.03 |
|  | Msg 3 PUSCH | 110.34 | 147.44 | 133.65 | 121.75 | -0.9 |
|  | eMBB PUSCH | 103.68 | 140.78 | 126.99 | 121.75 | -6 |
|  | VoIP PUSCH w/o repetition | 101.73 | 138.83 | 125.04 | 121.75 | 4.7 |
|  | VoIP PUSCH w/ repetition | 116.43 | 153.53 | 139.74 | 121.75 | -10 |

Observation 3: For SubUrban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- Based on MPL evaluation results, PUCCH, Msg3 PUSCH, eMBB PUSCH and VOIP PUSCH coverage are the bottleneck channel

- Based on MIL/MCL evaluation results, eMBB PUSCH,Msg3 PUSCH and PUCCH Format3 coverage are the bottleneck channel

Per the agreement in RAN1#102 e-meeting, payload size of SIP message is 1500 byte and 1second transmission period can be assumed. The packet size of SIP message can be calculated via 1500*8/50= 240 bits with 20ms data arriving interval. It should be noted that the actual payload size is much smaller than that of VoIP, i.e. 320 bits. With this regard, SIP message carrying by PDSCH is certainly not the bottleneck as downlink VoIP meets the requirement. Accordingly, at least SIP message carried by PDSCH doesn’t need to be evaluated independently.

Regarding performance evaluation of MSG4, based on agreement in RAN1#102 e-meeting, payload size of MSG4 message is 1040bits. In this case, Data rate of MSG4 is about 1Mbps which is lower than data rate (10Mbps) of PDSCH. In this regard, if PDSCH isn’t the bottleneck, MSG4 isn’t the bottleneck as well.

With observation 1, 2, 3, we have the following proposal:

Proposal 1: PUCCH and PUSCH coverage enhancement should be studied for FR2.

Conclusion

This contribution discussed the baseline performance and simulation assumptions for FR2, we have following observations:

Observation 1: For Urban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- Based on MPL evaluation results, PUCCH, Msg3 PUSCH, eMBB PUSCH and VOIP PUSCH coverage are the bottleneck channel.

- Based on MIL/MCL evaluation results, eMBB PUSCH, Msg3 PUSCH and PUCCH format3 coverage are the bottleneck channel.

Observation 2: For Indoor scenario,

- Based on MPL evaluation results, no bottleneck channel is identified.

- Based on MIL/MCL evaluation results, eMBB PUSCH, Msg3 PUSCH and VoIP PUSCH coverage are the bottleneck channel.

Observation 3: For SubUrban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- Based on MPL evaluation results, PUCCH, Msg3 PUSCH, eMBB PUSCH and VOIP PUSCH coverage are the bottleneck channel

- Based on MIL/MCL evaluation results, eMBB PUSCH,Msg3 PUSCH and PUCCH Format3 coverage are the bottleneck channel

With observation 1, 2, 3, we have the following proposal:

Proposal 1: PUCCH and PUSCH coverage enhancement should be studied for FR2.

References

  - R1-2007480, [102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of Phase 1 - link budget template

Appendix: LLS assumptions and performance evaluation results

Table10: LLS assumptions for PUSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| Carrier | 28GHz | 28GHz | 28GHz |
| Duplex configuration | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDSU (S: 11D:3G:0U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDSU (S: 11D:3G:0U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDSU (S: 11D:3G:0U) |
| SCS | 120KHz | 120KHz | 120KHz |
| Number of antenna elements for BS | 128, (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 | 2 | 2 |
| Number of antenna elements for UE | 8, one panel:(M, N, P) = (2,2,2) | 8, one panel:(M, N, P) = (2,2,2) | 8, one panel:(M, N, P) = (2,2,2) |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| Number of DMRS | For eMBB/VoIP: 1 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 30km/h: 2 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 30km/h: 2 OS For Msg 3: 3 OS |
| Occupied channel bandwidth | 30 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 30 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 4 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 |
| Modulation | QPSK | QPSK | QPSK |
| Coding Rate | TDD config 1: eMBB: CR=379/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: eMBB: CR=308/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=379/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=379/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=308/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=308/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; |
| Channel Coding | LDPC | LDPC | LDPC |
| Channel Mode | TDL-A, 100ns | TDL-A, 100ns TDL-C, 100ns | TDL-A, 100ns TDL-C, 100ns |
| UE Mobility: | 3km/h | 3km/h for indoor, 30km/h for outdoor | 3km/h for indoor, 30km/h for outdoor |
| Channel estimation | True | True | True |
| Target BLER | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 |
| PUSCH repetition | w/o repetition | w/o repetition | w/o repetition |
| Waveform | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM |

Table11: LLS assumptions for PUCCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| Carrier | 28GHz | 28GHz | 28GHz |
| Format type | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits |
| SCS | 120KHz | 120KHz | 120KHz |
| Number of antenna elements for BS | 128, (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 | 2 | 2 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| DMRS configuration for PUCCH | w/o additional DMRS | w/o additional DMRS | w/o additional DMRS |
| Occupied channel bandwidth | 1 PRB | 1 PRB | 1 PRB |
| Channel Mode | TDL-A, 30ns | TDL-A, 100ns TDL-C, 100ns | TDL-A, 30ns TDL-C, 30ns |
| UE Mobility: | 3km/h | 3km/h for indoor, 30km/h for outdoor | 3km/h for indoor, 30km/h for outdoor |
| Channel estimation | True | True | True |
| PUCCH repetition | 1/2 | 1/2 | 1/2 |
| Frequency hopping | On | On | On |

Table12: LLS assumptions for PDSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| Carrier: | 28GHz | 28GHz | 28GHz |
| BWP | 100MHz | 100MHz | 100MHz |
| SCS | 120KHz | 120KHz | 120KHz |
| Number of antenna elements for BS | 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) |
| Number of TxRUs for BS | 2. | 2 | 2 |
| UE receive chains | 2T2R | 2T2R | 2T2R |
| length of PDCCH | 12 OS | 12 OS | 12 OS |
| Configuration of DMRS | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. |
| Data rate | 25Mbps for eMBB 320 bits with 20ms data arriving for VOIP Msg4 payload size:1040bits | 25Mbps for eMBB 320 bits with 20ms data arriving for VOIP Msg4 payload size:1040bits | 1Mbps for eMBB 320 bits with 20ms data arriving for VOIP Msg4 payload size:1040bits |
| PRBs/TBS/MCS | full bandwidth for eMBB 320 bits with 20ms data arriving for VOIP | full bandwidth for eMBB 320 bits with 20ms data arriving for VOIP | full bandwidth for eMBB 320 bits with 20ms data arriving for VOIP |
| Channel Coding | LDPC | LDPC | LDPC LDPC |
| Channel Model | TDL-A, | TDL- A, TDL-A | TDL- A, TDL-C |
| DS | 30ns | 100ns | 100ns 100ns |
| UE velocity: | 3km/h | 3km/h for indoor, 30km/h, for outdoor. | 3km/h for indoor, 30km/h, for outdoor. |
| Channel estimation | True | True | True |
| Target BLER | 10% BLER for eMBB 2% BLER for VOIP | 10% BLER for eMBB 2% BLER for VOIP | 10% BLER for eMBB 2% BLER for VOIP |
| UL DL configuration if TDD is assumed: | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) |

Table13: LLS assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Carrier | 28GHz |
| SCS | 120kHz |
| UE speed | 3kmph, 30kmph |
| Channel model | TDL-A, TDL-C |
| Aggregation level | 16 |
| Payload | 40 bits + 24CRC bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | 1-port precoder cycling |
| BLER for PDCCH | 1% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | 4 |

Table14: LLS assumptions for SSB

| Parameters | Values |
| --- | --- |
| Carrier | 28GHz |
| Periodicity | 20ms |
| SCS | 120KHz |
| UE speed | 3kmph, 30kmph |
| Channel model | TDL-A, TDL-C |
| BLER for SSB | 1% BLER |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |