3GPP TSG RAN WG1 Meeting #103-e	R1-2006455

Oct 26th – Nov 13th, 2020

Agenda Item:	8.8.1.1

Source:	IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

Title:	Baseline coverage performance for uplink in FR1

Document for:	Discussion and decision

# Motivation

In RAN#86, NR coverage enhancement was identified as one RAN work area for Rel-17 [1]. The SI requires to identify the performance targets for coverage enhancement, and study the potential solutions for coverage enhancements for the identified scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study additional enhanced solutions for FR2, if any

  - Evaluate the performance of the potential solutions based on link level simulations.

In RAN1#101e and RAN1#102e, several agreements were made regarding the parameters to be used for baseline evaluations of the Rel-15/16 5G NR to identify the bottleneck channels so that necessary enhancements could be identified for Rel-17 [2]. In this contribution, we give the baseline evaluations for the UL PUSCH channel and make several observations regarding the same. In our companion contributions [3], [4], [5] and [6] we propose way forwards to enhance the coverage performance of the UL channels.

# Baseline Link Level Evaluations for Coverage Enhancement

We consider the eMBB and VOIP settings as per the agreements in [2]. The link budget results are attached as an excel sheet to our contribution. We focus on the rural long distance TDD setting.

## VOIP evaluations

The following simulation criteria is considered in our analysis

Table 1: Simulation settings for VOIP

| Parameter | Value |
| --- | --- |
| Frequency | 4 GHz (TDD) |
| Frame structure for TDD | DDDSU only for 4GHz; No scheduling for UL in S slot. |
| BWP | 100MHz |
| SCS | 30 kHz |
| Channel Model | TDL-D, 30ns (valid for Rural) |
| UE velocity | 3 kmph |
| Frequency Hopping | No |
| Waveform | DFT-s-OFDM |
| BLER | 2% rBLER |
| HARQ | Yes |
| UE Tx | 1 |
| gNB Rx | 2 |
| Number of TxRUs for gNB | 2 receive chains simulated in LLS |
| DMRS configuration | 2 symbols, no multiplexing with data. 2nd and 11th OFDM symbols. Type-I |
| TBS size | 320 bits |
| Repetition | Type-B, 2 repetitions |
| PUSCH duration | 14 OS |
| PUSCH | Mapping type-A |
| Modulation | MCS 1; QPSK, pi/2 BPSK Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM 38.214 For pi/2 BPSK, the q=1 in the MCS table. |
| Channel Estimation | Non-Ideal (channel is estimated using appropriate methods) |

For the VOIP simulations, we generate 1 new transport block (TB) every 20ms and drop it at the end of 20ms if it is not delivered. The TDD frame structure has 8 UL slots in 20ms time frame which are used for the transmission of the voice packets. The SINR to achieve 2% rBLER from these results is used in the link budget. For these settings, the SNR was identified to be -12 dB for QPSK and -13.5 dB for pi/2 BPSK. The summary of the findings for MCL/MIL/MPL as per the templates agreed in [7] are shown below. Apart from the above assumptions for link, the following assumptions were made for the link budget evaluations –

| Parameter | Value |
| --- | --- |
| Fc | 4GHz |
| BS Height | 35m |
| UE Height | 1.5m |
| Lognormal shadow fading std deviation (dB) | 6 dB |
| Pathloss Model | Rural, LOS, O2O |
| Number of antenna elements | 64 |
| Number of RF chains in LLS | 2 |
| Antenna gain per element | 8 dBi |
| Antenna gain loss | 13 dB (60 degree away from boresight in Azimuth and 10 degree away from boresight in Elevation) |
| Cable loss | 3 dB |
| BS Noise Figure | 5 dB |
| Thermal Noise Density | -174 dBm/Hz |
| Receiver Intf Density | -174 dBm/Hz (same or below noise floor, valid for large cells, identified using SLS) |
| Receiver implementation margin (dB) | 2 dB |
| Shadow fading margin | 9.87 dB |
| HARQ gain | Considered in LLS |

Table: Parameters for VOIP link budget evaluations

| The required SNR | MCL | MIL | MPL | Range | LOS/ NLOS | O2I/ O2O | Key assumptions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -12 | 136.21 | 143.26 | 133.39 | 7.3 km | LOS | O2O | QPSK |
| -13.5 | 140.71 | 147.76 | 137.89 | 9.5 km | LOS | O2O | Pi/2 BPSK with Pmax=26 dBm, average power is 23 dBm, antenna gain correction factor is 13 dB |
| -13.5 | 143.71 | 150.76 | 140.89 | 11.3 km | LOS | O2O | Pi/2 BPSK with Pmax=29dBm, average power is 23 dBm, antenna gain correction factor is 13 dB |
| -13.5 | 140.71 | 157.76 | 147.89 | 16.9 km | LOS | O2O | Pi/2 BPSK with Pmax=26dBm, average power is 23 dBm, antenna gain correction factor is 3 dB |
| -13.5 | 143.71 | 160.76 | 150.89 | 20.1km | LOS | O2O | Pi/2 BPSK with Pmax=29dBm, average power is 23 dBm, antenna gain correction factor is 3 dB |

Table: Link evaluation for VOIP PUSCH for 320 bits

For detailed link budget evaluation along with the assumptions, please see the excel sheets attached in our contribution.

### Observations

It is clear that one way to improve the data rate is to improve the transmission power and also by employing low PAPR waveforms such as pi/2 BPSK. Specifically, we should evaluate the performance of pi/2 BPSK with power boosting up to 26 dBm for duty cycles of UL < 50% and until 29 dBm for UL duty cycles <25%, to enhance the coverage performance and cell edge data rates.

## eMBB evaluations

The following simulation criteria is considered in our analysis

Table 3: Simulation settings for eMBB

| Parameter | Value |
| --- | --- |
| Frequency | 4 GHz (TDD) |
| Frame structure for TDD | DDDSU only for 4GHz; No scheduling for UL in S slot. |
| BWP | 100MHz |
| SCS | 30 kHz |
| Channel Model | TDL-D, 30ns (valid for Rural, LOS, O2O) |
| UE velocity | 3 kmph |
| Frequency Hopping | No |
| Waveform | DFT-s-OFDM |
| BLER | 10% iBLER |
| HARQ | No |
| Repetition | No |
| UE Tx | 1 |
| gNB Rx | 2 |
| Number of TxRUs for gNB | 2 receive chains simulated in LLS |
| DMRS configuration | 2 symbols, no multiplexing with data. 2nd and 11th OFDM symbols. Type-I For pi/2 BPSK, Rel-16 DMRS is used. |
| TBS size | Chosen to achieve 100 kbps, 272 for pi2BPSK and 288 for QPSK |
| PUSCH duration | 14 OS |
| PUSCH | Mapping type A |
| Modulation | Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM 38.214 MCS 3 for QPSK MCS 1 for pi/2 BPSK |
| PRBs | 6 PRBs for pi/2 BPSK 4 PRBs for QPSK |
| Spectrum shaping | 2-tap shaping for pi/2 BPSK modulation |
| Channel Estimation | Non-Ideal (channel is estimated using appropriate methods) |

For the eMBB simulations, we generate 1 new transport block (TB) every UL slot of the DDDSU frame structure. We have not considered HARQ and repetitions for this analysis. The link level performance results are shown below. The SINR to achieve 10% iBLER from these results is -2.3 dB for QPSK and -5.95 dB for pi/2 BPSK.

Figure: Link Level Performance with simulation settings in Table-3 to achieve 100 kbps

The summary of the findings for MCL/MIL/MPL as per the templates agreed in [7] are shown below. The parameters used for the same are also shown.

| Parameter | Value |
| --- | --- |
| Fc | 4GHz |
| BS Height | 35m |
| UE Height | 1.5m |
| Lognormal shadow fading std deviation (dB) | 6 dB |
| Pathloss Model | Rural, LOS, O2O |
| Number of antenna elements | 64 |
| Number of RF chains in LLS | 2 |
| Antenna gain per element | 8 dBi |
| Antenna gain loss | 13 dB (60 degree away from boresight in Azimuth and 10 degree away from boresight in Elevation) |
| Cable loss | 3 dB |
| BS Noise Figure | 5 dB |
| Thermal Noise Density | -174 dBm/Hz |
| Receiver Intf Density | -174 dBm/Hz (same or below noise floor, valid for large cells, identified using SLS) |
| Receiver implementation margin (dB) | 2 dB |
| Shadow fading margin | 9.87 dB |
| HARQ gain | Assumed 0 dB |

Table: Parameters for eMBB link budget evaluations

| The required SNR | MCL | MIL | MPL | Range | LOS/ NLOS | O2I/ O2O | Key assumptions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -2.31 | 129.53 | 136.58 | 126.71 | 5 km | LOS | O2O | QPSK |
| -5.95 | 134.41 | 141.46 | 131.59 | 6.6 km | LOS | O2O | Pi/2 BPSK with Pmax=26, average power is 23 dBm, antenna gain correction factor is 13 dB |
| -5.95 | 137.41 | 144.46 | 134.59 | 7.8 km | LOS | O2O | Pi/2 BPSK with Pmax=29dBm, average power is 23 dBm, antenna gain correction factor is 13 dB |
| -5.95 | 134.41 | 151.46 | 141.59 | 11.8 km | LOS | O2O | Pi/2 BPSK with Pmax=26, average power is 23 dBm, antenna gain correction factor is 3 dB |
| -5.95 | 137.41 | 154.46 | 144.59 | 14 km | LOS | O2O | Pi/2 BPSK with Pmax=29dBm, average power is 23 dBm, antenna gain correction factor is 3 dB |

Table: Link evaluation for eMBB PUSCH for 100 kbps

### Observations

One of the ways to improve the data rate is to improve the transmission power and also by employing low PAPR waveforms such as pi/2 BPSK. Specifically, we should evaluate the performance of pi/2 BPSK with power boosting up to 26 dBm for duty cycles of UL < 50% and until 29 dBm for UL duty cycles <25%, to enhance the coverage performance and cell edge data rates.

We also present several enhancements in [3], [4], [5] and [6] that should be used for NR coverage improvement.

# Baseline Multi-cell System Simulation

In this section, we show the system level evaluations in the form of SINR CDF and spectral efficiency/ data rate calculations for the rural setup. We specifically show the benefits of employing the pi/2 BPSK modulation with power boosting as per the frame structure.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Antennas at BS | 8×4×2= 64 antenna elements for fc=4 GHz |
| Tx RUs per sector | 8 |
| Inter-site Distance | 30 Km |
| Antenna at UE | 1 |
| Pathloss Model | RMa-LMLC [ITU M.2412] |
| Fast Fading Model | RMa [3GPP TR 38.901] |
| System Bandwidth | 10 MHz |
| User Density | 10 per sector |
| User Distribution | 40% indoor, 40% outdoor, 20% outdoor in car |
| Mechanical/Electrical Tilt | 90 in GCS/92 in LCS |
| BS/UE Height | 35 m/1.5 m |
| UE transmit power | 23 dBm |
| BS/UE antenna gain | 8 dBi/0 dBi |
| BS Noise Figure | 3 dB |
| Resource allocation | Equal allocation |
| Receiver Type | MMSE |
| Uplink Power Control | P0=-118 dBm, alpha=0.9 |
| Interference Modelling | Explicit |
| CQI Tables | For the current scheme, existing CQI tables in Rel-15/Rel-16 are used. For the proposed scheme, enhanced CQI tables with lower code rates are used. |

Table 3: SLS Setup

To support lower SNRs encountered in large cell scenarios, we propose a new CQI table as shown below

| CQI | Modulation | code-rate × 1024 | Efficiency |
| --- | --- | --- | --- |
| 1 | QPSK/BPSK | 11/q | 0.0107 |
| 2 | QPSK/BPSK | 14/q | 0.0136 |
| 3 | QPSK/BPSK | 21/q | 0.0205 |
| 4 | QPSK/BPSK | 36/q | 0.0351 |
| 5 | QPSK/BPSK | 63/q | 0.0615 |
| 6 | QPSK/BPSK | 106/q | 0.1036 |
| 7 | QPSK/BPSK | 156/q | 0.1523 |
| 8 | QPSK/BPSK | 240/q | 0.2344 |
| 9 | QPSK/BPSK | 386/q | 0.3770 |
| 10 | QPSK/BPSK | 716/q | 0.6016 |
| 11 | QPSK/BPSK | 898/q | 0.8770 |
| 12 | QPSK/BPSK | 1204/q | 1.1758 |
| 13 | 16QAM | 378 | 1.4766 |
| 14 | 16QAM | 490 | 1.9141 |
| 15 | 16QAM | 616 | 2.4063 |

Table: Enhanced CQI table for coverage enhancement

The SLS results with the proposed enhancements using pi/2 BPSK modulation scheme are shown below.  In the results below, the current scheme refers to the evaluations done using the current CQI table in Rel-15/Rel-16 specifications. The results using the proposed scheme refers to the evaluations done using the proposed CQI table. Whenever pi/2 BPSK modulation is used, Pcmax is allowed to be boosted to 26 dBm and 29 dBm based on the UL duty cycle.

Figure: CDF of UL SINR and UL Throughput with ISD=30 Km and 40% duty cycle in the frame structure for UL. For a 40% duty cycle case, proposed scheme uses Ptx=26 dBm with pi/2 waveform

| Metric | fc=4GHz |  |
| --- | --- | --- |
|  | Current | Proposed |
| 5% UE throughput | 12.26 Kbps | 21.00 Kbps |
| 50% UE throughput | 74.47 Kbps | 142.2 Kbps |
| Mean UE throughput | 83.75 Kbps | 149.2 Kbps |
| Mean Cell throughput | 837.5 Kbps | 1.492 Mbps |

Table 4: Comparison with the current and proposed methods for coverage enhancement - 40% duty cycle

Below we show results when the UL occupancy is 20% of the entire frame structure i.e., 20% duty cycle. In this case, the Pcmax, for the case of pi/2 BPSK can be allowed to go to 29dBm while the average still remains at 23 dBm.

Figure: CDF of UL SINR and UL Throughput with ISD=30 Km and 20% duty cycle in the frame structure for UL. For a 20% duty cycle case, proposed scheme uses Ptx=29 dBm with pi/2 waveform.

| Metric | fc=4GHz |  |
| --- | --- | --- |
|  | Current | Proposed |
| 5% UE throughput | 6.130 Kbps | 21.79 Kbps |
| 50% UE throughput | 37.23 Kbps | 100.4 Kbps |
| Mean UE throughput | 41.87 Kbps | 104.9 Kbps |
| Mean Cell throughput | 418.7 Kbps | 1.049 Mbps |

Table 4: Comparison with the current and proposed methods for coverage enhancement - 20% duty cycle

## Observations

For ISD=30 Km, in case of 40% duty cycle, with Ptx=26 dBm, we observe 2x gains and in case of 20% duty cycle, with Ptx=29 dBm, we observe close to 4x gains. As seen, new enhanced code rates should be supported to enhance the coverage. One such solution in the proposed new CQI tables.

Note that, and ISD of 30km at 4 GHz carrier frequency in rural areas is a substantial improvement over the existing releases.

# References

[1] RAN1 Chairman’s Report, RAN1#101e, June 2020.

[2] RP-193240, New SID on NR coverage enhancement

[3] R1-2006456, PUSCH coverage enhancements, IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

[4] R1-2006457, PUCCH coverage enhancements, IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

[5] R1-2007905, PUSCH coverage enhancements, IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

[6] R1-2007906, Coverage enhancements for Msg3, IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

[7] RAN1 Chairman’s Report, RAN1#102e, August 2020.