3GPP TSG RAN WG1 #103-e		R1-2007922

e-Meeting, October 26th – November 13th, 2020

Title: 	Remaining issues in Mode-1

Source: 	ZTE, Sanechips

Agenda item:	7.2.4

Document for:	Discussion and decision

Introduction

The remaining Mode 1 issues are discussed in this contribution, including HARQ report, HARQ RTT, grants overlapping and type 1 codebook.

Discussion

HARQ report

For sidelink HARQ report, a pending discussion in last RAN1 meeting is shown as follow:

Whether to support exceptional reports to the gNB, especially for the case where nothing is transmitted for a DG?

Email discussion on Issue 1.3-2: Are there are other exceptional reports to the gNB (e.g., nothing to transmit for DG, etc.)? - Yes (please provide details) FL summary: The issues described by the companies supportive of specifying further exceptional cases, do not result in a complete breakdown of the system. If they happen, they only result in some inefficient operation. Some of the supporters have even stated that these are optimizations. - No further specification is necessary FL Proposed conclusion: - No additional exceptional reports to the gNB are specified at this point. Final conclusion: No agreement. The discussion continues to next meeting.

In our opinion, in case of no transmission for a DG, option B is preferred due to two aspects:

Firstly, when the sidelink DG is received with NDI toggled, the Tx UE use the received sidelink grant to determine PSCCH duration(s) and PSSCH duration(s) for initial transmission and, if available, retransmission(s) of a single MAC PDU. It is unnecessary to skip the SL transmission because UE can decide some link adaptive Tx parameters such as MCS, DMRS,MIMO by itself. After that, the UE will generate HARQ feedback according to the reception of PSFCH with corresponding granted PSSCH transmission. And no further specification is necessary.

Secondly, when the sidelink DG is received with NDI not toggled, nothing to transmit on the DG means that the corresponding MAC PDU has been positively acknowledged on sidelink, but unfortunately gNB determines a NACK, then a DG for retransmission is received by the Tx UE, but the Tx UE would skip this grant. In this case, the Tx UE will determine ACK with the former information of initial transmission of the same TB, this can be covered by the current spec and also no further specification is necessary.

In all, it can conclude that no further specification is necessary for sidelink HARQ report.

No further specification is necessary for sidelink HARQ report.

HARQ RTT

To avoid resource waste for retransmission in case the former transmission has been successfully transmitted, it has been agreed when sidelink HARQ feedback is enabled, gNB should guarantee that the minimum time gap between PSFCH reception and next scheduled PSCCH/PSSCH retransmission is Tpre +delta(ms). Tpre has been defined as the minimum time gap between a last PSFCH reception occasion and the PUCCH resources or PUSCH resources to report HARQ-ACK information. For sidelink HARQ report, Tpre has considered all the factors such as PSFCH reception, HARQ codebook generation, Rx-Tx change, etc. For the minimum time gap between PSFCH reception and next scheduled PSCCH/PSSCH retransmission, the following factors should be taken into account: PSFCH reception, Rx-Tx change, PSCCH/PSSCH retransmission preparation time. In which, PSCCH/PSSCH retransmission preparation time could be much smaller than that of HARQ codebook generation. As an implementation, after the first PSCCH/PSSCH transmission, the preparation of retransmission has been nearly completed and just wait for transmission, it will not consume much time. So taking Tpre as the minimum time between PSFCH reception and next scheduled PSCCH/PSSCH retransmission seems sufficient.

“delta=0” is supported for minimum time gap between PSFCH reception and next scheduled PSCCH/PSSCH retransmission

Another issue is how to define the conditions of application the HARQ RTT minimum time, there are two options to be discussed for the applicability:

- Option A. The gNB provides a grant for transmission in a pool with PSFCH resources.

- Option B. SL HARQ feedback is used

For option B, gNB needs additional information to decide whether SL HARQ is enabled, so for simplicity, option A is preferred.

gNB applies HARQ RTT limitation when providing a grant for transmission in a pool with PSFCH resources

Whether it is necessary to clarify that up to one SL transmission is scheduled or configured in a SL slot in mode 1

One mode 1 issue not treated in last RAN1 meeting is "whether it is necessary to clarify that up to one SL transmission is scheduled or configured in a SL slot in Mode 1" [2]. Note that, per specification wise, the limitation of up to one PSSCH transmission per slot (e.g., due to Pc,max applicable to only one PSSCH per slot) is different from the limitation of up to one PSSCH scheduling/grant from the gNB for one SL slot.

When multiple resource pools are configured for a mode 1 UE and PSSCH resources in some of resource pools are FDMed in the same slot, as shown in Figure 1, it should be clarified that whether a UE can receive multiple PSSCH schedulings/grants from the gNB for SL slot n.

Figure 1 FDMed PSSCH in two resource pool

With the consensus that one UE can transmit at most one PSSCH in one sidelink slot, there is a question on whether the multiple grants to a UE can allow more than one granted transmission occasions/resources in a same slot. If multiple grants can assign transmission occasions in the same slot but UE can only use one of them to perform transmission, some regulations should be defined to ensure the gNB and UE would not have different understanding regarding to the granted PSSCH dropping. In the SL PHY procedure agenda, selection of one SL transmission from multiple transmission candidates always involves priority comparison, which is transparent to gNB. In a word, with the time-overlapping grants, gNB may gain less after a dropping of PSSCH, but the specification still needs to define the rule of PSSCH dropping (e.g. based on the similar rule in current SL prioritization).

In RAN1 #102-e meeting, mode 2 had discussed how to restrict selection of resources for different TBs to different slots, assuming this is resolved in MAC layer, then it is better for mode 1 UE to has the same restriction. In order to avoid the situation to perform prioritization between SL grants, it is suggested that the gNB would not assign multiple grants to a UE where the multiple grants can have overlapping PSSCH transmission in a slot.

UE does not expect to receive multiple grants that can assign more than one PSCCH/PSSCH transmissions in a same slot. A corresponding 38.214 TP is given below.

- 8.1.2 Resource allocation In sidelink resource allocation mode 1: - for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.3 of [6, TS 38.213]. - a UE does not expect that more than one PSCCH/PSSCH transmission in a same slot are either scheduled or configured by SL grants across configured transmission resource pools on a carrier.

Corrections for Type-1 codebook

In this section three points for corrections for type-1 codebook are given respectively, and lastly a final TP is provided.

- Point 1:

In sidelink physical procedure , it has been agreed that the number of logical slots in a resource pool within 10240ms period is not limited to be an integer multiple of the PSFCH resource period, and for the location of PSFCH slots where the PSFCH resource period is N, Logical slot index #0, #N, #2N, …, within 10240 ms period is used [1]. When the number of logical slots in a resource pool within 10240ms happen to be not an integer multiple of the PSFCH resource period, the PSSCH transmission occasions corresponding to the first PSFCH slot with 10240ms is no longer N. In this case if we still generate type-1 codebook according to PSFCH resource period N, there will be some confusion because we cannot find N PSSCH transmission occasions corresponding to the first PSFCH slot with 10240ms. So type-1 codebook should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

Type 1 codebook should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

- Point 2:

In current 38.213, multiple resource pools are allowed to be configured, but the set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions only consider one resource pool but the procedure in the specification does not distinguish which resource pool is used.

- 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool …… The cardinality of the set MA defines a total number M of PSFCH reception occasions corresponding to the HARQ-ACK information bits. ……

From the yellow marked text above, it seems sub-bullet c) defines multiple resource pools with a set of sidelink resource pool bitmaps, but just only one but unknown which one resource pool is used to generate the set MA in the following pseudo-code and the HARQ-ACK information bits, this is not correct when multiple resource pools with different PSFCH period are configured.

In current 38.213 specification when multiple resource pools are configured, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits

All the configured resource pools should be considered for Type-1 codebook generation

- Point 3:

As analyzed in section 2.3, a UE just performs one SL PSCCH/PSSCH transmission in one specific resource pool according to the grant for a SL slot even there are multiple resource pools configured in this slot. If PSSCH transmission occasions in different resource pools overlap in the same slot and associate with the same PUCCH/PUSCH slot, it may need not to generate all the HARQ feedback information to the PSSCH transmission occasions in the slot, because HARQ feedback of the overlapped but unused PSSCH occasions must be NACK, let us take slot n in figure 1 in section 2.3 as example to analyze the HARQ feedback information case by case:

- Case1: If resource pool 1 has a grant and the corresponding PSSCH transmits correctly, then ACK is generated for resource pool 1 and NACK is generated for resource pool 2

- Case2: If resource pool 2 has a grant and the corresponding PSSCH transmits correctly, then ACK is generated for resource pool 2 and NACK is generated for resource pool 1

- Case3: If there is no grant in slot n, then NACK is generated for both resource pool 1 and resource pool 2

- Case4: If resource pool 1 has a grant and the corresponding PSSCH transmits not correctly(or the grant is lost), then NACK is generated for both resource pool 1 and resource pool 2

- Case5: If resource pool 2 has a grant and the corresponding PSSCH transmits not correctly(or the grant is lost), then NACK is generated for both resource pool 1 and resource pool 2

As can be seen that in all the cases above, there is a NACK information generated, which comes from the unused resource pool, that NACK can be seen as a redundant information, so we just need to generate only one HARQ feedback information for one candidate slot for type 1 HARQ codebook generation, regardless of whether there are overlapped PSSCH transmission occasions from multiple resource pool in the same slot, that means if 8 FDMed resource pools are configured in one SL slot, only one HARQ feedback bit is generated, rather than 8 bits, so that the HARQ feedback overhead can be reduced.

When PSSCH resources in multiple resource pools are FDMed in the same slot, there will be only one HARQ feedback information generated for the slot when generating the type 1 HARQ codebook.

- Proposed TP:

- Based on the analysis in this section above, a text proposal is provided for type-1 codebook correction.

For corrections of Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0 -index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual If the SL slot corresponding to nF has not been counted MA=MA∪j; j=j+1; nF=nF+1; end end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while

Conclusion

This paper concludes with the following proposals and observations:

Observation 1:	In current 38.213 specification when multiple resource pools are configured, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits

Proposal 1: No further specification is necessary for sidelink HARQ report.

Proposal 2: "delta=0" is supported for minimum time gap between PSFCH reception and next scheduled PSCCH/PSSCH retransmission

Proposal 3: gNB applies HARQ RTT limitation when providing a grant for transmission in a pool with PSFCH resources

Proposal 4: UE does not expect to receive multiple grants that can assign more than one PSCCH/PSSCH transmissions in a same slot. A corresponding 38.214 TP is given below.

- 8.1.2 Resource allocation In sidelink resource allocation mode 1: - for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.3 of [6, TS 38.213]. - a UE does not expect that more than one PSCCH/PSSCH transmission in a same slot are either scheduled or configured by SL grants across configured transmission resource pools on a carrier.

Proposal 5: Type 1 codebook should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

Proposal 6: All the configured resource pools should be considered for Type-1 codebook generation

Proposal 7: When PSSCH resources in multiple resource pools are FDMed in the same slot, there will be only one HARQ feedback information generated for the slot when generating the type 1 HARQ codebook.

Proposal 8: For corrections of Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0 -index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual If the SL slot corresponding to nF has not been counted MA=MA∪j; j=j+1; nF=nF+1; end end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while

References

- Chairman note RAN1#102-e

- R1-2006961 Feature lead summary#2 on Resource allocation for NR sidelink Mode 1