3GPP TSG RAN WG1 #103-e	R1-2007928

e-Meeting, October 26th – November 13th, 2020

Agenda item:		8.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:		Simulation Results for NR from 52.6 GHz to 71 GHz

Document for:		Discussion and Decision

# Introduction

This contribution captures the LLS and SLS simulation results for the NR > 52.6 GHz study item [1] consistent with the simulations assumptions captured in the draft TR [2].

# Link level evaluation results

LLS performance results were produced for PDSCH BLER with and without ICI.

Some observations:

- WITH CPE compensation, 120 or 240kHz cannot be used for 64-QAM, and 960kHz performs the best

- With CPE compensation, 480kHz suffers 0.7dB – 1.3dB loss compared to 960kHz for MCS 22 depending on the channel. This loss is due to inter-carrier interference caused by phase noise. Note that although here we only compare 400MHz bandwidth due to the assumption of maximum FFT size of 4096, this performance difference becomes higher if we use larger bandwidth, because it would make ICI effect even more significant.

- CPE compensation is enough and in fact the best method for 960kHz for smaller bandwidth, because it does not suffer inter-carrier interference. Using ICI compensation does not provide any gain, and can even deteriorate the performance. However, for 2GHz bandwidth, there is some gain from ICI compensation.

- ICI compensation is truly beneficial only for lower SCS such 120,240 and 480kHz when higher-order modulation is used (such as 64-QAM)

- It is also observed that in the ICI compensation algorithm, filter length has a clear impact on the comparison between different SCSs, because filter length should be optimized according to the SCS and/or allocation bandwidth

- NCP is enough up to 960kHz SCS at least up to 20ns RMS delay spread

Table B.2.7-1: LLS template: SINR in dB achieving PDSCH BLER of 10% /1%

| Tdoc / Source | MCS | Channel | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007928 / Source 1 | 7 | TDL-A, 5ns | 3.2/ 6 (X for 10% BLER, Y for 1% BLER) | 3.1/5.6 | 3.1/5.8 | 3.2/5.4 | 1.9/3.4 |
|  |  | TDL-A, 10ns | 2.7/4.7 | 2.6/4.7 | 2.6/4.6 | 2.6/4.9 | 2/3.2 |
|  |  | TDL-A, 20ns | 2.3/4.2 | 2.3/4.1 | 2.4/4.2 | 2.6/4.5 | 1.8/3.1 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 16 | TDL-A, 5ns | 11.9/14.6 | 11.4/14 | 11/13.3 | 10.9/13.4 | 9.9/11.4 |
|  |  | TDL-A, 10ns | 11.2/13.4 | 10.7/13 | 10.3/12.2 | 10.3/12.5 | 9.8/11.2 |
|  |  | TDL-A, 20ns | 11.1/13.3 | 10.4/12.1 | 10.1/11.9 | 10.4/12.5 | 9.9/11.5 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 22 | TDL-A, 5ns | inf/inf | inf/inf | 17.5/20.9 | 16.2/18.6 | 16.2/19.0 |
|  |  | TDL-A, 10ns | inf/inf | inf/inf | 17.1/20.7 | 16.1/18.2 | 16.1/18.2 |
|  |  | TDL-A, 20ns | inf/inf | inf/inf | 16.8/19.3 | 16.1/17.9 | 16.3/18.8 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  | Additional report/notes: - CP type normal - antenna configuration for CDL model - waveform in case of PUSCH - PTRS configuration every 2nd RB (CPE compensation) - DMRS configuration front-loaded - any optional or other assumption/parameters used not as in the baseline |  |  |  |  |  |  |

Table B.1.1.7-2: LLS template: SINR in dB achieving PDSCH BLER of 10% /1% with ICI u=1

| Tdoc / Source | MCS | Channel | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007928 / Source 7 | 7 | TDL-A, 5ns | 3.3/ 5.8 | 3.3/6.1 | 3.7/6.4 | 4.4/7.3 | 2.2/3.8 |
|  |  | TDL-A, 10ns | 2.6/4.5 | 2.7/5.1 | 3.3/5.7 | 4.0/6.2 | 2.2/3.8 |
|  |  | TDL-A, 20ns | 2.4/4.2 | 2.5/4.4 | 2.9/5.3 | 3.9/6.4 | 2.1/3.4 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 16 | TDL-A, 5ns | 11.2/14.1 | 11.1/13.2 | 11.1/13.4 | 11.4/13.9 | 9.9/11.7 |
|  |  | TDL-A, 10ns | 10.6/13.0 | 10.3/12.4 | 10.4/12.1 | 11.0/13.0 | 9.8/11.5 |
|  |  | TDL-A, 20ns | 10.1/12.3 | 9.9/11.8 | 10.1/12.0 | 11.1/13.3 | 9.7/11.2 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 22 | TDL-A, 5ns | 19.9/inf | 16.9/19.9 | 16.4/18.6 | 16.5/19.0 | 15.1/ 16.8 |
|  |  | TDL-A, 10ns | 19.3/inf | 16.4/18.7 | 16.0/17.9 | 16.2/18.5 | 15.2/16.9 |
|  |  | TDL-A, 20ns | 19.0/inf | 16.1/17.9 | 15.6/17.7 | 16.3/18.4 | 15.0/16.7 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  | Additional report/notes: - CP type normal - antenna configuration for CDL model - waveform in case of PUSCH - PTRS configuration every 2nd RB (ICI compensation,u=1) - DMRS configuration front-loaded - any optional or other assumption/parameters used not as in the baseline |  |  |  |  |  |  |

Table B.1.1.7-3: LLS template: SINR in dB achieving PDSCH BLER of 10% /1% with ICI u=2

| Tdoc / Source | MCS | Channel | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007928 / Source 7 | 7 | TDL-A, 5ns | 3.4/ 6.0 | 3.5/6.2 | 4.3/7.0 | 5.3/8.3 | 2.4/4.2 |
|  |  | TDL-A, 10ns | 2.7/5.0 | 3.1/5.2 | 3.7/6.1 | 5.0/7.6 | 2.3/3.8 |
|  |  | TDL-A, 20ns | 2.5/4.5 | 2.8/4.8 | 3.7/5.6 | 4.9/8.1 | 2.3/4.0 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 16 | TDL-A, 5ns | 11.0/13.7 | 11.1/13.6 | 11.3/14.0 | 12.2/15.4 | 10.1/11.9 |
|  |  | TDL-A, 10ns | 10.2/12.7 | 10.7/13.1 | 10.8/13.1 | 11.8/14.2 | 10.0/11.9 |
|  |  | TDL-A, 20ns | 10.1/12.2 | 10.1/12.2 | 10.6/12.4 | 11.7/14.1 | 9.8/11.4 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 22 | TDL-A, 5ns | 18.1/21.7 | 16.6/19.4 | 16.4/18.8 | 17.1/20.0 | 15.1/17.3 |
|  |  | TDL-A, 10ns | 17.4/20.5 | 16.3/18.6 | 16.1/18.3 | 16.6/19.1 | 15.2/16.8 |
|  |  | TDL-A, 20ns | 17.1/19.8 | 15.9/17.5 | 16.0/17.8 | 16.9/19.3 | 15.1/16.7 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  | Additional report/notes: - CP type normal - antenna configuration for CDL model - waveform in case of PUSCH - PTRS configuration every 2nd RB (ICI compensation,u=2) - DMRS configuration front-loaded - any optional or other assumption/parameters used not as in the baseline |  |  |  |  |  |  |

Table B.1.1.7-4: LLS template: SINR in dB achieving PDSCH BLER of 10% /1% with ICI u=3

| Tdoc / Source | MCS | Channel | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007928 / Source 7 | 7 | TDL-A, 5ns | 3.6/6.1 | 3.9/6.5 | 4.8/7.6 | 6.5/9.7 | 2.6/4.5 |
|  |  | TDL-A, 10ns | 2.9/5.1 | 3.2/5.2 | 4.3/6.8 | 6.2/8.8 | 2.6/4.4 |
|  |  | TDL-A, 20ns | 2.6/4.5 | 3.1/5.0 | 4.0/6.2 | 6.2/9.1 | 2.5/4.2 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 16 | TDL-A, 5ns | 11.2/13.9 | 11.0/14.3 | 11.6/14.1 | 13.1/16.1 | 10.2/12.2 |
|  |  | TDL-A, 10ns | 10.4/12.8 | 10.5/13.3 | 11.2/13.6 | 12.9/15.5 | 10.1/11.8 |
|  |  | TDL-A, 20ns | 10.0/12.1 | 10.3/12.4 | 11.0/13.0 | 12.9/15.7 | 10.0/11.6 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-D, 20ns |  |  |  |  |  |
|  |  | CDL-D, 30ns |  |  |  |  |  |
|  | 22 | TDL-A, 5ns | 17.5/21.6 | 16.6/19.5 | 16.8/19.5 | 18.2/21.0 | 15.2/16.7 |
|  |  | TDL-A, 10ns | 17.1/19.8 | 16.2/18.1 | 16.4/18.5 | 17.6/20.3 | 15.3/16.9 |
|  |  | TDL-A, 20ns | 16.7/19.1 | 15.9/17.5 | 16.2/18.3 | 18.2/21.0 | 15.2/16.9 |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  |  | CDL-B, 20ns |  |  |  |  |  |
|  |  | CDL-B, 50ns |  |  |  |  |  |
|  | Additional report/notes: - CP type normal - antenna configuration for CDL model - waveform in case of PUSCH - PTRS configuration every 2nd RB (ICI compensation,u=3) - DMRS configuration front-loaded - any optional or other assumption/parameters used not as in the baseline |  |  |  |  |  |  |

### B.1.2       Evaluation results for PRACH

LLS performance results were produced for PRACH preamble midsection probability of 1% and corresponding to false alarm probability.

Some observations:

- Performance is dependent on selected Ncs

- Time offset also has impact on results but it is smaller than Ncs impact

- Larger subcarrier spacing will likely need higher Ncs

- PN model has relatively smaller impact on performance

Table B.1.2-7-1: LLS template: SINR in dB achieving PRACH preamble misdetection probability of 1% and corresponding false alarm probability

| Tdoc / Source | Channel | 120KHz | 240KHz | 480KHz | 960KHz |
| --- | --- | --- | --- | --- | --- |
| R1-2007928 / Source 1 | TDL-A, 5ns | -9.6 | -9.5 | -9.4 | -9.9 |
|  | TDL-A, 10ns | -10.3 | -10.1 | -9.8 | -9.8 |
|  | TDL-A, 20ns | -9.5 | -8.9 | -8.8 | -9.5 |
|  | CDL-B, 20ns |  |  |  |  |
|  | CDL-B, 50ns |  |  |  |  |
|  | CDL-D, 20ns |  |  |  |  |
|  | CDL-D, 30ns |  |  |  |  |
|  | Additional report/notes: 1. PRACH format A3, L=139 2. Ncs={13,13,69,69} for each SCS, TDL-A,20ns with 240kHz also with Ncs=69. 3. antenna configuration for CDL model 4. Agreed PN model used in TX and RX side. Time offset assumption of 0-0.385us corresponding to ISD of 0-100m. False alarm probability of 0.1%. |  |  |  |  |

# Simulation Results & Discussion

The following SLS results are produced. Throughput results for DL-only traffic at low, medium and high load points for omnidirectional LBT, directional LBT and without any LBT in indoor scenario A are shown in Fig. 1.

Some observations:

- when compared to a system without LBT, omnidirectional LBT degrades DL and UL throughput with increasing load, and roughly 25% loss in median throughput can be observed at high load. Correspondingly, omnidirectional LBT increases transmission delays at medium and high load points.

- at low load, omnidirectional LBT can provide roughly the same 5%ile and median throughput as a system without LBT and even provide some UL throughput improvement at high percentile.

- in the case with DL-only traffic, both omnidirectional and directional LBT cause roughly 10-20% performance degradation at 50%ile and high percentile points with medium and high load. At 5%ile point, directive LBT provides roughly 10% TP improvement at low and medium load but causes some 5%ile TP loss at high load.

Table B.2.7-1: System level evaluation results for indoor scenario A

| Tdoc / Source | Cases |  | Case 1 (DL:UL 50:50, No-LBT) |  |  | Case 2 (DL:UL 50:50, Omni-LBT) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007928 / Source 1 | Traffic load Metrics |  | Low load | Medium load | High load | Low load | Medium load | High load |
|  | DL UPT (Mbps) | 5%ile | 2362 | 1187 | 631 | 2344 | 987 | 490 |
|  |  | 50%ile | 9137 | 5266 | 3083 | 8720 | 4391 | 2313 |
|  |  | 95%ile | 19548 | 16680 | 13333 | 18462 | 14979 | 11405 |
|  |  | mean | 9857 | 6843 | 4444 | 9280 | 5767 | 3544 |
|  | DL delay (ms) | 5%ile | 11 | 13 | 16 | 12 | 14 | 19 |
|  |  | 50%ile | 24 | 41 | 70 | 25 | 49 | 93 |
|  |  | 95%ile | 91 | 181 | 341 | 92 | 219 | 440 |
|  |  | mean | 34 | 61 | 111 | 35 | 74 | 143 |
|  | UL UPT (Mbps) | 5%ile | 1528 | 870 | 508 | 1484 | 682 | 342 |
|  |  | 50%ile | 4080 | 3198 | 2246 | 4041 | 2723 | 1721 |
|  |  | 95%ile | 7654 | 6968 | 6175 | 9643 | 6704 | 5456 |
|  |  | mean | 4297 | 3527 | 2723 | 4207 | 3107 | 2218 |
|  | UL delay (ms) | 5%ile | 28 | 31 | 35 | 22 | 32 | 40 |
|  |  | 50%ile | 53 | 68 | 96 | 53 | 79 | 125 |
|  |  | 95%ile | 141 | 247 | 424 | 144 | 317 | 630 |
|  |  | mean | 65 | 94 | 146 | 66 | 112 | 189 |
|  | Arrival rate (files/s) |  | 1.5 | 2.75 | 4 | 1.5 | 2.75 | 4 |
|  | BO |  | 25% | 54% | 78% | 25% | 58% | 82% |
|  | Additional report/notes: - LBT Procedures: No-LBT, Omni-LBT - Cases: - Case 1: No-LBT, DL:UL 50:50 - Case-2: Omni-LBT, DL:UL 50:50 |  |  |  |  |  |  |  |

Table B.2.7-2: System level evaluation results for indoor scenario A

| Tdoc / Source | Cases |  | Case 3 (No-LBT, DL:UL 100:0) |  |  | Case 4 (Omni-LBT, DL:UL 100:0) |  |  | Case 5 (Directional-LBT, DL:UL 100:0) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007928 / Source 1 | Traffic load Metrics |  | Low load | Medium Load | High Load | Low Load | Medium Load | High Load | Low Load | Medium Load | High Load |
|  | DL UPT (Mbps) | 5%ile | 2627 | 902 | 412 | 2388 | 861 | 350 | 2748 | 1050 | 395 |
|  |  | 50%ile | 9630 | 5859 | 3022 | 8675 | 4954 | 2466 | 9122 | 5352 | 2669 |
|  |  | 95%ile | 19548 | 17116 | 13723 | 17575 | 15021 | 11576 | 17955 | 15201 | 12067 |
|  |  | mean | 10204 | 7072 | 4455 | 9178 | 6076 | 3669 | 9596 | 6441 | 3930 |
|  | DL delay (ms) | 5%ile | 11 | 13 | 16 | 12 | 14 | 19 | 12 | 14 | 18 |
|  |  | 50%ile | 22 | 37 | 72 | 25 | 44 | 88 | 24 | 40 | 81 |
|  |  | 95%ile | 82 | 239 | 524 | 90 | 251 | 617 | 78 | 205 | 546 |
|  |  | mean | 31 | 69 | 139 | 35 | 77 | 166 | 31 | 65 | 151 |
|  | Arrival rate (files/s) |  | 2.0 | 3.5 | 5.0 | 2.0 | 3.5 | 5.0 | 2.0 | 3.5 | 5.0 |
|  | BO |  | 23% | 56% | 84% | 26% | 58% | 89% | 24% | 58% | 92% |
|  | Additional report/notes: - LBT Procedures: No-LBT, Omni-LBT and Directional LBT - Cases: Case 1: No-LBT, DL:UL 100:0 Case 2: Omni-LBT, DL:UL 100:0 Case 3: Directional-LBT, DL:UL 100:0 |  |  |  |  |  |  |  |  |  |  |

Figure 1: DL throughput per packet for DL-only traffic.

# Conclusion

This contribution provides simulation results for the indoor office environment consistent with the criteria

# References

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71 GHz”, Intel Corporation

- R1-2007340, “Draft TR 38.808 v002: Study on supporting NR from 52.6 GHz to 71 GHz”, Moderator (Intel Corporation)