3GPP TSG RAN WG1 Meeting #103-e																R1-2007943

e-Meeting, October 26 – November 13, 2020

Source:			Intel Corporation

Title:					Considerations on performance evaluation for NR in 52.6-71GHz

Agenda item:	8.2.3

Document for:	Discussion and Decision

Introduction

At RAN#86 the new Study Item on supporting NR from 52.6 GHz to 71 GHz was approved [1]. It was decided to study the feasibility of NR operation in the frequency range using the waveforms already supported in NR Rel-16. However, a successful application of current NR waveforms in such high carrier frequency requires careful selection of the OFDM numerology, which should account for both state-of-the-art hardware impairments and the channel propagation conditions in 52.6–71GHz band. It is also expected that the design of NR signals/channels may require modifications (e.g. to comply with the regulatory requirements [2]), which can have a substantial impact on system performance.

In order to make a reasonably detailed evaluation of the NR performance considering the above-mentioned conditions, both link-level and system-level simulations are required. In this contribution, we discuss the metrics we use to evaluate NR performance in 52.6–71GHz band and propose updates to the link- and system-level simulation assumptions for use in the SI.

System-Level Evaluation Methodology

Discussion on metrics to evaluate

Our system-level evaluations in 52.6–71GHz band are focused on channel propagation conditions and the ability of an OFDM system to operate in those. One challenge for an OFDM system with a large bandwidth and a high carrier frequency is reduced CP length, which may result in an increase of intersymbol interference level. In this section, we discuss the metrics we use to assess intersymbol interference impact on an OFDM system.

### RMS delay spread

The common method to indirectly assess intersymbol interference level is to compare a CP length with a channel delay spread statistics at the receiver. The root mean square delay spread can be calculated per each serving link as follows:

,

where	Hn	is a complex channel coefficient for each cluster n (also referred as complex channel tap)

τn	is a cluster delay for each cluster n

N	is a number of clusters (channel taps) in a channel impulse response (CIR)

The details of complex channel taps generation can be found in [4, Section 7.5].

Please note, that in order to compute the effective delay spread experienced by a receiver, the channel tap magnitude should account for a beamforming applied at both transmitter and receiver. Therefore, the channel tap magnitude should be calculated from the complex channel tap generated at Step 12 of fast fading modelling, not borrowed from a power delay profile generated at Step 6. Please also note, that in the case of Doppler spread modelling the squared magnitude of a channel tap should be averaged over time.

Proposal 1:

- Use root mean square effective channel delay spread at the receiver as a metric for system-level evaluation of NR in 52.6–71GHz

### Intersymbol interference SIR

The more precise way to assess the intersymbol interference impact on OFDM performance is to calculate a signal-to-interference energy ratio over the receiver FFT window. This method allows to set a clear threshold for an acceptable intersymbol interference level (e.g. based on the supported SINR range). Given the ISI threshold and the ISI statistics collected on system level per each serving link for different OFDM numerologies, it is possible to select the numerologies that can support an operation in 52.6–71GHz band based on the clear data.

Given that ISI is independent of the noise and the other types of interference, it is desired to ensure the ISI SIR level is higher than the operating SINR range. We assume QAM64 could be the highest order modulation for NR in 52.6–71GHz given the hardware impairments. Therefore, the ISI SIR should be above 30 dB not to cause a significant performance degradation.

Many links will operate at SINR levels below 30 dB, so keeping the ISI SIR above 30 dB for all the links may be an excessive requirement. Moreover, most of the links which are expected to have lower ISI SIR are NLoS links, which are expected to have low SINR at the same time. Therefore, we think having at least 80% of the links with ISI SIR above the threshold is a reasonable requirement.

The ISI signal-to-interference ratio is the ratio of the current OFDM symbol energy Es collected over the receiver FFT window to the energy of the previous and next OFDM symbol(s) Eintrf collected over the same FFT window:

|  | where, Hn is a complex channel coefficient for each cluster n tn is a cluster delay for each cluster n t0 is a starting position of the current OFDM symbol selected by the receiver tCP is the cyclic prefix length tFFT is the FFT window length |
| --- | --- |
| Useful signal (the current OFDM symbol) |  |
| ISI from the previous symbol(s) and the next symbol(s) |  |

The formulas above consist of the numerical integration over a channel impulse response (CIR) time shift τ and the sum of CIR taps < | Hn |2 >, which timings tn satisfy the condition below the sum sign. Please note, that the integrands have quantized form since their value changes only when some CIR tap is added to/removed from the sum. Therefore, a non-uniform grid over the time shift τ with the values that correspond to adding/removing of CIR taps from the sum is recommended for precise integration. One more thing to mention is that the time averaging of CIR taps | Hn |2 should be performed only in case of sub-symbol channel fluctuation modelling (e.g. due to Doppler spread).

Figure 1 illustrates the timings mentioned in the formulas. The blue shaded area corresponds to a cyclic prefix and the grey shaded area is an FFT window. The vertical black bars are the example CIR taps, with the height representing their power. The zero at the axis is a moment of the current OFDM symbol energy arrival to the receiver.

Figure 1. Receiver timing example

It can be seen, that the intersymbol interference SIR for each particular OFDM symbol depends both on the CIR realization and the selection of the t0, tCP and tFFT timings. While tCP and tFFT are tied with the selected OFDM numerology, the choice of the OFDM symbol starting position t0 is left up to the receiver implementation. Although it will not be specified in NR, we think it is worthwhile to discuss some reasonable t0 selection algorithms in order to agree on realistic ISI SIR calculation assumptions.

Proposal 2:

- Use intersymbol interference signal to interference ratio as a metric for system-level evaluation of NR in 52.6–71GHz

- Assume the acceptable intersymbol interference level criteria is having 80% of links with intersymbol of 30dB SIR or higher

### Discussion on an OFDM symbol starting position

The common assumption for time domain resynchronization in NR is using matched filters during PSS/SSS, TRS (and potentially other RS) reception. It allows a receiver to detect and update the position of the CIR peak tmax with the resynchronization periodicity. We assume that the resynchronization periodicity is high enough to ensure that CIR change between resynchronizations doesn’t affect peak position tmax and ISI SIR value significantly. We also assume the receiver implementation that adjust an OFDM symbol starting position t0 based on the detected CIR peak position tmax after each resynchronization, i.e. t0 is a function of tmax for each OFDM symbol.

One option is to simply set t0 = tmax. However, in some cases it may increase ISI level. For example, when tmax ≠ 0 (which is common for NLoS CIR after analog beamforming) receiver incurs ISI from both the previous and the next OFDM symbol(s).

Another option is to set t0 = 0. This approach eliminates ISI from the next OFDM symbol(s), however it may reduce the overall SIR given the CIR is longer than the CP length.

The third option is to have a fixed offset t0 = tmax – Δt. It can result in a starting position that minimizes ISI level for a given channel statistics, while still allows a simple implementation. In order to find the proper size of the offset Δt for the deployment scenarios we consider for NR in 52.6–71GHz, we have done some evaluations. Some of the results are provided in Figure 2 and Figure 3 below.

The goal was to determine a single Δt value, which can keep ISI level low in different channel conditions. The evaluation was done for both normal and extended CP. As we proposed above, we have used 20th percentile of ISI SIR CDF as a metric in the evaluation. Different portions of the CP length were tested as a fixed Δt offset (“offset max tap” curves). In addition, the upper bounds on the possible ISI SIR for a given OFDM numerology are added for a reference (“perfect” lines). They correspond to a dynamic Δt adjustment that maximizes ISI SIR for each CIR realization (perfect FFT window position).

As can be seen, the proper selection of Δt is very important for ISI reduction: the difference in the metric can be more than 50 dB given the same OFDM numerology. Another thing is the behavior of the curves is different for different channel models, so no single Δt value can minimize ISI in all cases. It means a reasonable compromise, which provides a fair performance, needs to be found.

Figure 2. Intersymbol interference level as a function of FFT window position (normal CP)

Figure 3. Intersymbol interference level as a function of FFT window position (extended CP)

The figures show that usually-LoS scenarios such as InH and InF-SH always have ISI SIR ≥ 30 dB. So Δt selection can be based solely on the other two scenarios without sacrificing the performance. It is good to ensure as many OFDM numerologies as possible satisfy the acceptable ISI level criteria in UMi and InF-DL. InF-DL curves have an extremum at Δt = 0.4*CP and UMi curves also have good ISI SIR performance at this point. Thinking this way, we propose to assume Δt is set equal to 40% of CP length in a reasonable NR receiver implementation for 52.6–71GHz.

Proposal 3:

- Assume the dynamic FFT window placement based on the 40% CP length offset from the detected CIR peak for intersymbol interference SIR calculation

Updates to Simulation assumptions

Table 1 show agreed system level simulation assumption from RAN1 #102-e meeting.

Table 1. System level simulation assumptions

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz Optional: 70 GHz |
| Subcarrier Spacing [kHz] | For 2000MHz BW: - 960 kHz - optional: 120, 240, 480 kHz For 400MHz BW: - 120 kHz - optional: 240, 480, 960 kHz Note: Other than value above, companies are encouraged to evaluating using subcarrier spacing values determined to be feasible from LLS study. Values for the subcarrier spacing may be revisited after further investigation from LLS study. |
| Bandwidth [MHz] | 2000 MHz 400 MHz Note: Channel bandwidth evaluated may be revisited after further investigation. |
| Number of RB | For 2000 MHz: - N/A (120 kHz), - N/A (240 kHz), - 320 (480 kHz) (optional), - 160 (960 kHz), - 80 (1920 kHz), For 400 MHz: - 256 (120 kHz), - 128 (240 kHz), - 64 (480 kHz), - 32 (960 kHz), - N/A (1920 kHz) For other channel bandwidths: - Companies are asked to provide information. Companies are encouraged to utilize linearly scaled PRB sizes for a given bandwidth based on above. |
| Deployment Scenario | - Scenario indoor-A (for two operator case) - Scenario indoor-C (for single operator case) Secondary scenarios: - Scenario outdoor-B Optional: - other scenarios listed below Indoor Office: Scenario Indoor-A) InH open office model: Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, ISD = 20m, BS randomly deployed within 10m x 10m virtual box, minimum distance between BS of different operators is 2m. Optional: single operator deployment in Scenario Indoor-A Scenario Indoor-B) small InH open office model: Office box 20m x 20 m, 1 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS randomly deployed within 10m x 10m virtual box, minimum distance between BS of different operators is 2m. Scenario Indoor-C) InH open office model: Office box 120m x 50 m, 12 BS per operator, 1 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, ISD = 20m Scenario Indoor-D) InH open office model: Office box 120m x 50 m, 6 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, ISD = 20m FFS: if the office box scenario can be reduced down to 50m x 50m Scenario Indoor-E) InH open office model: Office box 120m x 80 m, 3 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, a=20m, b=40m, c=20m, and d=40m Dense Urban: Scenario Outdoor-A) Dense Urban with 1 layer Hexagonal grid, single layer, 3 sectors per site, 7 sites locations, BS height 10m, UE height 1.5m, ISD = 150m FFS: whether ISD needs to be smaller optional: Reducing deployment size from 7 sites to 1 site with wrap around Scenario Outdoor-B) Dense Urban with 2 layers Macro layer (sub 7GHz – not necessarily need to be simulated for the 60GHz evaluation): Hexagonal grid, single layer, 3 sectors per site, 7 sites locations BS height 25m, UE height 1.5m, ISD = 100m, fixed BS position Micro layer (above 52.6 GHz): BS height 10m, UE height 1.5m, 2 operator, 2 BS per hexgrid per operator, random position within macro hexagonal grid per operator, minimum distance between TRP and UE: 10m, minimum distance between micro gNBs’ of the same operator: 10m optional: Reducing deployment size from 7 sites to 1 site with wrap around. Scenario Outdoor-C) Dense Urban with 1 layer Hexagonal grid, single layer, 3 sectors per site, 3 sites locations, BS height 10m, UE height 1.5m, ISD = 150m Indoor Factory Hall: Scenario Factory-A) Indoor factory with Dense cluster & low BS (InF-DL) Grid, 300m x 150m x 10m factor hall ISD 50m, BS height 1.5m, UE height 1.5m, Typical clutter size 2m, Clutter height 6m, Clutter density 60% Scenario Factory-B) Indoor factory with sparse clutter & High BS (InF-SH) Grid, 300m x 150m x 10m factor hall ISD 50m, BS height 8m, UE height 1.5m, Typical clutter size 10m, Clutter height 2m, Clutter density 20% |
| UE distribution | Average of 5 or 10 UE per BS UE are either 100% indoor or 100% outdoor depending on deployment scenario. |
| Channel Model | InH open office: - gNB-to-gNB and gNB-to-UE links: InH – office channel & PL model from TR38.901, indoor – open office LOS probability from TR38.901 (optional: indoor – mixed office LOS probability from TR38.901) - UE-to-UE links: InH – office channel & PL model from TR38.901, indoor – mixed office LOS probability from TR38.901 Dense Urban: - gNB-to-gNB and gNB-to-UE links: UMi street canyon channel & PL model from TR38.901 - UE-to-UE links: [outdoor to outdoor D2D channel & PL model from TR36.843 Section A.2.1.2], [(optional: UMi street canyon channel & PL model from TR38.901)] Indoor factory: - gNB-to-gNB and gNB-to-UE links: InF channel & PL model from TR38.901 - UE-to-UE links: [InF channel & PL model from TR38.901] Note: 3D distance between an gNB and a UE is applied. 3D distance is also used for LOS probability and break point distance. Note: channel models in brackets, [ ], are working assumption and may be revisited. Note: For D2D channel model used for UE-to-UE links companies should report how they scaled the model to 60 GHz. |
| Mobility | 3 km/hr |
| BS Antenna Configuration (Mg,Ng,M,N,P) | For outdoor macro/sectorized scenarios: (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH) For outdoor micro-layer scenarios: (Mg,Ng,M,N,P) = (1,3,8,16,2) with (0.5 dv, 0.5 dH) Note: 3 Panel single sector gNB with {0,+120,-120} degree boresight orientations. The gNB will only utilize 1 panel at given moment. For indoor scenarios: (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH) optional: (Mg,Ng,M,N,P) = (1,1,8,16,2) per pol with (0.5 dv, 0.5 dH) |
| BS Antenna Pattern | For outdoor scenarios: - Antenna power pattern given in Table 7.3-1 of TR38.901 (with exception of antenna element gain) For indoor/factory scenarios: - Antenna power pattern given in Table A.2.1-7 of TR38.802 for ceiling mount (with exception of antenna element gain) |
| BS Antenna element gain | 5 dBi |
| UE Antenna Configuration (Mg,Ng,M,N,P) | Configuration 1: (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) Configuration 2 (optional): (Mg,Ng,M,N,P) = (1,2,4,4,2) with (0.5 dv, 0.5 dH) Note: In both configurations, the 2 panels are back-to-back with panel selection done the at receiver. The UE will only utilize 1 panel at a given moment. |
| UE Antenna Pattern | Antenna power pattern given in Table A.2.1-8 of TR38.802 For indoor factory scenarios: Boresight orientation should be fixed in all simulation drops For other scenarios: Boresight orientation should be randomized between [0, 360) in the horizontal plane in each simulation drop Note: Companies to provide information about boresight orientation (e.g. random orientation, vertical to ground, parallel to ground, etc) |
| UE Antenna element gain | 5 dBi |
| BS Power Limitation | 40 dBm EIRP Optional: 60 dBm EIRP Maximum TxP adjusted to meet EIRP limits |
| UE Power Limitation | 25 dBm EIRP with 21 dBm max TxP Optional: 40dBm EIRP with 21 dBm max TxP |
| BS NF | 7 dB |
| UE NF | 10 dB Optional: 13dB |
| Transmission Rank | Rank adaptative transmission between Rank 1 and 2 |
| PDCCH Overhead | 2 symbol per slot |
| DMRS Overhead | 1 symbol per slot |
| CSI-RS Overhead | Companies to provide information |
| SRS Overhead | Companies to provide information |
| Other Overhead | Companies to provide information |
| Data Processing Latency | UE processing timeline in microseconds are assumed to be same as 120 kHz SCS PDSCH/PUSCH processing latency Optional: UE processing timeline in microseconds are assumed to be half of 120 kHz SCS PDSCH/PUSCH processing latency |
| TDD DL/UL Ratio | Companies to provide information (if applicable) |
| CSI feedback | Ideal feedback |
| Additive Rx EVM | Note: additive Rx EVM values may be revisited after LLS study |
| Traffic Model | FTP Model 3 (27Mbyte file) Optional: - Full buffer, - FTP Model 1 (27, 8 Mbyte file), - FTP Model 3 (0.5, 2, 8, 16 Mbyte file) |
| UE Receiver | MMSE-IRC |
| Cell selection criteria | Random select from strongest RSRP with 1 dB HO Margin Note: UE with RSRP below a -71 dBm + 10 log10( bandwidth/2GHz ) are not considered in simulation and not counted toward UE distribution count |
| DL/UL Traffic Ratio | 50% DL, 50% UL Optional: 100% DL, 0% UL, 80% DL, 20% UL 0% DL, 100% UL |
| Channel access modelling | Companies to report details of LBT procedure and parameters (e.g. ED, CWmax, COT, etc.) if LBT procedure is used in the evaluations. |
| Synchronization Assumption | Companies are asked to provide information on the synchronization assumption made between operators for 2 operator deployment scenarios. |

### Update of Indoor A Scenario Description

Indoor A scenario is described as “Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, ISD = 20m, BS randomly deployed within 10m x 10m virtual box,  minimum distance between BS of different operators is 2m.”  and followed by the deployment illustration figure.

Based on our understanding, the y-axis distance from the wall to the 10m x 10m virtual box is 7.5m at the bottom as well as at the top. Additionally, the x-axis distance from the wall to the closest 10m x 10m virtual box is 5m. Based on this, we can draw more accurate illustration of indoor A in Figure 4.

Figure 4. Illustration of indoor scenario A

Since the BS are deployed randomly within the 10m x 10m virtual box, there cannot be a fix ISD definition. However, if we assume ISD refers to the distance between centers of the 10m x 10m virtual box, we can have some definition. In this case, the x-axis ISD is 20m, but the y-axis ISD is 25m. If the ISD of 20m is taken more literally, one may assume that there is 12.5m y-axis distance from the wall to the 10m x 10m virtual box at the top of the layout.

We suggest updating the indoor A scenario description to avoid any ambiguity, and update the figure for indoor A as follows:

- Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, x-axis ISD = 20m and y-axis ISD = 25m, where ISD is define by the distance between two adjacent 10m x 10m virtual box, BS randomly deployed within 10m x 10m virtual box,  minimum distance between BS of different operators is 2m.”

Proposal 4:

- Update the indoor A description as follows:

- Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, x-axis ISD = 20m and y-axis ISD = 25m, where ISD is define by the distance between two adjacent 10m x 10m virtual box, BS randomly deployed within 10m x 10m virtual box,  minimum distance between BS of different operators is 2m.”

### Clarification of Ceiling Mounted BS Antenna Orientation

For ceiling mounted BS for Indoor A and C deployments, BS has asymmetric number of elements in the horizontal and vertical domain. In addition, due to the asymmetric nature of the BS positioning within the indoor A and C deployment boundary, the ceiling mounted BS may have different beamwidth resolution in x-axis and y-axis domain. Depending on whether BS has applied rotation, before 90 degree down tilt to create a ceiling mounted antenna, the longer side of the antenna array may be facing different x-y axis. Figure 5 show an illustration of ceiling mounted BS antenna array seen from the bird’s eye view depending on rotation applied to the BS antenna.

Figure 5. Illustration of ceiling mounted BS antenna with 0 and 90 rotation

In verify whether selection of the antenna rotation has significant impact to the simulation results, we have checked the geometry and corresponding system level performance. Figure 6 shows the DL geometry for two different BS antenna rotation configurations for Indoor A scenario. In the figure 4x8 and 8x4 correspond to 0 and 90 rotation of the BS antenna before the down tilt, respectively. We observed similar statistics for various RSRP distribution between BS and BS, BS and UE, and UE and UE links, which we have omitted in this contribution.

Figure 6. DL geometry comparison between 0 and 90 rotation of the BS antenna before the down tilt

Based on the study, we believe the antenna rotation does not significantly impact the observations of the system level simulations. However, we suggest that companies provide information on the antenna rotation of the BS for the Indoor A and C deployment scenario.

Proposal 5:

- Companies are encouraged to provide ceiling mounted BS antenna rotation for indoor A and C deployment scenario.

### Inclusion of non-ceiling mounted BS for Indoor A Scenario

For the mixed operator scenario, we noticed that all BS are fully ceiling mounted and antennas are facing downwards that maximize antenna coverage to the UEs and minimize potential BS to BS interference. While for enterprise deployments, it is typical to have only ceiling mounted antennas to maximize LOS probability and beam gain, for non-organized deployments, where the BS positions can randomly vary and there may be multiple operators co-existing in the same environment, it will be difficult to assume only ceiling mounted BS antennas exist. In fact, for many of the pico-cell or Wi-Fi access point (AP) disaggregated and non-centralized deployments, it is far more typical to see non-ceiling mount deployments. These pico-cell/APs are generally wall mounted or place on top of shelves or tables. Therefore, to better reflect the non-organized deployments of the multi-operator scenario, we suggest to include non-ceiling mounted BS as an option for indoor A scenario.

Proposal 6:

- Proposed to include non-ceiling mounted BS as an option for indoor A scenario, to better reflect non-organized deployments of the multi-operator scenario.

Link-Level Evaluation Methodology

The link-level simulation parameters agreed from RAN1 #102-e are summarized in Table 2.

Table 2. Link level simulation assumptions

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz Optional: 70 GHz |
| Subcarrier Spacing [kHz] | PDSCH/PUSCH: - {120, 240, 480, 960} kHz -optional: 1920 kHz Optional: - if evaluated companies are asked to provide information on other channels/signals and subcarrier spacing |
| Bandwidth [MHz] | PDSCH/PUSCH: - {400, 2000} MHz Optional: - Companies are asked to provide information if other bandwidths are evaluated Note: Evaluation of listed channel bandwidth does not mean RAN1 has agreed to support such channel bandwidth and are only for evaluation purposes to obtain useful insights. |
| Number of RB | For 400 MHz: - 256 (120 kHz), - 128 (240 kHz), - 64 (480 kHz), - 32 (960 kHz), - N/A (1920 kHz) For 2000 MHz: - N/A (120 kHz), - N/A (240 kHz), - 320 (480 kHz) (optional), - 160 (960 kHz), - 80 (1920 kHz), For other channel bandwidths: - Companies are asked to provide information. Companies are encouraged to utilize linearly scaled PRB sizes for a given bandwidth based on above. Note: Other bandwidth and sub-carrier spacing combinations can be optionally used. |
| Waveform | For PDSCH: CP-OFDM For PUSCH: CP-OFDM and DFT-s-OFDM |
| CP Type | Normal CP Extended CP Note: ECP is not expected to be applicable in all SCS and channel conditions, and companies providing results for ECP are encouraged to provide evaluation results with motivation/justification of simulated ECP cases |
| Channel Model | TDL model as defined in of TR38.901 Section 7.7.2: - TDL-A (5ns, 10ns, 20ns DS) - optional DS for consideration: 40ns, 60ns DS CDL model as defined in of TR38.901 Section 7.7.1: - CDL-B (20ns, 50ns DS) - CDL-D (20ns, 30ns DS) with K-factor = 10 dB - optional DS for consideration: 100ns DS Optional modification CDL-B/D model (a) Indoor Office NLOS: CDL-B (20 ns DS), and Indoor Office LOS: CDL-D (20 ns DS) - Use mean angular spread values from Table 7.5.6-Part2 (for ASD, ASA, and ZSA) and Table 7.5-10 (for ZSD) - Use mean angles of CDL-B/D for desired mean angles as baseline (no angle translation) - Note that the angular spread values in the table are quoted in log units - Mean K-factor for CDL-D from Table 7.5.6-Part2 (9 dB) (b) UMi – Street Canyon NLOS: CDL-B (50 ns DS), and UMi – Street Canyon LOS: CDL-D (30 ns) - Use mean angular spread values from Table 7.5.6-Part1 (for ASD, ASA, and ZSA) and Table 7.5-8 (for ZSD). - Use mean angles of CDL-B/D for desired mean angles as baseline (no angle translation) - Note that the angular spread values in the table are quoted in log units - Use mean K-factor for CDL-D from Table 7.5.6-Part1 (7 dB) Note: Mean angular spread values are used as desired AS value to scale the ray angles as described in TR38.901 section 7.7.5.1. As baseline, the ray angles are not translated, meaning (TR38.901 section 7.7.5.1). If companies perform translation of the ray angles they are encouraged to report the details. The mean K-factor is used to scale the tap powers as described in TR38.901 section 7.7.6. Note 2: for TDL/CDL model, the delay spread (DS) value mentioned is the delay spread scaling value (i.e. corresponding to normalized delay of 1.0). Note 3: Other models (either TDL or CDL) with DS values not listed are optional. Note 4: Companies are encouraged to provide evaluation results with motivation/justification of simulated DS values. |
| Antenna Configuration (Mg,Ng,M,N,P) | For TDL model: - 2x2 - 1x2 (optional) For CDL model: Configuration 1: - (Mg,Ng,M,N,P) = (1,1,8,16,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,4,4,2) UE with (0.5 dv, 0.5 dH) Configuration 2: - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) |
| Mobility | 3 km/hr |
| PA Model | Optional: - Companies to provide modelling (in lieu of pre-loaded Tx EVM) |
| gNB TRP PN Model | 3GPP TR38.803 example 2 BS PN profile Optional: - If other PN profile is used, companies to provide information on the modelling used Note: companies to provide information about the LO distribution model assumed in the simulations. |
| UE PN Model | 3GPP TR38.803 example 2 UE PN profile Optional: - If other PN profile is used, companies to provide information on the modelling used Note: companies to provide information about the LO distribution model assumed in the simulations. |
| Pre-loaded Tx EVM | Optional: - 3% at Tx (In lieu of PA model), - If other values are used companies are asked to provide information on the values selected for simulation. |
| Additive Rx EVM | Optional: - 5% at Rx, - If other values are used companies are asked to provide information on the values selected for simulation. |
| I-Q Imbalance | Optional: - (-26dBc), - (-31dBc), - If other values are used companies are asked to provide information on the values selected for simulation. |
| Frequency Offset | Optional: - 0.1 ppm (for PDSCH/PUSCH) - 5, 10, 20 ppm (for initial access) |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 Note: companies are asked to provide information the precoding scheme (including granularity) used in the evaluations. |
| PDSCH SLIV | (S=2, L=12) Optional:(S=0, L=14) Note: Starting symbol, S, (indexed from 0) and length, L. |
| DMRS Configuration | 1 DMRS symbol (front loaded), or 2 DMRS symbols at (2,11) symbol index Note: no data multiplexing is assumed in DMRS symbols |
| PTRS Configuration | For CP-OFDM: (K = 4, L = 1) or (K = 2, L = 1) Note: PTRS per K number of PRBs, and PTRS every L number of OFDM symbols For DFT-s-OFDM: (Ng = 2, Ns = 2, L = 1) (Ng = 2, Ns = 4, L = 1) (Ng = 4, Ns = 2, L = 1) (Ng = 4, Ns = 4, L = 1) (Ng = 8, Ns = 4, L = 1) Note: Ng number of PT-RS groups, Ns number of samples per PT-RS group, and PTRS every L number of DFT-s-OFDM symbols Note 2: companies are asked to provide the PT-RS configuration used for DFT-s-OFDM simulation among the listed above, where the selection of the PT-RS is chosen such that it provides similar overhead as the chosen PT-RS configuration for PUSCH CP-OFDM (if simulated). |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): - MCS 7 (QPSK), - MCS 16 (16QAM), - MCS 22 (64QAM), From MCS Table 2 (TS38.214): - MCS 27 (256QAM) (optional) Assume NohPRB = 0 for MCS calcuations. Note: If normal CP and extended CP are to be compared, companies are asked to provide information on the MCS values used that provide similar payload sizes for the comparison. Companies to provide actual code rate used in the evaluations. |

Simulation of Rank 2 Cases

All the link level simulation evaluations assume use rank 1 transmission. For short range communications, the LOS probability does increase and can potentially limit the use of higher transmission ranks. However, use of polarized antennas does allow support of rank 2 transmission even in LOS links. In fact, use of polarized antennas and high probability of LOS links in short BS to UE distance in indoor deployments creates higher received signal and generates even more opportunities for rank 2 transmission than typically visible in outdoor deployments. Compared to rank 1 transmission, rank 2 transmission does have higher SNR requirements.

Given that we expect rank 2 transmission to occur quite frequently, we suggest to add rank 2 transmission as an option in the link level simulation assumptions.

Proposal 7:

- Propose to add rank 2 transmission as an option in the link level simulation assumptions.

Conclusion

In this contribution we discussed the reasonable assumptions for link- and system-level evaluation of NR in 52.6–71GHz band, as well as the metrics for the evaluation. Based on the discussion, we make the following observations and proposals:

Proposal 1:

- Use root mean square effective channel delay spread at the receiver as a metric for system-level evaluation of NR in 52.6–71GHz

Proposal 2:

- Use intersymbol interference signal to interference ratio as a metric for system-level evaluation of NR in 52.6–71GHz

- Assume the acceptable intersymbol interference level criteria is having 80% of links with intersymbol of 30dB SIR or higher

Proposal 3:

- Assume the dynamic FFT window placement based on the 40% CP length offset from the detected CIR peak for intersymbol interference SIR calculation

Proposal 4:

- Update the indoor A description as follows:

- Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, x-axis ISD = 20m and y-axis ISD = 25m, where ISD is define by the distance between two adjacent 10m x 10m virtual box, BS randomly deployed within 10m x 10m virtual box,  minimum distance between BS of different operators is 2m.”

Proposal 5:

- Companies are encouraged to provide ceiling mounted BS antenna rotation for indoor A and C deployment scenario.

Proposal 6:

- Proposed to include non-ceiling mounted BS as an option for indoor A scenario, to better reflect non-organized deployments of the multi-operator scenario.

Proposal 7:

- Propose to add rank 2 transmission as an option in the link level simulation assumptions.

References

- RP-193227, “New SID: Study on supporting NR from 52.6GHz to 71 GHz,” Intel Corporation, RAN#86, Dec 2019

- ETSI EN 302.567. V2.1.1., “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU,” 2017-07.