3GPP TSG RAN WG1 Meeting #103-e	R1-2007963

e-Meeting, Oct 26th – Nov 13th, 2020

Title:	Text proposal on the MsgA configuration in unlicensed band

Source: 	ZTE, Sanechips

Agenda Item:	7.2.1

Document for:	Discussion and Decision

# Introduction

This document discusses the issue of MsgA configuration in unlicensed band when multiple RB sets are configured.

And the corresponding TP is provided.

# MsgA configuration in unlicensed band

The following agreement has been made in RAN1#98bis, while it is not reflected in the current specifications.

Agreements: - All msgA PUSCH occasions and the associated msgA RACH occasions are confined within a single 20 MHz carrier/LBT bandwidth

In RAN1#102-e meeting, there were some discussions in the NR-U session [1] whether the MsgA PRACH and PUSCH should be confined to a single RB set if multiple RB sets are configured in the unlicensed band. We think the discussion is not needed given the above agreement. The following change to TS 38.213 is proposed.

Proposal 1:

- Adopt the following TP#1 to TS38.213

Reasons for change To capture the agreement in the specification Summary of changes All msgA PUSCH occasions and the associated msgA RACH occasions are confined within a single RB set Specs/Sections impacted TS 38.213, Section 8.1A Consequences if not approved: MsgA PRACH or PUSCH may across multiple RB sets ======================== Start of TP1 for TS 38.213 =============================== 8.1A PUSCH for Type-2 random access procedure *** Unchanged text omitted *** A UE determines a first interlace or first RB for a first PUSCH occasion in an active UL BWP respectively from interlaceIndexFirstPO-MsgA-PUSCH or from frequencyStartMsgA-PUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of interlaces or a number of RBs provided by nrofInterlacesPerMsgA-PO or by nrofPRBs-perMsgA-PO, respectively. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgA-PUSCH. A number Nf of PUSCH occasions in the frequency domain of an UL BWP is provided by nrofMsgA-PO-FDM. For operation with shared spectrum channel access, all PUSCH occasions and the associated RACH occasions are confined within a single RB set. *** Unchanged text omitted *** ======================== End of TP1 ===============================

# Reference

[1] R1-2007192, FL summary for initial access signals and channels for NR-U, Moderator (Qualcomm Incorporated)