3GPP TSG RAN WG1 #103-e		R1-2007976

e-Meeting, Oct. 26th - Nov. 13th, 2020

Title: 	Discussion on applications, traffic model and evaluation methodology for XR

Source: 	ZTE, Sanechips

Agenda item:	8.14.1

Document for:	Discussion and decision

Introduction

A new SID on XR evaluation has been approved in RP-193241 with the following scope. In this paper, we discuss the applications, traffic model and evaluation methodology related to XR.

- Confirm XR and Cloud Gaming applications of interest - Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc. - Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios - Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs Note 1: eURLLC SI/WI work relevant to XR should be taken into consideration. Note 2: Traffic model for the performance evaluation shall be based on the standardization in SA WG4

Discussions

Applications

The applications of interest for XR have been extensively studied with outcome captured in TR26.928. 23 representative use cases (e.g. streaming of Immersive 6DoF/Untethered Immersive Online Gaming/XR meeting) are elaborated therein together with their corresponding service type, enabling delivery architecture, QoS/QoE requirements etc. Additionally, 7 delivery architectures and their traffic characteristics are synthesized in the TR. Accordingly, the associations between the applications of interest/delivery architectures in RP-193241[1] and several exemplary use cases are encapsulated in Table 1. For some use cases more than one delivery architectures are involved and function presumably under different phases of the use cases.

Table 1 Use cases for some applications of XR

| Delivery Architecture/Application of interest | Use Cases |
| --- | --- |
| VR1: Viewport dependent streaming | Streaming of Immersive 6DoFEmotional Streaming; AR Streaming with Localization Registry; Immersive 6DoF Streaming with Social Interaction |
| VR2: Split Rendering: Viewport rendering with Time Warp in device | Untethered Immersive Online Gaming; Immersive Game Spectator Mode; 5G Online Gaming party |
| AR1: XR Distributed Computing | Real-time communication with the shop assistant; 5G Online Gaming party |
| AR2: XR Conversational | XR Meeting; AR animated avatar calls; 360-degree conference meeting; AR avatar multi-party calls; 3D shared experience |
| CG: Cloud Gaming | 5G Online Gaming party |

The latency and packet error rate aspects of low latency/ultra reliability services were investigated and defined in the form of 5QI within SA group. Consequently the uRLLC RAN study and work items carried out evaluations/specification work to deliver the latency or reliability requirement. Among the services investigated by Rel-16, a glimmer of what should be characteristic of XR services has been captured under 5QI 80 with a Non-GBR (guaranteed bit rate) token. The corresponding latency requirement is 10ms and the reliability requirement is 10-6.

Now that the SA group evolved to capture the XR applications requiring NW to provide target data rate in the order of up to102 Mbps under corresponding joint (PDB, PER) constraint comparable to that of 5QI 80. We believe RAN level efforts are needed to carry out comprehensive evaluations of XR applications of interest and specify, if needed, the necessary enhancements on top of NR eMBB and uRLLC.

In the meantime, up to one single (data rate, PDB, PER) requirement would be sufficient for the evaluation of a given application of interest and while this task seems straightforward for the 4 other categories, the appropriate requirement of cloud gaming for evaluation is worth discussing.

First of all, the delivery architecture for CG for evaluation purpose needs to be settled. Among the 8 delivery architectures, we believe generalized split rendering services is suitable considering the service characteristic of VR assimilates well that of a vast type of CG at least for DL. In the mean time, various 3D objects from CG applications could be delivered with reduced RTT and satisfying quality thanks to this architecture. Typical data rate characteristics of this architecture in 26.928 could therefore be used.

Secondly, it was captured in 26.928 the CG applications are supposed to need RTT requirements within {50,100,150,200}ms and in 23.501 that the RTT requirements could be as aggressive as 5ms. The RTT value and the corresponding PDB needs to be settled among the candidates.

Proposal 1: The applications of interest of the study item include at least the following listed in RP-193241.

- VR1: Viewport dependent streaming - VR2: Split Rendering: Viewport rendering with Time Warp in device - AR1: XR Distributed Computing - AR2: XR Conversational - CG: Cloud Gaming

- Up to one single (Rate, PDB, PER) requirement is evaluated for each application of interest

- For delivery of CG applications, generalized split rendering architecture and its relevant traffic characteristics are considered.

TR26.928 captures in section 6.3 a summary of traffic characteristics, from which the (Rate, PDB, PER) requirement for each applications of interest could be derived accordingly. To be specific,

- For VR service delivered through viewpoint dependent streaming, the reliability requirement is fixed as 10-6, the rate range is fixed as 25Mbps.

- For Split rendering with pose correction, the rate range is fixed as 100Mbps and the DL PDB is fixed as 20ms.

- For XR conversational, the rate could as large as 50Mbps at the beginning and almost constant or fluctuating depending on media consumption

For conversational services such as XR meeting, the number of participants is also supposed to be more than other services.

Observation 1: Considering the differentiation of the three requirements, the applications of interest in RP-193241 could be summarized into the following categories:

- Cat. 1 Reduced rate or PDB requirement for stringent PER target

- Cat .2 Reduced PER for stringent rate/PDB requirement

- Cat. 3 Balanced requirement on Rate/PDB/PER subject to bursty traffic and high number of UEs

Traffic Characteristics

In this section, we further discuss the traffic characteristics from evaluation perspective. In general, the traffic characteristics should first settle the traffic type, i.e. whether the packet size and the interval arrival rate is fixed, and accordingly detailed aspects such as {packet size, packet arrival rate} for fixed packet size/arrival rate case and {packet size distribution and the shaping parameter, inter arrival time distribution} for variant packet size/arrival rate case. In addition, PDB and PER from the application of interest are also needed in the simulation. We discuss in the corresponding sub-sections for DL and UL some typical characteristics that could be used for evaluations.

DL Traffic Characteristics

For VR services, as mentioned in section 1, the data rate is captured as 25Mbps and traffic file size distribution and traffic periodicity range is fixed in 26.928. Denote the packet arrival rate as R, The file size could therefore be calculated as 25/ R Mbit. Given the typical DL traffic for XR services are video streaming at 60, 90 or 120 fps [2]. The traffic file size could be derived as 0.417, 0.278, or 0.208Mbit. PDB for evaluation could be set as that of 5QI use case {6, 8, 9} covering video streaming use cases which is 300ms.

As elaborated in the section 1, the architecture of cloud gaming could be generalized split rendering whose data rate and reliability are 100Mbps and 10-3respectively [2]. Regarding PDB, cloud gaming applications could admit differentiated RTT requirement ranging from the following in 26.928.

- Ultra-Low-Latency applications: roundtrip interaction delay threshold of at most 50ms latency. - Low-Latency applications: roundtrip interaction delay threshold of at most 100ms latency. - Moderate latency applications: roundtrip interaction delay threshold of at most 200ms latency. - Non- critical latency applications: roundtrip interaction delay threshold higher than 200ms latency.

Take 50ms as the RTT requirement, the DL PDB is influenced by

- The UL transmission content i.e. whether the pose information or some additional local scene information is needed;

- The UE side efficiency of performing split rendering.

In all, the delay budget for DL could be up to more than half of the RTT considering the time reserved for UE side processing.

Other than the characteristics of fixed traffic type, during the workshop on XR, Gaussian/Pareto distributions are proposed to assimilate the fluctuating traffic. We believe down-selection is needed in RAN1#103-e meeting from the two distributions. Moreover, other periodicity values and the modeling of variant inter arrival rates needs to be considered.

Proposal 2: The following 2 categorizations of XR representative services for DL services as the starting point

- 25Mbps,10-6, 300, fixed periodicity, fixed packet size

- 100Mbps, 10-3, [5-30] fixed periodicity and fixed packet size/statistical model for fluctuating traffic

- FFS periodicity values and the modeling of variant inter arrival rate

- FFS statistical model for fluctuating traffic, target to down-select in RAN1#103-e between Gaussian distribution and Pareto distribution

Another line of thinking is whether/how to model the fluctuating traffic for e.g. the AR service where data rate peeks at the beginning and fluctuates depending on media consumption. We believe the following alternatives could be considered depending on the actual applications of interest:

Alt 1:  Gaussian and Pareto distribution should be considered as feasible candidates.

Alt 2:  The mixed traffic model consisting of two or more fixed traffic type corresponding to different data rate.

Alt 1 could assimilate well the rate fluctuation related to the media consumption. Alt 2 could assimilate the traffic that stays stable at different rates during different periods.

With Alt 1 in use, the parameters are derived following a three-step methodology:

Step 1: Generate the mean packet size according to the packet arrival rate and the data rate requirement.

Step 2: Determine the minimum file size or the standard derivation parameter based on the relationship between the mean data rate and the minimum/maximum (i.e. truncated) data rate.

Step 3: Determine the remaining parameter, if any, by jointly considering the mean file size and the parameter obtained from step 2.

We demonstrate the use of the methodology to Pareto distribution via the numerical example of data rate = 100Mbps and packet arrival rate = 60fps. Similar to that of the fixed packet size, the mean packet size is 0.2083Mbyte.

Further we set the min file size as 0.1 Mbyte considering the data rate shrinks no more than half of the mean data rate. The shaping parameter could thus be obtained as 1.9 by considering the mean packet size and the min file size. On the other hand. The mean value can be estimated as kxmink-1, where xmin is the minimum file size and k is the shaping parameter. The derived parameters are encapsulated in Table 2.

Table 2 Exemplary parameters for Pareto distribution following a three-step methodology

| Mean Packet Size (Mbytes) | 0.2083 |
| --- | --- |
| Location parameter k | 0.1 |
| Shape parameter alpha | 1.9 |
| Remarks：The DL rate is 100Mbps and the packet arrival rate is 60fps |  |

Observation 2: One or both alternatives for modelling fluctuating traffic may be considered for AR service.

Proposal 3: Adopt the three-step methodology to derive the traffic models for Gaussian/Pareto distribution of file size.

Step 1: Generate the mean packet size according to the packet arrival rate and the data rate requirement.

Step 2: Determine the minimum file size or the standard derivation parameter based on the relationship between the mean data rate and the minimum/maximum (i.e. truncated) data rate.

Step 3: Determine the remaining parameter, if any, by jointly considering the mean file size and the parameter obtained from step 2.

Regarding the one way PDB derivation from RTT, the UE capability and actual service type should be considered. If the UE should perform additional operations such as rendering or scene concatenation, the overall delay budget needs to consider the time consumed also. This would put more stringent requirement on DL PDB.

Observation 3: The UE side processing and UE capability needs to be considered to determine the DL PDB from RTT.

UL Traffic Characteristics

For UL traffic characteristics, the PER requirement is similar to that of DL traffic characteristics. From PDB perspective, the setting should be considered jointly with the DL PDB in case RTT is provided. We believe the same value range as that of DL could be considered to reflect both DL/UL transmission could be relatively more consuming. From rate perspective, the following two types of categorizations could be considered for evaluation purpose.

- Cat.1: The UL traffic is essentially the HTTP applications.

- Cat.2: The UL traffic could include uploading large size of files including the (concatenated) images/3D objects.

From 26.928 and 802.11m, the HTTP application type of UL traffic should be generated considering a fixed packet size of 350Bytes HTTP application MAC packet and a arrival rate of 1000Hz. On the other hand, UL rate of AR services could be used to model the traffic of the other category.

Accordingly, the following proposal is drawn.

Proposal 4: The following 2 categorizations of XR representative services for UL services as the starting point.

- 28kbps,10-6, 300, fixed periodicity, fixed packet size

- 2.7-50Mbps, 10-3, [5-30] fixed periodicity and fixed packet size/statistical model for fluctuating traffic

- FFS periodicity values and the modeling of variant inter arrival rate

- FFS statistical model for fluctuating traffic, target to down-select in RAN1#103-e between Gaussian distribution and Pareto distribution

Evaluation

With the above discussions, evaluations could be done to verify whether additional techniques on top of NR eMBB/uRLLC would be needed to deliver the desired traffic under the reliability/delay requirement.

Simulation Parameters

Considering the use cases, two scenarios (i.e. the Umi and Indoor Hotspot scenarios) would be relevant for the XR evaluations. Detailed simulation parameters are attached in the appendix A. Some clarifications are provided in the following regarding several rows of the simulation table.

- Bandwidth and duplex mode

Considering the data rate requirement, a suitable configuration of bandwidth could be up to 100MHz and in our simulation, the bandwidth is dedicated to DL. FDD is used in the XR simulation evaluations in this paper.

- Number of UEs

The number of UEs in the simulation have major difference. We believe at least for cloud gaming up to 10 UEs would be a suitable setting.

- Antenna configuration

In order to support higher data rate, more antennas should be considered, for example 64Tx in gNodeB and 4Rx in UE side.

Evaluation Metrics

Assuming the UE should satisfy the latency, reliability, data rate requirement, the capacity could be reflected in the following alternatives:

Alt 1: The capacity is measured by the number/percentage of UEs achieving the given (rate, reliability, latency) requirement that could be supported.

Alt 2: Statistical metrics regarding the CDF of the UPT (User perceived throughput).

Alt 1 provides an overall performance indicator from system perspective. Alt 2 provides a detailed picture considering UE specific capacity information. 5% or mean UPT could be used to provide a UE based QoE reflecting the experience of cell-edged UE or a UE in general. Given the above 2 alternatives have been used in uRLLC and could reflect differentiated levels of information, we believe they could be re-used for XR evaluations as well.

Additionally, coverage could be reflected by performing link budget calculation such as MPL and compare it with ISD. The link budget template from [4] could be re-used with some modifications in the following aspects:

BLER/latency for link level simulations

10% is set as target BLER for PDSCH. This needs to be revised and aligned with that of XR PER performance. For example, 10-3 should be considered as a candidate for CG application given reTx is considered as infeasible for some scenarios.

In the meantime, the latency requirement needs to set appropriately in accordance with the PDB for the services types where retransmission are necessary.

PRBs/TBS/MCS for eMBB for PDSCH/PUSCH

In Coverage Enhancement study, the PRBs/TBS/MCS settings for FR1 is left to companies' report while a reference is fixed for FR2 where full bandwidth could be employed for rate requirement as aggressive as 25Mbps. We believe for XR services, it would be good to use the PRBs setting similar to that of the 25Mbps requirement of FR2, i.e. dedicate the full bandwidth for coverage evaluation in DL. In this way, a unified observation could be drawn in terms of MPL in use. For UL, the PRBs/TBS/MCS setting should be determined according to the traffic evaluated.

Proposal 5: Consider the capacity metric and coverage enhancement metric in XR evaluation.

- Capacity metric should include both the number/percentage of UEs satisfying the PDB/PER/Rate requirement and the statistical metrics regarding the CDF of UPT.

- Coverage metric should use the service based metric i.e. calculate MPL and compare it with the ISD. For the link level simulations used to generate the MPL, the following discussion points should be settled.

- For DL, the number of PRBs should be generated by considering the full bandwidth dedicated. TBS should be determined by an agreed MCS.

- For UL, the PRBs/TBS/MCS should be set in accordance with the UL traffic.

- Target BLER should be set in accordance with the traffic, e.g. 10-3 or 10-6.

Preliminary Evaluation Results

Some preliminary system simulation results in Umi and Indoor Hotspot scenarios are shown respectively in Figure 1 and Figure 2. PER is 0.1% and PDB is 10ms in the XR simulation evaluations.

Figure 1 Ratio of satisfied UEs in Umi scenario

Figure 2 Ratio of satisfied UEs in Indoor Hotspot scenario

Based on system simulation results above, it can be observed that the ratio of satisfied UEs with 0.1%PER decreases with the increment of the number of UEs per cell in both Umi and Indoor Hotspot scenarios. Assuming outage is 10%, when data rate is 30Mbps, 60Mbps and 100Mbps, there are 10 UEs, 5UEs and 3UEs per cell which satisfy the requirement of latency and reliability in Umi scenario respectively and there are 10 UEs, 6UEs and 4UEs per cell which satisfy respectively the requirement of latency and reliability in Indoor Hotspot scenario. Based on the above analysis, it can be also observed that the number of UEs per cell which satisfy the requirement of latency and reliability decreases with the increment of data rate in both Umi and Indoor Hotspot scenarios.

The CDF curves of average throughput per user are shown in Figure 3, wherein average throughput per user is obtained based on the packets successfully transmitted within PDB. It is found that some users can receive a packet successfully in 3 slots and the rate is more than 1100Mbps. And also average throughput per user decreases with the increment of the number of UEs per cell.

Figure 3 Date rate is 100Mbps in Umi Scenario

Other results on resource utilization and average throughput per user are provided in Appendix B. To further explain, 1, 2, 3 in X-axis of Figure B.1 and Figure B.2 correspond to 30Mbps, 60Mbps, and 100Mbps, respectively.

Observation 4: Up to 10 UEs could be supported under the target reliability, rate requirement in both Umi and Indoor Hotspot scenarios, and when date rate is 30Mbps.

Observation 5: Up to 5 UEs could be supported under the target reliability, rate requirement in both Umi and Indoor Hotspot scenarios when date rate is 60Mbps.

Observation 6: Up to 3UEs could be supported under the target reliability, rate requirement in both Umi and Indoor Hotspot scenarios when date rate is 100Mbps.

Observation 7: The ratio of satisfied UEs with 0.1%PER decreases with the increasing of the number of UEs per cell in both Umi and Indoor Hotspot scenarios.

Observation 8: Some users can received a packet successfully in 3 slot and the rate is more than 1100Mbps when data rate is 100Mbps.

Conclusion

In this paper, we provided a categorization of the applications of interest of XR SID and some thoughts on the traffic characteristics for DL/UL respectively. Evaluation methodologies in aspects of capacity/coverage are elaborated. Preliminary results in terms of capacity is provided. The following observations/proposals are made in this paper.

Observation 1: Considering the differentiation of the three requirements, the applications of interest in RP-193241 could be summarized into the following categories:

- Cat. 1 Reduced rate or PDB requirement for stringent PER target

- Cat .2 Reduced PER for stringent rate/PDB requirement

- Cat. 3 Balanced requirement on Rate/PDB/PER subject to bursty traffic and high number of UEs

Observation 2: One or both alternatives for modelling fluctuating traffic may be considered for AR service.

Observation 3: The UE side processing and UE capability needs to be considered to determine the DL PDB from RTT.

Observation 4: Up to 10 UEs could be supported under the target reliability, rate requirement in both Umi and Indoor Hotspot scenarios, and when date rate is 30Mbps.

Observation 5: Up to 5 UEs could be supported under the target reliability, rate requirement in both Umi and Indoor Hotspot scenarios when date rate is 60Mbps.

Observation 6: Up to 3UEs could be supported under the target reliability, rate requirement in both Umi and Indoor Hotspot scenarios when date rate is 100Mbps.

Observation 7: The ratio of satisfied UEs with 0.1%PER decreases with the increasing of the number of UEs per cell in both Umi and Indoor Hotspot scenarios.

Observation 8: Some users can received a packet successfully in 3 slot and the rate is more than 1100Mbps when data rate is 100Mbps.

Proposal 1: The applications of interest of the study item include at least the following listed in RP-193241.

- VR1: Viewport dependent streaming - VR2: Split Rendering: Viewport rendering with Time Warp in device - AR1: XR Distributed Computing - AR2: XR Conversational - CG: Cloud Gaming

- Up to one single (Rate, PDB, PER) requirement is evaluated for each application of interest

- For delivery of CG applications, generalized split rendering architecture and its relevant traffic characteristics are considered.

Proposal 2: The following 2 categorizations of XR representative services for DL services as the starting point

- 25Mbps,10-6, 300, fixed periodicity, fixed packet size

- 100Mbps, 10-3, [5-30] fixed periodicity and fixed packet size/statistical model for fluctuating traffic

- FFS periodicity values and the modeling of variant inter arrival rate

- FFS statistical model for fluctuating traffic, target to down-select in RAN1#103-e between Gaussian distribution and Pareto distribution

Proposal 3: Adopt the three-step methodology to derive the traffic models for Gaussian/Pareto distribution of file size.

- Step 1: Generate the mean packet size according to the packet arrival rate and the data rate requirement.

- Step 2: Determine the minimum file size or the standard derivation parameter based on the relationship between the mean data rate and the minimum/maximum (i.e. truncated) data rate.

- Step 3: Determine the remaining parameter, if any, by jointly considering the mean file size and the parameter obtained from step 2.

Proposal 4: The following 2 categorizations of XR representative services for UL services as the starting point.

- 28kbps,10-6, 300, fixed periodicity, fixed packet size

- 2.7-50Mbps, 10-3, [5-30] fixed periodicity and fixed packet size/statistical model for fluctuating traffic

- FFS periodicity values and the modeling of variant inter arrival rate

- FFS statistical model for fluctuating traffic, target to down-select in RAN1#103-e between Gaussian distribution and Pareto distribution

Proposal 5: Consider the capacity metric and coverage enhancement metric in XR evaluation.

- Capacity metric should include both the number/percentage of UEs satisfying the PDB/PER/Rate requirement and the statistical metrics regarding the CDF of UPT.

- Coverage metric should use the service based metric i.e. calculate MPL and compare it with the ISD. For the link level simulations used to generate the MPL, the following discussion points should be settled.

- For DL, the number of PRBs should be generated by considering the full bandwidth dedicated. TBS should be determined by an agreed MCS.

- For UL, the PRBs/TBS/MCS should be set in accordance with the UL traffic.

- Target BLER should be set in accordance with the traffic, e.g. 10-3 or 10-6.

References

RP-19324, “ New SID on XR Evaluations for NR”, RAN#86, QC.

26.928, “Extended Reality (XR) in 5G”.

23.501, “System architecture for the 5G System (5GS); Stage 2”.

R1-2007480, RAN1#102-e.

- Appendix A Simulation parameters

TableA.1simulation parameters for Indoor Hotspot scenario

| Parameters | Value |
| --- | --- |
| Scenario | 5G UMI |
| Inter-BS distance | 200m |
| Carrier frequency | 3.5 GHz |
| Duplex Mode / Simulation bandwidth | 100 MHz |
| SCS | 30KHz |
| BS Antenna Configuration | 64 Tx antenna ports, and (dH, dV) = (0.5λ, 0.8λ); 64TX:(M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 8, 4); The antenna tilt is 6 degrees. |
| UE Antenna Configuration | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |
| Transmit Power | 44 dBm at TRP, and 23 dBm at UE |
| Antenna Height | 10 m for BS and 1.5 m for UE |
| Antenna Element Gain | 8dBi for BS and 0 dBi for UE |
| Receiver Noise Figure | 5 dB for BS and 7 dB for UE |
| UE distribution | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |
| Number of UEs per cell | up to 10 |
| Scheduling Algorithm | MU-MIMO+ PF |
| HARQ/repetition | HARQ retransmission |
| Channel estimation | Realist |
| BS receiver | MMSE-IRC |

TableA.2 simulation parameters for Indoor Hotspot scenario

| Parameters | Value |
| --- | --- |
| Scenario | Indoor Hotspot |
| Inter-BS distance | 20m |
| Carrier frequency | 3.5 GHz |
| Duplex Mode / Simulation bandwidth | 100 MHz |
| SCS | 30KHz |
| BS Antenna Configuration | 64 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ) 64TX:(M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 8, 4); The antenna tilt is 90 degrees. |
| UE Antenna Configuration | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |
| Transmit Power | 30 dBm at TRP, and 23 dBm at UE |
| Antenna Height | 3 m for BS and 1.5 m for UE |
| Antenna Element Gain | 5dBi for BS and 0 dBi for UE |
| Receiver Noise Figure | 5 dB for BS and 7 dB for UE |
| UE distribution | 100% indoor; Use 3km/h for modeling fading channel; |
| Number of UEs per cell | up to 10 |
| Scheduling Algorithm | MU-MIMO+ PF |
| HARQ/repetition | HARQ retransmission |
| Channel estimation | Realist |
| BS receiver | MMSE-IRC |

- Appendix B Simulation results

Resource utilization

Figure B.1 Resource utilization in Umi scenario

Figure B.2 Resource utilization in Indoor Hotspot scenario

Average throughput

Figure B.3 Date rate is 30Mbps in Umi Scenario

Figure B.4 Date rate is 60Mbps in Umi Scenario

Figure B.5 Date rate is 100Mbps in Umi Scenario