3GPP TSG-RAN WG1 Meeting #103-e	R1-2007984

e-Meeting, 26th October – 13th November, 2020

Agenda Item:	8.2.3

Source:	Ericsson

Title:	Evaluation results for NR in 52.6 to 71 GHz

Document for:	Discussion, Decision

# 1	Introduction

This document contains link and system level evaluation results for operation of NR in 52.6 to 71 GHz using the agreed reporting templates in [1].

# 2	Link Level Evaluation Results

### 2.1.1	Link level evaluations for CP-OFDM

In [2], we provide extensive comparison between the TR 38.803 Ex 2 models that have been used by RAN1 discussion so far and the new phase noise models proposed in RAN4. To compare the impact of different phase noise models, we consider the following three sets of phase noise models:

- PN model set 1

  - BS: Ex2 BS

  - UE: Ex2 UE

- PN model set 2

  - BS: Ex2 BS

  - UE: R4-2011494 (ref [7])

- PN model set 3

  - BS: R4-2010176 DM=0 dB (ref [6])

  - UE: R4-2010176 DM=5 dB (ref [6])

For the TDL-A channel with 10 and 40 ns, we have presented and discussed the link performance with CPE or simple ICI compensation for PN model set 1, 2, and 3 in [2]. The SNR values in dB, which achieves 10% and 1% PDSCH BLER, for these PN model sets are provided in section 2.1.1.

#### 2.1.1.1	TDL channel model

In this section, we provide link-level evaluation results in terms of DL-SCH block error rate (BLER) vs. signal to noise ratio (SINR) for TDL-A channel to assist in the proper selection of SCS. In all link-level simulations for OFDM, current Rel-15 PTRS configurations (K=2 and L=1) are used, and we compare CPE compensation only vs. ICI compensation. PN model set 1 is assumed. According to the evaluation assumptions in [1], three channel bandwidths are evaluated.

- 400 MHz

  - 256 RBs @ 120 kHz SCS

  - 128 RBs @ 240 kHz SCS

  - 64 RBs @ 480 kHz SCS

- 1600 MHz

  - 256 RBs @ 480 kHz SCS

  - 128 RBs @ 960 kHz SCS

- 2000 MHz

  - 160 RBs @ 960 kHz SCS

In Figure 1, we provide BLER for MCS 22, MCS 16, and MCS 7 with CPE compensation. The results show phase noise has negative impacts on link performance of smaller sub-carrier spacings particularly for higher MCSs when only CPE compensation is performed. It is noted that, when the delay spreads are low, a system with smaller number of sub-carriers generally has better performance than one with larger number of sub-carriers due to less phase noise power aggregated by all the sub-carriers. However, with larger delay spreads, systems with large SCS start to suffer from inter-symbol interference (ISI). For the example of 960 kHz SCS, link performance error floor starts to develop for the 64QAM in a channel with 40 ns average delay spreads.

In Figure 2, we provide BLER for the same agreed setups using three-tap de-ICI compensation (i.e., u=1). The results show that, with simple ICI compensation, link performance of smaller sub-carrier spacings can be brought on par with that of larger sub-carrier spacings particularly in the range of typical link adaptation target of 10% BLER for channels with low to moderate average delay spreads. Therefore, there is no need to drastically increase SCS, e.g., to 960 kHz, to combat phase noise.

However, for large delay spreads, systems with larger SCS still incur ISI-induced performance losses and, as a result, are outperformed by systems with smaller SCS.  The SNR values in dB, which achieves 10% and 1% PDSCH BLER, are shown in Table 1 and  Table 2 for only CPE and ICI compensation, respectively.

Figure 1:  BLER for TDL-A with 5 ns, 10 ns, 20 ns, 40 ns with CPE compensation (performance results for MCS 7, MCS 16, and MCS 22 are plotted with red, black, and blue curves, respectively). PN model set 1 is assumed.

Figure 2: BLER for TDL-A with 5 ns, 10 ns, 20 ns, and 40 ns with ICI compensation (performance results for MCS 7, MCS 16, and MCS 22 are plotted with red, black, and blue curves, respectively). PN model set 1 is assumed.

To further summarize and crystallize the above link performance results, we compare, in Figure 3, the 64QAM link performance of the following:

- 960 kHz SCS with CPE compensation (1.6 GHz BW)

- 480 kHz SCS with ICI compensation (1.6 GHz BW)

It can be observed that, for channels with small delay spreads (5/10 ns), link performance of both setups is comparable with negligible differences. However, for channels with moderate delay spreads (20/40 ns), 480 kHz SCS with ICI compensation can outperform 960 kHz SCS with CPE compensation substantially.

Phase noise induced performance issues for the OFDM waveform in the 52.6 – 71 GHz frequency range can be effectively addressed with the Rel-15 PTRS structure and simple ICI compensation algorithms. Performance with SCS of 480 kHz with simple ICI compensation is on par or better than the performance with 960 kHz with CPE compensation only.

Figure 3: BLER for TDL-A with 5 ns, 10 ns, 20 ns, and 40 ns for 480 kHz (with ICI compensation) and 960 (with CPE compensation) for MCS 22. PN model set 1 is assumed.

#### 2.1.1.2	CDL-B channel model

In this section, we provide link-level evaluation results in the presence of phase noise for CDL-B channel as defined in of TR38.901 Section 7.7.1 [4] for the following three channel BW.

- 400 MHz

  - 256 RBs @ 120 kHz SCS

  - 128 RBs @ 240 kHz SCS

  - 64 RBs @ 480 kHz SCS

- 1600 MHz

  - 256 RBs @ 480 kHz SCS

  - 128 RBs @ 960 kHz SCS

- 2000 MHz

  - 160 RBs @ 960 kHz SCS

In all link-level simulations for OFDM, current Rel-15 PTRS configurations (K=2 and L=1) are used, and we compare CPE compensation only vs. ICI compensation. According to the evaluation assumptions in [1],  we provide performance results for the following scenarios:

- CDL-B(20 ns), 4x8 gNB, 2x2 UE

- CDL-B(50 ns), 4x8 gNB, 2x2 UE

For beam forming, dual panel operation with UE (nearly) perfectly oriented towards the mean AoA is considered. As shown in Figure 4 and Figure 5, with simple ICI compensation, the link performance for smaller sub-carrier spacings, e.g., 240 kHz and 480 kHz, are on par with that of 960 kHz. Therefore, there is no need to drastically increase SCS, e.g., to 960 kHz, to combat phase noise. The SNR values in dB, which achieves 10% and 1% PDSCH BLER, are shown in Table 1 and  Table 2 for  only CPE and ICI compensation, respectively.

Figure 4: BLER for CDL-B (20 ns) with 4x8 gNB and 2x2 UE arrays per polarization for CPE (left) and ICI (right) compensation. Performance results for MCS7, MCS 16 and MCS 22 are plotted with red, black, and blue curves, respectively. PN model set 1 is assumed.

Figure 5: BLER for CDL-B (50 ns) with 4x8 gNB and 2x2 UE arrays per polarization for CPE (left) and ICI (right) compensation. Performance results for MCS7, MCS 16 and MCS 22 are plotted with red, black, and blue curves, respectively. PN model set 1 is assumed.

#### 2.1.1.3	CDL-D channel model

In this section, we provide link-level evaluation results in the presence of phase noise for CDL-D channel for the following three channel BW.

- 400 MHz

  - 256 RBs @ 120 kHz SCS

  - 128 RBs @ 240 kHz SCS

  - 64 RBs @ 480 kHz SCS

- 1600 MHz

  - 256 RBs @ 480 kHz SCS

  - 128 RBs @ 960 kHz SCS

- 2000 MHz

  - 160 RBs @ 960 kHz SCS

In Figure 6 and Figure 7, we compare CPE compensation only vs. ICI compensation using a 3-tap de-ICI filter (i.e., u=1) for the following scenarios:

- CDL-D (20 ns), 4x8 gNB, 2x2 UE

- CDL-D (30 ns), 4x8 gNB, 2x2 UE

Given the nature of the LOS channels, there is generally no observable performance degradation due to excessive delay spreads for sub-carrier spacing up to 960 kHz. In all cases, simple ICI compensation procedure results in the link performance of smaller sub-carrier spacings to be on par with that of the larger sub-carrier spacings. Therefore, there is no need to drastically increase SCS, e.g., to 960 kHz, to combat phase noise.  The SNR values in dB, which achieves 10% and 1% PDSCH BLER, are shown in Table 1 and  Table 2 for  only CPE and ICI compensation, respectively.

Figure 6: BLER for CDL-D (20 ns) with 4x8 gNB and 2x2 UE arrays per polarization for CPE (left) and ICI (right) compensation. Performance results for MCS7, MCS 16 and MCS 22 are plotted with red, black, and blue curves, respectively. PN model set 1 is assumed.

Figure 7: BLER for CDL-D (30 ns) with 4x8 gNB and 2x2 UE arrays per polarization for CPE (left) and ICI (right) compensation. Performance results for MCS7, MCS 16 and MCS 22 are plotted with red, black, and blue curves, respectively. PN model set 1 is assumed.

### 2.1.2	Link level evaluations for DFTS-OFDM

For DFT-s-OFDM, PTRS samples are mapped before the DFT operation (subsampled time domain) which enable full phase noise estimation and compensation using interpolation in time domain. For DFT-s-OFDM evaluation, we consider PT-RS configuration with Ng = 8, Ns = 4, L = 1. Our results demonstrate that the Rel-15 PTRS design address phase noise issues for all considered sub-carrier spacings in the 52.6 – 71 GHz frequency range. Therefore, it is not necessary to drastically increase the SCS e.g., to 960 kHz, for the purposes of combating phase noise. The SNR values in dB, which achieves 10% and 1% PUSCH BLER, are shown in Table 7.

Phase noise induced performance issues for the DFT-s-OFDM waveform in the 52.6 – 71 GHz frequency range can be addressed with the Rel-15 uplink PTRS structure and currently supported SCS values, e.g., 120 kHz.

Figure 8: BLER for TDL-A with 5 ns (left) and 10 ns (right). Performance results for MCS 7, MCS 16, and MCS 22 are plotted with red, black, and blue curves, respectively. PN model set 1 is assumed.

## 2.1	Tabulated Evaluation results for PDSCH/PUSCH

### 2.1.1	Evaluation results for PDSCH

In the section, we provide link-level evaluation results in terms of BLER vs. SNR for different channel model, delay spreads, subcarrier spacings, and bandwidths. To compare the impact of different phase noise models, we consider the following three sets of phase noise models (see a description on phase noise modeling in Section 3.3.1 of our companion paper [5]):

- PN model set 1

  - BS: Ex2 BS

  - UE: Ex2 UE

- PN model set 2

  - BS: Ex2 BS

  - UE: R4-2011494 (ref [7])

- PN model set 3

  - BS: R4-2010176 DM=0 dB (ref [6])

  - UE: R4-2010176 DM=5 dB (ref [6])

In all link-level simulations for OFDM, current Rel-15 PTRS configurations (K=2 and L=1) are used, and we compare CPE compensation only vs. ICI compensation.

Table 1: SNR in dB achieving PDSCH BLER of 10% or 1% with CPE compensation for PN model set 1.

| Tdoc / Source | MCS | Channel |  | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 480 kHz/1.6 GHz | 960KHz / 1.6 GHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | 7 | TDL-A, 5ns |  | 3.1/5.5 | 2.9/5.3 | 2.8/5.1 | 2.2/3.7 | 2.2/3.9 | 2.1/3.8 |
|  |  | TDL-A, 10ns |  | 2.6/4.6 | 2.4/4.5 | 2.5/4.3 | 2.1/3.4 | 2.2/3.7 | 2.2/3.6 |
|  |  | TDL-A, 20ns |  | 2.3/4.0 | 2.3/3.9 | 2.5/4.2 | 2.2/3.6 | 2.4/3.8 | 2.4/3.7 |
|  |  | TDL-A, 40 ns |  | 2.2/3.7 | 2.4/3.9 | 2.7/4.3 | 2.5/3.8 | 2.8/4.2 | 2.8/4.2 |
|  |  | CDL-B, 20ns |  | 2.6/5 | 2.3/4.6 | 2.3/4.6 | 1.7/3.2 | 1.7/3.2 | 1.7/3.2 |
|  |  | CDL-B, 50ns |  | 1.8/3.4 | 1.8/3.2 | 1.9/3.4 | 1.7/2.9 | 1.8/3 | 1.8/3 |
|  |  | CDL-D, 20ns |  | 0.3/1.5 | 0.2/1.4 | 0.1/1.2 | 0.2/1.3 | 0.2/1.4 | 0.2/1.4 |
|  |  | CDL-D, 30ns |  | 0.3/1.5 | 0.2/1.4 | 0/1.2 | 0.2/1.4 | 0.2/1.4 | 0.2/1.4 |
|  | 16 | TDL-A, 5ns |  | 11.8/14.3 | 11.3/13.88 | 11.0/13.4 | 10.5/12.4 | 10.0/11.9 | 10.0/11.8 |
|  |  | TDL-A, 10ns |  | 11.3/13.5 | 10.8/13.0 | 10.6/12.6 | 10.3/11.9 | 9.8/11.4 | 9.9/11.3 |
|  |  | TDL-A, 20ns |  | 11.0/12.9 | 10.5/12.3 | 10.3/12.0 | 10.2/11.6 | 9.9/11.2 | 9.9/11.3 |
|  |  | TDL-A, 40 ns |  | 10.8/12.5 | 10.4/11.9 | 10.4/11.9 | 10.5/11.7 | 10.8/12.4 | 11.0/12.7 |
|  |  | CDL-B, 20ns |  | 11.2/13.8 | 10.7/13 | 10.5/13.0 | 10.0/11.7 | 9.4/10.9 | 9.4/10.9 |
|  |  | CDL-B, 50ns |  | 10.5/12.3 | 10.0/11.7 | 9.9/11.5 | 9.8/11.2 | 9.4/10.5 | 9.5/10.7 |
|  |  | CDL-D, 20ns |  | 8.7/9.9 | 8.2/9.4 | 8.0/9.2 | 8.3/9.6 | 7.8/9.0 | 7.9/9.1 |
|  |  | CDL-D, 30ns |  | 8.7/9.6 | 8.2/9.4 | 8.0/9.2 | 8.3/9.6 | 7.8/8.9 | 7.9/9.1 |
|  | 22 | TDL-A, 5ns |  | -/- | -/- | 17.2/19.7 | 21.1/- | 16.1/18.2 | 16.3/18.6 |
|  |  | TDL-A, 10ns |  | -/- | -/- | 16.8/20.0 | 21.3/- | 15.9/17.8 | 16.1/18.3 |
|  |  | TDL-A, 20ns |  | -/- | -/- | 16.6/19.6 | 21.4/- | 16.1/18.0 | 16.3/18.6 |
|  |  | TDL-A, 40 ns |  | -/- | -/- | 16.7/20.1 | 22/- | 20.7/- | 21.6/- |
|  |  | CDL-B, 20ns |  | -/- | -/- | 16.7/20.4 | 21.4/- | 15.4/17.5 | 15.8/18.1 |
|  |  | CDL-B, 50ns |  | -/- | -/- | 16.2/19.5 | 21.5/- | 15.5/17.4 | 15.9/18.3 |
|  |  | CDL-D, 20ns |  | 21.8/- | 17.4/- | 14.0/15.8 | 17.0/- | 13.6/15.0 | 13.9/15.5 |
|  |  | CDL-D, 30ns |  | 21.8/- | 17.4/- | 13.9/15.8 | 16.9/- | 13.5/14.9 | 13.9/15.5 |
|  |  |  | Additional report/notes: - PN model set 1: BS: Ex2 BS and UE: Ex2 UE - CPE compensation - Normal CP - antenna configuration for CDL model Configuration 2: - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) - PTRS: K=2, L=1 - DMRS configuration: 2 DMRS symbols at (2,11) - No TRS, No CSI-RS - The effective CR for MCS22, MCS16, and MCS 7 are 0.685, 0.678, and 0.539, respectively. |  |  |  |  |  |  |

Table 2: SNR in dB achieving PDSCH BLER of 10% or 1% with ICI compensation for PN model set 1.

| Tdoc / Source | MCS | Channel |  | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 480 kHz/1.6 GHz | 960KHz / 1.6 GHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | 7 | TDL-A, 5ns |  | 3.2/5.6 | 3.2/5.6 | 3.3/3.9 | 2.3/4.3 | 2.4/4 | 2.4/4 |
|  |  | TDL-A, 10ns |  | 2.7/4.7 | 2.7/4.9 | 2.9/3.6 | 2.2/4.0 | 2.4/3.8 | 2.4/3.8 |
|  |  | TDL-A, 20ns |  | 2.4/4.0 | 2.5/4.1 | 3.0/4.7 | 2.3/3.6 | 2.7/4.1 | 2.6/4.0 |
|  |  | TDL-A, 40 ns |  | 2.3/3.8 | 2.6/4.1 | 3.2/4.8 | 2.6/3.9 | 3.0/4.5 | 3.0/4.5 |
|  |  | CDL-B, 20ns |  | 2.6/5.1 | 2.5/4.8 | 2.8/5.1 | 1.8/3.3 | 2.0/3.5 | 2.0/3.5 |
|  |  | CDL-B, 50ns |  | 1.9/3.5 | 2.0/3.4 | 2.4/3.9 | 1.8/3.0 | 2.1/3.3 | 2.1/3.3 |
|  |  | CDL-D, 20ns |  | 0.4/1.5 | 0.4/1.6 | 0.6/1.7 | 0.3/1.5 | 0.5/1.7 | 0.5/1.7 |
|  |  | CDL-D, 30ns |  | 0.3/1.5 | 0.4/1.6 | 0.5/1.7 | 0.3/1.5 | 0.5/1.7 | 0.5/1.7 |
|  | 16 | TDL-A, 5ns |  | 11.3/13.7 | 11.0/13.4 | 11.2/13.6 | 10.0/11.7 | 10.1/13.4 | 10.0/13.4 |
|  |  | TDL-A, 10ns |  | 10.8/12.9 | 10.5/12.6 | 10.7/12.7 | 9.8/11.3 | 9.9/11.4 | 9.9/11.3 |
|  |  | TDL-A, 20ns |  | 10.4/12.2 | 10.2/11.9 | 10.4/12.1 | 9.7/11.0 | 10.0/11.3 | 10.0/11.3 |
|  |  | TDL-A, 40 ns |  | 10.2/11.8 | 10.0/11.5 | 10.6/11.9 | 10.0/11.1 | 10.9/12.4 | 11.0/12.5 |
|  |  | CDL-B, 20ns |  | 10.6/14.0 | 10.3/12.7 | 10.6/13.1 | 9.5/11.0 | 9.4/11 | 9.5/11 |
|  |  | CDL-B, 50ns |  | 9.9/11.6 | 9.7/11.2 | 10.0/11.6 | 9.3/10.5 | 9.5/10.6 | 9.5/10.7 |
|  |  | CDL-D, 20ns |  | 8.2/9.4 | 8.0/9.1 | 8.2/9.3 | 7.9/9.1 | 7.9/9.1 | 8.0/9.2 |
|  |  | CDL-D, 30ns |  | 8.2/9.4 | 8.0/9.1 | 8.1/9.3 | 7.9/9.1 | 7.9/9 | 8.0/9.1 |
|  | 22 | TDL-A, 5ns |  | 18.5/- | 17.0/19.6 | 16.3/18.7 | 16.1/18.0 | 15.6/17.4 | 15.5/17.3 |
|  |  | TDL-A, 10ns |  | 18.2/21.3 | 16.5.18.7 | 15.8/17.8 | 15.8/17.5 | 15.4/16.9 | 15.3/16.8 |
|  |  | TDL-A, 20ns |  | 17.8/20.6 | 16.2/18.1 | 15.5/17.2 | 15.6/17 | 15.5/16.9 | 15.5/16.9 |
|  |  | TDL-A, 40 ns |  | 17.5/20.6 | 16.0/17.6 | 15.7/17.2 | 15.7/17.1 | 19.3/- | 19.4/- |
|  |  | CDL-B, 20ns |  | 18.2/- | 16.5/19.1 | 15.7/18.2 | 15.7/17.6 | 14.8/16.5 | 14.9/16.5 |
|  |  | CDL-B, 50ns |  | 17.5 /20.8 | 15.8/17.6 | 15.0/16.7 | 15.2/16.6 | 14.8/16.1 | 14.9/16.4 |
|  |  | CDL-D, 20ns |  | 14.7/16.4 | 13.7/15.0 | 13.2/14.5 | 13.5/14.9 | 13.1/14.3 | 13.3/14.5 |
|  |  | CDL-D, 30ns |  | 14.7/16.4 | 13.7/15.0 | 13.2/14.5 | 13.5/14.9 | 13.0/14.3 | 13.3/14.5 |
|  |  |  | Additional report/notes: - PN model set 1: BS: Ex2 BS and UE: Ex2 UE - ICI compensation - Normal CP - antenna configuration for CDL model Configuration 2: - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) - PTRS: K=2, L=1 - DMRS configuration: 2 DMRS symbols at (2,11) - No TRS, No CSI-RS - The effective CR for MCS22, MCS16, and MCS 7 are 0.685, 0.678, and 0.539, respectively. |  |  |  |  |  |  |

Table 3: SNR in dB achieving PDSCH BLER of 10% or 1% with CPE compensation for PN model set 2

| Tdoc / Source | MCS | Channel |  | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 480 kHz/1.6 GHz | 960KHz / 1.6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | TDL-A, 10ns |  | 10.6/12.7 | 10.5/12.6 | 10.6/12.6 | 10.1/11,5 | 10.0/11.5 |
|  |  | TDL-A, 40ns |  | 10.1/11.58 | 10.1/11.5 | 10.3/11.7 | 10.1/11.4 | 11.2/12.9 |
|  | 22 | TDL-A, 10ns |  | 17.8/20.3 | 17.3/19.7 | 16.5/18.7 | 17.6/20.3 | 16.9/18.9 |
|  |  | TDL-A, 40ns |  | 17.2/19.0 | 16.8/18.5 | 16.4/18.1 | 17.7/20.8 | -/- |
|  |  |  | Additional report/notes: - PN model set 2: BS: Ex2 BS and UE: R4-2011494 (ref [7]) - CPE compensation - Normal CP - PTRS: K=2, L=1 - DMRS configuration: 2 DMRS symbols at (2,11) - No TRS, No CSI-RS - The effective CR for MCS22, MCS16, and MCS 7 are 0.685, 0.678, and 0.539, respectively. |  |  |  |  |  |  |

Table 4: SNR in dB achieving PDSCH BLER of 10% or 1% with ICI compensation for PN model set 2

| Tdoc / Source | MCS | Channel |  | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 480 kHz/1.6 GHz | 960KHz / 1.6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | TDL-A, 10ns |  | 10.6/12.7 | 10.5/12.6 | 10.8/12.8 | 10.0/11.4 | 10.0/11.5 |
|  |  | TDL-A, 40ns |  | 10.1/11.6 | 10.1/11.6 | 10.6/12.0 | 10.0/11.2 | 11.2/12.9 |
|  | 22 | TDL-A, 10ns |  | 17.4/19.7 | 17.0/19.2 | 16.4/18.4 | 16.4/18.0 | 15.9/17.4 |
|  |  | TDL-A, 40ns |  | 16.8/18.4 | 16.4/18.0 | 16.2/17.8 | 16.3/17.8 | 21.0/- |
|  |  |  | Additional report/notes: - PN model set 2: BS: Ex2 BS and UE: R4-2011494 (ref [7]) - ICI compensation - Normal CP - PTRS: K=2, L=1 - DMRS configuration: 2 DMRS symbols at (2,11) - No TRS, No CSI-RS - The effective CR for MCS22, MCS16, and MCS 7 are 0.685, 0.678, and 0.539, respectively. |  |  |  |  |  |  |

Table 5: SNR in dB achieving PDSCH BLER of 10% or 1% with CPE compensation for PN model set 3

| Tdoc / Source | MCS | Channel |  | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 480 kHz/1.6 GHz | 960KHz / 1.6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | TDL-A, 10ns |  | 10.6/12.7 | 10.5/12.6 | 10.5/12.6 | 10.0/11.5 | 10.0/11.4 |
|  |  | TDL-A, 40ns |  | 10.1//11.6 | 10.0/11.5 | 10.3/11.7 | 10.0/11.3 | 11.2/12.9 |
|  | 22 | TDL-A, 10ns |  | 17.8/20.4 | 17.3/19.7 | 16.4/18.6 | 17.4/20.5 | 16.6/18.6 |
|  |  | TDL-A, 40ns |  | 17.3/19.1 | 16.8/18.6 | 16.3/18.0 | 17.6/21.3 | -/- |
|  |  |  | Additional report/notes: - PN model set 3: PN model: BS: R4-2010176 DM=0 dB (ref [6]) and UE: R4-2010176 DM=5 dB (ref [6]) - CPE compensation - Normal CP - PTRS: K=2, L=1 - DMRS configuration: 2 DMRS symbols at (2,11) - No TRS, No CSI-RS - The effective CR for MCS22, MCS16, and MCS 7 are 0.685, 0.678, and 0.539, respectively. |  |  |  |  |  |  |

Table 6: SNR in dB achieving PDSCH BLER of 10% or 1% with ICI compensation for PN model set 3

| Tdoc / Source | MCS | Channel |  | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 480 kHz/1.6 GHz | 960KHz / 1.6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | TDL-A, 10ns |  | 10.6/12.7 | 10.6/12.7 | 10.6/12.7 | 10.0/11.3 | 10.0/11.7 |
|  |  | TDL-A, 40ns |  | 10.1/11.6 | 10.1/11.6 | 10.4/11.8 | 10.0/11.2 | 11.2/12.8 |
|  | 22 | TDL-A, 10ns |  | 17.3/19.6 | 16.8/19.0 | 16.2/18.2 | 16.2/17.8 | 15.7/17.1 |
|  |  | TDL-A, 40ns |  | 16.7/18.4 | 16.3/17.8 | 16.1/17.7 | 16.1/17.6 | 20.2/- |
|  |  |  | Additional report/notes: - PN model set 3: BS: R4-2010176 DM=0 dB (ref [6]) and UE: R4-2010176 DM=5 dB (ref [6] ) - ICI compensation - Normal CP - PTRS: K=2, L=1 - DMRS configuration: 2 DMRS symbols at (2,11) - No TRS, No CSI-RS - The effective CR for MCS22, MCS16, and MCS 7 are 0.685, 0.678, and 0.539, respectively. |  |  |  |  |  |  |

### Evaluation results for PUSCH

In this section, the block error rate performance of PUSCH is evaluated for TDL-A channel with different delay spreads and for different combinations of SCS and bandwidth for PN model set 1 (BS: Ex2 BS and UE: Ex2 UE).

We assume that transform precoding is enabled and the resultant waveform is DFT-s-OFDM. For DFT-s-OFDM, PTRS samples are mapped before the DFT operation (subsampled time domain) which enable full phase noise estimation and compensation using interpolation in time domain.

Table 7: SNR in dB achieving PUSCH BLER of 10% or 1% with PN compensation for PN model set 1

| Tdoc / Source | MCS | Channel |  | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | 7 | TDL-A, 5ns |  | 3.3/5.6 | 3.0/5.0 | 3.0/5.0 | 3.1/4.6 |
|  |  | TDL-A, 10ns |  | 3.1/5.1 | 3.0/4.6 | 2.9/4.6 | 3.2/4.6 |
|  | 16 | TDL-A, 5ns |  | 11.2/13.6 | 10.5/12.5 | 10.4/12.4 | 10.2/11.7 |
|  |  | TDL-A, 10ns |  | 10.6/12.7 | 10.2/11.8 | 10.2/11.8 | 10.2/11.5 |
|  | 22 | TDL-A, 5ns |  | 17.3/20.1 | 16.5/19.1 | 16.0/18.3 | 15.5/17.2 |
|  |  | TDL-A, 10ns |  | 17.0/19.4 | 16.2/19.0 | 15.7/17.8 | 15.4/16.8 |
|  |  |  | Additional report/notes: - PN model set 1: BS: Ex2 BS and UE: Ex2 UE - PN compensation - Normal CP - PUSCH waveform: DFT-S-OFDM - PTRS: Ng = 8, Ns = 4, L =1 - DMRS configuration: 2 DMRS symbols at (2,11) |  |  |  |  |  |

## 2.2	Tabulated Evaluation results for PSS/SSS and PBCH

### 2.2.1	Evaluation results for PSS/SSS

Simulation results for PSS/SSS detection can be found in Table 8

Table 8: SINR in dB achieving cell ID detection probability of 90% by one-shot detection from PSS/SSS

| Tdoc / Source | Channel | 120KHz | 240KHz | 480KHz | 960KHz |
| --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | TDL-A, 5ns | -5.4 | -5.3 | -5.1 | -4.8 |
|  | TDL-A, 10ns | -5.4 | -5.4 | -4.8 | -4.4 |
|  | TDL-A, 20ns | -5.4 | -4.9 | -4.3 | -4.3 |
|  | TDL-A, 40ns | -5.1 | -4.6 | -4.1 | -4.0 |
|  | CDL-B, 20ns |  |  |  |  |
|  | CDL-B, 50ns |  |  |  |  |
|  | CDL-D, 20ns |  |  |  |  |
|  | CDL-D, 30ns |  |  |  |  |
|  | Additional report/notes: - frequency offset: 10ppm - the number and granularity of the frequency locations: -x...x, with granularity SCS/2 where x=ceil(freq_error/SCS)*SCS for coarse search followed by refinement using hypothesis bi-section - antenna configuration for CDL model: N/A - any optional or other assumption/parameters used not as in the baseline: Medium antenna correlation. Every 4th SSB use one of the following precoders sqrt(1/2) [1 1], [1 j], [1 -1] and [1 -j]. - false alarm rate: 1% - criteria for PSS detection success: Correct cell ID and frequency offset estimate error is less than SCS/4. - Simulation duration: 5000 SS/PBCH blocks |  |  |  |  |

### 2.2.2	Evaluation results and link budget for PBCH

Simulation results for PBCH reception can be found in Table 9 for individual blocks and in Table 10 for combination of 4 consecutively transmitted blocks.

Table 9: SINR in dB achieving PBCH BLER of 10%, using 1 transmission.

| Tdoc / Source | Channel | 120KHz | 240KHz | 480KHz | 960KHz |
| --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | TDL-A, 5ns | -5.0 | -5.3 | -5.2 | -4.7 |
|  | TDL-A, 10ns | -5.3 | -5.3 | -4.9 | -4.1 |
|  | TDL-A, 20ns | -5.3 | -5.0 | -4.4 | -4.1 |
|  | TDL-A, 40ns | -5.0 | -4.5 | -4.3 | -3.9 |
|  | CDL-B, 20ns |  |  |  |  |
|  | CDL-B, 50ns |  |  |  |  |
|  | CDL-D, 20ns |  |  |  |  |
|  | CDL-D, 30ns |  |  |  |  |
|  | Additional report/notes: - frequency offset: 10ppm - the number and granularity of the frequency locations: -x...x, with granularity SCS/2 where x=ceil(freq_error/SCS)*SCS for coarse search followed by refinement using hypothesis bi-section - antenna configuration for CDL model: N/A - any optional or other assumption/parameters used not as in the baseline: Medium antenna correlation. Every 4th SSB use one of the following precoders sqrt(1/2) [1 1], [1 j], [1 -1] and [1 -j]. - false alarm rate: 1% - criteria for PSS detection success: Correct cell ID and frequency offset estimate error is less than SCS/4. Simulation duration: 5000 SS/PBCH blocks |  |  |  |  |

Table 10: SINR in dB achieving PBCH BLER of 10%, using 4 transmissions.

| Tdoc / Source | Channel | 120KHz | 240KHz | 480KHz | 960KHz |
| --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | TDL-A, 5ns | -10.4 | -10.4 | -10.2 | -9.8 |
|  | TDL-A, 10ns | -10.3 | -10.3 | -9.9 | -9.5 |
|  | TDL-A, 20ns | -10.0 | -10.0 | -9.6 | -8.9 |
|  | TDL-A, 40ns | -9.9 | -9.6 | -9.0 | -8.3 |
|  | CDL-B, 20ns |  |  |  |  |
|  | CDL-B, 50ns |  |  |  |  |
|  | CDL-D, 20ns |  |  |  |  |
|  | CDL-D, 30ns |  |  |  |  |
|  | Additional report/notes: - frequency offset: 10ppm - the number and granularity of the frequency locations: -x...x, with granularity SCS/2 where x=ceil(freq_error/SCS)*SCS for coarse search followed by refinement using hypothesis bi-section - antenna configuration for CDL model: N/A - any optional or other assumption/parameters used not as in the baseline: Medium antenna correlation. Every 4th SSB use one of the following precoders sqrt(1/2) [1 1], [1 j], [1 -1] and [1 -j]. - false alarm rate: 1% - criteria for PSS detection success: Correct cell ID and frequency offset estimate error is less than SCS/4. Simulation duration: 5000 SS/PBCH blocks |  |  |  |  |

An example of how Maximum Coupling Loss (MCL) and Maximum Isotropic Loss (MIL) is calculated is shown in Table 11. The element and beamforming gain used in the link budget calculations assumes Antenna Configuration 2 in the link level evaluation assumptions in TR 38.808 [3] for BS and UE. For BS, 20 dBi antenna gain is used: (10*log10(4x8) + 5 (element gain) = 20. For the UE, 6 dBi gain is used which assumes that the UE uses wider beams during initial access for SSB reception (reduction from 11 dBi). SNR values in Table 11 are taken from Table 9.

Table 11: MCL and MIL calculation example for PBCH, 1 transmission, TDL-A 3km/h 20ns, using no FDM with RMSI (SSB/CORESET 0 multiplexing pattern 1)

| Number of Tx tries | 1 tx-try | 1 tx-try | 1 tx-try | 1 tx-try |
| --- | --- | --- | --- | --- |
| Delay Spread [ns] | 20 | 20 | 20 | 20 |
| SCS [kHz] | 120 | 240 | 480 | 960 |
| Bandwidth [MHz] | 28.8 | 57.6 | 115.2 | 230.4 |
| Noise level [dBm] 1) | -89.4 | -86.4 | -83.4 | -80.4 |
| Required SNR [dB] 2) | -5.3 | -5.0 | -4.4 | -4.1 |
| Tx antenna gain [dBi] 3) | 20 | 20 | 20 | 20 |
| Rx antenna gain [dBi] 3) | 6 | 6 | 6 | 6 |
| Max Tx power (according to regulations) [dBm] 4) | 17.6 | 20.0 | 20.0 | 20.0 |
| Transmit Power [dBm] 5) | 17.6 | 20.0 | 20.0 | 20.0 |
| Maximum coupling loss (MCL) [dB] 6) | 112.3 | 111.4 | 107.8 | 104.5 |
| Maximum isotropic loss (MIL) [dB] 7) | 138.3 | 137.4 | 133.8 | 130.5 |
| 1) Over used subcarriers, assuming a noise figure of 10 dB and thermal noise spectral density of -174 dBm/Hz. 2) SNR required to have 10% BLER for 1Tx/Rx x-pol pair. 3) Element gain + beamforming gain 4) Considering ETSI/FCC EIRP limit of 40 dBm, ETSI BRAN (EIRP) PSD limits of 23 dB/MHz, FCC conducted power limit of 27 dBm above 100 MHz and with proportionally reduced power below 100 MHz. 5) Equals Max Tx power (according to regulations) 6) MCL = (Max Tx power) - (noise level) - (required SNR) 7) MIL = MCL + (Tx antenna gain) + (Rx antenna gain) |  |  |  |  |

## 2.3	Tabulated Evaluation results for PRACH

### 2.3.1	Link-level evaluation results for PRACH

Link-simulation results for PRACH can be found in Table 12 (PRACH format A3) and Table 13 (PRACH format B4), and the misdetection rates are also illustrated in Figure 9 (delay spreads 5 ns and 10 ns) and Figure 10 (delay spreads 20 ns and 40 ns). In all cases a maximum propagation round-trip time (RTT) of 380 ns (corresponding to ISD 100 m) was used, but smaller RTTs, e.g. 100 ns observed in indoor environments, lead to similar performance. Additional simulation details are provided in the table footnotes.

Table 12: SINR in dB achieving PRACH preamble misdetection probability of 1% and corresponding false alarm probability, for PRACH format A3 (L = 139/571/1151 refers to the PRACH sequence length)

| Tdoc / Source | Channel | 120KHz | 240KHz | 480KHz | 960KHz |
| --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | TDL-A, 5ns | –7.3 / ≤0.1% (L=139) –14.5 / ≤0.1% (L=571) –17.6 / ≤0.1% (L=1151) |  | –9.4 / ≤0.1% (L=139) –15.4 / ≤0.1% (L=571) | –9.1 / ≤0.1% (L=139) |
|  | TDL-A, 10ns | –7.9 / ≤0.1% (L=139) –14.6 / ≤0.1% (L=571) –17.5 / ≤0.1% (L=1151) |  | –9.4 / ≤0.1% (L=139) –15.7 / ≤0.1% (L=571) | –8.6 / ≤0.1% (L=139) |
|  | TDL-A, 20ns | –8.5 / ≤0.1% (L=139) –14.4 / ≤0.1% (L=571) –17.6 / ≤0.1% (L=1151) |  | –8.9 / ≤0.1% (L=139) –15.8 / ≤0.1% (L=571) | –8.4 / ≤0.1% (L=139) |
|  | TDL-A, 40ns | –8.4 / ≤0.1% (L=139) –14.6 / ≤0.1% (L=571) –17.9 / ≤0.1% (L=1151) |  | –8.5 / ≤0.1% (L=139) –15.3 / ≤0.1% (L=571) | –6.8 / ≤0.1% (L=139) |
|  | Additional report/notes: 1. PRACH format A3 2. No cyclic shifts 3. Random propagation round-trip time, uniformly distributed over [0, 380 ns] (corresponding to ISD 100 m). 4. Delay estimation tolerance is 0.5 × PUSCH CP (with PUSCH SCS assumed same as PRACH SCS). 5. The receiver structure in R1-1609672 is used, with NNC = 1. 6. The detection threshold was selected to yield a maximum false-alarm probability of 0.1% across all SNRs. |  |  |  |  |

Table 13: SINR in dB achieving PRACH preamble misdetection probability of 1% and corresponding false alarm probability, for PRACH format B4 (L = 139/571/1151 refers to the PRACH sequence length)

| Tdoc / Source | Channel | 120KHz | 240KHz | 480KHz | 960KHz |
| --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | TDL-A, 5ns | –9.6 / ≤0.1% (L=139) –16.7 / ≤0.1% (L=571) –20.0 / ≤0.1% (L=1151) |  | –11.8 / ≤0.1% (L=139) –17.9 / ≤0.1% (L=571) | –12.0 / ≤0.1% (L=139) |
|  | TDL-A, 10ns | –10.2 / ≤0.1% (L=139) –16.7 / ≤0.1% (L=571) –19.6 / ≤0.1% (L=1151) |  | –11.8 / ≤0.1% (L=139) –18.1 / ≤0.1% (L=571) | –11.4 / ≤0.1% (L=139) |
|  | TDL-A, 20ns | –10.8 / ≤0.1% (L=139) –16.3 / ≤0.1% (L=571) –19.7 / ≤0.1% (L=1151) |  | –11.4 / ≤0.1% (L=139) –18.3 / ≤0.1% (L=571) | –11.3 / ≤0.1% (L=139) |
|  | TDL-A, 40ns | –10.8 / ≤0.1% (L=139) –16.3 / ≤0.1% (L=571) –19.4 / ≤0.1% (L=1151) |  | –11.3 / ≤0.1% (L=139) –17.8 / ≤0.1% (L=571) | –9.2 / ≤0.1% (L=139) |
|  | Additional report/notes: 1. PRACH format B4 2. No cyclic shifts 3. Random propagation round-trip time, uniformly distributed over [0, 380 ns] (corresponding to ISD 100 m). 4. Delay estimation tolerance is 0.5 × PUSCH CP (with PUSCH SCS assumed same as PRACH SCS). 5. The receiver structure in R1-9609672 is used, with NNC = 1 for all cases except for 120 kHz with L = 571 or L = 1151, where NNC = 2 is used. 6. The detection threshold was selected to yield a maximum false-alarm probability of 0.1% across all SNRs. |  |  |  |  |

(a)						         (b)

Figure 9: Misdetection rate vs SNR for TDL-C with max RTT 380 ns and (a) delay spread 5 ns or (b) delay spread 10 ns, for different PRACH formats, subcarriers spacings and sequence length L.

(a)						         (b)

Figure 10: Misdetection rate vs SNR for TDL-C with max RTT 380 ns and (a) delay spread 20 ns or (b) delay spread 40 ns, for different PRACH formats, subcarriers spacings and sequence length L.

### 2.3.2	Link budget examples for PRACH

Link budgets in terms of maximum coupling loss (MCL) and maximum isotropic loss (MIL) are derived for a few example cases in Table 14. A transmit antenna gain (element gain + beamforming gain) of 6 dBi and a receive antenna gain of 20 dBi were assumed, like for SSB, cf. Section 2.2. It should be noted that as long as the transmit antenna gain stays above 6 dBi, the relative performance of the different considered cases is independent of the assumed antenna gains. However, this fact is heavily dependent on the UE EIRP limit being 25 dBm. In order to illustrate performance if a larger UE EIRP is allowed, or if a lower transmit antenna gain is used, Table 15 shows corresponding link-budget results if the UE-specific power limits (UE conducted power limit of 21 dBm and UE EIRP limit of 25 dBm) are not considered (while all other regulatory limits are still adhered to).

Table 14: Example PRACH link budgets for format B4, delay spread 20 ns, and max RTT 380 ns

| Parameter | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PRACH format | B4 | B4 | B4 | B4 | B4 | B4 |
| SCS [kHz] | 120 | 120 | 120 | 480 | 480 | 960 |
| Sequence length L | 139 | 571 | 1151 | 139 | 571 | 139 |
| Delay spread [ns] | 20 | 20 | 20 | 20 | 20 | 20 |
| Max RTT [ns] | 380 | 380 | 380 | 380 | 380 | 380 |
| Frequency occupancy [MHz] | 16.68 | 68.52 | 138.12 | 66.72 | 274.08 | 133.44 |
| Noise level [dBm] 1) | -94.78 | -88.64 | -85.60 | -88.76 | -82.62 | -85.75 |
| Required SNR [dB] 2) | -10.8 | -16.3 | -19.7 | -11.4 | -18.3 | -11.3 |
| Tx (UE) antenna gain [dBi] 3) | 6 | 6 | 6 | 6 | 6 | 6 |
| Rx (gNB) antenna gain [dBi] 3) | 20 | 20 | 20 | 20 | 20 | 20 |
| Max Tx power according to regulations [dBm] 4) | 19.22 | 25.36 | 27.00 | 25.24 | 27.00 | 27.00 |
| Tx power backoff based on CM [dB] 5) | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 |
| Max UE conducted power (backed off) [dBm] 6) | 18.7 | 18.7 | 18.7 | 18.7 | 18.7 | 18.7 |
| Max Tx power based on UE EIRP [dBm] 7) | 19 | 19 | 19 | 19 | 19 | 19 |
| Tx power [dBm] 8) | 18.7 | 18.7 | 18.7 | 18.7 | 18.7 | 18.7 |
| Maximum coupling loss (MCL) [dB] 9) | 124.3 | 123.6 | 124.0 | 118.9 | 119.6 | 115.7 |
| Maximum isotropic loss (MIL) [dB] 10) | 150.3 | 149.6 | 150.0 | 144.9 | 145.6 | 141.7 |
| 1) Over used subcarriers, assuming a noise figure of 7 dB and thermal noise spectral density of -174 dBm/Hz. 2) SNR required to have 1% misdetection rate, from Table 13. 3) Element gain + beamforming gain, same value as for SSB, cf. Section 2.2. 4) Considering ETSI/FCC EIRP limit of 40 dBm, ETSI BRAN (EIRP) PSD limits of 23 dB/MHz, FCC conducted power limit of 27 dBm above 100 MHz and with proportionally reduced power below 100 MHz. 5) Backoff from max UE conducted power, based on 95th percentile cubic metric (CM), which according to [8] is 2.3 dB for all supported Zadoff-Chu sequence lengths (L = 139/571/1151). 6) UE conducted power limit 21 dBm minus Tx power backoff based on CM. 7) UE EIRP limit of 25 dBm minus Tx antenna gain. 8) Minimum of max Tx power according to regulations, max UE conducted power, and max Tx power based on UE EIRP. 9) MCL = (Tx power) - (noise level) - (required SNR) 10) MIL = MCL + (Tx antenna gain) + (Rx antenna gain) |  |  |  |  |  |  |

Table 15: Example PRACH link budgets for format B4, delay spread 20 ns, max RTT 380 ns, without UE-specific power limits

| Parameter | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PRACH format | B4 | B4 | B4 | B4 | B4 | B4 |
| SCS [kHz] | 120 | 120 | 120 | 480 | 480 | 960 |
| Sequence length L | 139 | 571 | 1151 | 139 | 571 | 139 |
| Delay spread [ns] | 20 | 20 | 20 | 20 | 20 | 20 |
| Max RTT [ns] | 380 | 380 | 380 | 380 | 380 | 380 |
| Frequency occupancy [MHz] | 16.68 | 68.52 | 138.12 | 66.72 | 274.08 | 133.44 |
| Noise level [dBm] 1) | -94.78 | -88.64 | -85.60 | -88.76 | -82.62 | -85.75 |
| Required SNR [dB] 2) | -10.8 | -16.3 | -19.7 | -11.4 | -18.3 | -11.3 |
| Tx (UE) antenna gain [dBi] 3) | 6 | 6 | 6 | 6 | 6 | 6 |
| Rx (gNB) antenna gain [dBi] 3) | 20 | 20 | 20 | 20 | 20 | 20 |
| Max Tx power according to regulations [dBm] 4) | 19.22 | 25.36 | 27.00 | 25.24 | 27.00 | 27.00 |
| Tx power [dBm] 5) | 19.22 | 25.36 | 27.00 | 25.24 | 27.00 | 27.00 |
| Maximum coupling loss (MCL) [dB] 6) | 124.8 | 130.3 | 132.3 | 125.4 | 127.9 | 124.0 |
| Maximum isotropic loss (MIL) [dB] 7) | 150.8 | 156.3 | 158.3 | 151.4 | 153.9 | 150.0 |
| 1) Over used subcarriers, assuming a noise figure of 7 dB and thermal noise spectral density of -174 dBm/Hz. 2) SNR required to have 1% misdetection rate, from Table 13. 3) Element gain + beamforming gain, same value as for SSB, cf. Section 2.2. 4) Considering ETSI/FCC EIRP limit of 40 dBm, ETSI BRAN (EIRP) PSD limits of 23 dB/MHz, FCC conducted power limit of 27 dBm above 100 MHz and with proportionally reduced power below 100 MHz. 5) Equals max Tx power according to regulations. 6) MCL = (Tx power) - (noise level) - (required SNR) 7) MIL = MCL + (Tx antenna gain) + (Rx antenna gain) |  |  |  |  |  |  |

# 3	System Level Evaluation Results

## 3.1	Tabulated System level evaluation results for Indoor Scenario

### 3.1.1	System level evaluation results for scenario A

Table 16: System level evaluation results for scenario A, with/without LBT

| Tdoc / Source |  | Cases |  | Case 1: no LBT |  | Case 2: ED -47dBm |  |  | Case 3: ED-68 dBm |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) |  | 5%ile | 5566 | 3889 | 2448 | 5451 | 3518 | 2310 | 5201 | 3250 | 2007 |
|  |  |  | 50%ile | 9244 | 7380 | 5670 | 8851 | 7057 | 5356 | 8840 | 6583 | 4813 |
|  |  |  | 95%ile | 11219 | 10363 | 9247 | 10778 | 9882 | 8542 | 10739 | 9543 | 7780 |
|  |  |  | mean | 9002 | 7358 | 5877 | 8627 | 7019 | 5473 | 8595 | 6618 | 4929 |
|  | DL delay (s) |  | 5%ile | 0.019 | 0.020 | 0.023 | 0.020 | 0.021 | 0.025 | 0.020 | 0.022 | 0.028 |
|  |  |  | 50%ile | 0.024 | 0.032 | 0.042 | 0.025 | 0.033 | 0.045 | 0.025 | 0.036 | 0.050 |
|  |  |  | 95%ile | 0.038 | 0.058 | 0.090 | 0.039 | 0.063 | 0.097 | 0.040 | 0.066 | 0.112 |
|  |  |  | mean | 0.026 | 0.036 | 0.049 | 0.027 | 0.038 | 0.053 | 0.027 | 0.040 | 0.059 |
|  | UL UPT (Mbps) |  | 5%ile | 1869 | 1139 | 493 | 1718 | 960 | 487 | 1727 | 836 | 342 |
|  |  |  | 50%ile | 3409 | 2570 | 1835 | 3197 | 2361 | 1667 | 3130 | 1994 | 1236 |
|  |  |  | 95%ile | 4741 | 4199 | 3592 | 4419 | 3856 | 3314 | 4337 | 3444 | 2727 |
|  |  |  | mean | 3392 | 2659 | 1968 | 3183 | 2426 | 1810 | 3120 | 2088 | 1375 |
|  | UL delay (s) |  | 5%ile | 0.045 | 0.050 | 0.059 | 0.048 | 0.055 | 0.063 | 0.049 | 0.062 | 0.077 |
|  |  |  | 50%ile | 0.064 | 0.090 | 0.130 | 0.069 | 0.098 | 0.142 | 0.071 | 0.123 | 0.197 |
|  |  |  | 95%ile | 0.113 | 0.202 | 0.405 | 0.121 | 0.231 | 0.415 | 0.122 | 0.273 | 0.602 |
|  |  |  | mean | 0.072 | 0.107 | 0.179 | 0.076 | 0.119 | 0.188 | 0.078 | 0.146 | 0.261 |
|  | Arrival rate (files/s) |  |  | 0.47 | 1.53 | 2.45 | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 |
|  | 𝜌DL |  |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.94 |
|  | BO |  |  | 0.10 | 0.34 | 0.54 | 0.11 | 0.37 | 0.57 | 0.11 | 0.41 | 0.62 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario A) with the same settings, report only for OP A; case 1: no-LBT, case 2: LBT with ED = -47dBm, case 3: LBT with ED = -68dBm 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 17: System level evaluation results for scenario A with Receiver assisted LBT

| Tdoc / Source |  | Cases |  | Case 1: no LBT |  | Case 2: RAL ED -47dBm |  |  | Case 3: RAL ED-68 dBm |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) |  | 5%ile | 5574 | 3830 | 2396 | 5426 | 3561 | 2385 | 5415 | 3573 | 2227 |
|  |  |  | 50%ile | 9250 | 7319 | 5607 | 8845 | 6971 | 5317 | 8880 | 6678 | 4936 |
|  |  |  | 95%ile | 11220 | 10326 | 9205 | 10776 | 9827 | 8674 | 10748 | 9417 | 8209 |
|  |  |  | mean | 9008 | 7303 | 5822 | 8629 | 6951 | 5484 | 8630 | 6660 | 5146 |
|  | DL delay (s) |  | 5%ile | 0.019 | 0.020 | 0.023 | 0.020 | 0.021 | 0.025 | 0.020 | 0.022 | 0.026 |
|  |  |  | 50%ile | 0.024 | 0.032 | 0.042 | 0.025 | 0.033 | 0.045 | 0.025 | 0.035 | 0.049 |
|  |  |  | 95%ile | 0.038 | 0.059 | 0.091 | 0.039 | 0.063 | 0.096 | 0.039 | 0.066 | 0.103 |
|  |  |  | mean | 0.026 | 0.036 | 0.050 | 0.027 | 0.038 | 0.053 | 0.027 | 0.040 | 0.056 |
|  | UL UPT (Mbps) |  | 5%ile | 1871 | 1116 | 469 | 1741 | 1016 | 527 | 1664 | 888 | 329 |
|  |  |  | 50%ile | 3413 | 2541 | 1808 | 3194 | 2362 | 1697 | 3175 | 2064 | 1287 |
|  |  |  | 95%ile | 4742 | 4181 | 3569 | 4411 | 3881 | 3364 | 4363 | 3513 | 2839 |
|  |  |  | mean | 3395 | 2635 | 1943 | 3182 | 2437 | 1839 | 3144 | 2150 | 1441 |
|  | UL delay (s) |  | 5%ile | 0.045 | 0.050 | 0.060 | 0.048 | 0.055 | 0.062 | 0.048 | 0.061 | 0.074 |
|  |  |  | 50%ile | 0.064 | 0.091 | 0.132 | 0.069 | 0.097 | 0.140 | 0.069 | 0.119 | 0.195 |
|  |  |  | 95%ile | 0.112 | 0.206 | 0.413 | 0.120 | 0.225 | 0.410 | 0.121 | 0.255 | 0.591 |
|  |  |  | mean | 0.072 | 0.109 | 0.181 | 0.076 | 0.118 | 0.186 | 0.078 | 0.140 | 0.256 |
|  | Arrival rate (files/s) |  |  | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 |
|  | 𝜌DL |  |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 | 1.00 | 0.98 | 0.94 |
|  | BO |  |  | 0.10 | 0.35 | 0.55 | 0.11 | 0.37 | 0.57 | 0.11 | 0.40 | 0.62 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario A) with the same settings, report only for OP A; case 1: no-LBT, case 2: receiver assisted LBT with ED = -47dBm, case 3: receiver assisted LBT with ED = -68dBm Receiver assisted LBT: the LBT procedure is evaluated at the receiver instead of transmitter. The LBT result is assumed to be available instantly at the transmitter without accounting any overhead for exchanging this information between the transmitter and the receiver.(refer to section 2.1.4 in [9] for more details) 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DLCOT sharing when traffic in both directions 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 18: System level evaluation results for scenario A with Dynamic LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: Dynamic LBT ED -47dBm |  |  | Case 3: Dynamic LBT ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5574 | 3830 | 2396 | 5644 | 3743 | 2522 | 5542 | 3713 | 2141 |
|  |  | 50%ile | 9250 | 7319 | 5607 | 9228 | 7295 | 5705 | 9166 | 7079 | 5371 |
|  |  | 95%ile | 11220 | 10326 | 9205 | 11223 | 10314 | 9083 | 11221 | 10192 | 8835 |
|  |  | mean | 9008 | 7303 | 5822 | 8997 | 7301 | 5831 | 8960 | 7108 | 5518 |
|  | DL delay (s) | 5%ile | 0.019 | 0.020 | 0.023 | 0.019 | 0.020 | 0.024 | 0.019 | 0.020 | 0.024 |
|  |  | 50%ile | 0.024 | 0.032 | 0.042 | 0.024 | 0.032 | 0.042 | 0.024 | 0.034 | 0.046 |
|  |  | 95%ile | 0.038 | 0.059 | 0.091 | 0.037 | 0.060 | 0.091 | 0.039 | 0.065 | 0.103 |
|  |  | mean | 0.026 | 0.036 | 0.050 | 0.026 | 0.036 | 0.049 | 0.026 | 0.038 | 0.054 |
|  | UL UPT (Mbps) | 5%ile | 1871 | 1116 | 469 | 1863 | 1134 | 581 | 1860 | 1066 | 434 |
|  |  | 50%ile | 3413 | 2541 | 1808 | 3432 | 2564 | 1900 | 3387 | 2398 | 1618 |
|  |  | 95%ile | 4742 | 4181 | 3569 | 4724 | 4191 | 3643 | 4670 | 3944 | 3212 |
|  |  | mean | 3395 | 2635 | 1943 | 3408 | 2646 | 2026 | 3369 | 2487 | 1727 |
|  | UL delay (s) | 5%ile | 0.045 | 0.050 | 0.060 | 0.045 | 0.050 | 0.058 | 0.045 | 0.054 | 0.066 |
|  |  | 50%ile | 0.064 | 0.091 | 0.132 | 0.064 | 0.091 | 0.124 | 0.065 | 0.098 | 0.152 |
|  |  | 95%ile | 0.112 | 0.206 | 0.413 | 0.112 | 0.194 | 0.349 | 0.114 | 0.213 | 0.468 |
|  |  | mean | 0.072 | 0.109 | 0.181 | 0.071 | 0.108 | 0.165 | 0.072 | 0.116 | 0.205 |
|  | Arrival rate (files/s) |  | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.97 | 1.00 | 0.99 | 0.95 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.10 | 0.35 | 0.55 | 0.10 | 0.36 | 0.58 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario A) with the same settings, report only for OP A; case 1: no-LBT, case 2: Dynamic LBT with ED = -47dBm, case 3: Dynamic LBT with ED = -68dBm Dynamic LBT: a node operates without LBT unless the receiver experiences a failure in reception due to a drop in SINR, which reflects a presence of interferer. Only then, the node switches to LBT. Besides, when the LBT is switched on, the RAL described in section 2.1.4 of [9] is used 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 19: System level evaluation results for scenario A with directional LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: directional LBT ED -47dBm |  |  | Case 3: directional LBT ED-47+X dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5574 | 3830 | 2396 | 5294 | 3497 | 2365 | 5321 | 3537 | 2274 |
|  |  | 50%ile | 9250 | 7319 | 5607 | 8849 | 6924 | 5256 | 8836 | 6948 | 5337 |
|  |  | 95%ile | 11220 | 10326 | 9205 | 10780 | 9816 | 8494 | 10768 | 9867 | 8609 |
|  |  | mean | 9008 | 7303 | 5822 | 8638 | 6914 | 5449 | 8613 | 6925 | 5461 |
|  | DL delay (s) | 5%ile | 0.019 | 0.020 | 0.023 | 0.020 | 0.021 | 0.025 | 0.020 | 0.021 | 0.025 |
|  |  | 50%ile | 0.024 | 0.032 | 0.042 | 0.025 | 0.034 | 0.046 | 0.025 | 0.034 | 0.045 |
|  |  | 95%ile | 0.038 | 0.059 | 0.091 | 0.040 | 0.064 | 0.097 | 0.040 | 0.066 | 0.098 |
|  |  | mean | 0.026 | 0.036 | 0.050 | 0.027 | 0.038 | 0.053 | 0.027 | 0.038 | 0.053 |
|  | UL UPT (Mbps) | 5%ile | 1871 | 1116 | 469 | 1733 | 1008 | 529 | 1716 | 1028 | 481 |
|  |  | 50%ile | 3413 | 2541 | 1808 | 3206 | 2342 | 1674 | 3225 | 2360 | 1683 |
|  |  | 95%ile | 4742 | 4181 | 3569 | 4453 | 3881 | 3370 | 4447 | 3841 | 3342 |
|  |  | mean | 3395 | 2635 | 1943 | 3193 | 2435 | 1813 | 3194 | 2427 | 1811 |
|  | UL delay (s) | 5%ile | 0.045 | 0.050 | 0.060 | 0.048 | 0.054 | 0.063 | 0.048 | 0.055 | 0.065 |
|  |  | 50%ile | 0.064 | 0.091 | 0.132 | 0.068 | 0.099 | 0.143 | 0.068 | 0.099 | 0.142 |
|  |  | 95%ile | 0.112 | 0.206 | 0.413 | 0.119 | 0.218 | 0.409 | 0.119 | 0.219 | 0.424 |
|  |  | mean | 0.072 | 0.109 | 0.181 | 0.076 | 0.118 | 0.185 | 0.076 | 0.118 | 0.188 |
|  | Arrival rate (files/s) |  | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.97 | 1.00 | 0.99 | 0.96 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.11 | 0.37 | 0.57 | 0.11 | 0.37 | 0.58 |
|  | Additional report/notes: 1. LBT procedure and parameters: directional LBT, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario A) with the same settings, report only for OP A; case 1: no-LBT, case 2: directional LBT with ED = -47dBm, case 3: directional LBT with ED = -47+x dBm (i.e., -47+15dBm at gNB, -47+6dBm at UE) 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DLCOT sharing when traffic in both directions 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 20: System level evaluation results for scenario A, with mixed LBT configuration

| Tdoc / Source | Cases |  | Case 1: no LBT (OpA ) |  |  | Case 2: ED -47dBm (OpB) |  |  | Case 3: mixed configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | (Op A , no LBT ) |  |  | (Op B, -47dBm ) |  |  |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5574 | 3830 | 2396 | 5072 | 3434 | 2204 | 5662 | 3761 | 2455 | 5106 | 3552 | 2219 |
|  |  | 50%ile | 9250 | 7319 | 5607 | 8787 | 6893 | 5292 | 9218 | 7378 | 5574 | 8796 | 6929 | 5238 |
|  |  | 95%ile | 11220 | 10326 | 9205 | 10793 | 9829 | 8587 | 11222 | 10295 | 9071 | 10774 | 9845 | 8585 |
|  |  | mean | 9008 | 7303 | 5822 | 8562 | 6869 | 5435 | 9009 | 7324 | 5780 | 8553 | 6900 | 5427 |
|  | DL delay (s) | 5%ile | 0.019 | 0.020 | 0.023 | 0.020 | 0.021 | 0.025 | 0.019 | 0.020 | 0.023 | 0.020 | 0.021 | 0.025 |
|  |  | 50%ile | 0.024 | 0.032 | 0.042 | 0.025 | 0.034 | 0.045 | 0.024 | 0.032 | 0.043 | 0.025 | 0.034 | 0.046 |
|  |  | 95%ile | 0.038 | 0.059 | 0.091 | 0.041 | 0.065 | 0.102 | 0.037 | 0.058 | 0.090 | 0.041 | 0.064 | 0.101 |
|  |  | mean | 0.026 | 0.036 | 0.050 | 0.027 | 0.038 | 0.053 | 0.026 | 0.036 | 0.050 | 0.027 | 0.038 | 0.053 |
|  | UL UPT (Mbps) | 5%ile | 1871 | 1116 | 469 | 1641 | 961 | 414 | 1901 | 1081 | 526 | 1673 | 999 | 477 |
|  |  | 50%ile | 3413 | 2541 | 1808 | 3186 | 2326 | 1638 | 3409 | 2546 | 1827 | 3198 | 2365 | 1683 |
|  |  | 95%ile | 4742 | 4181 | 3569 | 4399 | 3844 | 3338 | 4732 | 4171 | 3661 | 4382 | 3895 | 3404 |
|  |  | mean | 3395 | 2635 | 1943 | 3151 | 2406 | 1778 | 3397 | 2631 | 1962 | 3161 | 2439 | 1832 |
|  | UL delay (s) | 5%ile | 0.045 | 0.050 | 0.060 | 0.048 | 0.054 | 0.063 | 0.045 | 0.050 | 0.057 | 0.048 | 0.053 | 0.062 |
|  |  | 50%ile | 0.064 | 0.091 | 0.132 | 0.069 | 0.100 | 0.146 | 0.064 | 0.091 | 0.132 | 0.069 | 0.099 | 0.139 |
|  |  | 95%ile | 0.112 | 0.206 | 0.413 | 0.124 | 0.231 | 0.458 | 0.111 | 0.205 | 0.389 | 0.123 | 0.221 | 0.411 |
|  |  | mean | 0.072 | 0.109 | 0.181 | 0.077 | 0.120 | 0.201 | 0.071 | 0.108 | 0.174 | 0.077 | 0.119 | 0.187 |
|  | Arrival rate (files/s) |  | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 | 0.46 | 1.57 | 2.48 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.11 | 0.37 | 0.57 | 0.10 | 0.35 | 0.55 | 0.11 | 0.37 | 0.58 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario A); case 1: no-LBT for both OPs, case 2: LBT with ED = -47dBm for both OPs, case 3: no LBT for OP A, LBT with ED = -47dBm for OP B 4. Other metric(s) and definition if reported: no 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 3.1.2	System level evaluation results for scenario B

Table 21: System level evaluation results for scenario B, with/without LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: ED -47dBm |  |  | Case 3: ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5985 | 4816 | 3791 | 5692 | 4554 | 3681 | 5698 | 4427 | 3317 |
|  |  | 50%ile | 8011 | 6452 | 5082 | 7637 | 6109 | 4849 | 7665 | 5799 | 4421 |
|  |  | 95%ile | 10675 | 9374 | 7848 | 10288 | 8935 | 7474 | 10254 | 8533 | 6892 |
|  |  | mean | 8748 | 7294 | 5895 | 8350 | 6924 | 5638 | 8327 | 6633 | 5138 |
|  | DL delay (s) | 5%ile | 0.021 | 0.025 | 0.034 | 0.021 | 0.027 | 0.035 | 0.021 | 0.029 | 0.039 |
|  |  | 50%ile | 0.022 | 0.030 | 0.042 | 0.024 | 0.031 | 0.043 | 0.024 | 0.033 | 0.050 |
|  |  | 95%ile | 0.047 | 0.067 | 0.100 | 0.049 | 0.069 | 0.102 | 0.049 | 0.072 | 0.116 |
|  |  | mean | 0.029 | 0.040 | 0.058 | 0.030 | 0.042 | 0.059 | 0.030 | 0.044 | 0.068 |
|  | UL UPT (Mbps) | 5%ile | 2076 | 1643 | 1254 | 1944 | 1518 | 1152 | 1921 | 1443 | 989 |
|  |  | 50%ile | 2904 | 2338 | 1767 | 2710 | 2169 | 1648 | 2698 | 2018 | 1399 |
|  |  | 95%ile | 4392 | 3790 | 3240 | 4179 | 3550 | 2996 | 4134 | 3312 | 2534 |
|  |  | mean | 3361 | 2783 | 2209 | 3165 | 2583 | 2060 | 3125 | 2390 | 1749 |
|  | UL delay (s) | 5%ile | 0.051 | 0.066 | 0.085 | 0.053 | 0.070 | 0.096 | 0.054 | 0.081 | 0.120 |
|  |  | 50%ile | 0.058 | 0.087 | 0.114 | 0.061 | 0.086 | 0.124 | 0.063 | 0.102 | 0.153 |
|  |  | 95%ile | 0.142 | 0.255 | 0.349 | 0.155 | 0.228 | 0.363 | 0.156 | 0.268 | 0.434 |
|  |  | mean | 0.081 | 0.134 | 0.179 | 0.087 | 0.125 | 0.192 | 0.088 | 0.148 | 0.232 |
|  | Arrival rate (files/s) |  | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.97 | 1.00 | 0.99 | 0.95 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.10 | 0.37 | 0.57 | 0.11 | 0.38 | 0.61 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario B) with the same settings, report only for OP A; case 1: no-LBT, case 2: LBT with ED = -47dBm, case 3: LBT with ED = -68dBm 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 22: System level evaluation results for scenario B with receiver assisted LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: RAL ED -47dBm |  |  | Case 3: RAL ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5985 | 4816 | 3791 | 5717 | 4593 | 3704 | 5767 | 4492 | 3577 |
|  |  | 50%ile | 8011 | 6452 | 5082 | 7674 | 6068 | 4843 | 7534 | 5977 | 4699 |
|  |  | 95%ile | 10675 | 9374 | 7848 | 10284 | 8923 | 7474 | 10280 | 8767 | 7084 |
|  |  | mean | 8748 | 7294 | 5895 | 8364 | 6898 | 5623 | 8342 | 6791 | 5437 |
|  | DL delay (s) | 5%ile | 0.021 | 0.025 | 0.034 | 0.021 | 0.027 | 0.035 | 0.021 | 0.028 | 0.037 |
|  |  | 50%ile | 0.022 | 0.030 | 0.042 | 0.024 | 0.032 | 0.043 | 0.024 | 0.032 | 0.044 |
|  |  | 95%ile | 0.047 | 0.067 | 0.100 | 0.049 | 0.070 | 0.102 | 0.048 | 0.071 | 0.105 |
|  |  | mean | 0.029 | 0.040 | 0.058 | 0.030 | 0.042 | 0.059 | 0.030 | 0.043 | 0.061 |
|  | UL UPT (Mbps) | 5%ile | 2076 | 1643 | 1254 | 1946 | 1511 | 1154 | 1932 | 1416 | 981 |
|  |  | 50%ile | 2904 | 2338 | 1767 | 2715 | 2164 | 1681 | 2715 | 1955 | 1387 |
|  |  | 95%ile | 4392 | 3790 | 3240 | 4177 | 3502 | 2917 | 4096 | 3312 | 2549 |
|  |  | mean | 3361 | 2783 | 2209 | 3168 | 2562 | 2046 | 3124 | 2386 | 1744 |
|  | UL delay (s) | 5%ile | 0.051 | 0.066 | 0.085 | 0.053 | 0.071 | 0.098 | 0.055 | 0.079 | 0.121 |
|  |  | 50%ile | 0.058 | 0.087 | 0.114 | 0.061 | 0.086 | 0.123 | 0.063 | 0.096 | 0.154 |
|  |  | 95%ile | 0.142 | 0.255 | 0.349 | 0.152 | 0.225 | 0.353 | 0.154 | 0.250 | 0.417 |
|  |  | mean | 0.081 | 0.134 | 0.179 | 0.086 | 0.126 | 0.190 | 0.088 | 0.139 | 0.227 |
|  | Arrival rate (files/s) |  | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.97 | 1.00 | 0.99 | 0.96 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.11 | 0.37 | 0.57 | 0.11 | 0.39 | 0.60 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario B) with the same settings, report only for OP A; case 1: no-LBT, case 2: receiver assisted LBT with ED = -47dBm, case 3: receiver assisted LBT with ED = -68dBm Receiver assisted LBT: the LBT procedure is evaluated at the receiver instead of transmitter. The LBT result is assumed to be available instantly at the transmitter without accounting any overhead for exchanging this information between the transmitter and the receiver.(refer to section 2.1.4 in [9] for more details) 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DLCOT sharing when traffic in both directions 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 23: System level evaluation results for scenario B with dynamic LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: Dynamic LBT ED -47dBm |  |  | Case 3: Dynamic LBT ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5985 | 4816 | 3791 | 6049 | 4837 | 3799 | 6062 | 4734 | 3674 |
|  |  | 50%ile | 8011 | 6452 | 5082 | 7934 | 6424 | 5051 | 7934 | 6342 | 4958 |
|  |  | 95%ile | 10675 | 9374 | 7848 | 10701 | 9340 | 7845 | 10682 | 9042 | 7625 |
|  |  | mean | 8748 | 7294 | 5895 | 8731 | 7259 | 5882 | 8743 | 7105 | 5716 |
|  | DL delay (s) | 5%ile | 0.021 | 0.025 | 0.034 | 0.021 | 0.026 | 0.034 | 0.021 | 0.027 | 0.036 |
|  |  | 50%ile | 0.022 | 0.030 | 0.042 | 0.023 | 0.030 | 0.042 | 0.022 | 0.032 | 0.045 |
|  |  | 95%ile | 0.047 | 0.067 | 0.100 | 0.046 | 0.068 | 0.100 | 0.045 | 0.069 | 0.109 |
|  |  | mean | 0.029 | 0.040 | 0.058 | 0.029 | 0.041 | 0.058 | 0.029 | 0.042 | 0.063 |
|  | UL UPT (Mbps) | 5%ile | 2076 | 1643 | 1254 | 2084 | 1647 | 1266 | 2066 | 1622 | 1211 |
|  |  | 50%ile | 2904 | 2338 | 1767 | 2941 | 2358 | 1781 | 2927 | 2264 | 1716 |
|  |  | 95%ile | 4392 | 3790 | 3240 | 4460 | 3861 | 3271 | 4385 | 3721 | 3081 |
|  |  | mean | 3361 | 2783 | 2209 | 3394 | 2803 | 2271 | 3359 | 2700 | 2117 |
|  | UL delay (s) | 5%ile | 0.051 | 0.066 | 0.085 | 0.050 | 0.064 | 0.084 | 0.051 | 0.067 | 0.092 |
|  |  | 50%ile | 0.058 | 0.087 | 0.114 | 0.057 | 0.079 | 0.111 | 0.058 | 0.081 | 0.120 |
|  |  | 95%ile | 0.142 | 0.255 | 0.349 | 0.142 | 0.235 | 0.330 | 0.142 | 0.213 | 0.342 |
|  |  | mean | 0.081 | 0.134 | 0.179 | 0.080 | 0.125 | 0.173 | 0.081 | 0.118 | 0.179 |
|  | Arrival rate (files/s) |  | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.97 | 1.00 | 0.99 | 0.96 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.10 | 0.35 | 0.55 | 0.10 | 0.36 | 0.56 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario B) with the same settings, report only for OP A; case 1: no-LBT, case 2: Dynamic LBT with ED = -47dBm, case 3: Dynamic LBT with ED = -68dBm Dynamic LBT : a node operates without LBT unless the receiver experiences a failure in reception due to a drop in SINR, which reflects a presence of interferer. Only then, the node switches to LBT. Besides, when the LBT is switched on, the RAL described in section 2.1.4 of [9] is used 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 24: System level evaluation results for scenario B with directional LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: directional LBT ED -47dBm |  |  | Case 3: directional LBT ED-47+X dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5985 | 4816 | 3791 | 5757 | 4508 | 3574 | 5758 | 4532 | 3530 |
|  |  | 50%ile | 8011 | 6452 | 5082 | 7643 | 5985 | 4684 | 7652 | 6005 | 4742 |
|  |  | 95%ile | 10675 | 9374 | 7848 | 10260 | 8957 | 7527 | 10267 | 8901 | 7487 |
|  |  | mean | 8748 | 7294 | 5895 | 8369 | 6880 | 5581 | 8376 | 6867 | 5571 |
|  | DL delay (s) | 5%ile | 0.021 | 0.025 | 0.034 | 0.021 | 0.027 | 0.036 | 0.021 | 0.027 | 0.036 |
|  |  | 50%ile | 0.022 | 0.030 | 0.042 | 0.024 | 0.032 | 0.043 | 0.024 | 0.032 | 0.043 |
|  |  | 95%ile | 0.047 | 0.067 | 0.100 | 0.048 | 0.074 | 0.106 | 0.048 | 0.073 | 0.108 |
|  |  | mean | 0.029 | 0.040 | 0.058 | 0.030 | 0.043 | 0.061 | 0.030 | 0.043 | 0.062 |
|  | UL UPT (Mbps) | 5%ile | 2076 | 1643 | 1254 | 1936 | 1507 | 1095 | 1936 | 1508 | 1094 |
|  |  | 50%ile | 2904 | 2338 | 1767 | 2717 | 2112 | 1622 | 2716 | 2119 | 1593 |
|  |  | 95%ile | 4392 | 3790 | 3240 | 4161 | 3548 | 2993 | 4155 | 3571 | 2970 |
|  |  | mean | 3361 | 2783 | 2209 | 3159 | 2556 | 2033 | 3155 | 2563 | 2019 |
|  | UL delay (s) | 5%ile | 0.051 | 0.066 | 0.085 | 0.053 | 0.071 | 0.092 | 0.053 | 0.069 | 0.093 |
|  |  | 50%ile | 0.058 | 0.087 | 0.114 | 0.061 | 0.090 | 0.124 | 0.062 | 0.087 | 0.124 |
|  |  | 95%ile | 0.142 | 0.255 | 0.349 | 0.155 | 0.253 | 0.395 | 0.155 | 0.252 | 0.385 |
|  |  | mean | 0.081 | 0.134 | 0.179 | 0.087 | 0.135 | 0.199 | 0.087 | 0.134 | 0.199 |
|  | Arrival rate (files/s) |  | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.10 | 0.37 | 0.57 | 0.10 | 0.37 | 0.57 |
|  | Additional report/notes: 1. LBT procedure and parameters: directional LBT , CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario B) with the same settings, report only for OP A; case 1: no-LBT, case 2: directional LBT with ED = -47dBm, case 3: directional LBT with ED = -47+x dBm (i.e., -47+15dBm at gNB, -47+6dBm at UE) 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 25: System level evaluation results for scenario B with mixed LBT configurations

| Tdoc / Source | Cases |  | Case 1: no LBT (OpA ) |  |  | Case 2: ED -47dBm (OpB) |  |  | Case 3: mixed configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | (Op A , no LBT ) |  |  | (Op B, -47dbm ) |  |  |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5985 | 4816 | 3791 | 5794 | 4612 | 3656 | 6084 | 4923 | 3856 | 5640 | 4499 | 3581 |
|  |  | 50%ile | 8011 | 6452 | 5082 | 7619 | 6012 | 4740 | 7922 | 6429 | 5034 | 7582 | 5983 | 4831 |
|  |  | 95%ile | 10675 | 9374 | 7848 | 10250 | 8830 | 7401 | 10764 | 9375 | 7841 | 10273 | 8900 | 7527 |
|  |  | mean | 8748 | 7294 | 5895 | 8408 | 6880 | 5577 | 8790 | 7289 | 5894 | 8363 | 6848 | 5634 |
|  | DL delay (s) | 5%ile | 0.021 | 0.025 | 0.034 | 0.021 | 0.027 | 0.035 | 0.020 | 0.025 | 0.034 | 0.021 | 0.027 | 0.035 |
|  |  | 50%ile | 0.022 | 0.030 | 0.042 | 0.023 | 0.032 | 0.043 | 0.022 | 0.030 | 0.041 | 0.023 | 0.032 | 0.043 |
|  |  | 95%ile | 0.047 | 0.067 | 0.100 | 0.049 | 0.072 | 0.105 | 0.046 | 0.067 | 0.097 | 0.053 | 0.079 | 0.112 |
|  |  | mean | 0.029 | 0.040 | 0.058 | 0.031 | 0.043 | 0.061 | 0.029 | 0.040 | 0.057 | 0.031 | 0.045 | 0.062 |
|  | UL UPT (Mbps) | 5%ile | 2076 | 1643 | 1254 | 1987 | 1579 | 1192 | 2110 | 1639 | 1279 | 1975 | 1570 | 1170 |
|  |  | 50%ile | 2904 | 2338 | 1767 | 2742 | 2181 | 1653 | 2923 | 2329 | 1781 | 2777 | 2247 | 1703 |
|  |  | 95%ile | 4392 | 3790 | 3240 | 4132 | 3564 | 2986 | 4447 | 3825 | 3262 | 4165 | 3642 | 3035 |
|  |  | mean | 3361 | 2783 | 2209 | 3180 | 2609 | 2092 | 3388 | 2773 | 2241 | 3204 | 2666 | 2107 |
|  | UL delay (s) | 5%ile | 0.051 | 0.066 | 0.085 | 0.054 | 0.069 | 0.093 | 0.050 | 0.064 | 0.084 | 0.053 | 0.067 | 0.087 |
|  |  | 50%ile | 0.058 | 0.087 | 0.114 | 0.061 | 0.084 | 0.116 | 0.058 | 0.082 | 0.112 | 0.060 | 0.082 | 0.114 |
|  |  | 95%ile | 0.142 | 0.255 | 0.349 | 0.153 | 0.248 | 0.347 | 0.141 | 0.222 | 0.322 | 0.160 | 0.257 | 0.383 |
|  |  | mean | 0.081 | 0.134 | 0.179 | 0.087 | 0.129 | 0.185 | 0.080 | 0.121 | 0.173 | 0.087 | 0.130 | 0.189 |
|  | Arrival rate (files/s) |  | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 | 0.42 | 1.59 | 2.65 |
|  | 𝜌DL |  | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 0.99 |
|  | 𝜌UL |  | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.96 | 1.00 | 0.99 | 0.97 |
|  | BO |  | 0.10 | 0.35 | 0.55 | 0.11 | 0.37 | 0.57 | 0.10 | 0.36 | 0.54 | 0.11 | 0.37 | 0.57 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: 2 operators (scenario B); case 1: no-LBT for both OPs, case 2: LBT with ED = -47dBm for both OPs, case 3: no LBT for OP A, LBT with ED = -47dBm for OP B 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 3.1.3	System level evaluation results for scenario C

Table 26: System level evaluation results for scenario C, with/ without LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: ED -47dBm |  |  | Case 3: ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5736 | 4324 | 3083 | 5503 | 3993 | 2783 | 5475 | 3988 | 2651 |
|  |  | 50%ile | 9409 | 7759 | 6148 | 9095 | 7460 | 5892 | 9122 | 7335 | 5737 |
|  |  | 95%ile | 11217 | 10635 | 9494 | 10800 | 10224 | 9018 | 10804 | 10097 | 8884 |
|  |  | mean | 9186 | 7747 | 6317 | 8837 | 7367 | 5952 | 8849 | 7300 | 5853 |
|  | DL delay (s) | 5%ile | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 |
|  |  | 50%ile | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 |
|  |  | 95%ile | 0,04 | 0,05 | 0,08 | 0,04 | 0,06 | 0,09 | 0,04 | 0,06 | 0,09 |
|  |  | mean | 0,03 | 0,03 | 0,05 | 0,03 | 0,04 | 0,05 | 0,03 | 0,04 | 0,05 |
|  | UL UPT (Mbps) | 5%ile | 2016 | 1392 | 882 | 1879 | 1289 | 847 | 1868 | 1206 | 729 |
|  |  | 50%ile | 3598 | 2911 | 2255 | 3371 | 2690 | 2051 | 3348 | 2596 | 1907 |
|  |  | 95%ile | 4770 | 4367 | 3856 | 4495 | 4054 | 3622 | 4430 | 3987 | 3381 |
|  |  | mean | 3540 | 2931 | 2321 | 3321 | 2705 | 2155 | 3286 | 2625 | 2007 |
|  | UL delay (s) | 5%ile | 0,04 | 0,05 | 0,05 | 0,05 | 0,05 | 0,06 | 0,05 | 0,05 | 0,06 |
|  |  | 50%ile | 0,06 | 0,08 | 0,11 | 0,06 | 0,09 | 0,12 | 0,07 | 0,09 | 0,14 |
|  |  | 95%ile | 0,10 | 0,17 | 0,28 | 0,11 | 0,17 | 0,28 | 0,11 | 0,19 | 0,33 |
|  |  | mean | 0,07 | 0,09 | 0,14 | 0,07 | 0,10 | 0,14 | 0,07 | 0,11 | 0,16 |
|  | Arrival rate (files/s) |  | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 |
|  | 𝜌DL |  | 1,00 | 1,00 | 0,99 | 1,00 | 0,99 | 0,99 | 1,00 | 1,00 | 0,99 |
|  | 𝜌UL |  | 1,00 | 0,99 | 0,95 | 1,00 | 0,98 | 0,95 | 1,00 | 0,99 | 0,94 |
|  | BO |  | 0,10 | 0,35 | 0,55 | 0,11 | 0,37 | 0,57 | 0,11 | 0,37 | 0,58 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: single operator (scenario C); case 1: no-LBT, case 2: LBT with ED = -47dBm, case 3: LBT with ED = -68dBm 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both direction 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 27: System level evaluation results for scenario C with receiver assisted LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: RAL ED -47dBm |  |  | Case 3: RAL ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5736 | 4324 | 3083 | 5503 | 3993 | 2783 | 5520 | 3854 | 2712 |
|  |  | 50%ile | 9409 | 7759 | 6148 | 9095 | 7460 | 5892 | 9004 | 7348 | 5750 |
|  |  | 95%ile | 11217 | 10635 | 9494 | 10800 | 10224 | 9018 | 10782 | 9908 | 8533 |
|  |  | mean | 9186 | 7747 | 6317 | 8837 | 7367 | 5952 | 8801 | 7277 | 5834 |
|  | DL delay (s) | 5%ile | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,03 |
|  |  | 50%ile | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 |
|  |  | 95%ile | 0,04 | 0,05 | 0,08 | 0,04 | 0,06 | 0,09 | 0,04 | 0,06 | 0,09 |
|  |  | mean | 0,03 | 0,03 | 0,05 | 0,03 | 0,04 | 0,05 | 0,03 | 0,04 | 0,05 |
|  | UL UPT (Mbps) | 5%ile | 2016 | 1392 | 882 | 1879 | 1289 | 847 | 1908 | 1259 | 810 |
|  |  | 50%ile | 3598 | 2911 | 2255 | 3371 | 2690 | 2051 | 3367 | 2616 | 1985 |
|  |  | 95%ile | 4770 | 4367 | 3856 | 4495 | 4054 | 3622 | 4458 | 3968 | 3449 |
|  |  | mean | 3540 | 2931 | 2321 | 3321 | 2705 | 2155 | 3302 | 2652 | 2079 |
|  | UL delay (s) | 5%ile | 0,04 | 0,05 | 0,05 | 0,05 | 0,05 | 0,06 | 0,05 | 0,05 | 0,06 |
|  |  | 50%ile | 0,06 | 0,08 | 0,11 | 0,06 | 0,09 | 0,12 | 0,07 | 0,09 | 0,13 |
|  |  | 95%ile | 0,10 | 0,17 | 0,28 | 0,11 | 0,17 | 0,28 | 0,11 | 0,18 | 0,32 |
|  |  | mean | 0,07 | 0,09 | 0,14 | 0,07 | 0,10 | 0,14 | 0,07 | 0,10 | 0,16 |
|  | Arrival rate (files/s) |  | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 |
|  | 𝜌DL |  | 1,00 | 1,00 | 0,99 | 1,00 | 0,99 | 0,99 | 1,00 | 1,00 | 0,99 |
|  | 𝜌UL |  | 1,00 | 0,99 | 0,95 | 1,00 | 0,98 | 0,95 | 1,00 | 0,99 | 0,95 |
|  | BO |  | 0,10 | 0,35 | 0,55 | 0,11 | 0,37 | 0,57 | 0,11 | 0,38 | 0,57 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: single operator (scenario C); case 1: no-LBT, case 2: receiver assisted LBT with ED = -47dBm, case 3: receiver assisted LBT with ED = -68dBm Receiver assisted LBT: the LBT procedure is evaluated at the receiver instead of transmitter. The LBT result is assumed to be available instantly at the transmitter without accounting any overhead for exchanging this information between the transmitter and the receiver.(refer to section 2.1.4 in [9] for more details) 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 28: System level evaluation results for scenario C with dynamic LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: Dynamic LBT ED -47dBm |  |  | Case 3: Dynamic LBT ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5736 | 4324 | 3083 | 5758 | 4201 | 2943 | 5659 | 4000 | 2910 |
|  |  | 50%ile | 9409 | 7759 | 6148 | 9472 | 7718 | 6253 | 9464 | 7680 | 6048 |
|  |  | 95%ile | 11217 | 10635 | 9494 | 11231 | 10394 | 9183 | 11234 | 10513 | 9232 |
|  |  | mean | 9186 | 7747 | 6317 | 9211 | 7639 | 6265 | 9164 | 7591 | 6135 |
|  | DL delay (s) | 5%ile | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 |
|  |  | 50%ile | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 |
|  |  | 95%ile | 0,04 | 0,05 | 0,08 | 0,04 | 0,06 | 0,09 | 0,04 | 0,06 | 0,09 |
|  |  | mean | 0,03 | 0,03 | 0,05 | 0,03 | 0,03 | 0,05 | 0,03 | 0,03 | 0,05 |
|  | UL UPT (Mbps) | 5%ile | 2016 | 1392 | 882 | 2017 | 1373 | 888 | 1971 | 1308 | 784 |
|  |  | 50%ile | 3598 | 2911 | 2255 | 3627 | 2899 | 2236 | 3589 | 2855 | 2136 |
|  |  | 95%ile | 4770 | 4367 | 3856 | 4778 | 4411 | 3887 | 4755 | 4293 | 3734 |
|  |  | mean | 3540 | 2931 | 2321 | 3552 | 2940 | 2347 | 3522 | 2870 | 2228 |
|  | UL delay (s) | 5%ile | 0,04 | 0,05 | 0,05 | 0,04 | 0,05 | 0,05 | 0,04 | 0,05 | 0,06 |
|  |  | 50%ile | 0,06 | 0,08 | 0,11 | 0,06 | 0,08 | 0,11 | 0,06 | 0,08 | 0,12 |
|  |  | 95%ile | 0,10 | 0,17 | 0,28 | 0,10 | 0,16 | 0,27 | 0,10 | 0,17 | 0,28 |
|  |  | mean | 0,07 | 0,09 | 0,14 | 0,07 | 0,09 | 0,14 | 0,07 | 0,09 | 0,14 |
|  | Arrival rate (files/s) |  | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 |
|  | 𝜌DL |  | 1,00 | 1,00 | 0,99 | 1,00 | 1,00 | 0,99 | 1,00 | 1,00 | 0,99 |
|  | 𝜌UL |  | 1,00 | 0,99 | 0,95 | 1,00 | 0,99 | 0,96 | 1,00 | 0,99 | 0,95 |
|  | BO |  | 0,10 | 0,35 | 0,55 | 0,10 | 0,36 | 0,55 | 0,10 | 0,36 | 0,56 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: single operator (scenario C); case 1: no-LBT, case 2: Dynamic LBT with ED = -47dBm, case 3: Dynamic LBT with ED = -68dBm Dynamic LBT: a node operates without LBT unless the receiver experiences a failure in reception due to a drop in SINR, which reflects a presence of interferer. Only then, the node switches to LBT. Besides, when the LBT is switched on, the RAL described in section 2.1.4 of [9] is used 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both direction 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

Table 29: System level evaluation results for scenario C with directional LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: directional LBT ED -47dBm |  |  | Case 3: directional LBT ED-47+X dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 5736 | 4324 | 3083 | 5597 | 4045 | 2748 | 5597 | 4061 | 2795 |
|  |  | 50%ile | 9409 | 7759 | 6148 | 9109 | 7348 | 5801 | 9106 | 7363 | 5826 |
|  |  | 95%ile | 11217 | 10635 | 9494 | 10812 | 10036 | 8709 | 10812 | 10032 | 8709 |
|  |  | mean | 9186 | 7747 | 6317 | 8863 | 7281 | 5834 | 8863 | 7287 | 5856 |
|  | DL delay (s) | 5%ile | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 | 0,02 |
|  |  | 50%ile | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 | 0,02 | 0,03 | 0,04 |
|  |  | 95%ile | 0,04 | 0,05 | 0,08 | 0,04 | 0,06 | 0,09 | 0,04 | 0,06 | 0,09 |
|  |  | mean | 0,03 | 0,03 | 0,05 | 0,03 | 0,04 | 0,05 | 0,03 | 0,04 | 0,05 |
|  | UL UPT (Mbps) | 5%ile | 2016 | 1392 | 882 | 1893 | 1211 | 695 | 1893 | 1211 | 694 |
|  |  | 50%ile | 3598 | 2911 | 2255 | 3394 | 2667 | 1996 | 3395 | 2660 | 1998 |
|  |  | 95%ile | 4770 | 4367 | 3856 | 4485 | 4055 | 3492 | 4485 | 4106 | 3589 |
|  |  | mean | 3540 | 2931 | 2321 | 3331 | 2691 | 2081 | 3332 | 2698 | 2101 |
|  | UL delay (s) | 5%ile | 0,04 | 0,05 | 0,05 | 0,05 | 0,05 | 0,06 | 0,05 | 0,05 | 0,06 |
|  |  | 50%ile | 0,06 | 0,08 | 0,11 | 0,06 | 0,09 | 0,13 | 0,06 | 0,09 | 0,13 |
|  |  | 95%ile | 0,10 | 0,17 | 0,28 | 0,11 | 0,19 | 0,32 | 0,11 | 0,19 | 0,31 |
|  |  | mean | 0,07 | 0,09 | 0,14 | 0,07 | 0,10 | 0,16 | 0,07 | 0,10 | 0,16 |
|  | Arrival rate (files/s) |  | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 | 0,49 | 1,79 | 3,04 |
|  | 𝜌DL |  | 1,00 | 1,00 | 0,99 | 1,00 | 1,00 | 0,99 | 1,00 | 1,00 | 0,99 |
|  | 𝜌UL |  | 1,00 | 0,99 | 0,95 | 1,00 | 0,99 | 0,95 | 1,00 | 0,99 | 0,95 |
|  | BO |  | 0,10 | 0,35 | 0,55 | 0,11 | 0,37 | 0,58 | 0,10 | 0,37 | 0,58 |
|  | Additional report/notes: 1. LBT procedure and parameters: directional LBT , CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: no 3. Details of case: single operator (scenario C); case 1: no-LBT, case 2: directional LBT with ED = -, case 3: directional LBT with ED = -47+x dBm (i.e., -47+15dBm at gNB, -47+6dBm at UE) 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

## 3.2	Tabulated System level evaluation results for outdoor Scenario

Table 30: System level evaluation results for outdoor B, with/without LBT

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: ED -47dBm |  |  | Case 3: ED-68 dBm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2007984 / Ericsson | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 2324 | 1691 | 1196 | 2167 | 1646 | 1173 | 2189 | 1665 | 1186 |
|  |  | 50%ile | 5632 | 4694 | 3759 | 5338 | 4421 | 3535 | 5396 | 4418 | 3472 |
|  |  | 95%ile | 8365 | 7152 | 6155 | 7932 | 6904 | 5846 | 8026 | 7009 | 5888 |
|  |  | mean | 5749 | 4820 | 3951 | 5472 | 4570 | 3691 | 5493 | 4595 | 3676 |
|  | DL delay (s) | 5%ile | 0.023 | 0.026 | 0.031 | 0.024 | 0.028 | 0.034 | 0.024 | 0.029 | 0.035 |
|  |  | 50%ile | 0.038 | 0.049 | 0.069 | 0.041 | 0.052 | 0.074 | 0.040 | 0.052 | 0.074 |
|  |  | 95%ile | 0.074 | 0.101 | 0.162 | 0.076 | 0.106 | 0.177 | 0.075 | 0.110 | 0.189 |
|  |  | mean | 0.044 | 0.059 | 0.088 | 0.046 | 0.063 | 0.095 | 0.046 | 0.063 | 0.097 |
|  | UL UPT (Mbps) | 5%ile | 1363 | 1089 | 832 | 1287 | 988 | 735 | 1271 | 1003 | 727 |
|  |  | 50%ile | 2958 | 2599 | 2189 | 2779 | 2432 | 1986 | 2786 | 2385 | 1953 |
|  |  | 95%ile | 4171 | 3744 | 3367 | 3877 | 3522 | 3091 | 3899 | 3481 | 3064 |
|  |  | mean | 3022 | 2655 | 2260 | 2842 | 2480 | 2065 | 2841 | 2443 | 2021 |
|  | UL delay (s) | 5%ile | 0.048 | 0.053 | 0.059 | 0.051 | 0.056 | 0.064 | 0.051 | 0.058 | 0.068 |
|  |  | 50%ile | 0.073 | 0.087 | 0.112 | 0.078 | 0.095 | 0.130 | 0.078 | 0.097 | 0.129 |
|  |  | 95%ile | 0.124 | 0.165 | 0.258 | 0.134 | 0.185 | 0.283 | 0.131 | 0.183 | 0.297 |
|  |  | mean | 0.080 | 0.101 | 0.139 | 0.086 | 0.109 | 0.152 | 0.086 | 0.110 | 0.157 |
|  | Arrival rate (files/s) |  | 0.36 | 1.34 | 2.32 | 0.36 | 1.34 | 2.32 | 0.36 | 1.34 | 2.32 |
|  | 𝜌DL |  | 0.999 | 0.997 | 0.987 | 0.999 | 0.996 | 0.989 | 1.000 | 0.997 | 0.989 |
|  | 𝜌UL |  | 0.999 | 0.993 | 0.978 | 0.999 | 0.993 | 0.981 | 0.998 | 0.993 | 0.979 |
|  | BO |  | 0.100 | 0.350 | 0.550 | 0.105 | 0.368 | 0.570 | 0.105 | 0.366 | 0.569 |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm or -68dBm, CWS: CW_min = CW_max = 3 2. any assumptions/parameters used not as in the agreed baseline: single site, UMi street canyon channel & PL model from TR38.901 3. Details of case: 2 operators (scenario B) with the same settings, report only for OP A; case 1: no-LBT, case 2: LBT with ED = -47dBm, case 3: LBT with ED = -68dBm 5. Details of COT sharing if used in evaluation: 0.5ms COT for DL, 0.25ms COT for UL, DL COT sharing when traffic in both directions. 6. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. |  |  |  |  |  |  |  |  |  |  |

# 4	References

R1-2005193, Summary #3 of email discussions for [101-e-Post-NR-52_71_GHz]

R1-2007982, “On NR operations in 52.6 to 71 GHz”, Ericsson

3GPP TR 38.808, "Study on supporting NR from 52.6 GHz to 71 GHz," v0.0.2, September 2020.

3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

R1-2007982, "On NR operations in 52.6 to 71 GHz," Ericsson, RAN1#103-e, October 2020.

R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson

R4-2011494, “On PN model for 52.6~71GHz”, Huawei

R1-1912714, “Evaluation results for enhanced PUCCH and PRACH designs”, Ericsson, RAN1#99, November 2019.

R1-2007983, “Channel Access Mechanism”, Ericsson, RAN1#103-e, October 2020.