3GPP TSG RAN WG1 #103-e	                                          R1-2008024

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.8.1.1

Source: 	CMCC

Title: 	Discussion on the baseline performance in FR1

Document for:	Discussion and Decision

Introduction

In the RAN1#102e meeting, significant progresses had been accomplished. The definition of MPL, MCL, MIL has been clarified. And link budget template and parameter assumptions was discussed and determined during the meeting and email discussion. The related agreements could be found in the Annex.

In this contribution, we provide our views on the baseline coverage performance of FR1.

Baseline coverage performance

According to the simulation assumption defined in the #101e and #102e meeting, the baseline coverage performances for FR1 have been derived according to the modified IMT-2020 template which is agreed during the email discussion. The detailed simulation assumptions [1][2] could refer to the Annex 1. And the template used for the link budget could refer to Annex 2. For 2.6GHz, 26dBm is assumed as the maximum power of UE.

- 2.1 Baseline coverage performance of Dense Urban O2I

### Considering the performance metric of MPL

Figure 1 Coverage performance of Dense Urban O2I scenario in MPL

It is observed that the PUSCH with 1Mbps requirement is most limited channel of the system with MPL requirements of 112.58dB.

PUCCH format 3 with 22bit is the 2nd limited channel. And PUSCH of voip with 1 repetition, PUCCH format 3 with 11 bit and PRACH with format C2 are the 3rd group of limited channels with around 123dB MPL requirements.

PUSCH with msg 3, PUCCH format 1 and PRACH B4 with 30kHz subcarrier spacing are the fourth group of limited channel with around 125dB MPL.

The PRACH format 0 which requires 128dB MPL is the most not limited channel in the uplink. From perspective of the relative MPL comparison, the limited channels could be as below,

Observation 1:

From the perspective of relative MPL, the limited channels could be set into several groups:

- Group 1 (the most limited channel(s)): PUSCH of eMBB with 1Mbps requirements

- Group 2: PUCCH format 3 with 22 bit

- Group 3: PUSCH of voip with 1 repetition, PUCCH format 3 with 11 bit and PRACH with format C2

- Group 4: PUSCH with msg 3, PUCCH format 1 and PRACH B4 with 30kHz subcarrier spacing

- Group 5 (least limited channels): PRACH format 0

Proposal 1:

From the perspective of relative MPL, PUSCH with 1Mbps requirement is the most limited channel and should be enhanced with highest priority.

Considering that PRACH format 0 is deployed in the commercial network and PRACH B4 with 30kHz SCS has a higher priority, the PRACH format C2 and B4 with 15kHz SCS could be a lower priority for the enhancement. And PUSCH of voip with repetition 2 are not considered in the current network, which could also be not considered.

Proposal 2:

From the perspective of relative MPL, the channels that need enhancements are listed (from high priority to low),

- PUSCH with 1Mbps, PUCCH format 3 with 22bit, PUSCH of voip with 1 repetition, PUCCH format 3 with 11 bit, PUSCH with msg 3, PUCCH format 1, PRACH B4 with 30kHz SCS.

Considering 2.6GHz and UMa NLOS pathloss model from 38.901, the 500m ISD will require around 118 dB pathloss. The PUSCH with 1Mbps needs 5.42 dB enhancements. PUSCH with voip traffic do not need any enhancement. If the centre frequency is configured as 3.5GHz, 500m ISD requires 120.57dB. The PUSCH with 1Mbps needs 8 dB enhancements. The voip traffic in PUSCH does not need enhancements.

Observation 2:

From the perspective of absolute MPL, PUSCH with 1Mbps requirement cannot satisfy both 2.6GHz and 3.5GHz 500m ISD coverage.

Proposal 3:

From either absolute or relative MPL, the PUSCH with 1Mbps should be enhanced for coverage.

With multiple assumptions of carrier frequencies and inter site distance, the absolute pathloss requirements are shown below.

Table 1 The required Pahtloss (dB)

|  | Inter-site distance, m |  |  |  |
| --- | --- | --- | --- | --- |
| Centre frequency, GHz | 500 | 400 | 350 | 300 |
| 2.6 | 117.99 | 114.21 | 111.94 | 109.32 |
| 3.5 | 120.57 | 116.79 | 114.52 | 111.90 |
| 4.9 | 123.50 | 119.71 | 117.44 | 114.83 |

It could be observed that the downlink channels are not limited in the Urban O2I scenario. For common channel for UEs, such as PDCCH and PBCH, 8 wide beams and -6dB antenna gains loss are assumed to provide a broader coverage compared with UE specific channels. For the PDSCH with eMBB and voip, UE specific beam forming is assumed and the full antenna gain could be used. Considering the absolution coverage with 500m ISD, the downlink channel is also not limited under current assumption.

Observation 3:

Downlink channels in Dense Urban O2I scenario are not limited according to both absolute and relative MPL.

### Considering the performance metric of MIL

Figure 2 Coverage performance of Dense Urban O2I scenario in MIL

Frome the perspective of MIL performance metric, the PUSCH with 1Mbps is still the most limited channel of the system. And voip with 1repetition is the 2nd limited channel. Then PUSCH msg 3 and PUCCH format 3 with 22bits are the 3rd limited channels. PUCCH format 3 with 11bits is the 4th limited channel. PRACH and downlink channels are not limited

Observation 4:

From the perspective of MIL, the priority of channel which needs enhancement is list below from high to low,

- PUSCH with 1Mbps, PUSCH of voip, PUSCH with msg 3, PUCCH format 3 with 22bits, PUCCH format 3 with 11bits

### Considering the performance metric of MCL

Figure 3 Coverage performance of Dense Urban O2I scenario in MCL

Frome the perspective of MCL performance metric, the PUSCH with 1Mbps is still the most limited channel of the system. And voip with 1 repetition is the 2nd limited channel. Then PUCCH format 3 with 22bits are the 3rd limited channels. PUSCH msg 3 is the 4th limited channel. PUCCH format 3 with 11bits is the 5th limited channel. PRACH and downlink channels are not limited

Observation 5:

From the perspective of MCL, the priority of channel which needs enhancement is list below from high to low,

- PUSCH with 1Mbps, PUSCH of voip, PUCCH format 3 with 22bits, PUSCH with msg 3, PUCCH format 3 with 11bits

Since there is not many reference for absolute requirements of MCL and MIL, the relative comparison of MCL or MIL is proposed.

Proposal 4:

The relative MCL or MIL could be considered for the performance metric.

In summary, the PUSCH with 1Mbps is the most limited channel and could not satisfy the 500m ISD in urban O2I scenario. And if time allowed, PUSCH of voip, PUSCH format 3 with 22bits, PUSCH of msg 3 and PUCCH format 3 with 11bits should be considered for the enhancements.

Proposal 5:

If time allowed, PUSCH of voip, PUSCH format 3 with 22bits, PUSCH of msg 3 and PUCCH format 3 with 11bits should be considered for the enhancements.

- 2.2 Baseline coverage performance of Rural O2I

The baseline coverage performance of Rural O2I could be found below according to the performance metrics of MPL, MIL and MCL.

### Considering the performance metric of MPL

Figure 4 Coverage performance of Rural O2I scenario in MPL

### Considering the performance metric of MIL

Figure 5 Coverage performance of Rural O2I scenario in MIL

### Considering the performance metric of MCL

Figure 6 Coverage performance of Rural O2I scenario in MCL

It could be observed that PUSCH with 100 kbps is the most limited channel in the Rural O2I scenario. Then the PUSCH of voip and PUCCH format 3 with 22 bits are the 2nd limited channels. And the PUSCH with msg 3 and PUCCH format 13 with 11 bits are the 3rd limited channels.

Proposal 6:

In Rural O2I, the PUSCH with 100kbps is the most limited channel and needs enhancement.

Proposal 7:

If time allowed, PUSCH of voip, PUSCH format 3 with 22bits, PUSCH of msg 3 and PUCCH format 3 with 11bits should be considered for the enhancements.

Conclusions

In this contribution, we provide our views on the baseline coverage performance of FR1. The proposals and observations are as follows.

Observation 1:

From the perspective of relative MPL, the limited channels could be set into several groups:

- Group 1 (the most limited channel(s)): PUSCH of eMBB with 1Mbps requirements

- Group 2: PUCCH format 3 with 22 bit

- Group 3: PUSCH of voip with 1 repetition, PUCCH format 3 with 11 bit and PRACH with format C2

- Group 4: PUSCH with msg 3, PUCCH format 1 and PRACH B4 with 30kHz subcarrier spacing

- Group 5 (least limited channels): PRACH format 0

Observation 2:

From the perspective of absolute MPL, PUSCH with 1Mbps requirement cannot satisfy both 2.6GHz and 3.5GHz 500m ISD coverage.

Observation 3:

Downlink channels in Dense Urban O2I scenario are not limited according to both absolute and relative MPL.

Observation 4:

From the perspective of MIL, the priority of channel which needs enhancement is list below from high to low,

- PUSCH with 1Mbps, PUSCH of voip, PUSCH with msg 3, PUCCH format 3 with 22bits, PUCCH format 3 with 11bits

Observation 5:

From the perspective of MCL, the priority of channel which needs enhancement is list below from high to low,

- PUSCH with 1Mbps, PUSCH of voip, PUCCH format 3 with 22bits, PUSCH with msg 3, PUCCH format 3 with 11bits

Proposal 1:

From the perspective of relative MPL, PUSCH with 1Mbps requirement is the most limited channel and should be enhanced with highest priority.

Proposal 2:

From the perspective of relative MPL, the channels that need enhancements are listed (from high priority to low),

- PUSCH with 1Mbps, PUCCH format 3 with 22bit, PUSCH of voip with 1 repetition, PUCCH format 3 with 11 bit, PUSCH with msg 3, PUCCH format 1, PRACH B4 with 30kHz SCS.

Proposal 3:

From either absolute or relative MPL, the PUSCH with 1Mbps should be enhanced for coverage.

Proposal 4:

The relative MCL or MIL could be considered for the performance metric.

Proposal 5:

If time allowed, PUSCH of voip, PUSCH format 3 with 22bits, PUSCH of msg 3 and PUCCH format 3 with 11bits should be considered for the enhancements.

Proposal 6:

In Rural O2I, the PUSCH with 100kbps is the most limited channel and needs enhancement.

Proposal 7:

If time allowed, PUSCH of voip, PUSCH format 3 with 22bits, PUSCH of msg 3 and PUCCH format 3 with 11bits should be considered for the enhancements.

References

- [1] #101-e RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #101-e e-Meeting, May 25th – June 5th, 2020

- [2] #101-e RAN2 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e e-Meeting, August 17th – 28th, 2020, 2020

Annex 1- Simulation assumptions

Annex 2- link budget calculation using the modified template of IMT-2020

Annex 3- related agreements in the #102e meeting

The agreements related with link budget template and performance metric are as below.

Agreements:

- Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters

  - [For LLS based methodology, ] coverage bottleneck(s) identification is performed using at least [MCL and] MIL.

  - [MCL values can also be considered to compare channels with similar antenna (and antenna array) gain]

Agreements:

- MPL can be used as supplemental information for coverage bottleneck(s) identification

- The results based on MPL are to be captured in TR

  - Note: this is uself to show the achievable ISD.

- The definition of MPL shall be determined in RAN1

- RAN1 will not further discuss on specific values for the parameters related to MPL

  - IMT-2020 values are as a starting point, but:

    - companies may use other values, and

    - for the parameters that companies think IMT-2020 self-evaluation does not clearly define the values for some scenarios, it is up to companies to report

Agreements:

- RAN1 strives for satisfying appropriate targets identified by companies particularly operators

  - The targets may be in the form of one or more of the following:

    - 1. Scenario dependent targets, e.g., ISD/MPL

    - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP;

    - 3. Relative difference between channels, e.g, MIL(/[MCL])

  - Further values and details of such targets will be clarified at RAN1#103-e

  - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets.

Agreements:

- Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters

  - For LLS based methodology, coverage bottleneck(s) identification is performed using at least MIL or MCL (assuming the set of simuation assumptions)

    - Even when SLS is used to obtain some components of MIL or MCL, it is categorized as LLS based methodology.

    - MCL values can also be used to identify the coverage bottleneck(s) when applicable

      - “applicable” above means the following situation:

        - [comparing channels with similar antenna (and antenna array) gain, and/or

        - the simulation results with MIL from companies are diverse, and the comparison with MIL is not easy]

The agreements related with parameter assumptions are as below.

Agreements:

- Define PSD for DL Tx power, which is depend on deployment scenario

  - For 4GHz frequency,

    - For rural with long distance scenario, PSD is 24 and 33 dBm/MHz

    - For rural scenario, PSD is 24 and 33 dBm/MHz

    - For urban scenario, PSD is 24 and 33 dBm/MHz

  - For 2.6 GHz frequency,

    - For rural with long distance scenario, PSD is 33 dBm/MHz

    - For rural scenario, PSD is 33 dBm/MHz

    - For urban scenario, PSD is 33 dBm/MHz

  - For 700MHz, 2GHz frequency

    - For rural with long distance scenario, PSD is 36 dBm/MHz

    - For rural scenario, PSD is 36 dBm/MHz

    - For urban scenario, PSD is 36 dBm/MHz

- Modify the description of row(s) of link budget template:

  - Keep the meaning of Total transmit power (row (3) ) and adding a new row (3 bis):

    - (3bis) means the transmit power for occupied channel bandwidth for control channel (17a) or data channel (17b)

- Companies are requested to set appropriate values for parameters, which is used to determine total transmit power ( row (3) and/or (3bis) ), to satisfy the PSD value above

- Note: RAN1 will further check the consistency of the definition of row(s) in link budget table when the IMT-2020 based link budget tale is updated