3GPP TSG RAN WG1 #103-e			                                                R1-2008031

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.11.2.1

Source: 	CMCC

Title: 	Discussion on resource allocation for power saving

Document for:	Discussion and Decision

Introduction

In RAN#86 meeting, a new work item for NR sidelink enhancements was approved [1], in which the following objectives on resource allocation for power saving was identified:

Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

In this contribution, we provide our preliminary views on the resource allocation for power saving.

Discussion

In the new WID, to further enhance the resource allocation to reduce power consumption of UEs, it is recommended to adopt the partial sensing and the random resource selection in Rel-14 LTE-V as the baseline. Below we discuss partial sensing mechanism and random resource selection separately.

Partial sensing mechanism

For the partial sensing mechanism in Rel-14 LTE-V, UE only performs sensing in a subset of subframes relevant for resource allocation, instead of the entire sensing window for full sensing mechanism. If partial sensing is configured, the following steps are used, as shown in Figure 1:

- Step 1: UE determines by its implementation a set of subframes which consists of at least  subframes (the orange subframes in Figure.1) within the time interval ;

- Step 2: If a subframe  is included in the set of subframes in Step 1, UE shall monitor any subframe (like the blue subframes in Figure.1) if k-th bit of the high layer parameter gapCandidateSensing is set to 1;

- Step 3: UE excludes any candidate single subframe resource from the selection window via pre-defined rules which are the same procedure with resource selection with full sensing.

Figure 1: Illustration of Rel-14 LTE-V partial sensing.

It is noted that there are two understandings for the LTE-V partial sensing mechanism:

- The partial sensing only saves the power consumption of decoding and computation. In this understanding, when requested by higher layer at any subframe n to determine at least Y subframes within the selection window, the set of subframes can be arbitrarily selected, which is fully flexible. To do so, the UE has to buffer all resources, and only the power consumption of SCI decoding and computation are saved.

- The partial sensing saves the power consumption of both decoding/computation and PSCCH/PSSCH reception. In this understanding, a set of subframes which consists at least Y subframes are within a pre-defined pattern when requested by higher layer at subframe n, which achieves limited flexibility. UE is able to buffer only a subset of resources according to the pre-defined resource selection pattern. Therefore, the UE can save the power consumption of both PSCCH/PSSCH reception and decoding/computation at the cost of flexibility of resource selection.

Observation 1: There are two understandings for the LTE-V partial sensing mechanism:

- The partial sensing only saves the power consumption of decoding/computation.

- The partial sensing saves the power consumption of both decoding/computation and PSCCH/PSSCH reception.

Since the resource allocation for power saving in NR sidelink takes the Rel-14 LTE-V partial sensing as a starting point, the partial sensing mechanism of NR sidelink may implement based on the above two understandings. However, LTE-V’s partial sensing is designed particularly for periodic traffic and PUE does not need to perform reception. So potential issues can be identified if NR sidelink adopts the LTE-V partial sensing without any modifications, such as various resource reservation periods, the aperiodic traffic and dynamic reservations and diverse use cases.

- Various resource reservation periods

In LTE-V, it mainly focuses on the periodic traffic model with the reservation period to be a multiple of 100ms, therefore, a 1000ms sensing window is divided into 10 pieces with a sensing step of 100ms. The step size 100ms can work well since other period values are multiple of 100ms, that is, whenever a candidate resource in the selection window is reserved by other UEs, the PUE can recognize the resource reservation and exclude the resources via pre-defined rules.

However, NR sidelink identifies extra periodic traffic models, i.e. {(1…99), 100, 200, 300, 400, 500, 600, 700, 900, 1000} ms reservation period and up to 16 reservation period values can be (pre-)configured per resource pool. If other UE reserves resources with reservation period of 20ms (i.e. 40 slots), PUE would not be able to identify this reservation if the fixed step size 100ms is applied, since PUE does not perform sensing at slot .

Proposal 1: Partial sensing mechanism in NR sidelink needs to take into account the supported reservation period values {(1..99), 100, 200, 300, 400, 500, 600, 700, 900, 1000} ms in Rel-16.

To match the features of NR sidelink traffic models, the step size of sensing needs be re-considered, and the high layer parameter gapCandidateSensing should be enhanced accordingly. One simple way is to derive the step size per resource pool and based on all (pre-)configured reservation period values, i.e. the step size can be common divisor of all the period values in the (pre-)configured reservation period list, in this way UE can identify each resource reservation and exclude the corresponding candidate single slot resource regardless of reservation period of other UEs. In addition, in order to maximize the performance of power saving, maximum common divisor can be considered. Moreover, the number of bits of high layer parameter gapCandidateSensing needs to be modified to be ratio of sensing window size and the derived step size.

Proposal 2: The step size of partial sensing is derived based on the (pre-)configured reservation period values per resource pool:

- the step size can be common divisor of all the period values in the (pre-)configured reservation period list.

- The aperiodic traffic and dynamic reservations

LTE-V partial sensing mainly designed to deal with periodic traffic or reservations without considering aperiodic traffic as well as dynamic reservations, which would degrade the sensing performance if LTE-V’s partial sensing mechanism is directly applied to NR sidelink resource allocation mode-2 without any modifications. Take Figure.2 for example, if other UE dynamically reserves resources just before slot n (the green resources in Figure.2), the reserved resources would not be detected by UE configured with partial sensing, hence it may not exclude the corresponding candidate resource. In such a case, the data transmission is probable to be interfered and the reliability would be decreased.

Figure 2: Illustration of impact of aperiodic traffic or dynamic reservations

Proposal 3: Partial sensing mechanism in NR sidelink needs to take aperiodic traffic and dynamic reservations into consideration.

One possible solution may be adding an additional sensing window before the candidate single slot resources. That is, after UE determines a set of slots which consists of at least Y slots within the selection window, UE monitors all resources within time period [, ] if a slot  is included in the set of slots determined above. The selection of X needs to accommodate the maximum time of dynamic reservation. In this way, UE can identify the dynamic reservations caused by both periodic and aperiodic traffic and achieve better performance of resource selection. Moreover, considering the possibility that periodic traffic and aperiodic traffic may share a common resource pool, UE configured with partial sensing needs to sense all resources within [, ] and any slot , where the derivation of Pstep is discussed in Proposal 2.

Proposal 4: In NR sidelink partial sensing, if a slot  is included in the determined set of slots, UE monitors all resources within [, ] and slot .

- Diverse use cases

In addition, the typical use case of Rel-14 LTE-V partial sensing is V2P, where the pedestrian UEs identifies the need of power saving. In such a case, only transmitter (P-UE) uses partial sensing to perform resource selection for power saving and receiver (V-UE) would perform reception via full sensing mechanism. In R17 sidelink enhancement WI, solutions for power saving are targeted not only pedestrian UEs, but also UEs in public safety and commercial use cases (e.g., smart home, smart factory, etc.), in these scenarios, both transmitters and receivers identify the motivation for power saving. In such a case, enhancements for sensing pattern alignment should be studied, especially with second understanding of power saving.

Proposal 5: Considering public safety and commercial use cases where both transmitters and receivers have the requirement of power saving, enhancements for sensing pattern alignment should be studied.

- Other potential enhancements

For the second understanding of power saving, UE determines by its implementation a set of slots which consists at least Y slots according to a pre-defined resource selection pattern, since UE only buffers a subset of slots which are relevant to the determined set of slots in selection window to save power consumption of PSCCH/PSSCH reception. However, it is possible that the determined set of slots may not meet the requirement of remaining PDB, or the number of determined set of slots that satisfy the PDB may be limited. In such a case, the data transmission is highly probable to be interfered and jammed, and the transmission reliability is decreased. Moreover, the number of determined set of slots needs to be higher than a (pre-)configured parameter minNumCandidateSlot. In LTE-V partial sensing, the parameter is fixed regardless of the data priority and which resource pool the transmission would take place. However, the (pre-)configured parameter minNumCandidateSlot represents the level of power consumption and number of retransmissions. So it can be considered that the parameter minNumCandidateSlot is (pre-)configured parameter for each priority value and per resource pool.

Proposal 6: In NR sidelink partial sensing, UE shall determine by its implementation a set of slots which consists at least Y slots within selection window and Y shall be greater than or equal to a higher layer parameter minMumCandidateSlot, where minMumCandidateSlot is (pre-)configured for each priority value and per resource pool.

In NR sidelink mode-2, if the number of candidate single-slot resources remaining in the set SA after resource exclusion procedure is smaller than X% of the total number of candidate resources, the RSRP threshold will be repeatedly increased with step of 3dB until X% of single-slot resources remain. If partial sensing mechanism is configured, since the determined set of slots is only a subset of the resource selection window and the determination is totally by UE implementation, the determined set of slots may be not enough or suitable to meet the requirement of reliability. To be specific, when the determined set of slots is slightly greater than the higher layer parameter minNumCandidateSlot or if the selected Y subframes are just right highly occupied and interfered by other UEs, the less and unsuitable candidate single-slot resources may result in unlimited increase of RSRP threshold, which leads to severe transmission interference and further impact the transmission reliability.

Proposal 7: Partial sensing mechanism in NR sidelink needs to take into account the unlimited increasing of RSRP threshold.

One possible solution is to set an upper limit of the number of RSRP threshold increments or the maximum value of increased RSRP threshold. When the upper limit or the maximum value is reached, instead of continue increasing RSRP threshold, UE can increase the number of determined set of slots resulting in more candidate single-slot resources. With more candidate resources, the interference can be decreased and reliability is increased.

Proposal 8: In NR sidelink partial sensing, an upper limit of the number of RSRP threshold increments or the maximum value of increased RSRP threshold can be configured. When the upper limit or the maximum value is reached, UE increases the number of determined set of slots.

Random resource selection

In LTE-V, a UE can be configured to either perform random resource selection or partial sensing, and when both are configured, it is up to UE implementation to do one of them. Moreover, considering that Rel-17 sidelink enhancements are targeted not only pedestrian UEs, but also UEs in public safety and commercial use cases, it would be beneficial to improve the resource utilization efficiency if different use cases can share the same resource pool. In addition, different resource pools may be configured with overlapped resources, so appropriate collision control among different resource selection schemes, i.e. full sensing, partial sensing and random resource selection, is needed, especially for UE using random resource selection, where no sensing information is obtained.

One possible way is to indicate resource selection schemes via SCI and when transmitter performs resource selection, it determines whether to exclude a candidate single-slot resource based on separately configured RSRP thresholds for different resource selection schemes. Alternatively, if random resource selection is identified via SCI decoding and overlapping is observed between the reserved resource and candidate resource, UE is expected to exclude the corresponding candidate resource without RSRP comparison. When the number of remaining candidate single-slot resources after exclusion procedure is less than X% of total number of candidate resources, the increased RSRP threshold value can be different among resource selection schemes. Moreover, when UE reporting candidate resource set to higher layer for resource selection, additional information such as whether the candidate resource overlaps with resources reserved by UE using random resource selection can be reported together, which may be beneficial for MAC layer to perform resource selection.

Proposal 9: Consider to support different resource selection schemes to share a common resource pool to improve resource utilization efficiency.

Proposal 10: RAN 1 discusses collision control mechanism among different resource selection schemes and the following alternatives can be considered:

- Different RSRP thresholds or increased RSRP threshold value is (pre-)configured for different resource selection scheme;

- UE reports whether one candidate resource overlaps with resources reserved by random resource selection UE to higher layer for further resource selection.

Conclusions

In this contribution, we discuss on the resource allocation for power saving, and the following observation and proposals are provided:

Observation 1: There are two understandings for the LTE-V partial sensing mechanism:

- The partial sensing only saves the power consumption of decoding/computation.

- The partial sensing saves the power consumption of both decoding/computation and PSCCH/PSSCH reception.

Proposal 1: Partial sensing mechanism in NR sidelink needs to take into account the supported reservation period values {(1..99), 100, 200, 300, 400, 500, 600, 700, 900, 1000} ms in Rel-16.

Proposal 2: The step size of partial sensing is derived based on the (pre-)configured reservation period values per resource pool:

- the step size can be common divisor of all the period values in the (pre-)configured reservation period list.

Proposal 3: Partial sensing mechanism in NR sidelink needs to take aperiodic traffic and dynamic reservations into consideration.

Proposal 4: In NR sidelink partial sensing, if a slot  is included in the determined set of slots, UE monitors all resources within [, ] and slot .

Proposal 5: Considering public safety and commercial use cases where both transmitters and receivers have the requirement of power saving, enhancements for sensing pattern alignment should be studied.

Proposal 6: In NR sidelink partial sensing, UE shall determine by its implementation a set of slots which consists at least Y slots within selection window and Y shall be greater than or equal to a higher layer parameter minMumCandidateSlot, where minMumCandidateSlot is (pre-)configured for each priority value and per resource pool.

Proposal 7: Partial sensing mechanism in NR sidelink needs to take into account the unlimited increasing of RSRP threshold.

Proposal 8: In NR sidelink partial sensing, an upper limit of the number of RSRP threshold increments or the maximum value of increased RSRP threshold can be configured. When the upper limit or the maximum value is reached, UE increases the number of determined set of slots.

Proposal 9: Consider to support different resource selection schemes to share a common resource pool to improve resource utilization efficiency.

Proposal 10: RAN 1 discusses collision control mechanism among different resource selection schemes and the following alternatives can be considered:

- Different RSRP thresholds or increased RSRP threshold value is (pre-)configured for different resource selection scheme;

- UE reports whether one candidate resource overlaps with resources reserved by random resource selection UE to higher layer for further resource selection.

References

- RP-193231, New WID on NR sidelink enhancement, LG Electronics, Sitges, Spain.