3GPP TSG RAN WG1 #103-e																																																				R1-2008038

e-Meeting, October 26th – November 13th, 2020

Source:	CMCC

Title:	Discussion on scenarios for IoT NTN

Agenda Item:	8.15.1

Document for:	Discussion and Decision

Introduction

A new study item on NB-Io/eMTC support for Non-Terrestrial Network (IoT NTN) was approved in RAN#86 meeting, with the following justification and objectives [1]:

Justification IoT operation is critical in remote areas with low/no cellular connectivity for many different industries, including e.g.: - Transportation (maritime, road, rail, air) & logistics - Solar, oil & gas harvesting - Utilities - Farming - Environment monitoring - Mining etc. The capabilities of NB-IoT and eMTC are a good fit to the above, but will require satellite connectivity to provide coverage beyond terrestrial deployments, where IoT connectivity is required. There is an urgent need for a standardized solution allowing global IoT operation anywhere on Earth, in view of other solutions already available. It is important that satellite NB-IoT or eMTC be defined in a complementary manner to terrestrial deployments. Objective The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including: - Bands of interest in sub 6 GHz - Device type with PC3 or PC5 (LEO and GEO) - Satellite constellation orbit LEO and GEO - Transparent payload. - Link budget NOTE 1: This first objective will be based on the scenarios documented in TR 38.821. NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions. The second objective is, for the above identified scenarios, to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items: - Aspects related to random access procedure/signals [RAN1, RAN2] - Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2] - Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2] - Aspects related to HARQ operation [RAN2, RAN1] - General aspects related to timers (e.g. SR, DRX, etc.) [RAN2] - RAN2 aspects related to idle mode and connected mode mobility [RAN2] - RLF-based for NB-IoT - Handover-based for eMTC - System information enhancements [RAN2] - Tracking area enhancements [RAN2] NOTE 3: GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

In this contribution, we will share our view on scenarios applicable to NB-IoT/eMTC over NTN.

Discussion

In our view, NB-IoT/eMTC over NTN is valuable in the following scenarios,

- Remote areas with sparsely distributed terminals, e.g., oil & gas harvesting, farming, environment monitoring, etc.

- Transportation & logistics to guarantee anywhere connectivity to 5G network. In this case, service continuity between NTN and terrestrial network needs to be supported, i.e., UE may switch to terrestrial network if available due to lower power consumption.

(a) Remote areas with sparsely distributed terminals							 (b) Transportation & logistics

Figure 1: Potential deployment scenarios for IoT NTN

Observation 1: NB-IoT/eMTC over NTN is valuable in the following scenarios

- Remote areas with sparsely distributed terminals

- Transportation & logistics to guarantee anywhere connectivity to 5G network

Furthermore, it is important to study and clarify the following issues applicable to NB-IoT/eMTC application over NTN

- The global frequency policy for satellite communication in sub 6 GHz

- Link budget for worse conditions to guarantee anywhere connectivity to 5G network, including

  - Rain and cloud attenuation

  - Building entry loss, if indoor deployment is considered

  - Carriage and container penetration loss for logistics application

  - Vegetation loss for outdoor application

- Maximum transmission power assumption of a satellite, which impact the maximum supported IoT terminal number

- GNSS capability and accuracy assumption on position and/or a reference time and frequency determination

Proposal 1: Study and clarify the following issues applicable to NB-IoT/eMTC application over NTN

- The global frequency policy for satellite communication in sub 6 GHz

- Link budget for worse conditions to guarantee anywhere connectivity to 5G network, including

  - Rain and cloud attenuation

  - Building entry loss, if indoor deployment is considered

  - Carriage and container penetration loss for logistics application

  - Vegetation loss for outdoor application

- Maximum transmission power assumption of a satellite, which impact the maximum supported IoT terminal number

- GNSS capability and accuracy assumption on position and/or a reference time and frequency determination

Conclusions

In this contribution, we share our views on scenarios applicable to NB-IoT/eMTC over NTN. The observations and proposals are summarized as follows:

Observation 1: NB-IoT/eMTC over NTN is valuable in the following scenarios

- Remote areas with sparsely distributed terminals

- Transportation & logistics to guarantee anywhere connectivity to 5G network

Proposal 1: Study and clarify the following issues applicable to NB-IoT/eMTC application over NTN

- The global frequency policy for satellite communication in sub 6 GHz

- Link budget for worse conditions to guarantee anywhere connectivity to 5G network, including

  - Rain and cloud attenuation

  - Building entry loss, if indoor deployment is considered

  - Carriage and container penetration loss for logistics application

  - Vegetation loss for outdoor application

- Maximum transmission power assumption of a satellite, which impact the maximum supported IoT terminal number

- GNSS capability and accuracy assumption on position and/or a reference time and frequency determination

References

- RP-193235, “Study on NB-Io/eMTC support for Non-Terrestrial Network”, RAN#86, Sitges, Spain, December 9-13, 2019.