3GPP TSG RAN WG1 #103-e			R1-2008095

e-Meeting, October 26th – November 13rd, 2020

Agenda Item: 	7.2.4

Source:		Spreadtrum Communications

Title:		Remaining issues in NR sidelink mode 1 resource allocation

Document for:		Discussion and decision

Introduction

In this contribution, we provide our considerations on the remaining issues and corresponding TP in NR sidelink transmission mode 1, including UE procedure for transmitting PSCCH and reservation period in SCI for CG.

Discussion

UE procedure for transmitting PSCCH

In RAN1 #100e, the following agreements were achieved for configured grant type-1:

Agreements: - The mapping between the values of HPN signaled in DCI and HPN signaled in SCI is fixed for a TB, and is up to UE implementation. - For dynamic grant, the toggling of NDI in DCI is used as the toggling of NDI in SCI for the first SL transmission scheduled by the DCI. The SCI for the remaining transmissions scheduled by the DCI, if any, have the NDI untoggled with respect to the first SL transmission. - FFS NDI in SCI for PUCCH ACK-NACK error cases Agreements: - The HARQ process ID for each transmission in a resource corresponding to a SL configured grant is determined based on the formula used for UL configured grants. - The mapping with the values of HPN in SCI is fixed for a TB, and is up to UE implementation. Note: This corresponds to the HARQ process ID for the interaction between gNB and UE, if any distinction is made.

We have agreed that the mapping between the values of HPN signaled in DCI and HPN signaled in SCI is fixed for a TB, and how to set the NDI field in SCI is determined. The corresponding TP has also been discussed [1]. However, it is not included in section 16.4 of the latest TS 38.213.

Proposal 1: The TP about HPN and NDI in RAN1 #100e should be included in section 16.4 of TS 38.213.

TP #1

------------------------------------------- Start of Draft TP of 213 ------------------------------------- - 16.4 UE procedure for transmitting PSCCH A UE transmitting a PSCCH using Mode-1 shall set the contents of SCI format 0_1 as follows: - the UE shall set value of the HARQ process ID field as indicated by higher layers. - for an initial transmission scheduled by a DCI format 3_0 with CRC scrambled by SL-RNTI, the UE shall toggle the NDI field value in SCI format 0_1 if the NDI field value in DCI format 3_0 is toggled; otherwise the UE shall not toggle the NDI field value in SCI format 0_1. For the subsequent transmissions scheduled by the DCI format 3_0 with CRC scrambled by SL-RNTI, the UE shall not toggle the NDI field value in SCI format 0_1. A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] sets - "Resource reservation period" as an index in sl-ResourceReservePeriod-r16 corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource-r16 - the values of the frequency resource assignment field and the time resource assignment field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where: - N=minNselected, Nmax_reserve, where Nselected is a number of resources in the set Ry with slot indices yj, 0≤j≤Nselected-1, such that y0<y1<…<yNselected-1≤y0+31, and Nmax_reserve is provided by sl-MaxNumPerReserve - each resource, from the set of Ry resources, corresponds to LsubCH contiguous sub-channels and a slot in a set of slots {tySL}, where LsubCH is the number of sub-channels available for PSSCH/PSCCH transmission in a slot - t0SL,t1SL,t2SL,... is a set of slots in a sidelink resource pool [6, TS 38.214] - y0 is an index of a slot where the PSCCH with SCI format 1-A is transmitted. For decoding of a SCI format 1-A, a UE may assume that a number of bits provided by sl-NumReservedBits-r16 can have any value. ---------------------------------------------- End of Draft TP ---------------------------------------------

Reservation period in SCI for CG

NR V2X mode 1 and mode 2 are different sidelink resource allocation modes for acquiring transmission resources. For TX UEs obtaining resources via SL configured grant in mode 1, it’s better to set the “resource reservation period” field in SCI format 1_A to the CG periodicities, so that resource pool sharing between mode 1 and mode 2 UEs can be supported.

Proposal 2: For TX UEs obtaining resources via SL configured grant in mode 1, it’s better to set the “resource reservation period” field in SCI format 1_A to the CG periodicities, and adopt TP #2 in subsection 2.2.

TP #2

For configured grant, a TP for “resource reservation period” field in SCI 1_A is provided in section 16.1.5 of TS 38.213 as follows.

------------------------------------------- Start of Draft TP of 213 ------------------------------------- - 16.4 UE procedure for transmitting PSCCH A UE transmitting a PSCCH using Mode-1 shall set the contents of SCI format 0_1 as follows: - the UE shall set value of the resource reservation period field as indicated by periodSlCG, if the UE is provided periodSlCG. - the UE shall set value of the HARQ process ID field as indicated by higher layers. - for an initial transmission scheduled by a DCI format 3_0 with CRC scrambled by SL-RNTI, the UE shall toggle the NDI field value in SCI format 0_1 if the NDI field value in DCI format 3_0 is toggled; otherwise the UE shall not toggle the NDI field value in SCI format 0_1. For the subsequent transmissions scheduled by the DCI format 3_0 with CRC scrambled by SL-RNTI, the UE shall not toggle the NDI field value in SCI format 0_1. A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] sets - "Resource reservation period" as an index in sl-ResourceReservePeriod-r16 corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource-r16 - the values of the frequency resource assignment field and the time resource assignment field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where: - N=minNselected, Nmax_reserve, where Nselected is a number of resources in the set Ry with slot indices yj, 0≤j≤Nselected-1, such that y0<y1<…<yNselected-1≤y0+31, and Nmax_reserve is provided by sl-MaxNumPerReserve - each resource, from the set of Ry resources, corresponds to LsubCH contiguous sub-channels and a slot in a set of slots {tySL}, where LsubCH is the number of sub-channels available for PSSCH/PSCCH transmission in a slot - t0SL,t1SL,t2SL,... is a set of slots in a sidelink resource pool [6, TS 38.214] - y0 is an index of a slot where the PSCCH with SCI format 1-A is transmitted. For decoding of a SCI format 1-A, a UE may assume that a number of bits provided by sl-NumReservedBits-r16 can have any value. ---------------------------------------------- End of Draft TP ---------------------------------------------

Conclusions

Proposal 1: The TP about HPN and NDI in RAN1 #100e should be included in section 16.4 of TS 38.213.

Proposal 2: For TX UEs obtaining resources via SL configured grant in mode 1, it’s better to set the “resource reservation period” field in SCI format 1_A to the CG periodicities, and adopt TP #2 in subsection 2.2.

References

- [1] R1-2001410 - TP for TS 38.213 from [100e-NR-5G_V2X_NRSL-RA_Mode1-02]