3GPP TSG RAN WG1 Meeting #103-e                        	             R1-2008105

E-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.6.2

Source:	Spreadtrum Communications

Title:	Discussion on reduced PDCCH monitoring

Document for:	Discussion and decision

- Introduction

In Rel.17 NR, the RedCap UE should be defined to adapt the use case of IoT, e.g. industrial sensor, video surveillance and wearables. Based on the objective of the SID [1], the RedCap UE can have the following methodology to save power.

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1].

In this contribution, we discuss the PDCCH monitoring reduction for the RedCap UE. This contribution is a revision of R1-2006286.

- PDCCH monitoring reduction

It is common understanding that the more stringent requirement of power consumption should be applied for RedCap UE, and thus the reduction of PDCCH monitoring could be studied for the RedCap UE.

  - BD number reduction

The number of BD directly affect the power consumption for decoding PDCCH candidate.

In our evaluation results in Section 3, it can be seen that the power saving gain by reducing the number of BD by half is approximately 6%~14%. Therefore, reducing the number of BD should be considered.

Proposal 1: Reducing the number of BD can be considered.

Standard impact

The limitation of the maximum number of BD

The number of non-overlapping CCE number decides the UE complexity of channel estimation, and the number of BD decides the UE complexity of decoding of PDCCH candidates. The large number of BD/CCE provide the flexibility of allocating PDCCH candidate at gNB side, which can reduce PDCCH blocking rate at gNB side. However, the large number of BD/CCE will cause too large UE complexity and power consumption, which is not friendly at UE sider. Therefore, the maximum number of BD/CCE is specified in the standard for the eMBB UE.

As the number of BD directly affect the power consumption for decoding PDCCH candidate, whether the maximum number of BD can be further limited in the standard is worthy studying.

The further limiting the maximum BD number may have the following standards impact, such as

- The further limitation of the maximum number of BD, and

- PDCCH candidates dropping rule.

Proposal 2: The follow standard impact can be considered.

- The further limitation of the maximum number of BD

- PDCCH candidates dropping rule

The reduction of DCI size budget

DCI size budget directly impacts the BD/CCE number. It is common understanding reducing DCI size budget is helpful for BD/CCE limit.

However, limiting to small DCI size budget for hypotheses to blind decoding could result in spectral efficiency loss, since large number of padded bits are needed.

Fortunately, this is not critical for RedCap UE. Since RedCap UE may work with small data or fixed data packet, the dedicated DCI format can be designed for RedCap UE with the DCI size as common as possible. For example, one codeword and narrowband BWP can fit for the small data transmission, which cause DCI bits being reduced in some fields, e.g. multi-antenna information, carrier indicator and resource allocation.

For eMBB UE, at most 4 different DCI sizes are monitored by the UE per slot and at most 3 different DCI sizes are monitored per C-RNTI per slot. For RedCap UE, it is expected to reduce the DCI size budget to 1 or 2.

Proposal 3: The follow standard impact can be considered.

- DCI size budget reduction

- Evaluation

In this section, we provide some numerical analyses on the power saving gain by reducing the number of PDCCH candidates, which is based on the power consumption models agreed in last meeting and scaling factors provided in TR 38.840 [2].

  - Evaluation assumptions

Power scaling scheme

According to [2], the power scaling scheme for PDCCH candidates processing reduction is as following:

Power scaling scheme for PDCCH candidates processing reduction: - Scaling for the power reduction due to PDCCH candidates processing (e.g. AL/CCE/BD) reduction is modelled solely based on its effect on micro sleep portion of the PDCCH-only slot - The UE power scheme should include the portion of PDCCH processing time reduction in accordance to PDCCH candidates (e.g. AL/CCE/BD) reduction - Note: In the reference configuration, the first two symbols are PDCCH symbols - For power scaling for PDCCH candidate reduction (for same slot scheduling only): P(α) = α ∙ Pt + (1 – α) ∙ 0.7Pt - where α is the ratio of PDCCH candidates to the max number of PDCCH candidates in the reference configuration (α>0). Pt is the PDCCH-only power for same-slot scheduling.

UE power consumption model

- FR1 2RX

According to [3], the power consumption mode for redcap in the case of FR1 2RX is as following:

| Power State | Alt.4a |
| --- | --- |
| Deep Sleep (PDS) | 0.8 |
| Light Sleep (PLS) | 18 |
| Micro sleep (PMS) | 31 |
| PDCCH-only (PPDCCH) | 50 for same-slot scheduling, 40 for cross-slot scheduling |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 120 |
| PDSCH-only (PPDSCH) | 112 |
| SSB/CSI-RS proc. (PSSB) | 50 |
| Intra-frequency RRM measurement (Pintra) | · [60]Note4 (synchronous case, N=8, measurement only) · [80] Note4 (combined measurement and search) |
| Inter-frequency RRM measurement (Pinter) | [60] Note4 (neighbor cell search power per freq. layer) · [15080] Note4 (measurement only per freq. layer) · Micro sleep power assumed for switch in/out a freq. layer |

- FR1 1RX

In some case, PDCCH-only power may be smaller than micro sleep power after applying scaling factor. To address this issue, the following rules are proposed in last meeting. In our evaluation, Rule 1 is adapted.

Working assumption: Adopting the following rule for power determination - Rule 1: ‘Micro sleep’ power of 1 Rx is [0.8]x2 Rx ‘Micro sleep’ power - Rule 2: For both 1 Rx and 2 Rx configuration, - P(α) = max (Micro-sleep, α ∙ Pt + (1 – α) ∙ 0.7Pt)) - Pt is the PDCCH-only power for same slot and cross-slot scheduling cases.

The power consumption mode for redcap in the case of FR1 1RX is as following:

| Power State |  |
| --- | --- |
| Deep Sleep (PDS) | 0.8 |
| Light Sleep (PLS) | 18 |
| Micro sleep (PMS) | 25 |
| PDCCH-only (PPDCCH) | 35 for same-slot scheduling, 28 for cross-slot scheduling |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 84 |
| PDSCH-only (PPDSCH) | 78 |
| SSB/CSI-RS proc. (PSSB) | 35 |
| Intra-frequency RRM measurement (Pintra) | · [42]Note4 (synchronous case, N=8, measurement only) · [56] Note4 (combined measurement and search) |
| Inter-frequency RRM measurement (Pinter) | [42] Note4 (neighbor cell search power per freq. layer) · [15056] Note4 (measurement only per freq. layer) · Micro sleep power assumed for switch in/out a freq. layer |

- FR2 2RX

According to TR 38.840, UE power consumption model for FR2 2RX is as following.

| Power State | Values |
| --- | --- |
| Deep Sleep (PDS) | 1 |
| Light Sleep (PLS) | 20 |
| Micro sleep (PMS) | 45 |
| PDCCH-only | 175 |
| SSB or CSI-RS proc. | 175 |
| PDCCH + PDSCH | 350 |

- FR2 1RX

According to TR 38.840, UE power consumption model for FR2 1RX is as following.

| Power State | Values |
| --- | --- |
| Deep Sleep (PDS) | 1 |
| Light Sleep (PLS) | 20 |
| Micro sleep (PMS) | 45 |
| PDCCH-only | 122.5 |
| SSB or CSI-RS proc. | 122.5 |
| PDCCH + PDSCH | 245 |

Traffic models and DRX configuration

According to the agreement of the last meeting, the parameters and DRX configuration corresponding to various traffic models are shown in the following table.

|  | Instant messaging | VoIP | heartbeat |
| --- | --- | --- | --- |
| Model | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. | FTP model 3 |
| Packet size | 0.1 Mbytes |  | 100 Bytes |
| Mean inter-arrival time | 2 sec |  | 60 sec |
| DRX setting | C-DRX cycle: 320ms inactivity timer: 80ms On duration: 10ms | C-DRX cycle: 40ms inactivity timer: 10ms On duration: 4ms | C-DRX cycle: 640ms inactivity timer: 80ms, 200ms FR1 On duration: 10ms FR1 On duration: 5ms |

Reference configuration

The following reference configuration are adopted for FR1:

Reference Configuration for FR1 - Downlink: TDD - Subcarrier spacing (SCS): 30 kHz - Number of carrier: 1CC, - System Bandwidth: 20 MHz - PDCCH periodicity: 1 slot - PDCCH region of 2 symbol at beginning of a slot, - k0 = 0, - 36 PDCCH blind decoding,

The following reference configuration are adopted for FR2:

Reference configuration for FR2: - Downlink: TDD, FR2 - Subcarrier spacing (SCS): 120 kHz - Number of carriers: 1CC - System Bandwidth: 100 MHz - PDCCH periodicity: 1 slot - PDCCH region of 2 symbol at beginning of a slot - k0 = 0 - 20 PDCCH blind decoding

  - Evaluation results

For the power reduction evaluation for reduced number of BD, the follow case are considered in our evaluation.

- FR1, TDD, 1 RX

- FR1, TDD, 2 RX

- FR2, TDD, 1 RX

- FR2, TDD, 2 RX

For each test case, we evaluate the power saving gain with reducing the BD number compared to the scenario without BD number reduction. In our evaluation, PDCCH/PDSCH repetition for coverage recover is not considered. However, in our view, PDCCH and PDSCH with the same repetition number can also use the agreed UE power consumption model for the power reduction evaluation.

The evaluation time is 100 C-DRX for each test case. Furthermore, in order to simplify the evaluation, the power saving technologies agreed in R16 are not considered. We calculated the time occupied by various power states according to the agreed traffic model, as shown in the following table.

Table 1: Time distribution for different power state in FR1

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Time distribution/ms |  |  |  |
| Deep sleep | 29792 | 0 | 62920 | 62800 |
| Light sleep | 0 | 3150 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 2200 | 1000 | 1080 | 1200 |
| PDCCH+PDSCH | 8 | 40 | 0.5 | 0.5 |

Table 2: Time distribution for different power state in FR2

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Time distribution/ms |  |  |  |
| Deep sleep | 30260 | 0 | 63420 | 63300 |
| Light sleep | 0 | 3080 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 1746 | 910 | 580 | 700 |
| PDCCH+PDSCH | 2 | 10 | 0.125 | 0.125 |

Case1：FR1 2RX

Power distribution for different traffic mode without the BD number reduction in FR1 2RX can be seen from the follow table.

Table 3: Power distribution without the BD number reduction in FR1 2RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 47667.2 | 0 | 100672 | 100480 |
| Light sleep | 0 | 106560 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 220000 | 100000 | 108000 | 120000 |
| PDCCH+PDSCH | 1920 | 9600 | 120 | 120 |
| Total | 269587.2 | 216160 | 208792 | 220600 |

Power distribution with reducing the BD number by half in FR1 2RX can be seen from the follow table.

Table 4: Power distribution with reducing the BD number reduction by half in FR1 2RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 47667.2 | 0 | 100672 | 100480 |
| Light sleep | 0 | 106560 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 187000 | 85000 | 91800 | 102000 |
| PDCCH+PDSCH | 1800 | 9000 | 112.5 | 112.5 |
| Total | 236467.2 | 200560 | 192584.5 | 202592.5 |

Power saving gain of reducing the BD number by half in FR1 2RX can be seen from the follow table.

Table 5: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 12.3% |
| VoIP | 7.2% |
| Heartbeat (80ms) | 7.8% |
| Heartbeat (200ms) | 8.2% |

Power distribution with 25% reduction of the BD number for FR1 2RX can be seen from the follow table.

Table 6: Power distribution with 25% reduction if the BD number for FR1 2RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 47667.2 | 0 | 100672 | 100480 |
| Light sleep | 0 | 106560 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 203500 | 92500 | 99900 | 111000 |
| PDCCH+PDSCH | 1800 | 9000 | 112.5 | 112.5 |
| Total | 252967.2 | 208060 | 200684.5 | 211592.5 |

Power saving gain with 25% reduction of the BD number for FR1 2RX can be seen from the follow table.

Table 7: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 6.2% |
| VoIP | 3.7% |
| Heartbeat (80ms) | 3.9% |
| Heartbeat (200ms) | 4.1% |

Case2：FR1 1RX

Power distribution for different traffic mode without the BD number reduction in FR1 1RX can be seen from the follow table.

Table 8: Power distribution for different traffic mode without the BD number reduction in FR1 1RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 47667.2 | 0 | 100672 | 100480 |
| Light sleep | 0 | 106560 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 154000 | 70000 | 75600 | 84000 |
| PDCCH+PDSCH | 1344 | 6888 | 84 | 84 |
| Total | 203011.2 | 183280 | 176356 | 184564 |

Power distribution for different traffic mode with reducing the BD number by half in FR1 1RX can be seen from the follow table.

Table 9: Power distribution for different traffic mode with reducing the BD number by half in FR1 1RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 47667.2 | 0 | 100672 | 100480 |
| Light sleep | 0 | 106560 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 130900 | 59500 | 64260 | 71400 |
| PDCCH+PDSCH | 1260 | 6300 | 78.75 | 78.75 |
| Total | 179827.2 | 172360 | 165010.75 | 171958.75 |

Power gain for different traffic mode with reducing the BD number by half in FR1 1RX can be seen from the follow table.

Table 10: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 11.4% |
| VoIP | 6.0% |
| Heartbeat (80ms) | 6.4% |
| Heartbeat (200ms) | 6.8% |

Power distribution for different traffic mode with 25% reduction of the BD number in FR1 1RX can be seen from the follow table.

Table 11: Power distribution for different traffic mode with 25% reduction of the BD number in FR1 1RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 47667.2 | 0 | 100672 | 100480 |
| Light sleep | 0 | 106560 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 142450 | 64750 | 69930 | 77700 |
| PDCCH+PDSCH | 1260 | 6300 | 78.75 | 78.75 |
| Total | 191377.2 | 177610 | 170680.75 | 178258.75 |

Power gain for different traffic mode with 25% reduction of the BD number in FR1 1RX can be seen from the follow table.

Table 12: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 5.7% |
| VoIP | 3.1% |
| Heartbeat (80ms) | 3.2% |
| Heartbeat (200ms) | 3.4% |

Case3：FR2 2RX

Power distribution for different traffic mode without the BD number reduction in FR2 2RX can be seen from the follow table.

Table 13: Power distribution for different traffic mode without the BD number reduction in FR2 2RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 60520 | 0 | 126840 | 126600 |
| Light sleep | 0 | 123200 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 611100 | 318500 | 203000 | 245000 |
| PDCCH+PDSCH | 5600 | 28000 | 350 | 350 |
| Total | 677220 | 469700 | 330190 | 371950 |

Power distribution for different traffic mode with reducing the BD number by half in FR2 2RX can be seen from the follow table.

Table 14: Power distribution for different traffic mode with reducing the BD number by half in FR2 2RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 60520 | 0 | 126840 | 126600 |
| Light sleep | 0 | 123200 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 519435 | 270725 | 172550 | 200900 |
| PDCCH+PDSCH | 5180 | 25900 | 323.75 | 323.75 |
| Total | 585135 | 419825 | 299713.75 | 327823.75 |

Power gain for different traffic mode with reducing the BD number by half in FR2 2RX can be seen from the follow table.

Table 15: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 13.6% |
| VoIP | 10.5% |
| Heartbeat (80ms) | 9.2% |
| Heartbeat (200ms) | 11.9% |

Power distribution with 25% reduction of the BD number for FR2 2RX can be seen from the follow table.

Table 16: Power distribution for different traffic mode with 25% reduction of the BD number in FR2 2RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 60520 | 0 | 126840 | 126600 |
| Light sleep | 0 | 123200 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 565267.5 | 612.5 | 187775 | 226625 |
| PDCCH+PDSCH | 5180 | 25900 | 323.75 | 323.75 |
| Total | 630967.5 | 443712.5 | 314938.75 | 353548.75 |

Power gain for different traffic mode with 25% reduction of the BD reduction in FR2 2RX can be seen from the follow table.

Table 17: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 6.8% |
| VoIP | 5.5% |
| Heartbeat (80ms) | 4.6% |
| Heartbeat (200ms) | 4.9% |

Case4：FR2 1RX

Power distribution for different traffic mode without the BD number reduction in FR2 1RX can be seen from the follow table.

Table 18: Power distribution for different traffic mode without the BD number reduction in FR2 1RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 60520 | 0 | 126840 | 126600 |
| Light sleep | 0 | 123200 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 427770 | 222950 | 142100 | 171500 |
| PDCCH+PDSCH | 3920 | 19600 | 245 | 245 |
| Total | 488780 | 365750 | 269185 | 298345 |

Power distribution for different traffic mode with reducing the BD number by half in FR2 1RX can be seen from the follow table.

Table 19: Power distribution for different traffic mode with reducing the BD number by half in FR2 1RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 60520 | 0 | 126840 | 126600 |
| Light sleep | 0 | 123200 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 363604.5 | 189507.5 | 120785 | 145775 |
| PDCCH+PDSCH | 3332 | 18130 | 208.25 | 208.25 |
| Total | 427456 | 330837 | 247833.25 | 272620 |

Power gain for different traffic mode with reducing the BD number by half in FR2 1RX can be seen from the follow table.

Table 20: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 13.1% |
| VoIP | 9.4% |
| Heartbeat (80ms) | 7.9% |
| Heartbeat (200ms) | 8.6% |

Power distribution for different traffic mode with 25% reduction of the BD number in FR2 1RX can be seen from the follow table.

Table 21: Power distribution for different traffic mode with 25% reduction of the BD number in FR2 1RX

| Power state | Instant messaging | VoIP | Heartbeat (Inactivity timer=80ms) | Heartbeat (Inactivity timer=200ms) |
| --- | --- | --- | --- | --- |
|  | Power distribution |  |  |  |
| Deep sleep | 60520 | 0 | 126840 | 126600 |
| Light sleep | 0 | 123200 | 0 | 0 |
| Micro sleep | 0 | 0 | 0 | 0 |
| PDCCH-only | 395687.25 | 206228.75 | 131442.5 | 158637.5 |
| PDCCH+PDSCH | 3332 | 18130 | 208.25 | 208.25 |
| Total | 459539.25 | 347558.75 | 258490.75 | 285482.5 |

Power gain for different traffic mode with 25% reduction of the BD number in FR2 1RX can be seen from the follow table.

Table 22: Evaluation results

| Traffic mode | Power gain |
| --- | --- |
| Instant messaging | 6.6% |
| VoIP | 5.0% |
| Heartbeat (80ms) | 4.0% |
| Heartbeat (200ms) | 4.3% |

Observation 1: The power saving gain by reducing the number of BD by half is approximately 6%~14% for different traffic mode.

- Conclusion

Based on the analyses and discussions, we have the following observations and proposals:

Proposal 1: Reducing the number of BD can be considered.

Proposal 2: The follow standard impact can be considered.

- The further limitation of the maximum number of BD

- PDCCH candidates dropping rule

Proposal 3: The follow standard impact can be considered.

- DCI size budget reduction

Observation 1: The power saving gain by reducing the number of BD by half is approximately 6%~14% for different traffic mode.

- Reference

- RP-193238 New SID on support of reduced capability NR devices.

- TR.38.840. NR; Study on UE power saving.

- 3GPP RAN1#102e Chairman’s Notes, August 17 - 28, 2020.