3GPP TSG RAN WG1 Meeting #103-e	R1-2008120

e-Meeting, October 26th – November 13th, 2020.

# Title:	Draft reply LS on sidelink configured grant handling

# Response to:	LS (R2-2008586) to RAN1 on sidelink configured grant handling

# Release:	Rel-16

# Work Item:	5G_V2X_NRSL-Core

Source:	RAN1 (Samsung)

To:	RAN2

Cc:

Contact Person:

Name:

Tel. Number:

E-mail Address:

# Attachments:	none

1. Overall Description:

RAN1 would like to provide reply on the following question from RAN2 in R2-2008586 [1].

Question

Is it possible to use the retransmission opportunities for initial transmission for a sidelink configured grant in case when the data was not available for the transmission opportunity for initial transmission?

RAN1 reply

From RAN1 perspective the case above is supported. Data arrival may occur later than earliest scheduled resource(s) within a period of sidelink configured grant, and UE can use the remaining resources for initial transmission.

In addition, in TS 38.213 [2], Section 16.5, RAN1 specified UE behaviour of reporting sidelink HARQ-ACK on PUCCH for the case of skipping resource scheduled by sidelink configured grant, as follows:

The UE generates an ACK if the UE does not transmit a PSCCH with a SCI format 1-A scheduling a PSSCH in any of the resources provided by a configured grant in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information.

If UE cannot use retransmission opportunities for initial transmission in case when the data was not available for initial transmission opportunity, UE will skip all resources within the period. According to TS 38.213, UE will report ACK to gNB, and gNB will not allocate resources by dynamic grant for potential retransmission corresponding to the sidelink configured grant. It will impact performance of configured grant thus should be avoided.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG RAN1 Meeting #104	-e	25th January – 5th February 2021			Online.

TSG RAN1 Meeting #104	-bis-e	12th – 20th April 2021					Online.

# References

- R2-2008586, LS to RAN1 on sidelink configured grant handling, Huawei