3GPP TSG RAN WG1 Meeting #103-e	R1-2008131

# e-Meeting,  October 26th - November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Mode2 for NR sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 16-Oct-2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Slots used for SL (sidelink) reception and transmissions of a resource pool are a subset of the slots available for sidelink reception and transmission. This subset is determined by a bitmap of length Lbitmap configured by higher layers. In the specifications SL slots are denoted by tiSL, whereas, SL slots that belong a resource pool are denoted as t'iSL. The slots of a SL resource pool t'iSL are numbered number successively 0, 1, … , wherein, i is the logical slot index. In section 8.1.4 of TS 38.214, the symbol tiSL is used instead of t'iSL when referring to SL slots of a SL resource pool. This should be corrected by using t'iSL instead of tiSL. In addition, in section 8.1.4 of TS 38.214, the UE procedure for determining the subset of resources includes step 5 where a UE shall exclude any candidate single-slot resource for all possible reservation periodicity values if the UE has not monitored a certain slot in the sensing window due to its own transmission. This resource exclusion procedure was designed assuming the worst case where the UE does not know the reservation periodicity of the other possible colliding UE(s). However, if we apply this principle considering for short reservation periodicities, this may lead to over excluding resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 8.1.4, for slidelink slots of a sidelink resource pool use symbol t'iSL instead of tiSL In section 8.1.4, remove step 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For section 8.1.4, ambiguity in the specification leading to incorrect sidelink operation. A UE might be required to monitor sidelink slots not in the slidelink resource pool. A UE might exclude candidate resources for resource selection unnecessarly and result in performance degradation when short reservation periodicity is configured at higher layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: The change in notation of SL slot is to avoid ambiguity and incorrect operation. A UE that doesn’t follow the removal of step 5 of the section 8.1.4, can over exclude sidelink resources, which an potential lead to a higher RSRP threshold for resource exclusion. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

*** Unchanged text is omitted ***

Notation:

t0SL,t1SL,t2SL,...t0'SL,t1'SL,t2'SL,...  denotes the set of slots which can belong to a sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot tySLty'SL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL) where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which can belong to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

3)	The internal parameter Th(pi,pj)  is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-ThresPSSCH-RSRP-List-r16, where i=pi+pj-1*8.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

6)5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSLtm'SL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot tmSLtm'SLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL tm+q×Prsvp_RX''SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SLtn''SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSLt0'SL,t1'SL,...,tTmax'SL, otherwise slot tn'SLtn''SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSLt0'SL,t1'SL,...,tTmax'SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

7)6)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

The UE shall report set SA to higher layers.

*** Unchanged text is omitted ***