3GPP TSG RAN WG1 Meeting #103-e	R1-2008133

# e-Meeting,  October 26th - November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PHY layer procedures for NR sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 16-Oct-2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 specified prioritization rules between PUCCH carrying sidelink HARQ-ACK and other UCIs or PUSCH. Handling of collision between PUCCH carrying SL HARQ-ACK and PRACH or PUSCH scheduled by RAR grant is a remaining prioritization case and needs to be speicfied. In CBRA case, this collision is possible since connected mode UE may be use dedicated RRC configuration. The RACH resource configured in dedicated RRC configuration may overlap with PUCCH resource. The PRACH/PUCCH resource overlapping issue also happened in legacy NR Uu system and UE cannot transmit PRACH and other UL TX simultaneously or within an internal corresponding to processing time of TA adjustment. Considering the prioritization between PSFCH/S-SS/PSBCH block transmission and transmission of PRACH or PUSCH scheduled by UL grant in RAR is specified as PRACH or PUSCH scheduled by UL grant in RAR is always prioritized, the rule can be similarly reused for SL HARQ-ACK report on PUCCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 16.2.4.3.1, add PUCCH carrying sidelink HARQ-ACK in the prioritization rule between PRACH and sidelink channels. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is lack of UE behaviour in the specification. It leading to UE dropping important transmission and impacts system performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 16.2.4.3.1	Prioritizations for sidelink and uplink transmissions

A UE performs prioritization between SL transmissions and UL transmissions after performing the procedures described in Clause 9.2.5 and in Clause 6.1 of [6, TS 38.214].

PSFCH transmissions in a slot have a same priority value as the smallest priority value among PSSCH receptions with corresponding HARQ-ACK information provided by the PSFCH transmissions in the slot.

A priority of S-SS/PSBCH block transmission is provided by sl-SSB-PriorityNR.

For prioritization between PSFCH/S-SS/PSBCH block transmission and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR, or a PUCCH with sidelink HARQ-ACK information report

-	if the UL transmission is for a PUSCH or for a PUCCH with priority index 1,

-	if sl-PriorityThresholdULURLLC is provided

-	the SL transmission has higher priority than the UL transmission if a smallest priority value of the SL transmission(s) is smaller than sl-PriorityThresholdULURLLC; otherwise, the UL transmission has higher priority than the SL transmission

-	else

-	the UL transmission has higher priority than the SL transmission

-	else

-	the SL transmission has higher priority than the UL transmission if the smallest priority value of the SL transmission(s) is smaller than sl-PriorityThreshold; otherwise, the UL transmission has higher priority than the SL transmission

A PRACH transmission, or a PUSCH scheduled by an UL grant in a RAR, has higher priority than a PSFCH or a S-SS/PSBCH block transmission or a PUCCH transmission with a sidelink HARQ-ACK information report.

### < Unchanged parts are omitted >