3GPP TSG RAN WG1 #103-e			R1-2008152

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.1.2.4

Source: 	Samsung

Title: 	Enhancements on HST-SFN

Document for:	Discussion and Decision

Introduction

In RAN1#102-e meeting, the following agreements were made for HST-SFN related enhancements [1]:

Agreement For the discussion purpose consider the following categorization of the enhanced DL transmission schemes - Scheme 1: - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - Scheme 2: - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner - PDSCH from TRPs is transmitted in SFN manner Agreement Study the following aspects of the enhanced transmission schemes: - For scheme 1: - Target DL physical channels, i.e., PDSCH only or PDSCH + PDCCH - Whether more than 2 QCL/TCI states are required and corresponding signaling details - Whether and how to indicate scheme 1 for differentiation with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states - QCL relationship between TRS and DMRS ports - Note: Other schemes/aspects are not precluded - For scheme 2: - Association of each MIMO layer of PDSCH to DM-RS antenna ports - Whether more than 2 QCL/TCI states are required and corresponding signaling details - Whether and how to indicate scheme 2 for differentiation with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states - Note: Other schemes/aspects are not precluded Agreement For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme: - 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation - 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step - 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step - Note: A second set of TRS resource(s) may be transmitted at 3rd step. Agreement Study TRP-based frequency offset pre-compensation including the following aspects: - Aspects related to indication of the carrier frequency determined based on the received TRS resource(s) in the 1st step - Option 1: Implicit indication of the Doppler shift(s) using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step - Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - Type of the uplink reference signals / physical channel used in the 2nd step, necessity of new configuration and corresponding signaling details - Option 2: Explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework - FFS: Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - CSI reporting aspects, configuration, quantization, signalling details, etc. - New QCL types/assumption for TRS with other RS (e.g., SS/PBCH), when TRS resource(s) is used as target RS in TCI state - New QCL types/assumptions for TRS with other RS (e.g., DM-RS), when TRS resource(s) is used as source RS in the TCI state - Target physical channels (e.g., PDSCH only or PDSCH/PDCCH) and reference signals that should be supported for pre-compensation - Signaling/procedural details on whether/how the pre-compensation is applied to target channels - Whether multiple sets of TRS and pre-compensation on TRS is needed in 3rd step. - Note: Other aspects/schemes are not precluded

This contribution provides Samsung’s view on the topics above.

UE-based enhancements for HST-SFN

A typical deployment scenario for HST system is to place multiple RRHs which are connected to a BBU and distributed along the railway. In such scenario, the NW can transmit DL data to an HST UE via SFN manner due to the difficulty to track accurate position of the UE. This type of transmission provides more reliable transmission than using one nearest RRH. In addition, received signal power at the UE side can be boosted via SFN transmission.

To reap the benefit of SFN transmission via HST, the set of participating RRHs in the transmission should be selected carefully. For example, the following can be considered:

i) Transmission delay of each RRH-UE link: It is beneficial to form the set of RRHs such that their aggregate SFN delay spread to UE is within a CP, otherwise UE would experience ISI.

ii) UE complexity: UE experiences multiple Doppler shifts, each coming from an RRH participating in the SFN transmission. As the number of RRHs increases, UE processing complexity also increases to track increased number of Doppler shifts.

As a result, NW firstly forms multiple SFN combinations, and then chooses an appropriate combination for transmission depending on the position and processing capability of UE. Figure 1 shows an example of transmission scenario when 4 RRHs are in the network and located equi-distantly with D. In this example, it is assumed that each RRH uses bi-directional beams, each with separate TRS and up to 4 adjacent RRHs (that is RRHs within 2D distance from a UE) participate in SFN transmission. In this set-up, the list of all possible SFN combinations can be found in Table 1.

Figure 1. Example of transmission scenario in bi-directional 4-RRH network

Table 1. List of possible SFN combinations in Figure 1

| # of RRHs for SFN | List of SFN combinations | # of combinations |
| --- | --- | --- |
| Single RRH (DPS) | #0L, #0R, #1L, #1R, #2L, #2R, #3L, #3R | 8 |
| Two RRHs | {#0L,#1L}, {#0R,#1L}, {#0R,#1R}, {#1L,#2L}, {#1R,#2L}, {#1R,#2R}, {#2L,#3L}, {#2R,#3L}, {#2R,#3R} | 9 |
| Three RRHs | {#0R,#1L,#2L}, {#0R,#1R,#2L}, {#1R,#2L,#3L}, {#1R,#2R,#3L} | 4 |
| Four RRHs | {#0R,#1R,#2L,#3L} | 1 |
| Total # of combinations |  | 22 |

When a NW relies on Rel-16 QCL framework, the Doppler shift, and Doppler spread property of SFNed target channel are inferred from a single TRS. It implies that when the NW is to use multiple SFN combinations for data and RS transmission it needs to configure the corresponding number of TRSs. In other words, if the NW is to use all SFN combinations according to Table 1, it needs to configure 22 TRSs. This would result in excessive use of NW resources only for TRS transmissions. According to our analysis, TRS overhead ratio from the total REs in NW is 36.5% for N = 8 RRHs as shown in Table 2, where the detailed assumptions are provided in the appendix. Note that the overhead ratio would further increase as the number of RRHs and/or the number of Tx beams per RRH increases, which can be excessive especially in FR2.

Observation 1. Rel-16 QCL framework requires excessive TRS overhead to support HST-SFN.

To solve the TRS overhead problem, we propose an extension of the QCL framework to allow the QCL properties, e.g., Doppler shift and Doppler spread associated with PDSCH can be inferred from multiple TRSs. In this extended QCL framework, NW does not need to configure separate TRS per each SFN combination. Rather, it only needs to configure one TRS per RRH per beam. Then NW indicates UE about QCL property of a SFNed target channel via a list of TRSs corresponding to the RRHs participating in SFN transmission. TRS overhead reduction according to the QCL framework enhancement is analyzed in Table 2 for the bi-directional N-RRH network. Note that the TRS overhead can be reduced from 36.5 % to 15.4% (57.9% ratio reduction) for N = 8.

Observation 2. QCL framework enhancement can reduce TRS overhead significantly compared to Rel-16 QCL framework for HST-SFN.

Proposal 1. Support QCL framework enhancement to enable multiple TRSs as QCL reference RSs for SFNed target channel, i.e., support Scheme 1 for UE enhancement.

Table 2. TRS overhead analysis for Rel-16 QCL framework vs. extended QCL framework

|  | # of TRSs |  | TRS overhead |  | TRS overhead ratio |  | Overhead ratio reduction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rel-16 QCL | QCL enhancement | Rel-16 QCL | QCL enhancement | Rel-16 QCL | QCL enhancement |  |
| N = 4 | 22 | 8 | 13,200 REs | 4,800 REs | 21.2% | 7.7% | 63.7% |
| N = 6 | 30 | 12 | 18,000 REs | 7,200 REs | 28.8% | 11.5% | 60.0% |
| N = 8 | 38 | 16 | 22,800 REs | 9,600 REs | 36.5% | 15.4% | 57.9% |

The extended QCL framework is also beneficial in reducing UE complexity in estimating Doppler shifts from TRS. In Rel-16 HST-SFN, the gNB transmits TRS in an SFNed manner and the UE has to track multiple Doppler shifts from the TRS. However, QCL framework enhancement allows gNB to transmit non-SFNed TRS and the UE is sufficient to track only a single Doppler shift per TRS. Figure 3 and Table 3 show the performance of Doppler shift tracking by a single SFNed TRS vs. three non-SFNed TRS in a 3-RRH network (estimation accuracy is improved by 64%). Assume that the UE tracks Doppler shifts of two nearest RRHs at a time. By using multiple non-SFNed TRS, the UE can achieve better Doppler shift tracking performance than SFNed TRS while requiring lower complexity. For example, the UE may suffer from large estimation errors from SFNed TRS as marked with red circles in the left figure. Such error occurs in the case that the transmission delays from RRHs to UE are almost the same so that their Doppler shifts are non-resolvable from SFNed TRS. If we use separate TRS per RRH, such error would not occur.

Figure 2. Doppler shift estimation in a 3-RRH network

Table 3. Mean absolute error of estimated Doppler shift

| SFNed TRS | Separate TRS per RRH | Error reduction by using separate RRH |
| --- | --- | --- |
| 113.4 Hz | 40.8 Hz | 72.6 Hz (64.0%) |

Observation 3. QCL framework enhancement that allows UE to track only a single Doppler shift per TRS in HST-SFN results in better estimation performance and lower complexity compared to Rel-16.

During RAN1#102-e meeting, it was under discussion in which target channels are applied the QCL framework enhancement for TRS. Rel-16 QCL framework defines that QCL target channel of TRS can be one of i) PDSCH, ii) PDCCH, iii) aperiodic TRS, iv) CSI-RS for channel measurement, v) CSI-RS for beam management. Since all the aforementioned channels are supported for Rel-16 spec-transparent SFN, there’s no reason to exclude any of them from the QCL enhancement. On the contrary, estimation performance of all the aforementioned channels would be improved via the QCL framework enhancement because of the improvement of tracking accuracy for QCL referred TRS, as shown in Table 3.

In designing QCL framework enhancement, one thing to decide is the maximum number of TRSs to indicate QCL reference. Considering that the HST-SFN deployment consists of linearly placed TRPs with equi-distance, transmission signals from the TRPs except the two nearest ones would be very small at UE because of the large pathloss. Thus, it is sufficient to support up to two TRSs for QCL framework enhancement.

Another thing to decide for QCL framework enhancement is how to indicate two TRSs as QCL references for SFNed target channel. In Rel-16 NC-JT, it is supported to indicate two TCI states for NC-JT PDSCH where the first TCI state is from TRP#1 while the second TCI state is from TRP#2. We can adopt the similar indication method for SFN purpose but the issue is whether NC-JT and SFN needs to be switched dynamically in HST scenario and if so, how UE can distinguish between NC-JT and SFN. In our view, both NC-JT and SFN are beneficial for HST scenario, where NC-JT can provide high throughput when UE is relatively slow so that its location can be accurately tractable, while SFN can provide high reliability when UE is fast. Since UE speed would change dynamically, it is beneficial for NW to support dynamic switching between NC-JT and SFN transmission. One way to support such dynamic switching is to introduce enhanced TCI state for SFN purpose only. In the enhanced TCI state, two pairs of QCL reference RSs are contained as in Table 4, where the first element in a pair is the QCL reference RS for QCL-TypeA/B/C and the second element is that for QCL-TypeD. Then, UE would know a PDSCH is SFNed if the PDSCH is indicated with a single enhanced TCI state. When a PDSCH is indicated with two legacy TCI states, it can be understood that the PDSCH is transmitted in NC-JT manner. The enhanced TCI state can be applied to PDCCH and CSI-RS as well to support SFN transmission on those channels.

Table 4. Example of RRC structure for enhanced TCI state.

TCI-State-r17 ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R qcl-Type1-r17 QCL-Info, qcl-Type2-r17 QCL-Info OPTIONAL, -- Need R ... }

Proposal 2. Regarding QCL enhancements to support per-TRP TRS in HST-SFN,

- Support PDSCH / PDCCH / CSI-RS as QCL target channel

- Support up to two TRSs for QCL reference

- Study indication mechanism to support dynamic switching between SFN and NC-JT

In RAN1#102-e meeting, it was under discussion whether PDSCH DM-RS needs to be transmitted in per-TRP manner as well as TRS. Per-TRP DM-RS transmission implies that each TRP participating in SFN are allocated different DM-RS ports, resulting in doubled number of DM-RS ports and doubled DM-RS overhead. Since per-TRP channel properties, such as Doppler and delay properties, can be already obtainable by per-TRP TRS, the necessity to introduce per-TRP DM-RS transmission is unclear considering its high overhead.

Proposal 3. Do not support per-TRP DM-RS transmission for HST-SFN, i.e., do not support Scheme 2 for UE enhancement.

gNB-based enhancements for HST-SFN

If the gNB can know the Doppler shift of the channel between each RRH and the UE, the gNB can pre-compensate Doppler shift at each RRH such that the UE experiences only a small residual Doppler shift at the received signal. Such pre-compensation can reduce UE complexity to track multiple Doppler shifts and improve the reliability of the received signal.

Figure 3 shows the PDSCH throughput comparison between UE-based enhancement scheme (per-TRP TRS) and gNB enhancement scheme (gNB pre-compensation), when there are two TRPs in the network and UE is located in the middle of the two TRPs. It is shown that gNB pre-compensation can improve the UE throughput significantly compared to the UE-based enhancement scheme.

Figure 3. Throughput comparison between UE enhancement and gNB enhancement schemes.

Observation 4. gNB pre-compensation can provide significant PDSCH throughput improvement compared to UE-based enhancement scheme.

In RAN1#102-e, the following three steps were defined for gNB pre-compensation procedures:

Agreement For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme: - 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation - 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step - 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step - Note: A second set of TRS resource(s) may be transmitted at 3rd step.

In the second step, UE informs Doppler shifts to the TRPs participating in SFN transmission via the two options:

- Option 1. UE transmits UL signal, e.g., SRS, and each TRP measures the UL signal to obtain Doppler shift

- Option 2. UE reports the measured Doppler shift from the 1st step via CSI report

Option 1 can be supported by gNB implementation by measuring Doppler shift from existing SRS for UL sounding purpose, so that no additional specification impact or RS overhead are needed. However, Option 2 requires additional specification support to define new UCI for Doppler shift reporting, and also requires additional UL resource for the new UCI. Hence in our view, Option 1 is preferred rather than Option 2.

Overall Doppler shift measurement procedure according to Option 1 is depicted in Figure 4. In Figure 4, UE receives separate TRSs from TRP#0 and TRP#1. Then, UE transmits an UL signal with frequency fc which are received with frequencies fm,0=fc+f0 and fm,1=fc+f1 at TRP#0 and TRP#1, respectively, where f0 and f1 are Doppler shifts at TRP#0 and TRP#1, respectively. From the UL signal, gNB obtains the frequency offset between TRP#0 and TRP#1, Δf=fm,0-fm,1=f0-f1, to be used for pre-compensation. In this procedure, when multiple TRSs are received with different frequencies the value of UL frequency fc is not defined in current specification so that it is up to UE implementation how to determine the value of fc.

There were discussions in RAN1#102-e meeting whether it is needed to indicate the UL frequency fc for UL signals used in pre-compensation purpose. In our view, indicating fc is not needed because the frequency offset measured at gNB, Δf, does not contain the component fc such that the pre-compensated value at gNB won’t differ regardless of fc.

Figure 4. Pre-compensation procedure according to Option 1.

Observation 5. In HST-SFN environment configured with multiple per-TRP TRSs, it is up to UE implementation how to determine UL frequency.

Proposal 4. Regarding the 2nd step for gNB pre-compensation procedure:

- Support Option 1 (Doppler shift measurement at TRP side)

- There’s no need to indicate UL frequency to UL signals/channels for pre-compensation purpose

In the third step, TRP#0 transmits signal with frequency fc while TRP#1 transmits signal with frequency fc-Δf for pre-compensation purpose. Then UE would receive SFNed signal from the two TRPs where frequency offset observed in the second step is removed.

One issue on the third step is to determine the target channel where the pre-compensation applies. In our view, it is needed to apply pre-compensation for PDSCH and PDCCH, and furthermore for TRS. As Rel-16 QCL framework mandates PDSCH and PDCCH to be in QCL linkage with TRS with QCL-TypeA, implying that Doppler shift and spread properties of PDSCH/PDCCH are inferred from the TRS, it is evident that TRS should be pre-compensated first to support pre-compensated PDSCH/PDCCH.

A drawback of supporting pre-compensated TRS is that the TRS needs to be UE specific since each UE faces different relative speed, resulting in the necessity of different pre-compensation value. For the system with lots of UEs, periodically transmitting such TRS imposes high RS overhead. An effective way to relieve such overhead is using AP-TRS only for pre-compensation, but it needs enhancement from the current spec. In Rel-16, the AP-TRS is restricted to be QCL’ed with P-TRS according to QCL-TypeA which implies that the AP-TRS cannot have different Doppler shift from its QCL’ed P-TRS, i.e., AP-TRS alone cannot be pre-compensated.

For pre-compensation of AP-TRS, we can introduce a new QCL type which does not contain Doppler shift property. Then, pre-compensated AP-TRS can be QCL’ed with non pre-compensated TRSs via the new QCL type as shown in Figure 4. Pre-compensated PDSCH or PDCCH can be QCL’ed with the AP-TRS via existing QCL type, e.g., QCL-TypeA and QCL-TypeD.

Proposal 5. Regarding the 3rd step for gNB pre-compensation procedure:

- Support pre-compensated AP-TRS to be used as QCL reference of pre-compensated PDCCH/PDSCH

- Introduce new QCL type to define QCL relationship between pre-compensated AP-TRS and non pre-compensated P-TRS

Conclusion

In this contribution, the following observations and proposals are given:

Observation 1. Rel-16 QCL framework requires excessive TRS overhead to support HST-SFN.

Observation 2. QCL framework enhancement can reduce TRS overhead significantly compared to Rel-16 QCL framework for HST-SFN.

Observation 3. QCL framework enhancement that allows UE to track only a single Doppler shift per TRS in HST-SFN results in better estimation performance and lower complexity compared to Rel-16.

Observation 4. gNB pre-compensation can provide significant PDSCH throughput improvement compared to UE-based enhancement scheme.

Observation 5. In HST-SFN environment configured with multiple per-TRP TRSs, it is up to UE implementation how to determine UL frequency.

Proposal 1. Support QCL framework enhancement to enable multiple TRSs as QCL reference RSs for SFNed target channel, i.e., support Scheme 1 for UE enhancement.

Proposal 2. Regarding QCL enhancements to support per-TRP TRS in HST-SFN,

- Support PDSCH / PDCCH / CSI-RS as QCL target channel

- Support up to two TRSs for QCL reference

- Study indication mechanism to support dynamic switching between SFN and NC-JT

Proposal 3. Do not support per-TRP DM-RS transmission for HST-SFN, i.e., do not support Scheme 2 for UE enhancement.

Proposal 4. Regarding the 2nd step for gNB pre-compensation procedure:

- Support Option 1 (Doppler shift measurement at TRP side)

- There’s no need to indicate UL frequency to UL signals/channels for pre-compensation purpose

Proposal 5. Regarding the 3rd step for gNB pre-compensation procedure:

- Support pre-compensated AP-TRS to be used as QCL reference of pre-compensated PDCCH/PDSCH

- Introduce new QCL type to define QCL relationship between pre-compensated AP-TRS and non pre-compensated P-TRS

References

- 3GPP RAN1, RAN1#102-e Chairman’s Note

- Appendix

Table 5. Evaluation assumptions

| Parameters | Values |
| --- | --- |
| TRP layout | Ds = 700 m, Dmin = 150 m |
| Channel model | HST-SFN channel model in TS36.101 |
| Carrier frequency / SCS | 1.9 GHz / 15 kHz |
| UE speed | 500km/h |
| gNB / UE antenna configuration | 2 Tx ports / 2 Rx ports |
| Number of RBs | 50 RBs |
| TRS periodicity / pattern | 10 ms, 2 slot pattern |