3GPP TSG RAN WG1 #103-e		R1-2008163

e-Meeting, October 26th – November 13th, 2020

Agenda Item:		8.3.4

Source:		Samsung

Title:		Discussion for propagation delay compensation enhancements

Document for:		Discussion and decision

# Introduction

In RAN 1 # 102-emeeting, some evaluation assumptions for propagation delay compensation were discussed and agreed:

Agreements:

- ± 8∙64∙Tc/2μ8*64*Tc/2 as the TA indicating error is assumed in the evaluation.

Agreements:

For 5GS synchronicity budget requirement,

- One Uu interface is assumed for smart grid.

- Two Uu interfaces are assumed for control-to-control.

Agreements:

For BS transmit timing error, further study the following three options:

- Option 1: 65 ns

- Option 2:±130ns for the indoor scenario and ±200ns for the smart grid scenario

- Option 3:82.5 ns

Agreements:

The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133 should be considered for evaluation of the time synchronization.

Agreements:

Asymmetry between downlink and uplink channel for control-to-control scenario is not considered.

Agreements:

100 ns is assumed for BS detecting error.

Agreements:

Timing advance adjustment accuracy defined in Table 7.3.2.2-1 in TS 38.133 is assumed for evaluation of the time synchronization.

Agreements:

Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization.

Moreover, the two main solutions to be further studied were identified as:

Agreements: The following options for propagation delay compensation are further studied in RAN1 - Option 1: TA-based propagation delay - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity). - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te) - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected) - Option 2: RTT based delay compensation: - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

In additional, it was agreed in RAN 2 that:

=> Discuss by email the delay components and understand the requirements with each component and agree on what needs to be addressed

=>	Introduce propagation delay compensation for the improved synchronisation accuracy requirement in case of in UL Time Synchronization

This contribution discusses propagation delay compensation from RAN 1 perspective.

# Propagation delay compensation

In RAN 2 #111-emeeting, it was agreed to introduce propagation delay compensation for the improved synchronisation accuracy requirement. Two main options were discussed in RAN 1: TA-based propagation delay and RTT based delay compensation. The accuracy, and pros/cons of the two methods were discussed below.

2.1 TA-based propagation delay

In general, for TA-based method, gNB estimation propagation delay and signals or pre-compensate the propagation delay to the UE. Based on the study in Rel-16 and the agreement, the estimation errors are related to subcarrier spacing and mainly consist of the following parts shown as figure 1:

- gNB transmit error (Tgte): There are three options for BS transmit timing error: 65ns, 130ns/200ns for different scenario respectively, 82.5ns.

- UE transmit timing error (Te): As defined in TS 38.133 Table 7.1.1-2, this error Te is about as ±12*64*Tc , ±8*64*Tc and ± 7*64*Tc for 15KHz, 30KHz and 60KHz respectively.

  - Note: besides Te, there may have UE estimation error of DL time synchronization Tee. In our understanding, in RAN 5 test case, it is assumed to be 0 to test the Te.

- gNB estimation error (Tge): This error is introduced when gNB estimates UL channel/signals to obtain propagation delay. PRACH may not able to provide enough accuracy of time estimation. SRS/UL DMRS or PUSCH can be used. The estimation accuracy depends on occupied BW. The error about ~100ns can be assumed with 20MHz bandwidth.

- TA  granularity error (TTAe): If using current TA granularity, i.e., 16∙64∙Tc/2μ, and the error is  ±8∙64∙Tc/2μ.

- Timing Advance adjustment accuracy (Tadj): Based on the requirement in TS 38.133, the accuracy of TA adjustment are ±256 Tc/ ±256 Tc/ ±128 Tc / ±32 Tc respectively for SCS 15/30/60/120 kHz.

If we assume UE compensate propagation delay based on the first TA command after PRACH (or a uplink signling with known TA in advance), there is no need to consider TA adjustment error.  UE can directly use the first TA command for time sync but not to adjust the TA. In another situation, assuming gNB adjust TA for propagation delay compensation, i.e., TA = 2 x propagation delay. gNB estimates TA based on  tgNBRx- t'gNBTx, where t'gNBTx is configured time resource allocation for PRACH. Since gNB cannot tell the error of Tgte and Te, therefore, the TA command to indicated to UE is TA =  tgNBRx- t'gNBTx+Tgte+Tee+Te=2*PD +Tgte+Tee+Te. For normal UL transmission, UE will apply this TA with an error of TTAe and  Tadj. However, if UE use TA command to calculate propagation delay PD’ = TA/2 based on the TA command, the calculation of propagation delay will not include the error of TA adjustment Tadj but only the indication error TTAe. If regular TA is applied, TA adjustment error need to be considered. Besides, for the time synchronization calculation, since it reference DL timing, additional transmission error need to be considered. Table 1 provide a summary of the above errors.

Figure 1

2.2 RTT based propagation delay

RTT based propagation delay was briefly discussed. The method is from positioning.  The main idea is that, the RTT time (double of propagation delay) is equal to ∆T1 +∆T2, where, ∆T1=tUERx-tgNBTx and ∆T2=tgNBRx-tUETx. Therefore, ∆T1 +∆T2 = (tUERx-tUETx)+ (tgNBRx-tgNBTx). In this method, it is also gNB to estimate the propagation delay. And gNB needs to know the offset at UE side between transmission and reception time. In addition, gNB needs to know when to receive the uplink transmission. In positioning, the information is from SRS-Pos from the LMF.

The main error comes from the following parts:

- gNB transmit error (Tgte): It happens when gNB transmit DL signaling at tgNBTx. This error is the offset between on actual gNB transmission time and the assumed gNB transmission time at gNB side.

- UE estimation error (Tee) and UE transmit timing error (Te): When UE estimate the signaling from gNB, it has estimation error. In addition, UE will transmit based on the indication from higher layer. It will additional cause transmit error.

- gNB estimation error (Tge): This error is introduced when gNB estimates UL channel/signals at tgNBRx. Similarly,  the accuracy depends on the occupied BW. Assuming similar occupied BW as TA-based method, the error can be also assumed to be ~100ns.

- UE Rx-Tx difference granularity error (TUEoffset): Because gNB need to estimate propagation delay with gNB Rx-Tx offset and UE Rx-Tx offset. In TS 38.133, the reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx)is defined with the resolution step of 2kTc, where: k =0,…,5 based on different configurations, with the estimation of PRS.

- Propagation delay indication granularity error (Tpd): gNB eventually need to signaling to UE about the propagation delay. Therefore, an additionally signaling to indicate propagation delay cannot be avoided. The granularity of propagation delay indication will also affect the total error.

Similarly, since RTT time if two times of propagation delay, the error for propagation delay estimation is half of the sum of the above Tgte, Te, Tge, TUEoffset . And when UE estimate time, gNB transmit error Tgte and the granularity error of propagation delay Tpd  needs to be additionally added.

Figure 2

2.3 Comparison of TA based method and RTT based method

From the above analysis on TA based method and RTT based method, we can see that, the fundamental of the two methods are the same. For TA based method, gNB estimate TA assuming UE Tx-Rx difference = 0, and gNB calculate gNB Tx-Rx different based on a virtual  t'gNBTx of the configured PRACH resource. Table 1 summarizes the total error from different method. With current procedure of TA and RTT method for positioning, the main different comes from the indication granularity.

For TA based method, there are three solutions.  Current TA procedure is to adjust uplink transmission time. Therefore, the granularity of current TA considered the required accuracy of UL reception at gNB side. However, in order to reduce the timing error for TNS, the granularity for propagation delay is much smaller. The finer granularity is not very useful for TA adjustment considering the hardware limited. On the other hand, finer TA granularity will cost a lot of overhead since it may happen more frequently for uplink transmission. Therefore, it is preferred to introduce a separate procedure to with a new dedicated signalling for propagation delay compensation.

Observation #1: Current TA procedure is to align the uplink signal arriving time at gNB sides, to achieve the target error of time estimation, big overhead is redundant for a normal TA procedure.

For position RTT based method, most of the configuration are from LMF other than gNB. In order to support RTT based solution, a whole new procedure and configurations needs to be introduced. In addition, to support current RTT based method, gNB and UE need to support PRS transmission and estimation. The specification impact of both RAN 1/2/3 is large and it may also have impacts on higher layer.

Observation #2: RTT-based solution requires PRS and positioning functionality. More impact in higher layer is expected while the gain compared with TA based solution is not clear.

Therefore, considering the above analysis and observation, a new procedure for propagation delay estimation can be considered.

Proposal #1: Introduce a new procedure for propagation delay estimation.

In order to improve the accuracy of time estimation, the following aspects can be considered:

- To introduce finer granularity to reduce the error TTAe caused by quantization, especial for smaller subcarrier space case, e.g., 15kHz and 30kHz.

- To reduce the estimation error Tge on UL reception time. In the analysis, SRS/UL DMRS or PUSCH is assumed for propagation delay estimation other than PRACH.

- To avoid TA adjustment error Tadj by transmit UL signal for propagation delay estimation with pre-known TA.

Other errors either are limited by hardware, or depend on UE or gNB detection performance.

Proposal #2: Introduce a finer granularity to indicate propagation delay.

Proposal #3: Consider to use SRS, UL DMRS or PUSCH for propagation delay estimation. A predefined TA can be used for transmission of the UL signalling for propagation delay estimation.

Table 1

| TA based method |  |  | Potential enhancement | RTT based method |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz |  | 15kHz | 30kHz |  |
| gNB transmit error (Tgte) | 65ns | 65ns | - | 65ns | 65ns | gNB transmit error (Tgte) |
| UE estimation error (Tee) | TBD | TBD | DL signalling, e.g., CSI-RS, PRS | TBD | TBD | UE estimation error (Tee) |
| UE transmit timing error (Te) | 391ns | 260ns | - | 391ns | 260ns | UE transmit timing error (Te) * |
| gNB estimation error (Tge) | 100ns | 100ns | UL signalling, e.g. SRS | 100ns | 100ns | gNB estimation error (Tge) |
| TA granularity (TTAe) | 260ns | 130ns | Finer granularity | 0.5~16 ns | 0.5~16 ns | UE Rx-Tx difference granularity error (TUEoffset): |
| Timing Advance adjustment accuracy (Tadj) | 130ns | 130ns | UL signaling with a pre-known TA. | TBD2 | TBD2 | Propagation delay granularity (Tpd) |
| Total Tgte +1/2*(Tgte+ Tee+ Te + Tge +TTAe ) | 437 ns +Tee/2(if any) | 342.5ns+Tee/2(if any) |  | （343~351）ns+(TBD+TBD2)/2 | （278~286）ns+(TBD+TBD2)/2 | Total Tgte +1/2*(Tgte+ Tee+ Te + Tge + TUEoffset+ Tpd) |

* For RTT based method, the transmission time needs to be further clarified.

# Conclusions

This contribution discusses propagation delay compensation from RAN 1 perspective. The following observations are made:

Observation #1: Current TA procedure is to align the uplink signal arriving time at gNB sides, to achieve the target error of time estimation, big overhead is redundant for a normal TA procedure.

Observation #2: RTT-based solution requires PRS and positioning functionality. More impact in higher layer is expected while the gain compared with TA based solution is not clear.

Based on the observations, the following proposals are made:

Proposal #1: Introduce a new procedure for propagation delay estimation.

Proposal #2: Introduce a finer granularity to indicate propagation delay.

Proposal #3: Consider to use SRS, UL DMRS or PUSCH for propagation delay estimation. A predefined TA can be used for transmission of the UL signalling for propagation delay estimation.

# References

- RP- 201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”

- TR 38.825 Study on NR Industrial Internet of Things (IoT)

- TS 38.133 Requirements for support of radio resource management v16.5.0