3GPP TSG RAN WG1 #103-e		R1-2008232

e-Meeting, October 26th – November 13th, 2020

Source:	OPPO

Title:	Text proposal of physical layer procedure for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Correction of PSSCH power control in TS38.213

It was agreed in RAN1#97 meeting that the power control of PSSCH can be based on DL PL only, or SL PL only, or both DL PL and SL PL. According to the RRC parameter, if p0-DL-PSCCHPSSCH is not configured, power control based on DL PL is disabled, and if p0-DL-PSCCHPSSCH is not configured, power control based on SL PL is disabled.

Agreements:

- For the SL open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss.

- When the SL open-loop power control is configured to use both DL pathloss and SL pathloss,

  - The minimum of the power values given by open-loop power control based on DL pathloss and the open-loop power control based on SL pathloss is taken.

    - (Working assumption) P0 and alpha values are separately (pre-)configured for DL pathloss and SL pathloss.

Existing description of PSSCH power control procedure in TS38.213 is confusing and there is some mistake in it. To make it clearer, we propose the following TP.

============================= Text proposal for 38.213 ==========================

16.2.1	PSSCH

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as:

-	if dl-P0-PSSCH-PSCCH-r16 and sl-P0-PSSCH-PSCCH-r16 are provided

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

-	elseif dl-P0-PSSCH-PSCCH-r16 is provided and sl-P0-PSSCH-PSCCH-r16 is not provided

PPSSCH(i)=minPCMAX,PMAX,CBR,PPSSCH,Di [dBm]

-	elseif dl-P0-PSSCH-PSCCH-r16 is not provided and sl-P0-PSSCH-PSCCH-r16 is provided

PPSSCH(i)=minPCMAX,PMAX,CBR,PPSSCH,SLi [dBm]

-	else

PPSSCH(i)=minPCMAX,PMAX,CBR [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of sl-MaxTxPower-r16 based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTransPower-r16 is not provided, then PMAX,CBR=PCMAX;

-

PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-    PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm]

PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i)

PPSSCH,D(i)=minPCMAX,PMAX,CBR

where

-	PO,D is a value of dl-P0-PSSCH-PSCCH-r16 if provided

-	αD is a value of dl-Alpha-PSSCH-PSCCH-r16, if provided; else, αD=1

-	PLD=PLb,f,c(qd) as described in Clause 7.1.1 except that

-	the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0

-	the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0

MRBPSSCH(i)iμ

PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL

PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i)

-	PO,SL is a value of sl-P0-PSSCH-PSCCH-r16, if provided

-	αSL is a value of sl-Alpha-PSSCH-PSCCH-r16, if provided; else, αSL=1

-	PLSL= referenceSignalPower – higher layer filtered RSRP, where

-	referenceSignalPower is obtained from a PSSCH transmit power per RE per antenna port of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient-r16, and

-	higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient-r16

-	MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration

The UE splits the power PPSSCH(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power.

A UE determines a power PPSSCH2,b,c(i) for a PSSCH transmission on a resource pool in the symbols where a corresponding PSCCH is transmitted in PSCCH-PSSCH transmission occasion i as

PPSSCH2(i)=10log10MRBPSSCHi-MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm]

where MRBPSCCH(i) is a number of resource blocks for the corresponding PSCCH transmission in PSCCH-PSSCH transmission occasion i.

The UE splits the power PPSSCH2(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power.

============================== End of text proposal  ============================

Clarification of time resource assignment in SCI

In current TS38.214 v16.2.0 section 8, two terminologies tkSL and tk'SL are used to indicate a slot that may belong to a resource pool and a slot that belongs to a resource pool based on bitmap indication respectively. During sensing procedure, UE should monitor  tkSL within sensing window. That means sensing UE should monitor all slots that may belong to a resource pool. The sensing UE needs to exclude reserved resources by decoded SCI of another UE. One issue is whether the time gap indication in SCI format 1-A, i.e. W=32 slot, is based on number of logical slot within the transmission resource pool of the UE transmitting the SCI, or based on the number of logical slot that may belong to a resource pool?

We think that should be the number of logical slot that may belong to a resource pool, i.e. tkSL. If that is based on number of logical slot of a resource pool, then the SCI indicates the logical slot within the transmission resource pool of the sensed UE during the sensing procedure. While the sensing UE cannot know the transmission resource pool of the sensed UE so that it cannot determine which slot is reserved by the sensed UE.

Proposal: The timing resource assignment in SCI format 1-A is based on logical slots that can belong to a resource pool, i.e. tkSL.

============================= Text proposal for 38.214 ==========================

8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields "Frequency resource assignment", "Time resource assignment" of the associated SCI format 1-A as described below.

"Time resource assignment" carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=t1

else

if t2-t1-1≤15

TRIV=30t2-t1-1+t1+31

else

TRIV=3031-t2+t1+62-t1

end if

end if

where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots that may belong to  a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31.

<not related>

============================== End of text proposal  ============================