3GPP TSG-RAN WG1 Meeting #103-e	R1-2008233

e-Meeting, October 26 – November 13, 2020

Agenda item:	5

Title:	Discussion on reply LS on new PQI support for PC5 communication

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

In a received LS from SA 2 in R1-2007514 (LS on new PQI support for PC5 communication), SA2 is requesting RAN1’s feedback on two questions relating to achievable performance of reusing the existing V2X mechanism for PS and commercial services with new standardized PQIs. In this contribution, we provide simulation results to show the expected performance of meeting the two PC5 services.

# 2. Discussion

In the SA2’s LS (R1-2007514), the first question is related to whether the new standardized PQIs for Public Safety services in Table 1 and commercial services in Table 2 can be supported in the AS layers.

Table 1: New Standardized PQIs for Public Safety

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New value#1 | GBR | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| New value#2 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| New value#3 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| New value#4 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| New value#5 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |

According to the above Table 1, the new PQI value with most stringent requirements is “New value #4” for Public Safety services. As such, we simulated only for this new value and provided our results in the following. The idea is, If the most stringent PQI requirements can be met for this new value, it is safe to assume that other new PQI values can be met as well. Simulation assumptions used to generate these results are provided in the Appendix.

|  | SNR = 5dB | SNR = 10dB | SNR = 25dB | SNR =35dB |
| --- | --- | --- | --- | --- |
| Success rate after 1st transmission | 0.991304 | 0.998353 | 0.999993 | 1 |
| Success rate after 2nd transmission | 0.006613 | 0.001261 | 7E-6 | 0 |
| Success rate after 3rd transmission | 0.001092 | 0.000206 | 0 | 0 |
| Success rate after 4th transmission | 0.000347 | 0.00007 | 0 | 0 |
| TB Error | 0.000643 | 0.00011 | 0 | 0 |

Observation 1: For Public Safety services in Table 1, the most stringent PER requirement of 10-6 in New value #4 can be supported at least when UE is in a good geometric condition (e.g. SNR above 25 and possibly even below).

Table 2: New Standardized PQIs for commercial service

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New value#1 | Delay Critical GBR | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset |
| New value#2 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging; |

Similarly, we simulated the new PQI value with most stringent requirements i.e. New value #1 for commercial services over PC5 interface, and we provided our results in the following. Simulation assumptions used to generate these results are also provided in the Appendix. Two different modulation schemes (16QAM and 64QAM) are used for comparison.

|  | 16QAM |  | 64QAM |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SNR = 25dB | SNR=30dB | SNR=25dB | SNR=30dB | SNR=35dB |
| Success rate after 1st transmission | 0.785649 | 0.958507 | 0.761927 | 0.968177 | 0.995108 |
| Success rate after 2nd transmission | 0.2048 | 0.040972 | 0.223648 | 0.031307 | 0.004892 |
| Success rate after 3rd transmission | 0.009551 | 0.000521 | 0.013003 | 0.000516 | 0 |
| Success rate after 4th transmission | 0 | 0 | 0.001171 | 0 | 0 |
| TB Error | 0 | 0 | 0.000251 | 0 | 0 |

Observation 2: For commercial services in Table 2, the most stringent PER requirement of 10-4 in New value #1 can be supported at least when UE is in a good geometric condition (i.e., SNR=25dB for 16QAM and SNR=30dB for 64QAM).

Conclusion 1: Based on the above provided simulation results and observations for performance evaluation of PS and commercial services, it can be concluded that the existing V2X mechanism is able to support these two services with new standardized PQIs over PC5 interface at least in FR1 with maximum bandwidth allocation.

In addition, it is stated in SA2’s LS (R1-2007514) that the MDBV values can be changed according to application requirements to override the default MDBV. For Example, for commercial service, to support required data rate of 10Gbps, the maximum MDBV value set by the application can be 12.5M byte. In SA2’s LS, the second question is related to what is the maximum MDBV value AS layer can support with the indicated PDB and PER in Table 2. In order to find the maximum MDBV value that can be supported by the AS layers, we simulated various MCS levels ranging from different modulation orders (i.e., 64QAM and 256QAM) and provided our results in the following.

|  | 64QAM |  |  |  |  | 256QAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCS=13 | MCS=15 | MCS=16 | MCS=17 | MCS=18 | MCS=22 | MCS=27 |
| Success rate after 1st transmission | 0.9364 | 0.847114 | 0.780917 | 0.660279 | 0.517549 | 0.225257 | 0.018385 |
| Success rate after 2nd transmission | 0.060613 | 0.141157 | 0.20285 | 0.315905 | 0.447047 | 0.671108 | 0.730412 |
| Success rate after 3rd transmission | 0.002988 | 0.009862 | 0.013011 | 0.019299 | 0.028002 | 0.079654 | 0.1937 |
| Success rate after 4th transmission | 0 | 0.001867 | 0.00316 | 0.003696 | 0.005036 | 0.016083 | 0.037572 |
| TB Error | 0 | 0 | 0.000062 | 0.000821 | 0.002365 | 0.007899 | 0.019913 |

Observation 3: It is observed the maximum MDBV value that can be supported by the AS layers using the existing V2X mechanism is when MCS=16 with 64QAM modulation with a maximum of 4 transmissions per TB and 30kHz SCS. The equivalent TB size is 295,176 bits. This translate into a maximum MDBV value of at least 36,897 bytes per 2ms.

Conclusion 2: Based on the above provided simulation results and observation, it can be concluded that the maximum MDBV value that can be supported by using the existing V2X mechanism is at least 36,897 bytes per 2ms in FR1 with maximum bandwidth allocation.

3. Draft reply LS answers

Answer to Q1): Based on RAN1 performance evaluation, it is concluded that the existing V2X mechanism is able to support these two services with new standardized PQIs over PC5 interface at least in FR1 with maximum bandwidth allocation.

Answer to Q2): Based on RAN1 performance evaluation, it is concluded that the maximum MDBV value that can be supported by using the existing V2X mechanism is at least 36,897 bytes per 2ms in FR1 with maximum bandwidth allocation.

Appendix: Simulation assumptions

Simulation assumptions used for performance evaluation of Q1

| Carrier frequency | FR1 |
| --- | --- |
| Subcarrier spacing | 30 kHz |
| Antenna configuration | 2Tx / 2Rx |
| SL bandwidth allocation | 12 PRBs (Table 1; New value #4) 270 PRBs (Table 2; New value #1) |
| Modulation | QPSK (Table 1; New value #4) 16QAM and 64QAM (Table 2; New value #1) |
| TB size | 256 bits (Table 1; New value #4) 160,000 bits (Table 2; New value #1) |
| Number of layers | 1 layer (Table 1; New value #4) 2 layers (Table 2; New value #1) |
| Simulated slots | 1,500,000 slots (Table 1; New value #4) 50,000 slots (Table 2; New value #1) |
| Maximum Tx per TB | 4 |
| Channel model | CDL model for V2X urban LOS |

Simulation assumptions used for performance evaluation of Q2

| Carrier frequency | FR1 |
| --- | --- |
| Subcarrier spacing | 30 kHz |
| Antenna configuration | 2Tx / 2Rx |
| SL bandwidth allocation | 270 PRBs |
| Modulation | 64QAM and 256QAM |
| TB size | Varies according to MCS level |
| Number of layers | 2 layers |
| Simulated slots | 20,000 slots |
| Maximum Tx per TB | 4 |
| Channel model | CDL model for V2X urban LOS |