3GPP TSG-RAN WG1 Meeting #103-e	R1-2008234

e-Meeting, October 26 – November 13, 2020

Title:	[Draft] Reply LS on UE capability for V2X

Response to:	R2-2008350

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	OPPO [to be RAN1]

To:	RAN2

Cc:	RAN4

Contact Person:

#### Name:	Kevin Lin

Tel. Number:

E-mail Address:	Kevin.Lin@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 thank RAN2’s LS in R2-2008350 and would like to provide our answer to the following question.

Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1

In RAN2’s understanding, network can configure sidelink parameters even for “a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1” (e.g., n47) from another band with Uu deployed in a cross-carrier manner, in “operator managed” geographical area(s) (as defined in TS 23.285 for LTE-V2X, and in TS 23.287 for NR-V2X).

Question: RAN2 respectfully asks RAN1 to provide feedback on the above NOTE: whether it implies from UE perspective that the cross-carrier configuration by NR Uu for a band “indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1” (e.g., n47) is not required to be supported by a UE in NR sidelink”?

Answer: In order to support NR Uu configuration in a cross-carrier manner for bands indicated with only the PC5 interface, the note will be updated in the next version of RAN1 UE features list for NR and LTE as “Note: configuration by NR Uu is not required to be supported in band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 in a non-operator managed geographical area.”

In addition, the followings will also be updated in FG 15-2:

- the note in the Notes column “This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise”, and

- the Mandatory/Optional column “Optional with capability signalling. For UE supports NR sidelink in licensed spectrum where gNB is defined or managed, UE must indicate this FG is supported.”

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above responses into account in their future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#104-e	25 January - 05 February 2021   	E-Meeting

RAN1#104b-e	12-20 April 2021		E-Meeting