3GPP TSG-RAN WG1 Meeting #103-e	R1-2008238

e-Meeting, October 26 – November 13, 2020

Agenda item:	8.11.1

Title:	Discussion on evaluation methodology for SL power saving

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

During the RAN1#102-e meeting, the evaluation methodology of R17 SL for power saving was discussed. Some agreements of reference configuration for power consumption model, scaling factor for adaption and power consumption level in each slot were achieved.

In this contribution, we continue to discuss the remaining details of evaluation methodology for SL power saving.

2. Discussions

For the baseline of evaluation, the following assumption was agreed:

Agreements: - For evaluation, the followings are baseline - 2 RX APs - 1 TX AP - 40 MHz for SL BWP size - Note that parameters or cases other than baseline is not precluded for evaluation, and companies are encouraged to provide the assumptions in details.

In our understanding, the baseline is used to unify the simulation assumption of all companies so that the simulation result of each company can be compared with each other. Therefore, we should put simulation assumptions and parameters into this baseline as much as possible to make it more concrete. At least, the traffic model, the number of TX UEs and the simulation environment ought to be included in this baseline. The profile made in RAN1#94bis in the following can be as a reference.

|  | Unicast | Multicast | Broadcast | Mixture |
| --- | --- | --- | --- | --- |
| Sidelink frequency (GHz) | 6, 30 | 6, 30 | 6, 30 | 6, 30 |
| Traffic models | Periodic: Medium intensity; [50] ms inter-packet arrival, [50]% vehicles generate packets. Aperiodic: Medium intensity, 100% vehicles generate packets. Periodic and aperiodic traffic are simulated separately. | Periodic: Medium intensity; [50] ms inter-packet arrival, [50]% vehicles generate packets. Aperiodic: Medium intensity, 100% vehicles generate packets. Periodic and aperiodic traffic are simulated separately. | Periodic: Medium intensity; [50] ms inter-packet arrival, [50]% vehicles generate packets Aperiodic: Medium intensity, 100% vehicles generate packets. Periodic and aperiodic traffic are simulated separately. | 33%, 33%, 34% vehicles generate unicast, multicast, broadcast packets, respectively. For each traffic type, 50% is periodic and 50% is aperiodic. Periodic: Medium intensity; 100 ms inter-packet arrival Aperiodic: Medium intensity |
| Simulation environment, UE drop and mobility | Highway: Option A Urban: Option A | Highway: Option A Urban: Option A | Highway: Option A Urban: Option A | Highway: Option A Urban: Option A |
| Number of Tx/Rx antenna elements for vehicle UE* | 2Tx/4Rx for 6 GHz FFS for 30 GHz | 2Tx/4Rx for 6 GHz FFS for 30 GHz | 2Tx/4Rx for 6 GHz FFS for 30 GHz | 2Tx/4Rx for 6 GHz FFS for 30 GHz |
| Antenna model for vehicle UE | Option 1 | Option 1 | Option 1 | Option 1 |
| Channel model | As defined | As defined | As defined | As defined |
| SL simulation bandwidth (MHz) | 20 MHz for 6 GHz 100 MHz for 30 GHz | 20 MHz for 6 GHz 100 MHz for 30 GHz | 20 MHz for 6 GHz 100 MHz for 30 GHz | 20 MHz for 6 GHz 100 MHz for 30 GHz |

Proposal 1: The simulation profile in RAN1#94bis should be as a reference in addition to the baseline achieved during RAN1#102-e.

Besides, both TX power of 0 dBm and 23 dBm are allowed during the discussion of last meeting. In one simulation of Mode 2, if different UEs use different TX powers, the performance of Mode 2 algorithm will decrease. For example, UE 1 with 23 dBm considers the resource used by UE 2 with 0 dBm as available during sensing. If these two UEs use a same resource, UE 2 will suffer a severe interference because of the high TX power of UE 1. Therefore, we suggest that only one type of TX power is utilized in one system level simulation. Otherwise, it is hard to distinguish that the degraded performance is due to the proposed power saving scheme of Mode 2 or the difference of TX powers.

Proposal 2: Either 0 dBm or 23 dBm is used during one time of simulation in FR1.

For power consumption level of each UE behaviour, many agreements have been reached in last meeting, which include:

Agreements: - For power consumption level, - Reuse three states of “Sleep” specified in TR38.840 including transition time/energy consumption - (Working assumption) For “PSCCH/PSSCH RX”, - In non-PSFCH-slot (i.e., the number of PSCCH/PSSCH symbols is 13), - the power consumption level is the same as that of “PDCCH+PDSCH” - For power consumption level of “PSCCH/PSSCH TX” - In non-PSFCH-slot (i.e. the number of PSCCH/PSSCH symbols is 13), - the power consumption level is the same as that of “UL” for long PUCCH or PUSCH - For power consumption level of “1st SCI/2nd SCI RX”, - the power consumption level is [0.7]* power consumption level of “PSCCH/PSSCH RX” - For power consumption level of “PSFCH TX”, - the power consumption level is [0.3]*power consumption level of “UL” for long PUCCH or PUSCH - (Working assumption) For power consumption level of “PSFCH RX”, - the power consumption level is power consumption level of “PDCCH-only” for cross-slot scheduling

During the last meeting, some companies propose to define the power consumption of “PSCCH/PSSCH TX or RX + PSFCH TX or RX”. In our understanding, this power consumption level can be calculated on basis of each component of it in order to reduce the difficulty and complexity of simulation. For example, the power consumption of “PSCCH/PSSCH TX+PSFCH TX” is equal to the power consumption of “PSCCH/PSSCH TX” plus the corresponding consumption of “PSFCH TX”. Furthermore, the above consumption level is appropriate for both FR1 and FR2 in our view.

Proposal 3: No additional definition of power consumption level for “PSCCH/PSSCH TX or RX + PSFCH TX or RX” is needed.

3. Conclusion

In this contribution, we discussed and provided our views and proposals on updating the evaluation methodology to include power saving aspect of NR sidelink operation. In summary, we have the following proposals.

Proposal 1: The simulation profile in RAN1#94bis should be as a reference in addition to the baseline achieved during RAN1#102-e.

Proposal 2: Either 0 dBm or 23 dBm is used during one time of simulation in FR1.

Proposal 3: No additional definition of power consumption level for “PSCCH/PSSCH TX or RX + PSFCH TX or RX” is needed.

4. References

[1]	Draft Report of 3GPP TSG RAN WG1 #102-e v0.2.0

[2] TR 37.885 v15.3.0 “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”, (2019-06)

[3]	TR 36.843 v12.0.1 “Study on LTE device to device proximity services”, (2014-03)

[4]	TR 38.840 v16.0.0 “Study on User Equipment (UE) power saving in NR”, (2019-06)