3GPP TSG RAN WG1 Meeting #103-e					          R1-2008287

e-Meeting , October 26th – November 6th, 2020

Agenda item:	8.7.1.1

Source:	Panasonic

Title:	On Potential Paging Enhancement

Document for:	Discussion/Decision

# Introduction

In the RAN1 #102e meeting, the agreements [1] regarding potential paging enhancement for power saving in the appendix were concluded.

To facilitate the discussion, this contribution provides some preliminary analysis for different paging enhancement schemes by using the agreed evaluation assumption and methodology.

# Paging enhancement

## Early indication

The principle of early paging indication is to inform UE before a target PO whether to start monitor the PDCCH occasions configured for paging. Hence, by this scheme, UE needs to wake up to monitor this early indication in the configured occasions in advance. If the indication is negative, which means there will be no paging for the UE, UE isnot required to monitor the upcoming PO. If the indication is positive, which means there will be paging PDCCH sent by gNB, UE shall continue monitor the PO.

As an intuitive observation, this indication brings no substantial or definite power saving benefit if looking at below two cases.

- In case there is no paging coming, the UE anyway needs to wake up to monitor the early indication, which is a similar operation with the legacy UE behavior of monitoring PO. But the active time could be potentially shorter.

- On the other hand, if there is paging in the upcoming PO, UE needs to wake up earlier and stay active in longer duration than legacy behavior.

Therefore, the optimization room for power saving may come from the case without paging. If UE active time could be reduced than normal paging monitoring procedure, it is possible to obtain some power saving gain. Based on the agreed evaluation, the analysis/calculation in the below tables on the early indication is provided, for low, medium and high SINR, respectively. In addition, the following assumptions are noted:

- As the power consumption of PDCCH-only (PPDCCH) and SSB/CSI-RS processing (PSSB) are same and both 50 unit, the calculation below does not distinguish the DCI-based, RS-based or sequence-based early indication.

- The early indication is assumed at the slot right before the PO. This means the result would be more optimized for the case when paging indication is positive.

Table.1 Power consumption calculation for early indication before PO. 3 SS bursts are used before PO for low SINR

| Baseline UE paging monitoring and power consumption |  |  | UE paging monitoring with early indication and power consumption |  |  |
| --- | --- | --- | --- | --- | --- |
| UE operations in a paging cycle | Time duration (ms) | Energy contribution | UE operations in a paging cycle | Time duration (ms) | Energy contribution |
| SSB processing | 2 | PSSB * 2 | SSB processing | 2 | PSSB * 2 |
| Light sleep | 18 | PLS * 18 + 100Note1 | Light sleep | 18 | PLS * 18 + 100Note1 |
| SSB processing | 2 | PSSB * 2 | SSB processing | 2 | PSSB * 2 |
| Light sleep | 18 | PLS * 18 + 100 Note1 | Light sleep | 18 | PLS * 18 + 100 Note1 |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 | SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 |
| Light sleep | 8 | PLS * 8 + 100 Note1 | Light sleep | 7 | PLS * 7 + 100 Note1 |
|  |  |  | Paging early indication detection | 1 | PPDCCH or PSSB |
| PO reception | 4 | (PPDCCH * (1 – RG) + PPDCCH+PDSCH * RGNote4) * 4 | PO reception or skipping depending on the indication | 4 | (PPDCCH+PDSCH * RGNote4 + PLS * (1 – RG) ) * 4 |
| Light sleep | 6 | PLS * 6 + 100 | Light sleep | 6 | PLS * 6 + 100 |
| Switch to another frequency layer | 0.5 | PMS * 0.5 | Switch to another frequency layer | 0.5 | PMS * 0.5 |
| Inter-freq. RRM measurement | 5 | (Pinter, search+meas * [1/4]Note2 + Pinter, meas-only * (1 – [1/4] Note2) ) * 5 | Inter-freq. RRM measurement | 5 | (Pinter, search+meas * [1/4]Note2 + Pinter, meas-only * (1 – [1/4] Note2) ) * 5 |
| Switch back to serving frequency | 0.5 | PMS * 0.5 | Switch back to serving frequency | 0.5 | PMS * 0.5 |
| Deep sleep | 1214 | PDS * 1214 + 450Note5 | Deep sleep | 1214 | PDS * 1214 + 450Note5 |
| (Total) | 1280 | 4117 + 280*RG | (Total) | 1280 | 4027 + 400*RG |
| Power saving gain = (90 - 120 * RG) / (4117 + 280*RG) |  |  |  |  |  |
| Note 1: Additional transition energy (relative power * ms) for light sleep as specified in Table 19 of TR 38.840 Note 2: Cell search rate for intra/inter-frequency RRM measurement Note 3: Scaling convention to combine two different types of UE operations as in Section 8.1.3 of TR 38.840 Note 4: RG is the paging rate to the UE group Note 5: Additional transition energy (relative power * ms) for deep sleep as specified in Table 19 of TR 38.840 |  |  |  |  |  |

Table.2 Power consumption calculation for early indication before PO. 2 SS bursts are used before PO for medium SINR

| Baseline UE paging monitoring and power consumption |  |  | UE paging monitoring with early indication and power consumption |  |  |
| --- | --- | --- | --- | --- | --- |
| UE operations in a paging cycle | Time duration (ms) | Energy contribution | UE operations in a paging cycle | Time duration (ms) | Energy contribution |
| SSB processing | 2 | PSSB * 2 | SSB processing | 2 | PSSB * 2 |
| Light sleep | 18 | PLS * 18 + 100 Note1 | Light sleep | 18 | PLS * 18 + 100 Note1 |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 | SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 |
| Light sleep | 8 | PLS * 8 + 100 Note1 | Light sleep | 7 | PLS * 7 + 100 Note1 |
|  |  |  | Paging early indication detection | 1 | PPDCCH or PSSB |
| PO reception | 4 | (PPDCCH * (1 – RG) + PPDCCH+PDSCH * RGNote4) * 4 | PO reception or skipping depending on the indication | 4 | (PPDCCH+PDSCH * RGNote4 + PLS * (1 – RG) ) * 4 |
| Light sleep | 6 | PLS * 6 + 100 | Light sleep | 6 | PLS * 6 + 100 |
| Switch to another frequency layer | 0.5 | PMS * 0.5 | Switch to another frequency layer | 0.5 | PMS * 0.5 |
| Inter-freq. RRM measurement | 5 | (Pinter, search+meas * [1/4]Note2 + Pinter, meas-only * (1 – [1/4] Note2) ) * 5 | Inter-freq. RRM measurement | 5 | (Pinter, search+meas * [1/4]Note2 + Pinter, meas-only * (1 – [1/4] Note2) ) * 5 |
| Switch back to serving frequency | 0.5 | PMS * 0.5 | Switch back to serving frequency | 0.5 | PMS * 0.5 |
| Deep sleep | 1234 | PDS * 1234 + 450Note5 | Deep sleep | 1234 | PDS * 1234 + 450Note5 |
| (Total) | 1280 | 3577 + 280*RG | (Total) | 1280 | 3487 + 400*RG |
| Power saving gain = (90 – 120* RG) / (3577 + 280*RG) |  |  |  |  |  |
| Note 1: Additional transition energy (relative power * ms) for light sleep as specified in Table 19 of TR 38.840 Note 2: Cell search rate for intra/inter-frequency RRM measurement Note 3: Scaling convention to combine two different types of UE operations as in Section 8.1.3 of TR 38.840 Note 4: RG is the paging rate to the UE group Note 5: Additional transition energy (relative power * ms) for deep sleep as specified in Table 19 of TR 38.840 |  |  |  |  |  |

Table.3 Power consumption calculation for early indication before PO. 1 SS burst is used before PO for high SINR

| Baseline UE paging monitoring and power consumption |  |  | UE paging monitoring with early indication and power consumption |  |  |
| --- | --- | --- | --- | --- | --- |
| UE operations in a paging cycle | Time duration (ms) | Energy contribution | UE operations in a paging cycle | Time duration (ms) | Energy contribution |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 | SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 |
| Light sleep | 8 | PLS * 8 + 100 Note1 | Light sleep | 7 | PLS * 7 + 100 Note1 |
|  |  |  | Paging early indication detection | 1 | PPDCCH or PSSB |
| PO reception | 4 | (PPDCCH * (1 – RG) + PPDCCH+PDSCH * RGNote4) * 4 | PO reception or skipping depending on the indication | 4 | (PPDCCH+PDSCH * RGNote4 + PLS * (1 – RG) ) * 4 |
| Deep sleep | 1266 | PDS * 1266 + 450Note5 | Deep sleep | 1266 | PDS * 1266 + 450Note5 |
| (Total) | 1280 | 2371.5 + 280*RG | (Total) | 1280 | 2281.5 + 400*RG |
| Power saving gain = (90 – 120* RG) / (2371.5 + 280*RG) |  |  |  |  |  |
| Note 1: Additional transition energy (relative power * ms) for light sleep as specified in Table 19 of TR 38.840 Note 2: Cell search rate for intra/inter-frequency RRM measurement Note 3: Scaling convention to combine two different types of UE operations as in Section 8.1.3 of TR 38.840 Note 4: RG is the paging rate to the UE group Note 5: Additional transition energy (relative power * ms) for deep sleep as specified in Table 19 of TR 38.840 |  |  |  |  |  |

For the baseline paging monitoring procedure, the components of power consumption are summarized in Table.4 below. The group paging rate RG = 0.1 is assumed.

Table 4. Baseline power consumption components for measurement, sleep and PO monitoring, assuming RG = 0.1

| Power components for Low/Medium/High SINR cases | Power consumption for measurement | Power consumption for sleep (incl. transition) | Power consumption for PO monitoring |
| --- | --- | --- | --- |
| Low | 853 (20.6%) | 3064 (73.9%) | 228 (5.5%) |
| Medium | 753 (20.9%) | 2624 (72.8%) | 228 (6.3%) |
| High | 195.5 (8.1%) | 1976 (82.4%) | 228 (9.5%) |

From the baseline power consumption for paging, it can be observed that:

- In the baseline case, the most power consumption component is the sleep, including deep and light sleep.

- The PO monitoring contributes less than 10% of the power consumption, even in the high SINR case.

- In the low and medium SINR case, the measurement and/or SSB processing contributes a solid portion (larger than 20%).

Observation 1: The optimization space to reduce paging monitoring is higher for high SINR case.

Observation 2: To reduce the power consumption for measurement/SSB processing may potentially provide substantial power saving gain due to the large portion it contributes.

In addition, with the above calculation in Table. 1, 2 and 3, the below chart in the Figure. 1 is also presented to understand the performance of the paging early indication.

Figure.1 Paging early indication performance comparison for low, media and high SINR case

It can be observed that:

- Paging early indication provides more power saving gain for high SINR case than for low SINR case, as the contribution from SSB processing is smaller.

- Power saving gain diminishes with larger group paging rate and disappears when group paging rate is higher than 0.75.

- Even when the group paging rate is close to zero, the power saving gain is still quite marginal.

Observation 3: Paging early indication can only provide slight power saving gain. Even in the high SINR case and with low group paging rate, which provides the best performance, the gain is still less than 4%.

Discussion for clarification of the power model on beam-related operation

Regarding the beam tracking for paging monitoring, UE could try to monitor all the occasions associated with more than one SSB index, as specified in TS38.304 section 7.1 paging:

- “In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.”

Although in the above numerical analysis, 4 slots PO reception are assumed for all cases, different SINR condition may lead to different operations. In good SINR case, it could be reasonable implementation that UE receives only one PO corresponding to the best quality SSB. However, in the medium and low SINR cases, soft combining or multiple reception of paging DCIs is also possible operation when the best beam of SSB is not sufficient. Hence, the number of PO reception slots could be differentiated between different cases.

Observation 4: For beam-sweeping operation, different PO monitoring durations could be different among different SINR conditions, which would impact on the baseline power consumption performance.

In our calculation, SSB measurement takes 2 slot. The current SSB design could support 4 SSBs with 2 slots which can support 4 beams with 15 kHz SCS. However, in our understanding, the current power model of PSSB = 50 is scaled from PSSB = 100 with 100 MHz bandwidth, which is not clear about one or two SSB is assumed. We tend to think only one SSB is received as it is close to the value of PPDCCH-only = 100, which assumes 2-symbol PDCCH region.

Observation 5: If beam-sweeping operation needs to be investigated for paging, the current power model needs to be clarified and updated.

## Sub-grouping for paging

The motivation of studying the sub-grouping for paging is to avoid unnecessary paging reception, i.e. in the case that CRC check for PDCCH is passed but there is no paging record in the PDSCH. Therefore, in terms of UE operation, it is equivalent to reducing the group paging rate. It can be seen from Table. 1-3 that the total energy for the three cases increase linearly with larger RG:

- Total energy for low SINR case = 4117 + 280*RG

- Total energy for medium SINR case = 3577 + 280*RG

- Total energy for high SINR case = 2371.5 + 280*RG

It is also obvious that the high SINR case may benefit more from sub-grouping than other cases. However, as the constant factor contributes the most part of power consumption, the power saving by reducing the RG may be quite limited. For example, the total energy for high SINR case decreases from 2399.5 to 2385.5, which has less than 1% power saving gain, if RG is reduced to 5% from 10%. Even in high traffic arrival rate scenarios, e.g. when RG is reduced to 10% from 50%, the power saving gain is only 4.67%. In some extreme cases such as the group paging rate is reduced by more than 50%, the power saving gain could be more obvious. Thus,

Observation 6: Except the extreme case that group paging rate can be dramatically reduced, the power saving gain solely provided by sub-grouping is marginal.

Therefore, from the numerical analysis based on the assumptions in Table.1-3 and all the above observations, we have the following observation:

Observation 7: To apply both paging early indication and sub-grouping together may provide more justified power saving gain.

Proposal 1: Support of paging early indication with sub-grouping information should be studied.

In the previous meeting, we achieved the below agreement.

Agreements: For potential paging enhancements, RAN1 to study the following candidate schemes: - Paging early indication before a target PO to indicate UE whether to monitor PDCCH scrambled with P-RNTI at the PO. Potential candidate indication methods include - DCI-based indication, e.g., based on - Extending existing DCI format 1_0 or 2_6 - New DCI format - RS-based or sequence-based indication, e.g., based on TRS/CSI-RS or SSS - Sub-grouping for paging, based on - Legacy paging DCI - Paging early indication - Additional reception occasions in time/frequency domain - Multiple P-RNTIs - Cross-slot scheduling for paging PDSCH - Other proposal is not precluded

It can be seen that RS/sequence-based paging early indication is a candidate scheme. In addition, as mentioned in Observation 2, the reduction of the power consumption from SSB measurement/processing is beneficial for the power saving of overall procedure. Thus CSI-RS occasion(s) near the PO is under discussion for such enhancement. Our companion contribution addresses it in more detailed level. Therefore, in our opinion, the enhancement of RS-based paging early indication and additional CSI-RS occasion(s) before PO can be optimized together for more efficient UE detection, e.g.

- The detection of both could be in the same slot for shorter UE active duration to save power

- Only RS detection operation is needed before paging PDCCH monitoring, which would potentially simplify the UE complexity and save power

The sub-grouping index could be associated with UE ID and the CSI-RS time/frequency domain resource and used sequence. All such information can be configured in SIB.

Proposal 2: Sub-grouping information could be carried in the RS-based paging early indication, which is associated with UE ID.

After UE detects the paging early indication and checks whether it belongs to the sub-group intended for paging, UE may choose to continue or stop the paging monitoring procedure in this paging cycle. One may argue that the RS-based sub-grouping indication is not flexible enough to accurately point to the paged UE. It is true that RS-based design is not friendly to support relatively large sub-group numbers in case the UE load is high in a PO. In this case, the further more dedicated sub-grouping information can be indicated by paging DCI.

Proposal 3: sub-grouping information can also be carried in the paging DCI for more refined sub-grouping indication.

## Cross-slot scheduling

As can be seen in the agreed simulation assumption, the power consumption is not distinguishable between same slot and cross-slot scheduling for paging PDCCH. The reason is the assumed bandwidth is 20 MHz for paging and the scaled power for PDCCH-only for same slot scheduling is 50, which is close to micro-sleep power consumption (45). Therefore, the power saving gain from the cross-slot scheduling can not be reflected by evaluation so far.

However, for other bandwidth larger than 20 MHz, cross-slot scheduling benefits are more obvious. Also from UE implementation point of view, having a minimum gap between paging PDCCH and PDSCH is more friendly for applying more relaxed processing timeline. It is also beneficial for applying sub-grouping in paging DCI, as more relaxed PDCCH processing timeline gives UE more time to determine whether to continue buffering and decoding paging PDSCH.

In general, as shown by the numerical analysis, applying only one of the solutions will not bring so much benefit. Applying all three solutions together may have more justified power saving gain in certain scenarios.

Proposal 4: Cross-slot scheduling for paging PDSCH should be supported together with paging early indication and sub-grouping.

# Conclusion

In this contribution, analysis on different paging enhancement schemes are provided. Based on that, the following conclusions are highlighted:

Observation 1: The optimization space to reduce paging monitoring is higher for high SINR case.

Observation 2: To reduce the power consumption for measurement/SSB processing may potentially provide substantial power saving gain due to the large portion it contributes.

Observation 3: Paging early indication can only provide slight power saving gain. Even in the high SINR case and with low group paging rate, which provides the best performance, the gain is still less than 4%.

Observation 4: For beam-sweeping operation, different PO monitoring durations could be different among different SINR conditions, which would impact on the baseline power consumption performance.

Observation 5: If beam-sweeping operation needs to be investigated for paging, the current power model needs to be clarified and updated.

Observation 6: Except the extreme case that group paging rate can be dramatically reduced, the power saving gain solely provided by sub-grouping is marginal.

Observation 7: To apply both paging early indication and sub-grouping together may provide more justified power saving gain.

Proposal 1: Support of paging early indication with sub-grouping information should be studied.

Proposal 2: Sub-grouping information could be carried in the RS-based paging early indication, which is associated with UE ID.

Proposal 3: sub-grouping information can also be carried in the paging DCI for more refined sub-grouping indication.

Proposal 4: Cross-slot scheduling for paging PDSCH should be supported together with paging early indication and sub-grouping.

# Reference

[1] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e

# Appendix: Conclusions from the RAN1#102e meeting

Agreements:

For study of Rel-17 paging enhancement, the following are assumed as a baseline for FR1 and FR2:

- Reference configuration for FR1/FR2 as specified in Section 8.1.1/8.1.2 of TR 38.840

  - Note: the setting for some PDSCH parameters may not be applicable for RedCap UEs

- Baseline paging cycle length: [1.28] second

- SS burst related assumptions:

  - 20 ms periodicity

  - 2 ms duration for serving cell RRM measurement, which can overlap with the one for synchronization before PO

  - FFS time/frequency tracking

- Measurement related assumptions:

  - 20 ms SMTC periodicity

  - 2 ms SMTC window for intra-frequency RRM measurement, assuming synchronized deployment

  - [5 ms SMTC window and 6 ms measurement gap for inter-frequency RRM measurement]

    - Note: RAN4 requirement assumes one frequency layer per measurement gap, and 0.5 ms is assumed for switch in/out a frequency layer

- Note: the inclusion of potential TRS/CSI-RS occasions can be considered

Agreements:

The following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode

- FFS: FR2 power consumption model for idle/inactive mode operations

| - Power State | Relative Power (FR1 reference from TR 8438.840) | Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 1 |
| Light Sleep (PLS) | 20 | 20 |
| Micro sleep (PMS) | 45 | 45 |
| PDCCH-only (PPDCCH) | 100 | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 (synchronization or serving cell measurement) | 50 |
| Intra-frequency RRM measurement (Pintra) |  150 (synchronous case, N=8, measurement only; Pintra, meas-only)  200 (combined search and measurement; Pintra, search+meas) |  [60] (synchronous case, N=8, measurement only; Pintra, meas-only)  [80] (combined search and measurement; Pintra, search+meas) |
| Inter-frequency RRM measurement (Pinter) | · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer | · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |  |

Agreements:

Group paging rate of 10% is assumed for the evaluation of Rel-17 paging enhancement

- FFS: Another group paging rate > 10%

- Note: If UE sub-grouping is applied, the sub-group paging rate can be reduced w.r.t. the total sub-group number for a PO

Agreements:

For the study on paging enhancements to reduce unnecessary paging reception, the following metrics are considered:

- UE power saving gain (relative to a given feature or overall)

- Impact to UE paging detection probability

  - FFS: Link level simulation assumptions

- System impact, including

  - Additional resource overhead and its implications

  - Impact to Rel-15/Rel-16 idle/inactive-mode UEs and connected-mode UEs

  - Impact to other legacy functionalities, including SI change and ETWS indication

  - [Note: NW energy consumption evaluation is not precluded]

Agreements:

- For the study of paging enhancement, 1, 2, or 3 SS burst processing is assumed before PO

  - Note: in choosing one or more values (1, 2, or 3) for the evaluations, companies to provide justification

Agreements

Send a LS with for evaluation methodology updates related to paging enhancement(s) to RAN2

- Draft LS in R1-2007355, which is endorsed, with final LS in R1-2007356 -> (typo fix) R1-2007425

Agreements:

For the study on paging enhancements, the following LLS assumptions are considered.

- For investigating the residual frequency error after one or multiple SS burst processing, at least -6 dB SNR should be considered

| Parameters | Values | Note |
| --- | --- | --- |
| Carrier Frequency | 4GHz (FR1) |  |
| Transmission BW | 20MHz (FR1) |  |
| Antenna Configuration | 1/2TX and 2/4 RX | Companies to report |
| Channels | TDL-C or CDL-C 300 ns delay spread 100 Hz Doppler shift |  |
| Frequency error | Uniform distribution in the range [-X, +X]ppm Companies to report the utilized X value(s) with justification | Modelled at the input of the considered paging channel/early indication design(s) |
| Paging PDCCH configuration | AL8, 41 info + 24 CRC bits, REG bundle size 6 | Companies to report additional setting(s), e.g., CORESET duration, etc. |
| Paging PDSCH configuration | Mapping type A, MCS 0, 48 PRB, TB scaling 1, DMRS type 1 with 2 additional DMRS | Companies to report additional setting(s), e.g., other TB scaling factor, other DMRS type(s), etc. |
| Paging early indication design(s) | Companies to report |  |

Agreements:

For potential paging enhancements, RAN1 to study the following candidate schemes:

- Paging early indication before a target PO to indicate UE whether to monitor PDCCH scrambled with P-RNTI at the PO. Potential candidate indication methods include

  - DCI-based indication, e.g., based on

    - Extending existing DCI format 1_0 or 2_6

    - New DCI format

  - RS-based or sequence-based indication, e.g., based on TRS/CSI-RS or SSS

- Sub-grouping for paging, based on

  - Legacy paging DCI

  - Paging early indication

  - Additional reception occasions in time/frequency domain

  - Multiple P-RNTIs

- Cross-slot scheduling for paging PDSCH

- Other proposal is not precluded