3GPP TSG RAN WG1 Meeting #103-e	R1-2008303

e-Meeting, October 26th – November 13th, 2020

Agenda item:		7.2.5

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 URLLC intra-UE prioritization enhancements (incl. draft CRs)

Document for:		Decision

# 1	Introduction

In this contribution, we summarize our input to the maintenance discussions on intra-UE prioritization/cancellation enhancements of Rel-16 URLLC & IIoT enhancements, with 3 independent draft CRs to 38.213 provided in the relevant sections below.

Section 2 includes the discussion of multiplexing and cancellation order between overlapping channels with different priorities. Section 3 discusses the remaining open issue when the UL channels are overlapping with semi-static DL symbols/SSB. Finally, in Section 4 we clarify the UE behaviour when PUCCH carrying positive SR is overlapping with high-priority PUSCH without UL-SCH.

# 2 Discussion on intra-UE multiplexing/cancellation order

One of the identified issues related to intra-UE multiplexing/cancellation order was identified and discussed in RAN1#102-e meeting [R1-2007432]. The main issue is whether the LP PUCCH and its related UCI should be cancelled right after the detection of any DCI scheduling overlapping HP channels. Two alternatives have been discussed extensively and as the final outcome, the following working assumption was agreed:

Working assumption

- Multiplexing/overriding/etc. is performed similar to Rel.15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc., are performed before cancellation.

- A UE cancels the transmission of a LP channel including any intermediate scheduled LP transmission that does not overlap with any LP channel, if any DCI schedules an overlapping HP transmission with the LP channel, before performing multiplexing/overriding HP channels if any.

- Multiplexing/overriding of HP channels is performed as if LP channels do not exist.

- A final HP channel is prioritized if it overlaps with a final LP channel, after performing multiplexing of HP channels

The above working assumption was implemented in the latest spec updates in TS 38.213, sec. 9 as can be seen from the quoted text below:

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability

However, in the above highlighted text in gray, it’s not clear when the UE should perform the cancellation of low-priority channel due to the reception of a PDCCH scheduling a high-priority channel that overlaps with the low-priority channel, i.e. whether the UE should cancel the low-priority channel before or after performing high-priority channels multiplexing triggered by the (newly) scheduled high-priority channel, if any. From the Working Assumption above, it’s clear that the UE should perform high-priority channels multiplexing triggered by the newly scheduled high-priority channel, if any, after cancelling the transmission of low-priority channel.

Proposal 1: Endorse the following draft CR to TS 38.213 Sec. 9 on intra-UE multiplexing and cancellation order (changes in green to be shown as track changes in the final CR)

## Draft CR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on intra-UE multiplexing and cancellation order |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2020-10-XX |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specifications on intra-UE multiplexing and cancellation order needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE should perform cancellation of low-priority, cancelled by a high priority channel scheduled by a PDCCH, before and after performing high-priority channels multiplexing triggered by the scheduled high-priority channel, if any. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior on intra-UE multiplexing and cancellation order, which could result in a different understanding between the UE and the network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

# 9	UE procedure for reporting control information

<omitted text>

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. Then,

-	if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission

-	if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission

where

-	the overlapping is applicable before or and after resolving overlapping among channels of larger priority index, if any, triggered by the first PUCCH or the first PUSCH scheduled by a DCI format in a PDCCH reception, as described in Clause 9.2.5

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability

<omitted text>

# 3 Discussion on overlapping between UL channels and semi-static DL symbols/SSB symbols

From FL summary on “Scheduling and HARQ” [R1-2007429], another issue which was agreed to be discussed in RAN1#102-e meeting but no conclusion is the overlapping case between UL channels and the semi-static DL symbols.

Firstly, we would like to point out the agreement from RAN1#101e:

Agreement:

After the UE determines the overlapping PUCCH or PUSCH for multiplexing/prioritization, the UE cancels the PUCCH or PUSCH that has overlapping with semi-static configured DL symbols or SSB symbols, and then the multiplexing/prioritization is performed among the non-cancelled overlapping transmissions.

Based on the agreement above, the following three steps have been taken as the starting point for discussion in RAN1#102-e meeting:

- Step 1: A UE follows Rel-15 behaviors for any intermediate procedure to determine the overlapping PUCCH or PUSCH for multiplexing/prioritization

- Step 2: UE cancels the ones that collides with semi-static DL symbols,

- Step 3: UE performs multiplexing/prioritization among the non-cancelled overlapping channels.

However, as indicated in [R1-2007429] and also in the email discussions, different companies have different understanding how these 3 steps are performed and what are the UE behaviours in these steps. It is of essential to classify the order of cancellation due to overlapping with semi-static configured DL symbols or SSB symbols and intra-UE multiplexing/prioritization, especially considering the agreed WA as discussed in Section 2. Our understanding is as follows.

- Step 1 is used to handle the overlapping channels with the same priority following the procedure of intra-UE multiplexing including possible multiplexing/prioritization process. This step applies to the overlapping channels with the same priority (either high-priority or low-priority channels) and the intermediate cancellation of LP channels which are overlapping with any HP transmission scheduled by DCI. Essentially this covers the first 3 steps from the WA agreed from last meeting (copied below):

  - Multiplexing/overriding/etc. is performed similar to Rel.15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc., are performed before cancellation.

  - A UE cancels the transmission of a LP channel including any intermediate scheduled LP transmission that does not overlap with any LP channel, if any DCI schedules an overlapping HP transmission with the LP channel, before performing multiplexing/overriding HP channels if any.

  - Multiplexing/overriding of HP channels is performed as if LP channels do not exist.

The possible outcome of this step is the multiplexed channel(s) resulting from the overlapping channels with the same priority (either high-priority or both high and low-priority channels).

- Then in Step 2 UE cancels all the channels which are overlapping with semi-static DL symbols/SSBs according to the procedure defined in Clause 11.1 and Clause 11.1.1 of TS 38.213. After Step 2, there is no transmission resource overlapping with semi-static DL symbols/SSB symbols anymore.

- Finally, in Step 3 UE handles the non-cancelled overlapping channels with different priorities i.e. prioritizing between low-priority channel and high-priority channel, i.e. the low-priority channels are dropped.

Proposal xx: Endorse the following draft CR to TS 38.213 Sec. 9 on intra-UE multiplexing/prioritization when the overlapping between UL channels and semi-static configured DL symbols/SSB symbols (changes as green text to be shown as track changes in the final CR):

## Draft CR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE behavior with overlapping UL channels and semi-static DL symbols/SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2020-10-XX |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specifications on intra-UE multiplexing and cancellation order needs to be clarified considering the cases where there are UL channels overlapping with semi-static DL symbols/SSB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE should determine valid resource (due to possible overlapping with semi-static DL symbols/SSB) before performing the prioritization process among channels with different priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior when UE is with overlapping UL channels and semi-static DL symbols/SSB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

# 9	UE procedure for reporting control information

<omitted text>

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. Then,

-	if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission

-	if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission

where

-	the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5. When the overlapping is applicable after resolving overlapping among channels of larger priority index, before resolving overlapping channels of different priority index, UE determines the valid transmission resource following the procedure as described in Clause 11.1 and Clause 11.1.1.

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH

-	Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs

-	if the overlapping group includes the first PUCCH

-	if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH and for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

-	if the overlapping group includes the first PUSCH

-	if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

If a UE would transmit the following channels that would overlap in time, after determining the valid transmission resource following the procedure as described in Clause 11.1 and Clause 11.1.1,

-	a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or

-	a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or

-	a first PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH and a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI report(s) without a corresponding PDCCH, or

-	a PUSCH of larger priority index with SP-CSI reports(s) without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH, or

-	a configured grant PUSCH of larger priority index and a configured PUSCH of lower priority index on a same serving cell

the UE is expected to cancel the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index.

<omitted text>

# 4 Handling overlapping between high-priority PUSCH without UL-SCH and SR (38.213 Section 9)

In the FL summary of [R1-2007033], one of the potential issues is about the scenario where high-priority PUSCH without UL-SCH collides with SR. Based on the agreement from RAN1#98bis, the priority of the A-CSI is indicated by Priority indicator within the DCI scheduling the PUSCH. As specified in TS 38.213, the SR is indicated by phy-PriorityIndex-r16 for each SR resource configuration in SchedulingRequestResourceConfig, with a priority index 0 or 1. If the UE is not provided a priority index for SR, the priority index is 0.

Following TS38.213, in case the UL channels of different priorities are overlapping, the UE does not transmit the ones of smaller priority index. However, looking at TS38.213 Section 9, related to the scenario where a PUSCH without UL-SCH overlaps with a PUCCH transmission with positive SR information, it says:

“If a UE would transmit on a serving cell a PUSCH without UL-SCH that overlaps with a PUCCH transmission on a serving cell that includes positive SR information, the UE does not transmit the PUSCH.”

There could be misunderstanding that positive SR is always transmitted independent of the PUSCH priority. On the other hand, it is worth to point out one paragraph from 38.213 Section 9 (which coming before the referred paragraph above) as shown below:

In the remaining of this Clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH.

The ambiguity might arise whether the above referred paragraph is applicable to the scenario of overlapping SR and PUSCH without UL-SCH or not since multiplexing of SR on PUSCH is not supported in NR. To make it clear, we would like to propose to update the relevant specification text.

Proposal 5: Endorse the following draft CR to TS 38.213 Sec. 9 to clarify that high priority PUSCH without UL-SCH is transmitted in case colliding with low-priority SR (added green text to be shown as changes in the final CR):

## Draft CR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE behaviour with overlapping PUCCH carrying positive SR and PUSCH without UL-SCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2020-10-XX |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With the introduction of the new Rel-16 PHY priority of PUSCH and SR, the Rel-16 behavior for UEs needs to be clarified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify, that in case overlapping with PUCCH carrying positive SR, a UE does not tranmit PUSCH without UL-SCH unless the PUSCH without UL-SCH is of a larger priority index than the positive SR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UEs behavior, when there is overlapping between positive SR and UL-PUSCH without UL-SCH and they are with different priorities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

# 9	UE procedure for reporting control information

<omitted text>

If a UE would transmit on a serving cell a PUSCH without UL-SCH that overlaps with a PUCCH transmission on a serving cell that includes positive SR information, the UE does not transmit the PUSCH, except when the PUSCH without UL-SCH is of a larger priority index than the positive SR.

<omitted text>