3GPP TSG RAN WG1 Meeting #103-e	R1-2008320

E-meeting, October 26- November 13, 2020

Agenda Item:	8.15.3

Source:	Huawei, HiSilicon

Title:	Other aspects to support IoT in NTN

Document for:	Discussion and Decision

# Introduction

In RAN#86, a new SI on NB-IoT/eTMC support for NTN was proposed and the first objective of this study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

NOTE 3: GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

In this contribution, we discuss some power-saving-related issues in IoT over NTN scenario.

# Discussion

## GNSS signal transmission

GNSS capability is assumed for IoT NTN terminal in this SI, which facilitates mobility management and physical layer procedure. For the purpose of continuous tracking, in terrestrial networks, GNSS is typically implemented as an independent module with dedicated memory and processor, thus IOT module and GNSS module can handle service requests simultaneously. To further reduce power consumption and cost of NTN-IoT devices, an integrated architecture can also be adopted with shared hardware resource between IoT module and GNSS module.

Figure 1 GNSS and IoT signal transmission

For integrated architecture, it is not a simple task to schedule GNSS and IoT module sequentially, as the GNSS search can take minutes depending on the signal strength, it may disrupt IoT-related service, e.g. paging/data transmitting, and vice versa. Thus, protocol-based scheduling is more suitable comparing with software scheduling.

Two power saving mechanisms are incorporated in NB-IoT as power saving mode (PSM) and extended discontinuous reception (eDRX). In PSM, a terminal enters a deep sleep mode by switching off most of its circuitry while staying registered to the network, and is unreachable from the network. Upon generation of UL data, it can wake up and monitor the control channel for UL grant. In eDRX, devices enter into an idle mode, deactivate monitoring the control channel for a defined period and re-enter the active state periodically to receive a paging message from the network for possible incoming data. To achieve a long battery life with NTN-IoT, its system should utilize more eDRX and PSM modes, GNSS signal can be scheduled during periods without any IoT activity.

Proposal 1: How to schedule GNSS signal transmission needs further study.

## Enhancement to power saving

It is reasonable to reuse power saving schemes in NB-IoT including PSM and eDRX, however, there is still room for further improvements. In terrestrial IoT network, both PSM/eDRX cycle consist of periodic active state and sleep state, while the state-switching depends the configured timer. In NTN, as the constellation topology is time-varying, the terminal switching to active state based on pre-configured period may not be able to access any satellite, as shown in figure 2:

Figure 2 Misalignment between terminal active-cycle and satellite visible window

Thus prediction-based enhancement can be considered with help of ephemeris data, terminals shall initiate state-switching based on both PSM/eDRX cycle and satellite trajectory predictions.

Proposal 2: How to optimize the PSM/eDRX cycle needs further study.

# Conclusions

In this contribution, we discuss some power-saving-related issues for IoT over NTN. The following proposals are presented:

Proposal 1: How to schedule GNSS signal transmission needs further study.

Proposal 2: How to optimize the PSM/eDRX cycle needs further study.

# References

RP-193235, New Study WID on NB-IoT/eMTC support for NTN

TR 38.821 V16.00, Solutions for NR to support non-terrestrial networks (NTN)