3GPP TSG RAN WG1 Meeting #103-e	R1-2008328

E-meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.3

Source:	Huawei

Title:	Corrections on PDCCH monitoring of DCI format 2_5

Document for:	Discussion and Decision

Introduction

In RAN1 #102-e, there was some discussions on PDCCH monitoring for DCI format 2_5 and the following recommendations were made [1]:

For the editors:

An editorial correction to TS 38.213 should capture that an IAB-MT monitors PDCCH candidates in Type-3-PDCCH CSS (all cell types) and USS sets for DCI formats with CRC scrambled by AI-RNTI in addition to the candidates/formats already captured in Section 10.1 of 38.213:

10.1 UE procedure for determining physical downlink control channel assignment A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets - a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or bysearchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG - a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG - a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell - a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI onthe primary cell of the MCG - a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = commonfor DCI formatswith CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI,or PS-RNTI and,only for the primary cell, C-RNTI,MCS-C-RNTI, or CS-RNTI(s), and - a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specificfor DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI,CS-RNTI(s),SL-RNTI, SL-CS-RNTI, or SL-L-CS-RNTI.

The recommendation to the editor is to add text under section 14 (instead of 10.1) of 38.213 since it is only applicable for IAB-MTs and not UEs.

Since the above correction has not been captured in the latest specification, this contribution provided a TP to capture the IAB-MT behaviour for PDCCH monitoring for DCI format 2_5.

Discussion

As agreed in RAN1 100bis-e, an IAB-MT needs to monitor DCI format 2_5 in a CSS set and a USS set if configured by higher layers.

Agreements: Confirm DCI Format 2_0 and DCI Format 2_5 can be monitored by an IAB-MT in at least a common search space. The same number of aggregation levels and candidates can be separately configured for both DCI Format 2_0 and DCI Format 2_5.

Agreements: DCI Format 2_0 is not monitored by an IAB-MT in a UE(MT)-specific search space. DCI Format 2_5 can be additionally monitored by an IAB-MT in a UE(MT)-specific search space. Signaling details (e.g. whether the configuration is in the existing UE-specific search space configuration or a new MT-specific search space configuration is left up to RAN2).

Hence, we have the following proposal

Proposal: Capture the following TP in 38.213

- 14	Integrated access-backhaul operation

*** Unchanged text is omitted ***

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the IAB-MT obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the IAB-MT detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the IAB-MT expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot. An IAB-MT monitors PDCCH candidates in a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common and a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by AI-RNTI.

Conclusions

Based on the discussion, we have the following proposal:

Proposal: Capture the following TP in 38.213

- 14	Integrated access-backhaul operation

*** Unchanged text is omitted ***

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the IAB-MT obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the IAB-MT detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the IAB-MT expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot. An IAB-MT monitors PDCCH candidates in a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common and a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by AI-RNTI.

- References

3GPP chairman’s notes RAN1#102-e

3GPP chairman’s notes RAN1#100bis-e