3GPP TSG-RAN WG1 Meeting #103-e	R1-2008344

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.8.1.2

Source:	Ericsson

Title:	Link and System Evaluation of Coverage for FR2

Document for:	Decision

# 1	Introduction

In this paper, we continue our evaluations in [4] and [5] of coverage evaluations for a set of channels in different scenarios with frequencies in Frequency Range 2 (FR2). The set is selected in the same way as for FR1 in [2] include channels used for the initial access procedure as well as when sending data and considers both uplink as well as downlink and allows to identify potential bottlenecks in terms of channels that significantly limit overall coverage. Additional results in this paper include the performance of Msg4 and the performance of PUSCH with fixed MCS and HARQ. A spreadsheet including results with the agreed link budget template is provided as an attachment. Note that methodologies need for the link budget template for MPL (maximum propagation loss) calculation including how to determine propagation margins and macro diversity gains are provided in [2].

For FR2, two different deployments, both with a carrier frequency of 30GHz, are studied, a dense urban macro deployment with base stations deployed outdoors as well as an indoor hotspot scenario with base stations deployed indoors, see also [3]. In both cases we consider TDD and 100MHz carrier bandwidth as well as advanced antenna solutions.

As discussed in [1] and [2], the evaluations involve link simulations to determine the required link quality in terms of signal-to-noise and interference ratio (SINR) that is required to reach acceptable error rates, and system level simulations that determine coverage, or probability of reaching, those SINRs.

Link level models and results are presented in Section 2, and system level models are presented in Section 3.

# 2	Link level models and results

In actual implementations, both at the base station side as well as the terminal side, various forms of beamforming solutions operating in the time domain or the frequency domain or a combination of both are available. Such solutions are sometimes referred to as analogue, digital, or hybrid solutions. Analogue implementations of time domain beamforming in the context of arrays of dual polarized element pairs, can be done for each polarization separately, whereas the processing of the two polarizations can be done in the frequency domain in digital baseband. Such a solution can be regarded as a hybrid solution.

Furthermore, link performance is expected to depend not only on the antenna configuration but also on the beamforming implementation and algorithms. This means that the bottlenecks will depend not only on the standard but also the beamforming implementation. In any case, the size of the array, in terms of the number of dual polarized elements pairs provides an upper bound on beamforming capability, and it is therefore proposed to consider base station arrays of 8x8 dual polarized antenna pairs for the indoor scenario and 16x16 dual polarized pairs for an outdoor macro scenario. At the terminal side, two panels oriented in different directions is assumed, where each panel consists of four dual polarized elements pairs.

We evaluate two high level configurations shown in Table 1, with one intended for indoor deployments and another for outdoor deployments. In both cases, a 100MHz TDD carrier using 120kHz subcarrier spacing is considered.

| - Carrier frequency | - 30GHz |  |
| --- | --- | --- |
| - Duplex | - TDD 3DL:1UL |  |
| - Bandwidth | - 1x100MHz (66PRBs) |  |
| - Subcarrier spacing | - 120kHz |  |
| - BS antenna config | - Indoor 8x8x2 (8x8 dual polarized element pairs) - Case Analogue: Time domain beamforming per polarization with 2T2R Case Digital: Frequency domain beamforming with up to 128T128R | - Outdoor 16x16x2 (16x16 dual polarized element pairs) - Case Analogue: Time domain beamforming per polarization with 2T2R Case Digital: Frequency domain beamforming with up to 512T512R |
| - UE antenna config | - Multiple 2x2x2 panels (4 dual polarized antenna pairs) with panel selection and time domain beamforming per polarization with 1T2R |  |
| - Waveform | - DL: OFDM, UL: DFT-S-OFDM |  |

Table 1 Carrier configuration for FR2

- As proposed in [1] and also discussed in [2] for FR1, the antenna capabilities are modeled partly on link level and partly on system level. Transmit and receive diversity as well as spatial multiplexing operate on fast fading time scales, and it appears natural to model them on link level. Striking a balance between complexity and accuracy while still allowing to account for up to two-layer spatial multiplexing as well as first order benefits of transmit diversity with dual polarized arrays, two transmit antennas is proposed to be used for the downlink whereas a single transmit antenna is consider for the uplink initially. On the receiver side, two antennas should be considered primarily because of the diversity and spatial multiplexing benefits.

For baseline link level simulations, we in this contribution consider 2T for downlink and 1T for uplink according to Table 2. While such evaluations are not provided here, 2 UE transmit antennas may also be considered.

| - Carrier frequency | - 30GHz |
| --- | --- |
| - gNB | - 2T2R |
| - UE | - 1T2R, [2T2R] |

Table 2 Baseline link level simulation antenna configurations

- The link level performance does not only depend on carrier frequency and antenna configurations, assumptions are also needed when it comes to the power delay profile. For simplicity, the so-called tapped delay line channel models are considered, and following for the same reasoning as in [2], TDL-C with medium correlation and 3km/h terminal speed is a starting point.  Channel models used in the link level simulations are provided in Table 3.

| - Carrier frequency | - 30 GHz |
| --- | --- |
| - Model | - TDL-A, TDL-C |
| - Correlation | - medium correlation |
| - RMS delay spread (DS) | - 30, 100 ns |
| - Terminal speed | - 3, 30 km/h |

Table 3 Link level simulation power delay profiles

## 2.1	Channels and SNR requirements

- As described in the above in order to find the bottle necks signaling both during initial access as well as uplink and downlink transmission in connected mode will be considered. Not all signals and channels will be explicitly included. More specifically note

- System information as well as paging are not explicitly included. They use the PDCCH and PDSCH and are therefore implicitly included. The same holds also messages beyond Msg3 in the access procedure.

- Error rates in the order of 10% are also considered in addition to the perhaps commonly assumed 1%, This means that the access time for the UE increases which could be acceptable when in poor coverage. Selection of these error rates can have a strong impact on both the absolute and relative coverage of channels and further consideration of this can be found in [6].

- In a practical (analogue) implementation with time domain beamforming, a balance between coverage, latency and data rates needs to be found. For example, to transmit an SSB in certain direction may block the entire carrier for data transmission to user in another directions. Similarly, to receive an UL PRACH with a duration of twelve symbols may prevent the reception of PUSCH and PUCCH within the entire subframe, even though the bandwidth occupied is just a tiny fraction of the carrier bandwidth.

- Since we are interested in coverage, and not in very high data rates, explicit phase noise models as well as PTRS based compensation do not appear critical.

- The following proposals are made:

Proposals:

- As a starting point, neglect any constraints imposed by certain beamforming implementation such as the possibility to simultaneous receive or transmit with maximum gain in more than one direction.

- Neglect PTRS overhead and compensation algorithms.

- The link level simulation results and detailed simulation parameters are given in Table 6 and Table 7 in Appendix A.

# 3. System-level models and results

This chapter first presents system-level models and assumptions for high-band and then coverage results for different scenarios.

## 3.1	Models and assumptions

In Table 4 assumptions for desired and interfering signals for 30GHz are presented. In this case an (8,8,2,8,8) AAS is assumed, i.e. constituted by 8x8 cross-polarized antenna elements, grouped into (1x1) subarrays.  Only wideband time domain beamforming is used. The SSB is swept in 64 different directions, four vertical and 16 horizontal, and the best direction (strongest received signal) is used for the coverage calculations. It is further assumed that the PRACH is received using the same beam, and also that the following PUCCH and PUSCH transmission for message 2 use this beam. Following transmissions are assumed to use a grid of 64 beams, 4 vertical and 16 horizontal. It is further assumed that SSB transmission is concurrent between cells, so that an SSB transmission is interfered with another SSB transmission, and that the interferer activity is 100%. For other downlink channels it is assumed that they are interfered by PDSCH with 50% activity. For the uplink it is assumed that PRACH and PUSCH are interfered by PUSCH with an activity of 50%, and that PUCCH is interfered by PUCCH. For the uplink channels power control with full pathloss compensation (alpha = 1) is used with SNR targets according to Table 4.

Table 4. Desired and interfering signal assumptions for 30GHz.

| Channel | Desired signal beam | Interfering signal | Interferer activity | Power control SNR target |
| --- | --- | --- | --- | --- |
| SSB | GoB 4x16 | SSB | 1.0 |  |
| Msg2 Pdcch | GoB 4x16 | PDSCH | 0.5 |  |
| Msg2 Pdsch | GoB 4x16 | PDSCH | 0.5 |  |
| Msg4 Pdsch | GoB 4x16 | PDSCH | 0.5 |  |
| PDCCH | GoB 4x16 | PDSCH | 0.5 |  |
| PDSCH data | GoB 4x16 | PDSCH | 0.5 |  |
| Msg 1 PRACH | GoB 4x16 | PUSCH | 0.5 | 3dB |
| PUCCH | GoB 4x16 | PUCCH | 0.5 | 3dB |
| Msg3 PUSCH | GoB 4x16 | PUSCH | 0.5 | 10dB |
| CSI PUSCH | GoB 4x16 | PUSCH | 0.5 | 10dB |
| PUSCH Data | GoB 4x16 | PUSCH | 0.5 | 10dB |

Some additional parameters are presented in Table 5.

Table 5. Additional parameters for 30GHz.

| Parameter | 30GHz |
| --- | --- |
| BS power | 40dBm for Dense Urban, 23dBm for indoor hotspot |
| UE power | 12dBm TRP, 23dBm EIRP |
| BS noise figure | 5dB |
| UE noise figure | 7dB |

Lastly, results in this section for PDSCH carrying user data are from link simulations use link and rank adaption based on CSI feedback and HARQ with up to three retransmissions, which is expected to provide a more accurate reflection of the coverage NR can achieve. Further details on these simulation assumptions are available in [4]. The PUSCH simulations use a fixed MCS and HARQ, in accordance with the setups agreed for the coverage enhancement study.

Msg4 is simulated with HARQ, and results are reported for the coverage when 4 retransmissions are used where 1% residual BLER is achieved after the 4 transmissions. Transmission with 10% initial BLER are also reported for reference in order to gauge the impact of restricting to a single transmission.

## 3.1	Dense Urban, ISD 200m, 30GHz, 100MHz TDD

Figure 1 shows SINR distributions and coverage for different channels for the Dense Urban scenario. It is seen that the SINR differs between channels because of the differences in transmit power, bandwidth, antenna gains, and interference. For each channel the SINR requirement is marked with a star in the SINR distribution, and the relative coverage (or outage) can be read at the corresponding value on the vertical axis. The SINR requirements and outage levels for the lower quality requirements are also included in the figure legend. The relative coverage is also summarized in Figure 2. With the exception of Msg4, red bars in this figure represent 1% error rates and blue bars 10% error rates for control. For Msg4, the red bars represent 10% iBLER (which results in less coverage for Msg4 from fewer retransmissions), while the blue bars are for 1% rBLER (with 4 retransmissions). The red and blue bars respectively correspond to higher or lower data rates for PDSCH and PUSCH. PUSCH in this scenario has a data rate of 5 Mbps at an initial BLER of 10%, and a lower data rate when the residual BLER is relaxed to 1% (resulting in a higher initial BLER).  Figure 3 shows the isotropic loss corresponding to the coverage for each of the channels.

It is seen that quite good coverage is achieved. The channel with the worst coverage is PUSCH data. This is followed by downlink data.  When comparing Msg4, recall that it is at 1% rBLER in the ‘less difficult’ blue bars and uses 4 retransmissions, while the ‘more difficult’ red bars are reflective of performance without retransmission.

Observations:

- For the dense urban scenario, uplink data has the worst coverage. This is followed by uplink control channels, downlink data, and downlink control channels. There is no obvious bottleneck among the control channels.

  - PUSCH is generally followed by CSI on PUSCH or PUCCH, then PRACH and lastly HARQ-ACK or Msg3

  - Msg4 with 4 retransmissions at 1% rBLER has the best overall coverage.

The Dense Urban scenario studied here has 80% of the users indoors. Among the indoor users, 20% are located in high-loss buildings, which are difficult to enter at 30GHz. To assess the effect of this user distribution, results are plotted separately for outdoor users in Figure 4 and for users in low-loss buildings in Figure 5. (Note that while the assumptions of [5] are used, including 10 Mbps PUSCH data, the observations are relevant in general.) It is seen that very good outdoor coverage is achieved, with practically full coverage for control channels and downlink data, and 70% of users exceeding 10Mbps in uplink. Also, for users in low-loss buildings, very good control channel coverage is achieved, and 95% of the users exceed 100Mbps in downlink and 33% exceed 10Mbps in uplink.

Figure 1. SINR distributions and coverage for different channels for Dense Urban at 30GHz.

Figure 2. Coverage in terms of fraction of users with acceptable quality for Dense Urban at 30GHz.

Figure 3. Estimated maximum isotropic loss for different channels for Dense Urban at 30GHz.

Figure 4. SINR distributions and coverage for different channels for Dense Urban at 30GHz for outdoor UEs.

Figure 5. SINR distributions and coverage for different channels for Dense Urban at 30GHz for indoor UEs in low-loss buildings.

## 3.2	Indoor hotspot, ISD 20m, 30GHz, 100MHz TDD

In order to stress coverage, a modified version of the indoor hotspot scenario, with one single three-sector base station in the middle of the office floor, is studied.

Figure 6 shows SINR distributions and coverage for different channels. For each channel, the SINR requirement is marked with a star in the SINR distribution, and the relative coverage (or outage) can be read at the corresponding value on the vertical axis. The SINR requirements and outage levels for the lower quality requirements are also included in the figure legend. The relative coverage is also summarized in Figure 7. The isotropic loss in this scenario is too low to be meaningful to use as a measure for coverage.

It is seen that quite good coverage is achieved. The channel with the worst, but still well above 95%, coverage is PUSCH data. This is followed by downlink data.

Observation:

- For the indoor hotspot scenario, quite good coverage is achieved, so there is little meaningful difference between the coverage of any of the uplink and downlink channels.

Figure 6. SINR distributions and coverage for different channels for Indoor Hotspot at 30GHz.

Figure 7. Coverage in terms of fraction of users with acceptable quality for Indoor Hotspot at 30GHz.

## 3.3	Suburban, ISD 500m, 30GHz, 100MHz TDD

In order to assess coverage also in a suburban scenario, which has not yet been defined by 3GPP or ITU for NR / IMT-2020 for frequencies above 6GHz, such a scenario has been created by modifying the Rural scenario to use the line-of-sight probability of the Suburban Macro scenario from IMT-A. This is motivated by that for IMT-A, the main difference between the rural and suburban scenarios when it comes to pathloss calculation is the line-of-sight probability. Oher parameters from the rural scenario are kept, like the user distribution.

Results are presented in Figure 9, Figure 10, and Figure 11. It seen that the bottlenecks are the same as in the urban scenario.

Figure 9. SINR distributions and coverage for different channels for Suburban at 30GHz.

Figure 10. Coverage in terms of fraction of users with acceptable quality for Suburban at 30GHz.

Figure 11. Estimated maximum isotropic loss for different channels for Suburban at 30GHz.

## 3.4	Summary and discussion

The above evaluations can be summarized as:

Observation:

- Across the scenarios, uplink data is typically limiting. This is followed by uplink control or downlink data. Downlink control channels are more robust. There is no obvious bottleneck among the control channels.

Based on this it is proposed to focus coverage enhancements efforts where most needed:

Proposal:

- Focus coverage enhancements efforts on uplink and downlink data.

It should be noted that in this study the focus has been on coverage, and for most channels the configuration providing the best coverage has been selected. Some of these configurations are resource consuming, especially when used together with analogue beamforming, and could form capacity bottlenecks. For capacity reasons it could be motivated to study efficiency enhancements of channels forming such bottlenecks.

Observation:

- Maintaining downlink coverage may require relatively high amounts of resource, and so techniques achieving downlink coverage that maintain spectral efficiency may be of interest.

# Conclusion

In the previous sections we first made the following proposals related to evaluations:

Proposals:

- As a starting point, neglect any constraints imposed by certain beamforming implementation such as the possibility to simultaneous receive or transmit with maximum gain in more than one direction.

- Neglect PTRS overhead and compensation algorithms.

Related to system-level coverage and identification of bottlenecks, we then made the following observations and proposals:

Observations:

- For the dense urban scenario, uplink data has the worst coverage. This is followed by uplink control channels, downlink data, and downlink control channels. There is no obvious bottleneck among the control channels.

  - PUSCH is generally followed by CSI on PUSCH or PUCCH, then PRACH and lastly HARQ-ACK or Msg3

  - Msg4 with 4 retransmissions at 1% rBLER has the best overall coverage.

- For the indoor hotspot scenario, quite good coverage is achieved, so there is little meaningful difference between the coverage of any of the uplink and downlink channels.

- Across the scenarios, uplink data is typically limiting. This is followed by uplink control or downlink data. Downlink control channels are more robust. There is no obvious bottleneck among the control channels.

- Maintaining downlink coverage may require relatively high amounts of resource, and so techniques achieving downlink coverage that maintain spectral efficiency may be of interest.

Proposal:

- Focus coverage enhancements efforts on uplink and downlink data.

# References

R1-2006616, “Evaluation methodology for coverage enhancements”, Ericsson, RAN1 #102-e, August 2020.

R1-2008343, “Link and System Evaluation of Coverage for FR1”, Ericsson, RAN1 #103-e, October 2020.

3GPP TR 37.910, “Study on self evaluation towards IMT-2020 submission,” v16.1.0 (2019-09).

R1-2004353, “Simulation Parameters and Initial Results for FR2”, Ericsson, RAN1 #101-e, May 2020.

R1-2007049, “Link and System Evaluation of Coverage for FR2”, Ericsson, RAN1 #102-e, August 2020.

R1-2008422, “Coverage Parameter Sensitivity and Network Enhancement”, Ericsson, RAN1 #103-e, October 2020.

# Appendix A – Link-level results

In this appendix, the raw link-level results are presented, see Table 6 for 30GHz results and Table 7 for detailed link simulation assumptions.

Note that the error rates refer to residual BLER after re-transmissions (rBLER), except for DL data (PDSCH) and UL data (PUSCH) where the 10% numbers refer to intial BLER (iBLER).

Table 6 Link level SNR requirements for 30 GHz

| Channel/Signal | Case (if applicable) |  |  |  | Number of Rx antennas | Speed [km/h] | SNR [dB] for 10% , 1% error rate 1) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | DS 30 ns | DS 100 ns |
| Initial Access |  |  |  |  |  |  |  |  |
| SSB (P/S-SS and PBCH) |  |  |  |  | 2 | 3 | -10.4, -8.36 | -9.38, -7.81 |
|  |  |  |  |  | 2 | 30 | -10.39, -8.36 | -9.38, -7.8 |
| MSG1 PRACH | ISD 20 m |  |  |  | 2 | 3 | -15.7, -11.8 |  |
|  | ISD 200 m |  |  |  | 2 | 3 |  | -15.2, -11.9 |
|  | ISD 200 m |  |  |  | 2 | 30 |  | -15.2, -11.9 |
| MSG2 RAR (PDCCH+PDSCH) |  |  |  |  | 2 | 3 | -9, -5.6 | -9.1, -6.4 |
|  |  |  |  |  | 2 | 30 |  | -8, -5.8 |
| MSG3 RRC request (PDCCH+PUSCH) |  |  |  |  | 2 | 3 | -10.9, -8.9 | -10.9, -9.2 |
|  |  |  |  |  | 2 | 30 |  | -11.5, -10.1 |
| MSG4 contention resolution (PDSCH) |  |  |  |  | 2 | 3 | -4.8, -10.5 | -5.4, -10.9 |
|  |  |  |  |  | 2 | 30 |  | -4.8, -9.3 |
| Uplink and Downlink Data Transmission |  |  |  |  |  |  |  |  |
| DL assignment or UL Grant (PDCCH) |  |  |  |  | 2 | 3 | -8.8, -6.5 | -8.4, -6.3 |
|  |  |  |  |  | 2 | 30 |  | -8.4, -6.2 |
| DL data (PDSCH) | 1 PRB |  |  |  | 2 | 3 | -0.5, -5.2 | -0.5, -5.2 |
|  | 5 PRBs |  |  |  | 2 | 3 | -2.1, -8.1 | -2.2, -8.2 |
|  | 36 PRBs |  |  |  | 2 | 3 | -3, -8.9 | -4, -9.5 |
|  | 1 PRB |  |  |  | 2 | 30 |  | 0.5, -4 |
|  | 5 PRBs |  |  |  | 2 | 30 |  | -3, -7.9 |
|  | 40 PRBs |  |  |  | 2 | 30 |  | -4.6, -9.5 |
| ACK/NACK (PUCCH) | Format 1 |  | No repetitions |  | 2 | 3 | -7.0, -4.7 | -6.8, -4.8 |
|  | Format 1 |  | No repetitions |  | 2 | 30 |  | -6.8, -4.7 |
|  | Format 1 |  | 8 repetitions |  | 2 | 3 | -14.3, -11.5 | -14.0, -11.2 |
|  | Format 1 |  | 8 repetitions |  | 2 | 30 |  | -14.0, -11.2 |
|  | Format 3 |  | No repetitions |  | 2 | 3 | -11.2, -7.1 | -10.9, -6.5 |
|  | Format 3 |  | No repetitions |  | 2 | 30 |  | -10.8, -6.4 |
|  | Format 3 |  | 8 repetitions |  | 2 | 3 | -16.7, -12.7 | -16.3, -11.9 |
|  | Format 3 |  | 8 repetitions |  | 2 | 30 |  | -16.2, -12.2 |
| CSI feedback (PUCCH/PUSCH) | PUCCH | 11 bits |  | No rep. | 2 | 3 | -6.7, -2.7 | -6.1, -1.9 |
|  | PUCCH | 11 bits |  | No rep. | 2 | 30 |  | -6.0, -1.9 |
|  | PUCCH | 11 bits |  | 8 reps. | 2 | 3 | -12.7, -9.0 | -12.2, -8.2 |
|  | PUCCH | 11 bits |  | 8 reps. | 2 | 30 |  | -12.2, -8.2 |
|  | PUCCH | 22 bits |  | No rep. | 2 | 3 | -4.6, -0.9 | -3.9, 0.1 |
|  | PUCCH | 22 bits |  | No rep. | 2 | 30 |  | -3.8, 0.1 |
|  | PUCCH | 22 bits |  | 8 reps. | 2 | 3 | -10.9, -7.5 | -10.4, -6.5 |
|  | PUCCH | 22 bits |  | 8 reps. | 2 | 30 |  | -10.4, -6.6 |
|  | PUSCH | 11 bits |  | No rep. | 2 | 3 | -5.2, 0.7 | -5.4, -0.5 |
|  | PUSCH | 11 bits |  | No rep. | 2 | 30 |  | -5.2, 0.1 |
|  | PUSCH | 11 bits |  | 8 reps. | 2 | 3 | -9.7, -4.4 | -9.9, -5.4 |
|  | PUSCH | 11 bits |  | 8 reps. | 2 | 30 |  | -9.9, -4.8 |
|  | PUSCH | 22 bits |  | No rep. | 2 | 3 | -2.8, 2.7 | -2.8, 2.1 |
|  | PUSCH | 22 bits |  | No rep. | 2 | 30 |  | -2.6, 2.4 |
|  | PUSCH | 22 bits |  | 8 reps. | 2 | 3 | -7.9, -2.3 | -8.1, -3.5 |
|  | PUSCH | 22 bits |  | 8 reps. | 2 | 30 |  | -8.0, -3.0 |
| UL data (PUSCH) |  |  |  |  | 2 | 3 | 0.5 / -5.4 | 0.1 / -6.0 |
|  |  |  |  |  | 2 | 30 | 0.7 / -7.3 |  |

1) Error rate is residual BLER after re-transmissions (rBLER), except for DL data (PDSCH) and UL data (PUSCH) where the 10% numbers refer to intial BLER (iBLER) of 10%.

Table 7 Link level assumption details for 30 GHz

| Channel | Assumptions |
| --- | --- |
| SSB (P/S-SS and PBCH) | SSBs are transmitted with 20ms periodicity residual BLER after 4 retransmissions within MIB TTI of 80ms, UE is not assumed to know the SS/PBCH block index |
| MSG1 PRACH | Format B4 with 12 symbols 10% or 1% missed detection at 0.1% false alarm probability, with maximum timing estimation error 50% of the normal CP length 64 preambles per cell Initial timing offset uniformly distributed in [0, 0.077 µs] for an ISD of 20m and [0, 0.77 µs] for an ISD of 200m |
| MSG2 RAR (PDCCH+PDSCH) | PDSCH with 8 bytes payload, MCS 0 with transport block scale factor 0.25, 12 PRBs, 3 DMRS symbol, 9 symbols with PDSCH (and 2 symbols reserved for PDCCH) precoder cycling |
| MSG3 RRC request (PDCCH+PUSCH) | PUSCH with 7 bytes payload, MCS 0, 2 PRBs, 3 DMRS symbols 11 symbols with PUSCH, With 7 re-transmissions (8 attempts), using different frequency for different attempts. No PDCCH errors |
| MSG4 contention resolution (PDSCH) | 3 retransmissions, PDSCH with 130 bytes payload, MCS 0, 42 PRBs, 3 DMRS, 9 symbols with PDSCH (and 2 symbols reserved for PDCCH) Precoder cycling |
| DL assignment or UL Grant (PDCCH) | PDCCH using aggregation level 16 and DCI format 0_0 or 1_0 with payload of 40bits+24bits CRC CORESET 66 PRBs, 1 symbol, non-interleaved mapping, precoder cycling |
| DL data (PDSCH) | Link and rank adaption based on 20 slot 2 port wideband CSI feedback periodicity and HARQ with up to three retransmissions. 66 PRBs, 2 (3km/h) or 3 (30km/h) symbols with DMRS, PDSCH and DMRS mapped to 132 symbols (12 symbols reserved for PDCCH), fix MCS0, overhead due to CSI-RS and TRS with 20ms period 1/5/36PRB at 3km/h, 1/5/40PRB at 30km/h 10% for iBLER, 1% for rBLER after retransmission |
| ACK/NACK (PUCCH) | Format 1 with 2bit, 14 symbols long with 7 DMRS, frequency hopping with no or 4 repetitions Pr(DTX to ACK) <=1%, Pr(NACK to ACK) <=0.1%, Pr(ACK error) <=10% or 1% Format 3 with 4bit (3 A/N+ 1Sr), 14 symbols long with 4 DMRS and frequency hopping with no or 8 repetitions, BLER<=10 % or 1%PUCCH Format 3 using 14 symbols, 1 PRB, 4 DMRS and frequency hopping 4 bits payload for ACK/NACKS (three bits for 3DL:1UL TDD asymmetry and another bit for scheduling request) Pr(DTX to ACK) <=1%, Pr(NACK to ACK) <=0.1%, Pr(ACK error) <=10% or 1% |
| CSI feedback (PUCCH or PUSCH) | 11 (6 part_1 + 5 part_2) or 22 (12 part_1 + 10 part_2) bits for wideband CSI feedback for 4Rx 10%, 1% BLER 1 PRB, No or 8 repetitions PUCCH format 3 with 4 DMRS and frequency hopping PUSCH without multiplexing with data on PUSCH and no frequency hopping Type I wideband CSI feedback - 8+2=10 bits for 2 port feedback + 3bit CRI 1 PRB, no HARQ ACK/NACKs - PUCCH format 3 with 4 DMRS, with frequency hopping, or - PUSCH without multiplexing with data on PUSCH and no frequency hopping |
| UL data (PUSCH) | HARQ with up to five transmission attempts in total, 2 symbols with DMRS, fixed MCS 5, 30 PRBs, no frequency hopping, PUSCH and DMRS mapped to 14 symbols and no UCI overhead included 10% initial BLER (iBLER), 1% residual BLER after retransmissions (rBLER) |