3GPP TSG RAN WG1 #103-e		R1-2008363

e-Meeting, October 26th – November 13th, 2020

Agenda Item: 	8.5.1

Source:	Sony

Title:	Considerations on Additional Scenarios for Evaluation

Document for:	Discussion and Decision

# Introduction

NR positioning enhancement study item was approved and the updated SID is described in [1]. One of the objectives is to study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases.

Furthermore, to address the higher accuracy location requirements resulting from new applications and industry verticals, NR Positioning in Rel-17 should evaluate and specify enhancements and solutions to meet the following exemplary performance targets:

(a) For general commercial use cases (e.g., TS 22.261):

- sub-meter level position accuracy (< 1 m)

(b) For IIoT Use Cases (e.g., 22.804):

- position accuracy < 0.2 m

The target latency requirement is < 100 ms; for some IIoT use cases, latency in the order of 10 ms is desired.

In the previous RAN1 #102e meeting, several agreements were made. However, there are still some resolved issues which shall be discussed and addressed in this RAN1 #103e meeting, as the last meeting for the study item phase. In this document, we discuss our view on the commercial positioning requirements, latency, and general simulation assumptions, including reference signal bandwidth.

# Discussions

In RAN1 #101e meeting, the following agreements related to positioning accuracy and latency were made [2]:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

The above agreements are considered as initial requirements. Furthermore, some requirements were not fully agreed yet (i.e. the requirements in []). Subsequently, during RAN1 #102e, the positioning accuracy and latency were further discussed. However, there was no consensus and no related agreements were made. There were diverse views on the positioning accuracy and latency. Moreover, there was a new consideration to introduce positioning service level based on the required positioning requirements.

Regarding the positioning accuracy, we are supportive of the agreement made in RAN1 #101e. Our simulation results shown in our companion contribution in [3] indicate these positioning requirements can be fulfilled. We recognize that in some cases (e.g. InF-DH), it is quite challenging to meet these requirements. We can express in the study item report that the following requirements may not be fulfilled in the selected scenarios. Therefore, there is still opening for exploring new positioning techniques in the future releases.

Proposal 1: Our preferred positioning accuracy requirements:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

- Horizontal position accuracy (< 1 m) for 90%of UEs

- Vertical position accuracy (<3m) for 90% of UEs

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

- Horizontal position accuracy (< 0.2 m) for 90%of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

As indicated above, one of the stringent accuracies is 0.2m horizontal positioning accuracy. In the context of latency, which we will discuss later, the range can be in the order of 10ms. There was also a discussion to introduce 0.5 m accuracy. It is good to have stringent requirement. However, in our view, these stringent requirements may not always be needed depending on the application and UE/network constraints. Hence, we are supportive to introduce service level in positioning requirements in which each level represent both positioning accuracy and latency. The accuracy range can be from 0.2m to 3m. Note: Horizontal accuracy with 3 m is as used in NR Rel-16). The details of service level positioning requirements can be further discussed in RAN2.

Proposal 2: Support different service level in which each level represent both positioning accuracy and latency.

Positioning latency requirements includes both end-to-end positioning latency and physical layer positioning latency. On end-to-end latency, we agreed to send an LS to RAN2 for analysis of latency of NR positioning protocols defined in Rel.16 including any potential enhancements. Considering end-to-end latency may involve higher layer protocol and signaling to core-network, it is better to discuss the end-to-end latency after receiving the LS response.

Proposal 3: Define the end-to-end positioning latency after receiving the associated LS response from RAN2 as this requirement is beyond RAN1 scope.

On physical layer latency, the agreements on the definition of physical layer latency were made in RAN1#102e meeting [4]:

Physical Layer Latency Start and End times are defined as follows:

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives. | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

One simplified example for legacy signalling sequence affecting the end to end latency for UE assisted DL-only (DL-TDoA) is elaborated here, and shown in  Figure 1. LS sends a positioning measurement request to the UE via LPP protocol (and transparent to gNB). The time when the UE receives this request is considered as the start of physical layer latency. The UE performs measurements of the PRS from multiple gNB´s within a measurement gap as the PRS may have different configuration with the existing UE active bandwidth part (BWP). Once the UE has obtained the positioning measurement data, the UE sends an uplink scheduling request (SR) to the serving gNB. The serving gNB sends the corresponding UL grant in downlink control channel (PDCCH). The UE then finally transmits the positioning measurement reports in uplink data channel (PUSCH) to the LS. The time when the gNB manages to decode PUSCH is known as end of physical layer positioning latency. There are multiple steps involved within the start and the end as described above. In the last meeting, it was proposed to define 10ms as physical layer latency for IIoT use-cases.

Figure 1: Signalling mechanism in DL-TDOA positioning technique

Figure 1: Signalling mechanism in DL-TDOA positioning technique

PRS configuration

PRS configuration

SR request

Meas GAP

PRS transmission

Positioning meas request

UE

gNB

LS

POS meas

UL grant

POS meas_rep

PRS configuration

PRS configuration

SR request

Meas GAP

PRS transmission

Positioning meas request

UE

gNB

LS

POS meas

UL grant

POS meas_rep

Observation 1:  Multiple steps are involved between the start and end of physical layer latency. Hence, it is quite challenging to obtain 10ms physical layer latency.

We consider 10ms physical layer latency can be achieved by simplifying the existing legacy procedure. There are some steps that are considered necessary, such as positioning measurement. There are also some steps that may be skipped such as scheduling request and assuming the UE is granted for the uplink positioning measurement report (PUSCH).

Proposal 4: Physical layer latency of 10ms is achievable by simplifying the existing legacy procedure.

The baseline parameters for all scenarios in the evaluation of the positioning performance in Rel-17 were agreed in RAN1 101e and most of the parameters are inherited from TR 38.855 (See Appendix A). It covers both FR1 and FR2. It is observed some baseline parameters may not be practically deployed. For example, the bandwidth for FR1 and FR2 are 100 MHz and 400 MHz, respectively. These are basically the entire bandwidth for both frequency ranges. It may not be practical to allocate Positioning Reference Signal (PRS) in the entire bandwidth. However, the simulation results would still be useful as it can be considered as the upper-bound results.

We consider that using impractical/uncommon simulation assumptions / parameters may lead to imprecise conclusions of the study item. Furthermore, by using practical simulation assumptions / parameters may expose more positioning techniques in order to meet the stringent positioning requirements.  We have observed that the positioning estimation accuracy is heavily affected by the PRS bandwidth configuration as shown in our companion contribution [3].

Observation 2: Some of the scenario parameters may not be practically implemented (e.g. bandwidth for FR1 and FR2 are 100 MHz, and 400MHz, respectively). In practice, it may not be possible to achieve the required positioning accuracy.

Proposal 5:  The conclusion in the technical report should capture that the obtained simulation results may be based on optimistic parameter value(s) and selecting other parameter value(s) (e.g. smaller number) may affect/degrade the results.

# Summary

In this contribution, we discussed some aspects related to positioning requirements and remaining issues on scenarios. We have made the following observations:

Observation 1:  Multiple steps are involved between the start and end of physical layer latency. Hence, it is quite challenging to obtain 10ms physical layer latency.

Observation 2: Some of the scenario parameters may not be practically implemented (e.g. bandwidth for FR1 and FR2 are 100 MHz, and 400MHz, respectively). In practice, it may not be possible to achieve the required positioning accuracy.

We also made the following proposals:

Proposal 1: Our preferred positioning accuracy requirements:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

- Horizontal position accuracy (< 1 m) for 90%of UEs

- Vertical position accuracy (<3m) for 90% of UEs

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

- Horizontal position accuracy (< 0.2 m) for 90%of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

Proposal 2: Support different service level in which each level represent both positioning accuracy and latency.

Proposal 3: Define the end-to-end positioning latency after receiving the associated LS response from RAN2 as this requirement is beyond RAN1 scope.

Proposal 4: Physical layer latency of 10ms is achievable by simplifying the existing legacy procedure.

Proposal 5:  The conclusion in the technical report should capture that the obtained simulation results may be based on optimistic parameter value(s) and selecting other parameter value(s) (e.g. smaller number) may affect/degrade the results.

# References

- RP-202094 Revised SID: Study on NR Positioning Enhancements

- Chairman notes RAN1#101e meeting, May-June 2020.

- R1-2008364, “Discussion on Performance evaluation of Rel-17 positioning”, Sony, RAN1#103e, Oct-Nov 2020.

- Chairman notes RAN1#102e meeting, August 2020.

# Appendix A

Table: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: FFS |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |