3GPP TSG RAN WG1 Meeting #103-e	R1-2008380

e-Meeting, October 26th – November 13th, 2020

Source:		SoftBank

Title:	Target value for FR1 voice coverage enhancements

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Introduction

RAN1 reached the following conclusion on target performance metrics:

Agreements:

- RAN1 strives for satisfying appropriate targets identified by companies particularly operators

  - The targets may be in the form of one or more of the following:

    - 1. Scenario dependent targets, e.g., ISD/MPL

    - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP;

    - 3. Relative difference between channels, e.g, MIL(/[MCL])

  - Further values and details of such targets will be clarified at RAN1#103-e

  - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets.

In this contribution, we share our view on the target MCL value for VoIP.

Discussions

The target MCL of 147 dB is derived based on the following simple table for UMTS (R1-2003464):

Table 1. Uplink link budget for UMTS

| Tx Power | 23.0 | dBm |
| --- | --- | --- |
| Process gain | 25.0 | dB |
| Required Eb/N0 | 5.0 | dB |
| Thermal noise | -174.0 | dBm/Hz |
| Noise figure | 5.0 | dB |
| Bandwidth | 3.84 | MHz |
| MCL | 146.2 | dB |

This link budget is mapped to the latest link budget template as in Table 2 for the fair comparison with NR. Since uplink is the bottleneck for UMTS, only the link budget for UL is shown here.

Table 2. Link budget based on the agreed link budget template

| Scenarios for evaluation | UMTS | NR |
| --- | --- | --- |
| Channel for evaluation | Uplink | Uplink 700MHz |
| Transmitter |  |  |
| (2) Number of [transmit TxRUs] Note: this row is void (left empty) for uplink |  |  |
| (2a) Number of transmit chains modelled in LLS | 1 | 1 |
| (3) Total transmit power (dBm) | 23 | 23 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 3840000 | 180000 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) | 17.16 | 30.45 |
| (3c) bandwidth used for the evaluated channel (Hz) | 3840000 | 180000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23.00 | 23.00 |
| (5) total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) | 0 | 0 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) | 0 | 0 |
| (5b) antenna gain correction factor at antenna gain component 2 of transmitter (dB) | 0 | 0 |
| Receiver |  |  |
| (10a) Number of [receive TxRUs] Note: this row is void (empty) for downlink | 2 | 2 |
| (10b) Number of receive chains modelled in LLS | 2 | 2 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 0 | 0 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 0 | 0 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 | 0 |
| (13) Receiver noise figure (dB) | 5 | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -inf | -inf |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -169 | -169 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -103.16 | -116.45 |
| (19) Required SNR (dB) | 5 | -6.71 (Note1) |
| (20) Receiver implementation margin (dB) | 0 | 0 |
| (21) H-ARQ gain (process gain for UMTS) (dB) | 25 | 0 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -123.16 | -123.16 |
| (22bis) MCL = (3bis) - (22) + (5) + (11bis) (dB) | 146.16 | 146.16 |

Note 1: This is an informative value to achieve the same MCL as UMTS.

For NR link budget, many rows allow optional values or are up to companies. Furthermore, many scenarios and frequencies are defined for the evaluation. Therefore, we need some guidance which condition needs to be satisfied for the fair comparison with the MCL of 147dB. Note the observation for DL is also included here.

Scenario

In general, the voice coverage is achieved by low band(s) in the commercial network, and hence these spectra should be used for comparison. At least rural with 700MHz and/or 2GHz should be considered for the voice coverage evaluation. It is preferred to consider all combination of LOS/NLOS + indoor/outdoor.

(2) Number of [transmit TxRUs], (2a) Number of transmit chains modelled in LLS and (5) total antenna gain at antenna gain component 2 of transmitter

Downlink

Legacy BSs for voice coverage typically employ 2-TxRUs, and they will be reused even when 3G/LTE is refarmed into NR. Therefore, 2-TxRUs should be used as a baseline for the comparison. If 4-TxRUs is used for 2GHz system, 3dB gain should be subtracted for the fair comparison.

Uplink

It is appropriate to use k=1 for the comparison because k=1 will be widely used for low band frequency. It is not necessary to perform the comparison with k=2 because k=2 is optional value for the evaluation.

(3) Total transmit power (dBm)

Downlink

The agreed PSD values can be used because it is the typical value for the real deployment.

Uplink

The same condition (i.e. 23dBm) should be applied for the fair comparison.

(10a) Number of [receive TxRUs], (10b) Number of receive chains modelled in LLS and (11bis) total antenna gain at antenna gain component 2 of receiver

Downlink

For number of receiver chains, 2 and 4 is allowed for 2GHz while 2 is allowed for 700MHz. Basically the coverage should be evaluated with the minimum requirement for the UE. Therefore, k=2 should be used for the comparison.

Uplink

As discussed earlier, legacy BS in real life network employs 2-TxRUs, and hence 2-TxRUs should be used for the comparison. If 4-TxRUs is used for the comparison, 3dB gain should be subtracted for the fair comparison.

(15) Receiver interference density

In our view, noise-limited scenario is a typical case for out of voice service. Hence, we can consider zero interference for the comparison with the 147dB MCL. Even when non-zero value is used for the evaluation, the result can be used for comparison because it is a pessimistic one. Therefore, it is not necessary to adjust the target MCL value.

(20) Receiver implementation margin (dB)

For UMTS link budget, receiver implementation margin is not considered. Hence, we can consider zero or realistic value (proposed by companies) can be used for the comparison with the 147dB MCL.

Other values

The realistic values proposed by companies can be used for the comparison.

Summary and proposal

Given the discussion in section 2, we propose the following:

Proposal

- Confirm the target MCL value of 147 dB for FR1 VoIP under the following condition:

Rural with 700MHz and/or 2GHz senarios

2 trannsmit TxRUs and 2 receive TxRUs for BS

1 Tx chain and 2 Rx chains for UE

Zero receiver interference density

    - Even when non-zero value can be used, it is not needed to adjust the target value

Zero recevier implementation margin

    - Even when non-zero value can be used, it is not needed to adjust the target value