3GPP TSG RAN WG1 #103-e	R1-2008384

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Remaining issues on DL signals/channels for NR-U

Agenda Item:	7.2.2

Document for:	Discussion and Decision

# Introduction

In RAN1#102_e, the following TP for TS38.213 clause 11.1 was endorsed [1] [2].

If a UE is configured with ChannelAccessMode-r16 ='dynamic', if the UE is configured with availableRB-SetsToAddModList-r16 and availableRB-SetsToRelease-r16, the UE expects to be configured with co-DurationsPerCell ToAddModList-r16 and co-DurationsPerCellToReleaseList-r16 and/or slotFormatCombToAddModList and slotFormatCombToReleaseList.

In this contribution, we discuss remaining issues on DL signals and channels for NR-U.

# Remaining issues

# Channel occupancy duration in FBE

In the current TS38.213 [3], the duration to which the availability indicated by the available RB set indicator field apply is defined by using an indicated channel occupancy duration under Clause 11.1.1. In RAN1#102_e, it was agreed that availableRB-SetsToAddModList-r16 and availableRB-SetsToRelease-r16 can be configured without configuring co-DurationsPerCell ToAddModList-r16 and co-DurationsPerCellToReleaseList-r16 and/or slotFormatCombToAddModList and slotFormatCombToReleaseList. This means that, for FBE, the indicated channel occupancy duration can be identified without using either an SFI-index field or a channel occupancy duration field. RAN1 common understanding may be to consider the channel occupancy duration defined in Clause 4.3 of TS37.213 [4] as the indicated channel occupancy duration. However, it has not been captured in the specification yet.

Table 1: Definition of “indicated channel occupancy duration” for FBE

|  | SFI-index field | CO duration field | For FBE, “indicated channel occupancy duration” in Clause 11.1.1 of TS38.213 is identified by: |
| --- | --- | --- | --- |
| Case 1 | Configured | Configured | CO duration field value (already defined in Clause 11.1.1) |
| Case 2 | Configured | Not configured | SFI-index value (already defined in Clause 11.1.1) |
| Case 3 | Not configured | Configured | CO duration field value (already defined in Clause 11.1.1) |
| Case 4 | Not configured | Not configured | COT for FBE if any DL burst is detected in the COT (not defined yet) |

Therefore, it is proposed that, if neither SFI-index field nor CO duration field is configured, COT for FBE (i.e. COT defined in Clause 4.3 of TS37.213) should be considered as being within “indicated channel occupancy duration” if any DL burst is detected in the COT.

Proposal 1:

- If neither SFI-index field nor CO duration field is configured, COT for FBE (i.e. COT defined in Clause 4.3 of TS37.213) should be considered as being within “indicated channel occupancy duration” if any DL burst is detected in the COT.

  - Adopt Text proposal #1.

Text proposal #1 --------- beginning of text proposal for TS 38.213 ## 11.1 Slot configuration <omitted> If a UE is provided ChannelAccessMode-r16 ='dynamic' and is provided availableRB-SetsToAddModList-r16 and availableRB-SetsToRelease-r16, the UE expects to be provided co-DurationsPerCell ToAddModList-r16 and co-DurationsPerCellToReleaseList-r16 and/or slotFormatCombToAddModList and slotFormatCombToReleaseList. If neither CO-DurationPerCell-r16 nor slotFormatCombinationId is provided and if ChannelAccessMode-r16 = semistatic is provided, the procedures in Clause 11.1.1 apply with assuming a channel occupancy time as being indicated as the remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time. <omitted>

# Conclusion

In this contribution, we propose

Proposal 1:

- If neither SFI-index field nor CO duration field is configured, COT for FBE (i.e. COT defined in Clause 4.3 of TS37.213) should be considered as being within “indicated channel occupancy duration” if any DL burst is detected in the COT.

  - Adopt Text proposal #1.

# References

  - 3GPP RAN1#102_e Chairman’s note, May 2020.

  - R1-2007257, “Summary#2 of Email discussion [102-e-NR-unlic-NRU-DL_Signals_and_Channels-01] on DCI format 2_0 fields, Slot Format determination and UE behaviour”, Moderator (Lenovo), RAN1#102_e, August 2020.

  - 3GPP TS38.213v16.3.0, “Physical layer procedures for control (Release 16),” September 2020.

  - 3GPP TS37.213v16.3.0, “Physical layer procedures for shared spectrum channel access (Release 16),” September 2020.