3GPP TSG RAN WG1 #103-e	R1-2008386

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Remaining corrections for wideband operation for NR-U

Agenda Item:	7.2.2

Document for:	Discussion and Decision

Background

At RAN2#111-e meeting, a correction on RRC parameters for intra-cell guard bands are made to configure intra-cell guard bands per SCS. Following the correction, RAN1 specification should be also updated.

Remaining corrections for wideband operation

The latest TS38.331 specifies RRC parameters for intra-cell guard bands as follows [1].

intraCellGuardBandsDL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

intraCellGuardBandsUL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

As above, intra-cell guard band configuration is updated to include configuration per SCS (IntraCellGuardBandsPerSCS-r16). The structure of IntraCellGuardBandsPerSCS-r16 is as follows.

IntraCellGuardBandsPerSCS-r16 ::=      SEQUENCE {

guardBandSCS-r16                       SubcarrierSpacing,

intraCellGuardBands-r16                SEQUENCE (SIZE (1..4)) OF GuardBand-r16

}

Section 7 of TS38.214V16.3,0 doesn’t reflect this change. Therefore, we propose to update the spec. as follows. In addition, intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 are replaced by intraCellGuardBands-r16. This change should also be reflected.

Proposal: Adopt the following Text Proposal.

Text proposal --------- beginning of text proposal for TS 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBands-r16 for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier with SCS configuration μ, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xstartsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to SCS configuration μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise and RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise The RB set with index s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBands-r16 for SCS configuration μ, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to SCS configuration μ and carrier size Ngrid,xsize,μ. μNgrid,xsize,μFor either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1. For a carrier with SCS configuration μ, the UE expects N BWP,istart,μ=RB s0,xstart,μ and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1 for a BWP i configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier with SCS configuration μ, the UE is indicated that no intra-cell guard-bands are configured for the carrier and expects NRB-set,x>1. For SCS configuration μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For SCS configuration μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs. -------- Unchanged contents are omitted --------- end of text proposal

Conclusion

In this contribution, we have the following proposal:

Proposal: Adopt the following Text Proposal.

References

- 3GPP TS38.331V16.2.0, September 2020