3GPP TSG-RAN WG1 Meeting #103-e	R1-2008418

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.1

Source:	Ericsson

Title:	Maintenance of Two step RACH

Document for:	Decision

# 1	Introduction

This contribution addresses remaining issues of 2-step RACH in NR.

# 2	Discussion

## 2.1 	Timing indicator of PUCCH for HARQ feedback of MsgB

In 38.321, “HARQ Feedback Timing Indicator” is specified and included in success RAR to indicate the timing of PUCCH for MsgB HARQ feedback.

.

But “PDSCH-to-HARQ feedback timing indicator” term is used in 38.213 in section 8.2A, we propose to align this indicator term between MAC spec. and RAN1 spec.:

To avoid inconsistency with the MAC specification, use “HARQ Feedback Timing Indicator” in 38.213 section 8.2A for indication of the PUCCH timing for MsgB HARQ feedback, according to text proposal TP1.

---------------------------------------start of TP1 for 38.213 section 8.2A-----------------------------------------

------------------------------------------unchanged text omitted------------------------------------------

## 8.2A	Random access response - Type-2 random access procedure

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback HARQ Feedback tTiming iIndicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆, where n is a slot of the PDSCH reception and ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214]

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

------------------------------------------unchanged text omitted------------------------------------------

--------------------------------------------------------end of TP1------------------------------------------------------------

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	To avoid inconsistency with the MAC specification, use “HARQ Feedback Timing Indicator” in 38.213 section 8.2A for indication of the PUCCH timing for MsgB HARQ feedback, according to text proposal TP1.