3GPP TSG- RAN WG1 Meeting #103-e                                                   R1-2008436

# e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.6

Source:	Apple Inc.

Title:	Remaining issues on Rel-16 Multi-TRP enhancement

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussion on remaining issues on multi-TRP enhancement including remaining details for single-DCI mode and multi-DCI mode.

# Single DCI mode

## TCI states and PDSCH repetition mapping

It has been agreed that when 2 CDM groups and 2 TCI states are indicated, they are 1-to-1 mapped, where the first TCI state corresponds to the first CDM group, and the second TCI state corresponds to the second CDM group. Further 1 CDM group can be indicated for FDM/TDM schemes. Currently for SDM schemes, there is no definition of CDM to TCI mapping when 3 CDM groups are indicated.

Proposal 1: Adopt the following TP for 38.214 to clarify that 3 CDM groups should not be allowed for multi-TRP operation.

#### 5.1.6.2 DM-RS reception procedure <unrelated part omitted> When a UE is not indicated with a DCI that DCI field "Time domain resource assignment' indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM group in the DCI field "Antenna Port(s)", - the first TCI state corresponds to the CDM group of the first antenna port indicated by the antenna port indication table, and the second TCI state corresponds to the other CDM group. When a UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', UE shall not expect to be indicated with DM-RS port(s) within three CDM group in the DCI field "Antenna Port(s)".

# Multi-DCI mode

## Radio link monitoring impact

For multi-DCI mode, the maximum number of CORESETs has been increased from 3 to 5. However, for FR1, based on current 38.213, the maximum number of RS for RLM is 4 according to Table 1 below, where indicates the maximum number of RS configured for RLM.

Table 3-1:  and  as a function of maximum number  of SS/PBCH blocks per half frame

| 4 | 2 | 2 |
| --- | --- | --- |
| 8 | 6 | 4 |
| 64 | 8 | 8 |

Then it should be decided for multi-DCI mode, which CORESET(s) should be monitored. Although UE is configured in multi-DCI mode, some common control signaling should be transmitted from one TRP. Therefore, it seems necessary for UE to monitor the link quality for this TRP only. Thus, if the higher layer index is configured in CORESET(s), UE only need to monitor the CORESET(s) with one particular higher layer index.

Proposal 2: For multi-DCI mode, if higher layer index in a CORESET is configured, UE only needs to monitor the CORESET(s) with CORESETPoolIndex configured to be 0 for RLM.

- Adopt the following TP for 38.213.

# 5 Radio link monitoring <unrelated part omitted> If the UE is not provided RadioLinkMonitoringRS and the UE is provided for PDCCH receptions TCI states that include one or more of a CSI-RS - the UE uses for radio link monitoring the RS provided for the active TCI state for PDCCH reception if the active TCI state for PDCCH reception includes only one RS - if the active TCI state for PDCCH reception includes two RS, the UE expects that one RS has QCL-TypeD [6, TS 38.214] and the UE uses the RS with QCL-TypeD for radio link monitoring; the UE does not expect both RS to have QCL-TypeD - the UE is not required to use for radio link monitoring an aperiodic or semi-persistent RS - For and Lmax = 8, the UE selects the RS provided for active TCI states for PDCCH receptions in CORESETs associated with the search space sets in an order from the shortest monitoring periodicity. If more than one CORESETs are associated with search space sets having same monitoring periodicity, the UE determines the order of the CORESET from the highest CORESET index as described in Clause 10.1. - If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE selects the NRLM RS provided for active TCI states for PDCCH reception in CORESETs with CORESETPoolIndex equals to 0.

## Default PDSCH beam when PDCCH is present

In 38.214 section 5.1.5, it is defined that the PDCCH should be with a higher priority than default PDSCH beam as follows:

“Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).”

For multi-TRP operation, this priority rule should be defined for a TRP, i.e. CORESETPoolIndex. It is not reasonable for UE to apply a beam to receive PDCCH from a CORESETPoolIndex to receive both PDSCH from different TRPs.

Proposal 3: The priority rule for PDCCH and default PDSCH beam should be defined per CORESETPoolIndex

- Adopt the following TP for 38.214.

### 5.1.5 Antenna ports quasi co-location <unchanged part omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, where the PDCCH is from a CORESET with the same CORESETPoolIndex as the CORESET for the scheduling PDCCH, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). When a UE is configured with enableTwoDefaultTCIStates, if the offset between the reception of the DL DCI and the corresponding PDSCH or the first PDSCH transmission occasion is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme-r16 set to 'TDMSchemeA' or is configured with higher layer parameter repetitionNumber-r16, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

## URLLC related HARQ-ACK PUCCH enhancement

In Rel-15, a UE is not expected to transmit two HARQ-ACK PUCCH in the same slot. As part of Rel-16 Multi-DCI based Multi-TRP enhancement, separate HARQ-ACK PUCCH within a slot for different TRPs is supported in which UE can be configured to transmit two HARQ-ACK PUCCH in TDM way in the same slot. Correspondingly, FG16-2a-4 is agreed to indicate whether UE supports separate HARQ-ACK PUCCH for Multi-DCI based Multi-TRP operation with candidate values of bitmap {LongAndLong, LongAndShort, ShortAndShort} for the supported combinations of long/short PUCCH formats within a slot.

At the same time, Rel-16 URLLC enhancement agreed on sub-slot based HARQ-ACK feedback, where there can be one HARQ-ACK PUCCH per sub-slot. With this feature, the UE can support up to 2 or 7 HARQ-ACK PUCCHs per slot. Consequently, UE capability is designed to indicate whether UE supports sub-slot based HARQ-ACK feedback procedure, including FG11-3, FG11-3c/d/e/f, etc.

There was no discussion whether UE should support Multi-DCI based Multi-TRP operation (with separate HARQ-ACK feedback) with sub-slot based HARQ-ACK feedback since these two features were designed independently by two different groups. As results, this is an unclear area without explicitly UE capability indication. If these two features can be used together, the UE would be required to support two HARQ-ACK PUCCHs within each sub-slot, which is not covered by any of the existing UE FGs.

Two resolves the issues, in our view, there are two alternative

- Alt 1: Introduce UE capability to indicate whether UE supports sub-slot based HARQ-ACK PUCCH when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with sub-slot based HARQ-ACK PUCCH

In summary, we have the following proposal

Proposal 4: In terms of whether UE supports sub-slot based HARQ-ACK PUCCH when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions

- Alt 1: Introduce UE capability to indicate whether UE supports sub-slot based HARQ-ACK PUCCH when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with sub-slot based HARQ-ACK PUCCH. TP to 38.213 as follows

# 9 UE procedure for reporting control information <unchanged part omitted> In the remaining of this Clause, if a UE is provided subslotLengthForPUCCH-r16, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH-r16. If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured with subslotLengthForPUCCH-r16 in any PUCCH-Config of the active UL BWP. <unchanged part omitted>

Furthermore, Rel-16 URLLC enhancement allows two HARQ-ACK codebooks to be configured with different priorities with corresponding UE capability FG11-4 and FG11-4a. It has not been discussed whether Multi-TRP Multi-DCI supports two HARQ-ACK codebooks with different priorities, especially when UE supports and is configured with ACKNACKFeedbackMode = SeparateFeedback. More importantly, the corresponding UE capability was not discussed during the Rel-16 UE feature discussion. If these two features can be used together, the UE would be required to support up to 4 HARQ-ACK PUCCHs in a slot, which is not covered by any of the existing UE FGs. In order to resolve the issue, we have the following proposal:

Proposal 5: In terms of whether UE supports two HARQ-ACK codebooks with different priorities when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions

- Alt 1: Introduce UE capability to indicate whether UE supports two HARQ-ACK codebooks with different priorities when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with two HARQ-ACK codebooks with different priorities. TP to 38.213 as follows

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks - a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1 - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured to generate two HARQ-ACK codebooks. <unchanged part omitted>

For proposals 4 and 5, we have a preference of Alt 2, not supporting the simultaneous configurations of multi-DCI based multi-TRP operation and sub-slot-based HARQ-ACK feedback/two HARQ-ACK codebooks in Rel-16 for simplicity.

- We are at a late stage of Rel-16 maintenance, and the UE features are frozen already (even though new FG can still be added if necessary). Defining new FGs according to Alt 1 may not be very straightforward, because we have FG 11-3c/d/e/f and FG 11-4c/d/e/f/g/h that define the PUCCH capabilities (numbers and format combinations) for eURLLC features, and short/long PUCCH combinations defined in FG 16-2a-4. Careful investigation would be required to go through the list and identify the valid overall combinations.

- Multi-DCI based multi-TRP operation was designed for eMBB, not URLLC. The consequence of not allowing joint operation of the features is that multi-DCI based multi-TRP operation may not be used for eMBB traffic when a UE has mixed eMBB/URLLC traffic. So the impact is relatively limited.

# General Issue for both single-DCI and multi-DCI mode

## Default aperiodic CSI-RS beam

In RAN1 #102, the following agreement on default aperiodic CSI-RS beam has been achieved.

Agreement In multi-DCI based multi-TRP system, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is less than threshold beamSwitchTiming, the UE determines the QCL assumption for AP CSI-RS resource as follows: - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. - When a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet , the other DL signal refers to PDSCH associated with the same CORESETPoolIndex as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336} and enableBeamSwitchTiming-r16 is not provided, periodic CSI-RS, semi-persistent CSI-RS; - If there is no other DL signal with an indicated TCI state in the same symbols as the CSI-RS, - When a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet , the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs , which are configured with the same value of CORESETPoolIndex as the PDCCH triggering that AP CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH triggering that AP CSI-RS. - UE is not expected to receive AP CSI -RS and PDSCH /AP-CSI-RS associated with different CORESETPoolIndex in overlapped symbol(s). - UE is not expected to receive AP CSI-RS and SP/P CSI-RS with different QCL type D in overlapped symbol(s) Note: The above behavior is applied at least for the same carrier scheduling case. Agreement In single-DCI based multi-TRP system if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is less than threshold beamSwitchTiming , the UE determines the QCL assumption for AP CSI-RS resource as follows - If there is any other DL signal with indicated TCI state in the same symbols as AP CSI-RS, The UE applies QCL assumption of the other signal. - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, when the other signal is a PDSCH indicated with two TCI states, the UE applies the first TCI state of those two TCI states. - Otherwise, - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, the UE applies the first one of two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states - UE is not expected to receive AP CSI-RS and SP/P CSI-RS with different QCL type D in overlapped symbol(s) Note: The above behavior is applied at least for the same carrier scheduling case.

Based on the agreement, the following TP is proposed.

Proposal 6: Adopt the following TP for default aperiodic CSI-RS beam for multi-TRP operation

##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology <unrelated part omitted> A trigger state is initiated using the CSI request field in DCI. - When all the bits of CSI request field in DCI are set to zero, no CSI is requested. - When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to trigger states to the codepoints of the CSI request field in DCI. is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. - When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state. - For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS; - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the other DL signal refers to: - PDSCH associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306]; - aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336} and enableBeamSwitchTiming-r16 is not provided; - periodic CSI-RS; - semi-persistent CSI-RS. - - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states and the other signal is a PDSCH indicated with two TCI states, the UE applies the first TCI state of those two TCI states. - else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; - else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS. - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, the UE applies the first one of two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states. - UE is not expected to receive aperiodic CSI -RS and PDSCH/aperiodic CSI-RS associated with different CORESETPoolIndex in overlapped symbol(s). - UE is not expected to receive aperiodic CSI-RS and semi-persistent/periodic CSI-RS with different ‘QCL-TypeD’ in overlapped symbol(s). - A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

# Conclusion

In this contribution, we discussed the remaining issues for multi-TRP enhancement. Based on the discussion, the following proposals have been made:

Proposal 1: Adopt the following TP for 38.214 to clarify that 3 CDM groups should not be allowed for multi-TRP operation.

#### 5.1.6.2 DM-RS reception procedure <unrelated part omitted> When a UE is not indicated with a DCI that DCI field "Time domain resource assignment' indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM group in the DCI field "Antenna Port(s)", - the first TCI state corresponds to the CDM group of the first antenna port indicated by the antenna port indication table, and the second TCI state corresponds to the other CDM group. When a UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', UE shall not expect to be indicated with DM-RS port(s) within three CDM group in the DCI field "Antenna Port(s)".

Proposal 2: For multi-DCI mode, if higher layer index in a CORESET is configured, UE only needs to monitor the CORESET(s) with CORESETPoolIndex configured to be 0 for RLM.

- Adopt the following TP for 38.213.

# 5 Radio link monitoring <unrelated part omitted> If the UE is not provided RadioLinkMonitoringRS and the UE is provided for PDCCH receptions TCI states that include one or more of a CSI-RS - the UE uses for radio link monitoring the RS provided for the active TCI state for PDCCH reception if the active TCI state for PDCCH reception includes only one RS - if the active TCI state for PDCCH reception includes two RS, the UE expects that one RS has QCL-TypeD [6, TS 38.214] and the UE uses the RS with QCL-TypeD for radio link monitoring; the UE does not expect both RS to have QCL-TypeD - the UE is not required to use for radio link monitoring an aperiodic or semi-persistent RS - For and Lmax = 8, the UE selects the RS provided for active TCI states for PDCCH receptions in CORESETs associated with the search space sets in an order from the shortest monitoring periodicity. If more than one CORESETs are associated with search space sets having same monitoring periodicity, the UE determines the order of the CORESET from the highest CORESET index as described in Clause 10.1. - If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE selects the NRLM RS provided for active TCI states for PDCCH reception in CORESETs with CORESETPoolIndex equals to 0.

Proposal 3: The priority rule for PDCCH and default PDSCH beam should be defined per CORESETPoolIndex

- Adopt the following TP for 38.214.

### 5.1.5 Antenna ports quasi co-location <unchanged part omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, where the PDCCH is from a CORESET with the same CORESETPoolIndex as the CORESET for the scheduling PDCCH, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). When a UE is configured with enableTwoDefaultTCIStates, if the offset between the reception of the DL DCI and the corresponding PDSCH or the first PDSCH transmission occasion is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme-r16 set to 'TDMSchemeA' or is configured with higher layer parameter repetitionNumber-r16, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

Proposal 4: In terms of whether UE supports sub-slot based HARQ-ACK PUCCH when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions

- Alt 1: Introduce UE capability to indicate whether UE supports sub-slot based HARQ-ACK PUCCH when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with sub-slot based HARQ-ACK PUCCH. TP to 38.213 as follows

# 9 UE procedure for reporting control information <unchanged part omitted> In the remaining of this Clause, if a UE is provided subslotLengthForPUCCH-r16, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH-r16. If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured with subslotLengthForPUCCH-r16 for the corresponding PUCCH. <unchanged part omitted>

Proposal 5: In terms of whether UE supports two HARQ-ACK codebooks with different priorities when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions

- Alt 1: Introduce UE capability to indicate whether UE supports two HARQ-ACK codebooks with different priorities when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with two HARQ-ACK codebooks with different priorities. TP to 38.213 as follows

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks - a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1 - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured to generate two HARQ-ACK codebooks with different priorities <unchanged part omitted>

Proposal 6: Adopt the following TP for default aperiodic CSI-RS beam for multi-TRP operation

##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology <unrelated part omitted> A trigger state is initiated using the CSI request field in DCI. - When all the bits of CSI request field in DCI are set to zero, no CSI is requested. - When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to trigger states to the codepoints of the CSI request field in DCI. is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. - When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state. - For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS; - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the other DL signal refers to: - PDSCH associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306]; - aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336} and enableBeamSwitchTiming-r16 is not provided; - periodic CSI-RS; - semi-persistent CSI-RS. - - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states and the other signal is a PDSCH indicated with two TCI states, the UE applies the first TCI state of those two TCI states. - else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; - else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS. - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, the UE applies the first one of two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states. - UE is not expected to receive aperiodic CSI -RS and PDSCH/aperiodic CSI-RS associated with different CORESETPoolIndex in overlapped symbol(s). - UE is not expected to receive aperiodic CSI-RS and semi-persistent/periodic CSI-RS with different ‘QCL-TypeD’ in overlapped symbol(s). - A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

# Reference

- 3GPP TS 38.214 v16.3.0

- 3GPP TS 38.213 v16.3.0