3GPP TSG-RAN WG1 #103-e		R1-2008473

e-Meeting, October 26th – November 13th, 2020

Source: 	Apple Inc.

Title:                     Framework and principles for RedCap

Agenda item:	8.6.4

Document for:	Discussion and Decision

# 1 Introduction

In RAN #86 meeting, a new study item “Support of Reduced Capability NR devices” [1] was approved, targeting for use cases such as wireless sensors, video surveillance, and wearables. The approved SID include the following objective:

- Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1].

In this contribution, we provide our views on the definition of Redcap device type and related framework.

# 2. Discussion

Rel-17 Redcap study item includes three use cases, including industrial wireless sensor, video surveillance and wearable. The target performances in terms of data rate, latency and reliability were summarized in Table 1 below:

Table 1: Target performances of Redcap devices

|  | Data rate | Latency | Reliability |
| --- | --- | --- | --- |
| Industrial wireless sensor | <= 2 Mbps | Non-safety related: <=100ms Safety related: 5~10ms | 99%~99.99% |
| Video surveillance | Economic video: 2-4Mbps High-end: 7.5-25 Mbps | <=500ms | 99%~99.9% |
| Wearable | Peak rate: <=150/50 Mbps (DL/UL) Reference rate: 2~5Mbps |  |  |

In our view, 3GPP should target to introduce one Redcap device type to address all of the three Redcap use cases, which is desirable from economy of scale perspective to avoid market segment for Redcap devices. Redcap device type definition should include a minimum set of components, which are essential to differentiate them from legacy Rel-15 and Rel-16 and correspondingly serve as filter for network to identify the Redcap devices. With these considerations in mind, our views is that reduced maximum bandwidth and reduced #Rx antenna capabilities satisfy these requirements and are sufficient to be used for access control purpose. Note that other advanced UE capabilities to fulfil higher requirements (e.g. 150Mbps peak data rate for wearable) can still be reported in RRC_CONNECTED phase by using the existing Rel-15/Rel-16 UE capability framework besides these minimized UE capabilities signaled by Redcap device type. Similarly, the other cost reduction features can also be reported by UE capability, instead of part of Redcap devices type definition, which include reduced DL/UL modulation as well as the Half-duplex FDD function.

Observation 1:

- The existing UE capability framework can be used by Redcap devices to report advanced capability in addition to capabilities included in the Redcap device type definition.

Proposal 1: One Redcap device is defined by grouping the requirement of reduced bandwidth and reduced number of Rx antennas.

# 3. Conclusion

In this contribution, we have presented our views with the following proposal and observations:

Observation 1:

- The existing UE capability framework can be used by Redcap devices to report advanced capability in addition to capabilities included in the Redcap device type definition.

Proposal 1: One Redcap device is defined by grouping the requirement of reduced bandwidth and reduced number of Rx antennas.

# References

- RP-193238		New SID on Support of Reduced Capability NR devices