3GPP TSG RAN WG1 #103-e		R1-2008474

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.7.1.1

Source:	Apple Inc.

Title:	Potential paging enhancements for idle/inactive-mode UE power saving

Document for:	Discussion/Decision

# Introduction

A work item on UE Power Saving Enhancements for NR was approved in RAN#86 (Dec 2019, RP-193239), and the WID was further updated in RP-200938 (RAN#88, June 2020) [1]. One objective is about paging enhancements for idle/inactive-mode UEs:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

- NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting

In RAN1#102-e, the evaluation assumptions have been agreed. In addition, the following potential enhancements were agreed to be investigated:

Agreements:

For potential paging enhancements, RAN1 to study the following candidate schemes:

- Paging early indication before a target PO to indicate UE whether to monitor PDCCH scrambled with P-RNTI at the PO. Potential candidate indication methods include

  - DCI-based indication, e.g., based on

    - Extending existing DCI format 1_0 or 2_6

    - New DCI format

  - RS-based or sequence-based indication, e.g., based on TRS/CSI-RS or SSS

- Sub-grouping for paging, based on

  - Legacy paging DCI

  - Paging early indication

  - Additional reception occasions in time/frequency domain

  - Multiple P-RNTIs

- Cross-slot scheduling for paging PDSCH

- Other proposal is not precluded

In this contribution, we provide the power saving evaluation results for different paging enhancement schemes and propose the way forward.

# Power Saving Evaluation and Recommendations

The UE power consumption evaluation should account for the activities the UE need to perform in each paging cycle. It depends on the UE’s RF condition (which determines how many SSBs the UE needs to monitor for time/frequency tracking before waking up at the paging occasion (PO) and decoding paging DCI/PDSCH) and the RRM measurement activities (intra-frequency and inter-frequency RRM).

In order to understand the power saving gain in different cases, we consider the following baseline cases:

- Case 1: low SINR, 3 SSBs for time/frequency tracking before each PO, with intra-frequency and inter-frequency measurement in each cycle (Figure 1)

- Case 2: low SINR, 3 SSBs for time/frequency tracking before each PO, with intra-frequency and inter-frequency measurement relaxation (with measurement once every 3 paging cycles) (Figure 1)

- Case 3: Medium SINR, 2 SSBs for time/frequency tracking before each PO (Figure 2)

- Case 4: High SINR, 1 SSBs for time/frequency tracking before each PO (Figure 3)

The offset between SSB and PO can vary, depending on the position of the PO. To evaluate the impact, we consider two representative cases:

- Case A: the SSB and the PO are offset by 10 ms (note that the SSB periodicity is 20 ms)

- Case B: the PO is immediately after the SSB.

Figure 1 Case 1 (without RRM relaxation) and Case 2 (with RRM relaxation) – low SINR

Figure 2 Case 3 – medium SINR

Figure 3 Case 4 – high SINR

The following enhancements have been evaluated for power saving gains:

- Wake-up signal (WUS) / paging early indication (PEI)

  - It is assumed that WUS is transmitted immediately after the 3rd SSB before the PO regardless of SINR. As an example, Figure 4 shows the activities with WUS for mediums SINR.

  - PDCCH is assumed for WUS, and the power consumption for monitoring WUS is assumed to be the same as PDCCH-only slot.

- Sub-grouping only

  - The assumption is that the sub-group indication is provided in paging DCI.

- WUS + sub-grouping

  - It is assumed that sub-group indication is provided together with WUS.

We did not evaluate cross-slot scheduling because in the current power model, there is no difference between the power consumption in a PDCCH-only slot for same-slot or cross-slot scheduling. Therefore, no power saving gain would be expected with cross-slot scheduling only with the current model. However, WUS transmitted in advance can achieve the power saving from cross-slot scheduling already.

Figure 4 Case 3 with WUS - medium SINR

The power saving for the evaluated schemes for different cases is summarized in Table 1 when group paging rate is 10%. The number of sub-groups is 4. Detailed evaluation assumptions are provided in Appendix.

|  |  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- |
| WUS | SSB and PO have 10 ms offset | 26.3% | 32.4% | 26.2% | 8.9% |
|  | SSB and PO are adjacent | 25.5% | 29.6% | 20.3% | -1.9% |
| Sub-grouping | SSB and PO have 10 ms offset | 0.3% | 0.3% | 0.4% | 0.5% |
|  | SSB and PO are adjacent | 0.3% | 0.3% | 0.5% | 0.6% |
| WUS + sub-grouping | SSB and PO have 10 ms offset | 28.9% | 35.5% | 28.5% | 10.6% |
|  | SSB and PO are adjacent | 28.0% | 32.5% | 22.2% | 0.1% |

Table 1 Power saving gain for different enhancements with group paging rate of 10% and 4 sub-groups (when applicable)

There is one case where WUS shows negative gain, which is the high SINR case with SSB and PO adjacent to each other. The reason is that the UE activities in this case are already clustered together in time, and adding WUS before-hand only creates additional decoding. However, this does not necessarily mean the UE would need to spend more power, because the UE may choose not to monitor WUS but rely on the paging DCI/PDSCH only.

Comparing the case where there is 10 ms offset between SSB and PO and the case where SSB and PO are adjacent, we see larger relative gain for the case with offset. The reason is that the baseline power consumption in this case is larger due to SSB detection and PO monitoring being separate in time.

From the evaluation results, we have the following observations:

Observation 1: For the cases evaluated with group paging rate of 10%, WUS provides relative power saving gain up to 32%.

- WUS provides larger relative power saving gain for lower SINR compared to higher SINR due to more savings related to SSB for time/frequency tracking.

- WUS provides more gain when there is less RRM measurement activities.

Observation 2: For the cases evaluated with group paging rate of 10%, sub-grouping only provides negligible (<1%) power saving gain.

Observation 3: For the cases evaluated with group paging rate of 10%, sub-grouping on top of WUS provides additional 2~3% power saving gain.

Naturally the power saving from WUS and/or sub-grouping also depends on the assumed group paging rate. Figure 5 provides the power saving gain for different group paging rate with 4 sub-groups when applicable. Only the results for 10 ms offset between SSB and PO are provided here.

(a) WUS

(b) Sub-grouping

(c) WUS + sub-grouping

Figure 5 Power saving gain for different enhancements with 10 ms offset between SSB and PO

Observation 4: The power saving gain provided by WUS decreases as the group paging rate increases.

Observation 5: The power saving gain provided by sub-grouping only increases as the group paging rate increases. However, the gain is still <3% with group paging rate of 60% for the cases evaluated.

Observation 6: The power saving gain provided by WUS and sub-grouping together provides a good saving gain (24%-35% for low and medium SINR, ~10% for high SINR) across the range of the group paging rate.

- WUS and sub-grouping complement each other, with WUS providing more power saving for lower group paging rate, and sub-grouping providing more power saving for higher group paging rate.

- The incremental power saving gain provided by sub-grouping on top of WUS is significant for high group paging rate.

We observed quite significant power saving provided by WUS, especially for lower SINR UEs (which need power saving most) and relatively low group paging rate (which was considered as more typical in the real network). Therefore, we see it as beneficial to introduce WUS for idle/inactive UEs.

As for sub-grouping, we also see the power saving benefit, especially when the group paging rate becomes large. This can be useful in certain part of the network where the UE density is high. Another aspect to consider is how much additional complexity/overhead it is to support sub-grouping. If we use PDCCH to support WUS, it can be very straightforward to support sub-grouping by using some bits in DCI to indicate the sub-groups. On the other hand, if sequence-based WUS is supported, multiple sequences would be needed to support sub-groups. These aspects can be considered further after the WUS design is more mature.

Proposal: Introduce wake-up signal for UEs in idle/inactive mode. Further consider the support of sub-grouping.

# Conclusion

In this contribution, we evaluated the power saving gain for different paging enhancements, and made the following observations and proposal:

Observation 1: For the cases evaluated with group paging rate of 10%, WUS provides relative power saving gain up to 32%.

- WUS provides larger relative power saving gain for lower SINR compared to higher SINR due to more savings related to SSB for time/frequency tracking.

- WUS provides more gain when there is less RRM measurement activities.

Observation 2: For the cases evaluated with group paging rate of 10%, sub-grouping only provides negligible (<1%) power saving gain.

Observation 3: For the cases evaluated with group paging rate of 10%, sub-grouping on top of WUS provides additional 2~3% power saving gain.

Observation 4: The power saving gain provided by WUS decreases as the group paging rate increases.

Observation 5: The power saving gain provided by sub-grouping only increases as the group paging rate increases. However, the gain is still <3% with group paging rate of 60% for the cases evaluated.

Observation 6: The power saving gain provided by WUS and sub-grouping together provides a good saving gain (24%-35% for low and medium SINR, ~10% for high SINR) across the range of the group paging rate

- WUS and sub-grouping complement each other, with WUS providing more power saving for lower group paging rate, and sub-grouping providing more power saving for higher group paging rate.

- The incremental power saving gain provided by sub-grouping on top of WUS is significant for high group paging rate.

Proposal: Support wake-up signal for UEs in idle/inactive mode. Further consider the support of sub-grouping.

# Reference

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88e, June 2020.

# Appendix A: Evaluation Assumptions

| Parameter | Value |
| --- | --- |
| SCS | 30 kHz |
| Paging cycle | 1.28 sec |
| Group paging rate | 10% unless specified otherwise |
| SSB periodicity | 20 ms |
| SSB duration | 2 ms (for time/frequency tracking and serving cell measurement) |
| Intra-frequency RRM measurement | Assumes synchronized deployment, 20 ms SMTC periodicity, 2ms SMTC window that overlaps with serving cell SSB |
| Inter-frequency RRM measurement | 5 ms SMTC window and 6 ms measurement gap |
| PO duration | 2 ms |

# Appendix B: Power Consumption Model

| Power State | Relative Power (per slot) |
| --- | --- |
| Deep sleep | 1 |
| Light sleep | 20 |
| Micro sleep | 45 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| PDSCH-only | 112 |
| SSB/CSI-RS processing | 50 |
| Intra-frequency RRM measurement | 60 for measurement only, 80 for combined search and measurement (synchronous) Probability for intra-frequency cell search = ¼ |
| Inter-frequency RRM measurement | 60 for measurement only, 60 for neighbor cell search Probability for inter-frequency cell search = ¼ Micro sleep power assumed for switch in/out a frequency layer |