3GPP TSG RAN WG1 #103-e		                                    		  R1-2008483

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.8.2.1

Source:	InterDigital Inc.

Title:	PUSCH coverage enhancements

Document for:	Discussion and Decision

# Introduction

In RAN1#102e, the following agreement was made regarding PUSCH enhancements [1].

Agreements: - Prioritize the study on the performance and specification impacts on DM-RS enhancements for PUSCH, including - Cross-slot channel estimation - With a lower priority compared with cross-slot channel estimation (i.e., companies are encouraged to study it) - Lower density - E.g., DM-RS sharing among multiple PUSCH transmissions or lower DMRS density in the frequency domain. - Higher density - E.g., in time or frequency domain, e.g., 1-comb pattern - Adaptive configuration - DM-RS balancing among frequency hops Agreements: - Prioritize the study on the performance and specification impacts on time domain based solutions for PUSCH enhancements, including - Increase the number of repetitions for PUSCH repetition type A - PUSCH repetition with non-consecutive slots/on the basis of available slots for TDD - Note: whether increasing the number of PUSCH repetition for FDD depends on the outcome of AI 8.8.1.1. - Enhancement on PUSCH repetition Type B - E.g., actual repetition across the slot boundary, or the length of actual repetition larger than 14 symbols, etc. - TB processing at least over multi-slot PUSCH - e.g., single TB, sized for a single slot, but transmitted in parts over multiple slots; or single TB, sized for multiple slots, transmitted over multiple slots, and in conjunction with repetition, etc. - FFS - OCC spreading based repetition - Symbol-level repetition - TB interleaving - RV repetition - Early termination of PUSCH repetitions Agreements: - Study following power domain based solution for PUSCH enhancements - Waveform design to optimize MPR/A-MPR - [FDD high power UE] - Power boosting for pi/2 BPSK Note: if a LS to RAN4 (for the last two bullets) is deemed necessary, target sending the LS in the 1st week of RAN1#103-e

In this contribution, the possible enhancements related to time-domain enhancements, DM-RS bundling, extended slot transmission, TB processing over multiple slots and waveform design to minimize MPR are discussed.

# Time domain and DM-RS enhancements

## DM-RS in special slots and DM-RS bundling to enable cross-slot channel estimation

In TDD, the number of uplink slots is limited, creating a constraint for the number of DMRS symbols that can be placed in an uplink slot. There exists tradeoff between the number of DMRS placed and number of resource blocks (RB) required to achieve target data rate, i.e., as the number of DMRS symbols in an uplink slot increases, the number of RBs increases as well. Therefore, the number of DMRS symbols placed in an uplink slot should be minimized to maximize the power allocation for data without degrading channel estimation performance.

Observation 1: The number of DMRS symbols placed in an uplink slot should be minimized without sacrificing channel estimation performance

One approach is to take advantage of a special slot which can be placed before the uplink slot. Typically, several uplink symbols follow the guard symbols in a special slot and it is possible to place reference signals in the uplink symbols in the special slot. Thus, it is possible to place DMRS symbols in a special slot which can be used to assist channel estimation in the following uplink slot. An example of such placement is shown in Figure 1 where one UL DMRS symbol is placed in the special slot which is followed by an uplink slot.

Figure 1 An example of placement of UL DMRS in a special slot before uplink slot

Observation 2: DMRS symbol can be placed in a special slot which is placed before the uplink slot, allowing channel estimation across the slot boundary

The proposal here is similar to DMRS bundling in which DMRS placed outside of the uplink slot can be used for channel estimation for the corresponding slot. Thus, the following proposal is made.

Proposal 1:Support DMRS placement in a special slot which can be bundled with DMRS in the adjacent uplink slot

To allow joint channel estimation, bundled DMRS should be the same hop. Thus, frequency hopping pattern which incorporates all bundled DMRS in the same hop should be supported.

Proposal 2:Support frequency hopping pattern which contains bundled DMRS in the same hop.

In addition, DMRS bundling for PUSCH repetition should be supported to enhance channel estimation. DMRS bundling could be supported for repetition type A, type B or both. While Type A offers slot-basis repetition, Type B offers flexibility in resource mapping, allowing repetition to be placed over the slot boundary. For type B, whether bundling can be applied to both actual and nominal repetitions should be discussed.

Proposal 3: Support DMRS bundling for repetition type A and B

## Extended slot transmission

Related to the previous proposal, the PUSCH duration may be extended into the uplink symbols in the special slot. In this case, the PUSCH occupy entire uplink slot and possibly uplink symbols in the adjacent slot. In terms of DMRS placement, the DMRS in the special slot can be used as an additional DMRS, possibly improving quality of channel estimation. By extending the length of PUSCH, DMRS density in PUSCH can be decreased, improving spectral efficiency of PUSCH. An example of the extended slot is shown in Figure 2.

Proposal 4: Support the use of uplink symbols in the adjacent special slot to extend duration of PUSCH

Figure 2 An example of extended slot transmission

## Repetition over non-consecutive slots

In the current specification, repetition counter is incremented over any slots, and repetition for PUSCH is limited by the frame structure. For example, for the frame structure such as DDDSU, to achieve 2 repetitions for PUSCH, the repetition parameter repK=8 must be configured. This feature lacks flexibility in configurations, limiting the number of possible repetitions for PUSCH, and coverage performance is scarified. Therefore, the following proposal is made.

Proposal 5: Support repetition of PUSCH over non-consecutive slots

# TB processing at least over multi-slot PUSCH

In low SNR conditions, transmission based on repetitions with RV cycling is not always the best way to improve coverage, e.g. in scenarios of limited power headroom, limited cell capacity, or when stringent latency is required. Coverage enhancement by repetition can require a wider band frequency domain allocation to accommodate the TB size within a slot (e.g. more than 1 PRB).

Even if the TB size is not be large, latency requirements (e.g. VoIP or time-sensitive traffic) might not be met when numerous repetitions required to meeting the HARQ operating point. Further, from a cell load/capacity point of view, when the cell is serving many devices in cell-edge conditions, repetition may limit the number of served devices in the cell and can thus yield to starvation of non-GBR traffic.

Observation 3: 	Relying solely on repetitions to meet PUSCH coverage can have the following shortcomings:

1)	A non-narrow band frequency allocation, thus reducing the TB’s power spectral density

2)	An increase of latency required to transmit the TB/reach the required HARQ operating point

3)	Increased cell load, which may come at the cost of other service types or users in the cell.

Spreading the TB over multiple slots in the time domain, however, can improve the power spectral density. A single TB can be scheduled over multiple consecutive slots with a narrower frequency allocation. Modulated symbols can be mapped over multiple resources in the time domain to ensure a higher spectral density.

Proposal 6: Support TB scheduling over consecutive slots in the time domain without repetition

As initial analysis, iBLER performance of TB scheduled over multiple slots is presented in Figure 3.. The simulation parameters for the evaluation is shown below. In the proposed multi-slot PUSCH, the TB is spread out across multiple slots with one PRB allocated per slot.

Table 1 Simulation parameters for enhancement evaluation

|  | Baseline | Multi-slot PUSCH |
| --- | --- | --- |
| Channel, Center frequency | PUSCH, 4GHz, TDD |  |
| SCS, bandwidth, Target data rate | 30kHz, 100MHz, 100kbps |  |
| LLS channel, mobility | TDL-C, DS=300ns, 3km/hr |  |
| UE TX antenna, BS RX antenna | 1 x 4 |  |
| # of DMRS per slot | 3 |  |
| Channel estimation | Realistic, bundled DMRS |  |
| MCS index | #4 Table 6.1.4.1-1, TS 38.214 |  |
| # of PRB, # of slots | 10,1 | 1,10 |

From the figure, it is clear that by spreading the TB over 10 slots, in lower SNR regions up to nearly 2dB gain can be achieved. At iBLER=10%, nearly 1dB gain can be achieved with the multi-slot PUSCH. Further gain can be expected by optimizing the number of slots the TB is spread across. In the simulation, DMRS from all slots were used for joint channel estimation.

The UE alternatively can split the TB into multiple coded segments that are transmitted over multiple slots. For example, DFT codes or similar can be used for orthogonal coding scheme to produce the coded TB segments (e.g. applied directly on modulated symbols). When the cell load is high and the cell is capacity limited, the gNB can multiplex another UE on the same set of resources by using a different spreading code from the same base sequence.

Figure 3 Multi-slot vs. Baseline performance

Outer codes or a similar block coding scheme can be used to produce coded TB segments. If the outer code is applied on the output bits of channel encoder, there should not be a reduction on channel coding gain, as the same TB size used by the channel encoder is used. The outer encoder then applies on the TB after channel coding, and each coded TB segment is then modulated on a different slot.

Proposal 7 Support TB encoding for transmission of coded TB segments mapped over multiple slots

Given the transmission time for a multi-slot PUSCH can be long, a small portion of the PUSCH transmission may suffer worse channel conditions than remaining scheduled slots (e.g. due to deep fading, fast fading, or sporadic interference). It can thus be beneficial to retransmit a portion of the TB that was not successfully decoded. Given the TBS is small, CBG retransmission may not be possible or efficient. The gNB can request to retransmit symbols mapped to a given slot for example.

Proposal 8: Support partial TB retransmission for TBs transmitted over a multi-slot PUSCH

# Waveform design to optimize MPR/A-MPR

In NR, Maximum Power Reduction (MPR) is used to reduce the leakage caused by the uplink transmission on the adjacent channels. It consists of reducing the UE’s maximum transmission power so that the power level of the leakage can be reduced. The interference on adjacent channels is due to the imperfections of the waveform such as adjacent channel leakage, in-band emission and out-of-band emission.  In order to allow the UE to transmit with higher power, techniques to reduce the leakage/imperfections of the waveform may be considered. The adjacent channel leakage is mainly caused by the non-linearity of the power amplifier. Techniques to reduce the PAPR can help increasing the transmission power while keeping the power amplifier of the UE operating in the linear region, and consequently, UE’s transmission can have less leakage on the adjacent channels. MPR reduction techniques can be enabled when the UE is in coverage limited scenario. For example, the gNB can determine based on e.g. uplink reference signals or based on power head room report if the UE needs more transmission power to achieve the performance target. The gNB can then indicate to the UE to use PAPR reduction techniques in the subsequent uplink transmissions.

Proposal 9: Support methods to minimize MPR of the waveform

# Conclusion.

The following observations are made in this contribution.

Observation 1: The number of DMRS symbols placed in an uplink slot should be minimized without sacrificing channel estimation performance

Observation 2: DMRS symbol can be placed in a special slot which is placed before the uplink slot, allowing channel estimation across the slot boundary

Observation 3: 	Relying solely on repetitions to meet PUSCH coverage can have the following shortcomings:

1)	A non-narrow band frequency allocation, thus reducing the TB’s power spectral density

2)	An increase of latency required to transmit the TB/reach the required HARQ operating point

3)	Increased cell load, which may come at the cost of other service types or users in the cell.

This contribution proposes to support the following enhancements

Proposal 1:Support DMRS placement in a special slot which can be bundled with DMRS in the adjacent uplink slot

Proposal 2:Support frequency hopping pattern which contains bundled DMRS in the same hop.

Proposal 3: Support DMRS bundling for repetition type A and B

Proposal 4: Support the use of uplink symbols in the adjacent special slot to extend duration of PUSCH

Proposal 5: Support repetition of PUSCH over non-consecutive slots

Proposal 6: Support TB scheduling over consecutive slots in the time domain without repetition

Proposal 7 Support TB encoding for transmission of coded TB segments mapped over multiple slots

Proposal 8: Support partial TB retransmission for TBs transmitted over a multi-slot PUSCH

Proposal 9: Support methods to minimize MPR of the waveform

Reference

[1] RAN1 Chairman’s notes, RAN1#102e, Aug., 2020.