3GPP TSG RAN WG1 #103-e		                                    		  R1-2008492

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.5.4

Source:	InterDigital Inc.

Title:	Discussion on positioning failure and mobility modeling

Document for:	Discussion and Decision

# Introduction

In RAN1#102e, the following agreement was made.

Conclusion: For UE mobility, the details of the optional mobility model are left to companies. Agreement: - For reducing NR positioning latency, more efficient signaling & procedures will be investigated to enable a device to request and report positioning information, which may include, but not limited to, the following aspects: - DL PRS/UL SRS configuration, activation or triggering. - The request for positioning information (the assistance data, etc.). - The report of positioning information (the measurement report, etc.). - Note: It is not within RAN1 scope to analyze positioning architecture enhancements to enable such more efficient signaling & procedures. - Note: RAN1 does not make any assumptions on whether the LCS architecture specified in TS 23.273 is enhanced or not.

In this contribution, a general concept of positioning failure and modeling of mobility are described. The purpose of introduction of model for mobility to assist alignment among companies during evaluation of mobility for positioning.

# Positioning failure

From the Rel-17 targets, positioning requirements consists of components in both spatial dimension (e.g. horizontal, vertical) and temporal dimension (latency for determining positioning). In this regard, the positioning error can be interpreted as the inability to perform positioning measurement within a given accuracy and within a time constraint.

Based on the defination provided in SID, a similar interpretation can be used for modeling integrity as a function of location accuracy and timeliness for determining the UE position. As an example, considering an actual position of a UE given by P and the position determined when using a positioning method i given by Pi, the positioning error can be calculated as R = |Pi  – P|. For a positioning service provided to an LCS client, the positioning error can be determined as R’ = |PAL – P|, where PAL is the positioning information tolerated by the application and is within the alert limit (AL). For further bounding the positioning error, the statistical upper bound of the UE position represented by the protection limit (PL), can be included when determining the positioning error as R’’ = |PPL – P|.

For ensuring integrity it is necessary for the positioning error incurred when using a positioning method is within the error tolerated by the application, R < R’’ < R’. Typically, this condition can be considered as the expected/nominal operation for positioning. In the event of a failure where the determined positioning information exceeds the error tolerated by the application (i.e. R > R’ when Pi  > PAL  or R’’ > R’ when PPL  > PAL), as per the requirement for integrity, it is necessary for the application or LCS client to be provided with alert/warning messages. In addition, even in the case when the determined positioning information is within the error tolerated by the application but exceeds the protection limit (i.e. R’’ < R < R’), an alert message should be triggered to the application for indicating that the positioning information is misleading or the protection limit is no longer valid.

The design objective of positioning service, which may be to minimize the probability of positioning failure when using a positioning method i, is given by min[ Fi ]= min[ Pr(R > R’) ]. The probability of positioning failure can be modelled as a function or error sources and time as Fi = f(Ec, Etx , Eex , Eue ,T), where Ec is the faults/errors in the correction data, Etx is the faults in transmitting data to UE, Eex represents the external feared events, Eue represents the faults in UE and T is the time duration for determining the UE position.

In the case when different positioning methods are used, the overall probability of positioning failure can be approximated as (Fi)n where n is the number of different positioning methods applied. From the model, it can be inferred that to minimize the positioning failure and consequently, to ensure integrity, either the probability of failure of a given positioning method should be minimized or multiple positioning methods should be applied.

In general, the tolerated error may vary depending on the application/use case, where it can be assumed that safety related use cases may have more stringent integrity requirements. In real deployments, it is possible for the performance of a positioning method to degrade and impact the positioning accuracy due to the activation of faults within the error sources. In these cases, a mechanism to detect and report the activation of faults in different error sources in the end-to-end positioning system, including the RAN and UE, should be studied. Impact of positioning failure on report or request procedure for positioning information should also be investigated. For identifying the presence of positioning error a technique to validate the determined positioning information on a timely basis with respect to an alternative/reference positioning information may be considered.

Proposal 1 : Impact of positioning failure on positioning accuracy and latency should be investigated

# Mobility modelling using constant velocity model

In RAN1 analysis, mobility is included as part of the evaluation assumptions [1]. In target tracking, state-space based model is used to model maneuvering objects. Considering future evaluations involving automobile, trains or drones, the mobility model assumed in the evaluation should be forward-compatible. The state space model captures velocity and cartesian coordinates of UE [2]. In this contribution, nearly constant velocity and coordinated turn model is introduced.

Constant velocity model

The nearly constant velocity model, in which a UE moves in one direction at constant speed, can be expressed as follows [3] :

xk+1=1T000100001T0001xk+T220T00T220Tv(k) 				(1)

where xk=xxyyT, with Cartesian coordinates given by x and y, respectively. Sample time in the mobility model is given by T in the equation above. Velocity in each direction are given by x and y. Unpredictable disturbances in the mobility model are given vk=v0v1T where vi is AWGN with Evi=0 and varvi=σi2.

Coordinated turn model

The coordinated model with a constant turn rate Ω (radians per sec) can be expressed as follows :

xk+1=1sinΩTΩ0-1-cosΩTΩ00cosΩT0-sinΩT001-cosΩTΩ1sinΩTΩ00sinΩT0cosΩT000001xk+T2200T000T2200T000Tv(k) 		(2)

Where xk=xxyyΩT, and vk=v0v1v2T, where vi models unpredictable disturbance in the model, and is AWGN with Evi=0 and varvi=σi2.

Considering that the agreed speed for UE is 3km/hr, we propose to adopt 3km/h for velocity. For the coordinated turn model, considering that in the IIoT scenario, UE may make sharp turn to go around equipment or shelves, 30 degrees per second may be a suitable value for turn rate.

In addition, UE in practical environment may switch between models, i.e., move in one direction uniformly for a certain duration and turn 90 degrees. Movement of automated guided vehicles (AGV) may be pre-programmed and move in constant manner, e.g., move straight for X seconds and turn 90 degrees, and continue moving straight for Y seconds. Companies should disclose switching patterns between 2 models above. For this reason, the following proposal is made.

# Conclusion.

The following proposal is made in this contribution.

Proposal 1 : Impact of positioning failure on positioning accuracy and latency should be investigated

Reference

[1] RAN1 Chairman’s notes, RAN1#102e, Aug., 2020.

- [2] Y. Bar-Shalom, X. R. Li and T. Kirubarjan, “Estimation with Applications to Tracking and Navigation: Theory Algorithms and Software,” Wiley-Interscience, New York, NY, 2001.