3GPP TSG RAN WG1 #103-e		                        R1-2008497

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.4

Source: 	ASUSTeK

Title:  	Remaining issues on sidelink power control

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about remaining issues on sidelink power control, including:

- DL pathloss in case of Uu-interface L1 problem

- Maintenance of PSSCH power control

- Maintenance of PSFCH power control

- Discussion

2.1 DL pathloss in case of Uu-interface L1 problem

Agreements:

For open-loop power control based on DL pathloss, the UE calculates DL pathloss using

- The RS resource used for power control for PUSCH transmission scheduled by DCI format 0_0 when the UE monitors DCI format 0_0.

- A RS resource from the SS/PBCH block that the UE uses to obtain MIB when the UE does not monitor DCI format 0_0.

In RAN1 #101-E meeting [1], it is agreed that a UE can be configured to use DL pathloss for sidelink power control. When the UE is configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS for PUSCH power control scheduled by DCI format 0_0. When the UE is not configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS resource from the SS/PBCH block that the UE uses to obtain MIB.

RAN2 agrees that mode-1 UE is allowed to continue using the configured SL grant type 1 when beam failure occurs or T310 is running [2][3]. In case these Uu-interface L1 problems happens, the mode-1 UE may not detect DL RSs normally, especially the DL RS for PUSCH power control scheduled by DCI format 0_0. In other words, DL pathloss derived from DL RS for DCI format 0_0-based PUSCH may be not accurate and meaningless for sidelink power control. Thus, for a UE configured to monitor PDCCH for detection of DCI format 0_0, one simple way during Uu-interface L1 problems is change or fallback to use SS/PBCH block that the UE uses to obtain MIB.

Observation 1: In case of Uu-interface beam failure recovery or T310 is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 1:  When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Justification

- When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Text Proposal 1

According text proposal for DL pathloss used for sidelink power control is provided below.

<   Text Proposal 1 for 38.213 >

### 16.2.1 PSSCH A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where <Omited> - if dl-P0-PSSCH-PSCCH-r16 is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] <Omited> where - PO,D is a value of dl-P0-PSSCH-PSCCH-r16 if provided - αD is a value of dl-Alpha-PSSCH-PSCCH-r16, if provided; else, αD=1 - PLD=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 and the UE does not perform link recovery procedure on the serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 or when the UE is performing link recovery procedure on the serving cell c <Omited> ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i as - if p0-DL-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of p0-DL-PSFCH - αPFSCH is a value of alpha-DL-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 and the UE does not perform link recovery procedure on the serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 or when the UE is performing link recovery procedure on the serving cell c

Proposal 2: 	Adopt the text proposal 1 in updating of TS38.213 section 16.2.1 and 16.2.3, for DL pathloss in case of Uu-interface L1 problem.

2.2  Maintenance of PSSCH power control

Justification

In RRC, SL power configuration is included in resource pool configuration. Since SL pathloss based power control is not supported for groupcast and broadcast, the corresponding specification seems missing.

Text Proposal 2

According text proposal for PSSCH power control is provided below.

<   Text Proposal 2 for 38.213 >

### 16.2.1 PSSCH A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower-r16 based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTransPower-r16 is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH-r16 is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - else if sl-P0-PSSCH-PSCCH-r16 is provided and if cast type of the PSSCH transmission is Unicast - PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH-r16 if provided - αD is a value of dl-Alpha-PSSCH-PSCCH-r16, if provided; else, αD=1 - PLD=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH-r16 is provided and if cast type of the PSSCH transmission is Unicast - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH-r16, if provided - αSL is a value of sl-Alpha-PSSCH-PSCCH-r16, if provided; else, αSL=1 - PLSL= referenceSignalPower – higher layer filtered RSRP, where - referenceSignalPower is obtained from a PSSCH transmit power per RE per antenna port of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient-r16, and - higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient-r16

Proposal 3: 	Adopt the text proposal 2 in updating of TS38.213 section 16.2.1, for supported Cast type for SL pathloss based power control.

2.3  Maintenance of PSFCH power control

Justification

- When DL pathloss based power control is disable for PSFCH, UE will autonomously determine a number NTx,PSFCH≥1 of simultaneous PSFCH transmissions. The upper bound of  NTx,PSFCH is missing, such that Nmax,PSFCH≥NTx,PSFCH≥1.

- Parameter names between 38.331 and 38.213 need to be aligned.

Text Proposal 3

According text proposal for PSFCH power control is provided below.

<   Text Proposal 3 for 38.213  >

### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH - αPFSCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 - <Omited> - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2 such that Nmax,PSFCH≥NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

Proposal 4: 	Adopt the text proposal 3 in updating of TS38.213 section 16.2.3, for PSFCH power control.

- Conclusion

In this contribution, we have following proposals:

Observation 1: In case of Uu-interface beam failure recovery or T310 is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 1:  When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Proposal 2: 	Adopt the text proposal 1 in updating of TS38.213 section 16.2.1 and 16.2.3, for DL pathloss in case of Uu-interface L1 problem.

Proposal 3: 	Adopt the text proposal 2 in updating of TS38.213 section 16.2.1, for supported Cast type for SL pathloss based power control.

Proposal 4: 	Adopt the text proposal 3 in updating of TS38.213 section 16.2.3, for PSFCH power control.

- Reference

- 3GPP TSG RAN WG1 #101-E Chairman’s Notes

- Report of 3GPP TSG RAN2#107 meeting

- Report of 3GPP TSG RAN2#108 meeting

- 3GPP TS 38.213 V16.3.0 (2020-09), “NR; Physical layer procedures for control (Release 16)”

- 3GPP TS 38.331 V16.2.0 (2020-09), “NR; Radio Resource Control (RRC) protocol specification (Release 16)”