3GPP TSG RAN WG1 #103-e		R1-2008514

e-Meeting, October 26th – November 13th, 2020

Agenda Item: 7.2.6

Source: MediaTek Inc.

Title:	Remaining issues on multi-beam operation

Document for: Decision

# Introduction

In this contribution, we discuss maintenance of enhancements on multi-beam operation and provide the text proposals on remaining issues.

# Latency reduction for DL/UL beam managements

## RRC parameter misalignment for TCI state/spatial relation update across CCs by MAC CE

In Rel-16, TCI state/spatial relation update across CCs by MAC CE were introduced. In order to support these features, up to two lists of CCs for TCI state/spatial relation are defined in the highlighted part of TS 38.331 as below.

The current version of TS 38.331 v16.2.0

CellGroupConfig ::= SEQUENCE { cellGroupId CellGroupId, rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig OPTIONAL, -- Need N rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity OPTIONAL, -- Need N mac-CellGroupConfig MAC-CellGroupConfig OPTIONAL, -- Need M physicalCellGroupConfig PhysicalCellGroupConfig OPTIONAL, -- Need M spCellConfig SpCellConfig OPTIONAL, -- Need M sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig OPTIONAL, -- Need N sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex OPTIONAL, -- Need N ..., [[ reportUplinkTxDirectCurrent ENUMERATED {true} OPTIONAL -- Cond BWP-Reconfig ]], [[ bap-Address-r16 BIT STRING (SIZE (10)) OPTIONAL, -- Need M bh-RLC-ChannelToAddModList-r16 SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL, -- Need N bh-RLC-ChannelToReleaseList-r16 SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16 OPTIONAL, -- Need N f1c-TransferPath-r16 ENUMERATED {lte, nr, both} OPTIONAL, -- Need M simultaneousTCI-UpdateList1-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R simultaneousTCI-UpdateList2-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R simultaneousSpatial-UpdatedList1-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R simultaneousSpatial-UpdatedList2-r16 SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex OPTIONAL, -- Need R uplinkTxSwitchingOption-r16 ENUMERATED {switchedUL, dualUL} OPTIONAL, -- Need R uplinkTxSwitchingPowerBoosting-r16 ENUMERATED {enabled} OPTIONAL -- Need R ]] }

However, the RRC parameters are misaligned between TS 38.331 and RAN1 specs. Therefore, we would like to suggest the following TPs to fix this issue. As a minor typo, there are two commas after the second RRC parameter in the second TP.

Text proposal for TS 38.213 v16.3.0

10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > - if the UE is provided by simultaneousTCI-UpdateList1-r16 or simultaneousTCI-UpdateList2-r16 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by TCI-States with same activated tci-StateID value to CORESETs with index p in all configured DL BWPs of all configured cells in a list determined from a serving cell index provided by a MAC CE command < Unchanged parts are omitted >

Text proposal for TS 38.214 v16.3.0

6.2.1 UE sounding procedure < Unchanged parts are omitted > When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs is indicated by higher layer parameter simultaneousSpatial-UpdateList1-r16 or simultaneousSpatial-UpdateList2-r16, the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs. < Unchanged parts are omitted >

# Conclusion

In this document, we provided the text proposals for remaining issues on multi-beam operation.